3GPP TSG CT WG3 Meeting #143	C3-254162

# Sophia-Antipolis, FR, 13th – 17th October, 2025

Source:	Huawei

Title:	Pseudo-CR on updates and corrections to the API definition clauses of the AIMLES_DataManagement API

Spec:	3GPP TS 29.482 (V1.1.0)

Agenda item:	19.42 (Metaverse_App)

Document for:	Agreement

1. Introduction

The following issues have been identified in the definition of the API definition clauses of the AIMLES_ContextTransfer API:

-	Some incorrect resources names need to be fixed.

-	The content of clause 6.1.2.3.3.1 is missing.

-	The name of the "AIMLE Data Management Assistance Notification" should be made consistent everywhere, there are some occurrences of "AIMLE Data Management Assistance Event Notification".

-	There are some occurrences where the reference number of TS 29.122 is incorrect, "3" instead of "2".

-	Missing expiration time of the subscription, to be provided by the AIMLE Server in subscription creation responses and also potentially in subscription update/retrieval responses.

-	In Table 6.1.2.6.2.3-1, the first attribute should convey the status of the data management operation, not the data management operation type, as per the related stage 2 requirements.

-	In Table 6.1.2.6.2.4-1, the cardinality column is empty.

-	The name of the attributes conveying the data preparation requirements and data analysis requirements are incorrect in some clauses in the data model definition.

-	Various styles related issues.

-	Various terminology alignments and a few editorial corrections to enhance the quality and readability.

2. Reason for Change

Address the above-detailed issues.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.482 V 1.1.0.

* * * * Start of Changes * * * *

##### 6.1.2.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.2.3.1-1 depicts the resource URIs structure for the AIMLES_DataManagement API.

Figure 6.1.2.3.1-1: Resource URI structure of the AIMLES_DataManagement API

Table 6.1.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.2.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| AIMLE Data Management Assistance Subscriptions | /subscriptions | POST | Request the creation of an AIMLE Data Management Assistance Subscription. |
| Individual AIMLE Data Management Assistance Subscription | /subscriptions/{subscriptionId} | GET | Retrieve an existing "Individual AIMLE Data Management Assistance Subscription" resource. |
|  |  | PUT | Request the update of an existing "Individual AIMLE Data Management Assistance Subscription" resource. |
|  |  | PATCH | Request the modification of an existing "Individual AIMLE Data Management Assistance Subscription" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual AIMLE Data Management Assistance Subscription" resource. |

* * * * Next Changes * * * *

###### 6.1.2.3.2.3	Resource Standard Methods

* * * * Next Changes * * * *

6.1.2.3.2.3.1	POST

The HTTP POST method enables a service consumer to request the creation of a new AIMLE Data Management Assistance Subscription at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.2.3.2.3.1-1.

Table 6.1.2.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 6.1.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.2.3.2.3.1-3.

Table 6.1.2.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataMgmtAssistSubsc | M | 1 | Contains the parameters to request the creation of an AIMLE Data Management Assistance Subscription. |

Table 6.1.2.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataMgmtAssistSubsc | M | 1 | 201 Created | Successful case. The "Individual AIMLE Data Management Assistance Subscription" resource is successfully created and a representation of that created resource is returned is returned in the response body. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.2.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/aimles-dm/<apiVersion>/subscriptions{subscriptionId} |

* * * * Next Changes * * * *

6.1.2.3.3.1	Description

This resource represents an AIMLE Data Management Assistance Subscription managed by the AIMLE Server.

* * * * Next Changes * * * *

6.1.2.3.3.2	Resource Definition

Resource URI: {apiRoot}/aimles-dm/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.1.2.3.3.2-1.

Table 6.1.2.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.2.1. |
| subscriptionId | string | Represents the identifier of the "Individual AIMLE Data Management Assistance Subscription" resource. |

* * * * Next Changes * * * *

###### 6.1.2.3.3.3	Resource Standard Methods

* * * * Next Changes * * * *

6.1.2.3.3.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing "Individual AIMLE Data Management Assistance Subscription" resource at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.2.3.3.3.1-1.

Table 6.1.2.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.2.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.2.3.3.3.1-3.

Table 6.1.2.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.2.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataMgmtAssistSubsc | M | 1 | 200 OK | Successful case. The requested "Individual AIMLE Data Management Assistance Subscription" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.2.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.2.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

* * * * Next Changes * * * *

6.1.2.3.3.3.4	DELETE

The HTTP DELETE method allows a service consumer to request the deletion of an existing "Individual AIMLE Data Management Assistance Subscription" resource at the AIMLE Server.

Table 6.1.2.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.2.3.3.3.4-2 and the response data structures and response codes specified in table 6.1.2.3.3.3.4-3.

Table 6.1.2.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.2.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIMLE Data Management Assistance Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.2.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.2.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

* * * * Next Changes * * * *

#### 6.1.2.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this resource in this release of the specification.

* * * * Next Changes * * * *

##### 6.1.2.5.1	General

Notifications shall comply to clause 6.6 of 3GPP TS 29.549 [14].

Table 6.1.2.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| AIMLE Data Management Assistance Notification | {notifUri} | POST | This service operation enables the AIMLE Server to notify a previously subscribed service consumer with AIML Data Management Assistance related report(s). |

* * * * Next Changes * * * *

##### 6.1.2.5.2	AIMLE Data Management Assistance Notification

* * * * Next Changes * * * *

###### 6.1.2.5.2.1	Description

The AIMLE Data Management Assistance Notification is used by the AIMLE Server to notify a previously subscribed service consumer on AIML Data Management Assistance related report(s).

* * * * Next Changes * * * *

###### 6.1.2.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.1.2.5.2.2-1.

Table 6.1.2.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Represents the callback URI encoded as a string formatted as URI. |

* * * * Next Changes * * * *

6.1.2.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.2.5.2.3-1 and the response data structures and response codes specified in table 6.1.2.5.2.3-2.

Table 6.1.2.5.2.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataMgmtAssistNotif | M | 1 | Represents the AIMLE Data Management Assistance Notification. |

Table 6.1.2.5.2.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The AIMLE Data Management Assistance Notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.2.5.2.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

Table 6.1.2.5.2.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

* * * * Next Changes * * * *

##### 6.1.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.2.6.1-1 specifies the data types defined for the AIMLES_DataManagement API.

Table 6.1.2.6.1-1: AIMLES_DataManagement API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| DataMgmtAssistSubsc | 6.1.2.6.2.2 | Represents the AIMLE Data Management Assistance Subscription. |  |
| DataMgmtAssistSubscPatch | 6.1.2.6.2.7 | Represents the requested modifications to an AIMLE Data Management Assistance Subscription. |  |
| DataMgmtAssistNotif | 6.1.2.6.2.3 | Represents the AIMLE Data Management Assistance Notification. |  |
| DataMgmtOp | 6.1.2.6.3.3 | Represents the data management operation type. |  |
| DataPrepReqs | 6.1.2.6.2.4 | Represents the data preparation requirements. |  |
| DataAnalysisReqs | 6.1.2.6.2.5 | Represents the data analysis requirements. |  |
| DataOperReqs | 6.1.2.6.2.6 | Represents the data operation requirements. |  |

Table 6.1.2.6.1-2 specifies data types re-used by the AIMLES_DataManagement API service.

Table 6.1.2.6.1-2: AIMLES_DataManagement API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AimleClientId | Clause 6.1.1.6.3.2 | Represents the unique identifier of the AIMLE Client. |  |
| DateTime | 3GPP TS 29.122 [2] | Represents a date and a time. |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| Uri | 3GPP TS 29.122 [2] | Represents a URI. |  |

* * * * Next Changes * * * *

###### 6.1.2.6.2.2	Type: DataMgmtAssistSubsc

Table 6.1.2.6.2.2-1: Definition of type DataMgmtAssistSubsc

| Attribute name | Data type | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| dataMgmtOp | DataMgmtOp | M |  | 1 |  | Contains the requested data management operation type. |  |  |  |
| dataPrepReqs | DataPrepReqs | C | 0..1 | Contains the data preparation requirements. This attribute shall be present only if the requested data management type within the "dataMgmtOp" attribute is "DATA_PREPARATION". |  |
| dataAnalysisReqs | DataAnalysisReqs | C | 0..1 | Contains the data analysis requirements. This attribute shall be present only if the requested data management type within the "dataMgmtOp" attribute is "DATA_ANALYSIS". |  |
| clientList | array(AimleClientId) | C |  | 1..N |  | Contains the list of the identifier(s) of the AIMLE Client(s) for which data management should be performed. (NOTE) |  |  |  |
| selCriteria | FFS | C |  | 0..1 |  | Contains the selection criteria for finding suitable AIMLE Clients for data management. (NOTE) |  |  |  |
| notifUri | Uri | M |  | 1 |  | Contains the URI via which the notifications shall be delivered. |  |  |  |
| expTime | DateTime | O | 0..1 | Contains the expiration time of the subscription. This attribute may be present only in the response to an AIMLE Data Management Assistance Subscription Creation/Update/Retrieval request. |  |
| suppFeat | SupportedFeatures | C |  | 0..1 |  | Contains the list of supported features among the ones defined in clause 6.1.2.7. This attribute shall be present only when feature negotiation is required. |  |  |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

Editor’s NOTE: The content of the clientCriteria attribute is FFS.

* * * * Next Changes * * * *

###### 6.1.2.6.2.3	Type: DataMgmtAssistNotif

Table 6.1.2.6.2.3-1: Definition of type DataMgmtAssistNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| status | FFS | M | 1 | Contains the status of the data management operation. |  |
| aggrDataPrepOut | FFS | C | 0..1 | Contains the outputs for data preparation. This attribute shall be present if the data management operation type within the "dataMgmtOp" attribute of the corresponding subscription is "DATA_PREPARATION". |  |
| aggrDataAnaOut | FFS | C | 0..1 | Contains the outputs for data analysis. This attribute shall be present if the requested data management operation type within the "dataMgmtOp" attribute of the corresponding subscription is "DATA_ANALYSIS". |  |
| timeStamp | DateTime | O | 0..1 | Contains the timestamp of the AIMLE Data Management Assistance Notification. |  |

Editor’s NOTE: The encoding of the "aggrDataPrepOut" attribute, the "aggrDataAnaOut" and "status" attributes is FFS.

* * * * Next Changes * * * *

###### 6.1.2.6.2.4	Type: DataPrepReqs

Table 6.1.2.6.2.4-1: Definition of type DataPrepReqs

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataSetId | string | M | 1 | Contains the identifier of the dataset. |  |
| dataOperReqs | arra(DataOperReqs) | M | 1 | Contains the requirements for data preparation. |  |

* * * * Next Changes * * * *

###### 6.1.2.6.2.5	Type: DataAnalysisReqs

Table 6.1.2.6.2.5-1: Definition of type DataAnalysisReqs

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataSetId | string | M | 1 | Contains the identifier of the dataset. |  |
| dataOperReqs | array(DataReqs) | M | 1 | Contains the requirements for data analysis. |  |

* * * * Next Changes * * * *

###### 6.1.2.6.2.6	Type: DataOperReqs

Table 6.1.2.6.2.4-1: Definition of type DataOperReqs

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataId | string | M | 1 | Contains the identifier of the dataset. |  |
| dataSetFeatId | string | M | 1 | Contains the identifier of the data set feature. |  |
| dataOperationFunc | FFS | M | 1 | Contains the function that performs the data analysis. It may be an identifier of a function if the function is available locally at the UE or an executable included in the request. |  |

Editor’s NOTE: The content of the "dataOperationFunc" attribute is FFS.

* * * * Next Changes * * * *

###### 6.1.2.6.2.7	Type: DataMgmtAssistSubscPatch

Table 6.1.2.6.2.2-1: Definition of type DataMgmtAssistSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataPrepReqs | DataPrepReqs | O | 0..1 | Contains the updated data preparation requirements. |  |
| dataAnalysisReqs | DataAnalysisReqs | O | 0..1 | Contains the updated data analysis requirements. |  |
| notifUri | Uri | O | 0..1 | Indicates the URI via which the notifications shall be delivered. |  |

* * * * Next Changes * * * *

###### 6.1.2.6.3.3	Enumeration: DataMgmtOp

The enumeration DataMgmtOp represents the data management operation type. It shall comply with the provisions defined in table 6.1.2.6.3.3-1.

Table 6.1.2.6.3.3-1: Enumeration DataMgmtOp

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DATA_PREPARATION | Indicates that the data management operation type is data preparation. |  |
| DATA_ANALYSIS | Indicates that the data management operation type is data analysis. |  |

* * * * Next Changes * * * *

##### 6.1.2.7.1	General

For the AIMLES_DataManagement API, error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [14].

In addition, the requirements in the following clauses are applicable for the AIMLES_DataManagement API.

* * * * End of Changes * * * *