3GPP TSG-CT WG3 Meeting #143	C3-254189

# Sophia-Antipolis, FR, 13th – 17th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 1725 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of lastRepInd attribute definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The definition of lastRepInd attribute in 29.522 is not aligned with the definition in 29.569. ExtAiotAreaId data type is missing from the specific Data Types table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the definition of lastRepInd attribute to indicate that only value true is allowed. Add ExtAiotAreaId data type to Table 5.45.5.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between different TSs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.45.5.1, 5.45.5.2.6, A.43 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the following API: - TS29522_AIoT.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

5.45.5.1	General

This clause specifies the application data model supported by the AIoT API. Table 5.45.5.1-1 specifies the data types defined for the AIoT API.

Table 5.45.5.1-1: AIoT service specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AIoTNotif | 5.45.5.2.6 | Represents the AIoT Operations Notification. |  |
| CommandReq | 5.45.5.2.4 | Represents the AIoT Command request. |  |
| CommandResp | 5.45.5.2.5 | Represents the AIoT Command response. |  |
| CommandType | 5.45.5.3.3 | Represents the type of AIoT Command. |  |
| ExtAIoTArea | 5.45.5.2.7 | Represents the External AIoT Service Area. |  |
| ExtAiotAreaId | 5.45.5.3.2 | Represents the External AIoT Area identifier. |  |
| InventoryReq | 5.45.5.2.2 | Represents the AIoT Inventory request. |  |
| InventoryResp | 5.45.5.2.3 | Represents the AIoT Inventory response. |  |

Table 5.45.5.1-2 specifies data types re-used by the AIoT API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the AIoT API.

Table 5.45.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AIoTDevices | 3GPP TS 29.569 [83] | Represents the AIoT device(s) related information. |  |
| AiotDevPermId | 3GPP TS 29.571 [8] | Represents the permanent identifier of the AIoT Device. |  |
| Bytes | 3GPP TS 29.571 [16] | Represents a sequence of bytes. |  |
| CivicAddress | 3GPP TS 29.572 [34] | Represents a civic address. |  |
| DevicesRepInfo | 3GPP TS 29.569 [83] | Represents the AIoT device(s) related reporting information. |  |
| DurationSec | 3GPP TS 29.122 [4] | Represents a time duration in units of seconds. |  |
| GeographicArea | 3GPP TS 29.572 [34] | Represents a geographical area. |  |
| ProblemDetails | 3GPP TS 29.571 [8] | Represents error related information. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported feature(s) and is used to negotiate the applicability of the optional features. |  |
| Uinteger | 3GPP TS 29.571 [8] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.122 [4] | Represents a URI. |  |

# *** 2nd Change ***

5.45.5.2.6	Type: AIoTNotif

Table 5.45.5.2.6-1: Definition of type AIoTNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afTransId | string | M | 1 | Contains the identifier of the AF transaction to which the notification is related. |  |
| devicesRepData | array(DevicesRepInfo) | C | 1..N | Contains the AIoT device(s) related reporting information. (NOTE) |  |
| lastRepInd | boolean | C | 0..1 | Contains the Last Report Indication, i.e., indicates whether this is the last reporting from the AIOTF for the AIoT service operation identified by the "transId" attribute. - "true" indicates that this is the last report. This attribute shall be present only when this is the last reporting from the AIOTF for the AIoT service operation identified by the "transId" attribute. When present, this attribute shall be set to "true". The presence of this attribute set to the value "false" is forbidden. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

# *** 3rd Change ***

A.43	AIoT API

openapi: 3.0.0

info:

title: 3gpp-aiot

version: 1.0.0-alpha.2

description: |

API for UE Address service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.522 V19.4.0; 5G System; Network Exposure Function Northbound APIs.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/3gpp-aiot/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

paths:

/request-inv:

post:

summary: Request to perform an AIoT Inventory operation.

operationId: InventoryRequest

tags:

- AIoT Inventory Request (custom operation without associated resources)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/InventoryReq'

responses:

'200':

description: >

OK. The AIoT Inventory request is successfully received and processed, and

AIoT Inventory related information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/InventoryResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

AIoTOperationsNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AIoTNotif'

responses:

'204':

description: >

No Content. The AIoT Operations Notification is successfully received and

acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/request-cmd:

post:

summary: Request to perform an AIoT Command operation.

operationId: CommandRequest

tags:

- AIoT Command Request (custom operation without associated resources)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/CommandReq'

responses:

'200':

description: >

OK. The AIoT Command request is successfully received and processed, and

AIoT Command related information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/CommandResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

AIoTOperationsNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AIoTNotif'

responses:

'204':

description: >

No Content. The AIoT Operations Notification is successfully received and

acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

InventoryReq:

description: Represents the AIoT Inventory request.

type: object

properties:

afId:

type: string

targetArea:

$ref: '#/components/schemas/ExtAIoTArea'

targetDevices:

$ref: 'TS29569_Naiotf_AIoT.yaml#/components/schemas/AIoTDevices'

numDevices:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

timeInterval:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

- notifUri

anyOf:

- required: [targetArea]

- required: [targetDevices]

InventoryResp:

description: Represents the AIoT Inventory response.

type: object

properties:

afTransId:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afTransId

CommandReq:

description: Represents the AIoT Command request.

type: object

properties:

afId:

type: string

commandType:

$ref: '#/components/schemas/CommandType'

targetArea:

$ref: '#/components/schemas/ExtAIoTArea'

targetDevices:

$ref: 'TS29569_Naiotf_AIoT.yaml#/components/schemas/AIoTDevices'

numDevices:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

msgSize:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

offset:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

length:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

data:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

- commandType

- notifUri

anyOf:

- required: [targetArea]

- required: [targetDevices]

CommandResp:

description: Represents the AIoT Command response.

type: object

properties:

afTransId:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afTransId

AIoTNotif:

description: Represents the AIoT Operations Notification.

type: object

properties:

afTransId:

type: string

devicesRepData:

type: array

items:

$ref: 'TS29569_Naiotf_AIoT.yaml#/components/schemas/DevicesRepInfo'

minItems: 1

lastRepInd:

type: boolean

default: false

description: >

Contains the Last Report Indication, i.e., indicates whether this is the last reporting

from the AIOTF for the AIoT service operation identified by the transId attribute.

true indicates that this is the last report.

This attribute shall be present only when this is the last reporting from the AIOTF

for the AIoT service operation identified by the transId attribute.

When present, this attribute shall be set to true. The presence of this attribute set

to the value false is forbidden.

required:

- afTransId

anyOf:

- required: [devicesRepData]

- required: [lastRepInd]

ExtAIoTArea:

description: Represents the AIoT area.

type: object

properties:

extAreaIds:

type: array

items:

$ref: '#/components/schemas/ExtAiotAreaId'

minItems: 1

geographicAreas:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

minItems: 1

civicAddresses:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CivicAddress'

minItems: 1

oneOf:

- required: [extAreaIds]

- required: [geographicAreas]

- required: [civicAddresses]

#

# SIMPLE DATA TYPES

#

ExtAiotAreaId:

description: Represents the External AIoT Area identifier.

type: string

#

# ENUMERATIONS

#

CommandType:

anyOf:

- type: string

enum:

- READ

- WRITE

- PERMANENT_DISABLE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the type of AIoT Command.

Possible values are:

- READ: Indicates that the AIoT Command is Read (i.e., retrieve information).

- WRITE: Indicates that the AIoT Command is Write (i.e., provision information).

- PERMANENT_DISABLE: Indicates that the AIoT Command is Permanent Disable (i.e., disable

the capability to transmit information).

#

# Data types describing alternative data types or combinations of data types

#

*** End of Changes ***