3GPP TSG-CT WG3 Meeting #143	C3-254232

Sophia-Antipolis, France, 13 -17 October, 2025

Source:	Ericsson

Title:	Pseudo-CR for limited alignment for VFL Inference with VFL Training

Spec:	3GPP TS 29.530 V1.0.0

Agenda item:	19.39

Document for:	Agreement

1. Introduction

23.320 specifies Naf_VFLTraining and Naf_VFLInference services.

2. Reason for Change

Naf_VFLInference service is not aligned with VFL Training service in following issues:

-	Support of GET operation for individual subscriptions.

-	Possibility of updating the notification correlation id in Patch operation.

-	Possibility of updating VFL inference subscriptions per analytics event in Patch operation.

-	Nomenclature of Individual Subscriptions.

-	The name of the attribute for reporting requirements in the VflInferSub and VflInferSubPatch data types ("vflReportInfo" attribute) does not reflect that it is a requirement.

-	Different names are used for the attributes corresponding to the notification correlation id where applicable.

-	The parameters section in on the TS29530_Naf_VFLInference.yaml is specific per operation.

3. Conclusions

It is considered convenient that Naf_VFLInference service is updated to be  aligned with Naf_VFLTraining service in all these issues.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.530 V1.0.0.

# *** 1st Change ***

# 4	Overview

The Application Function Artificial Intelligence/Machine Learning (AI/ML) Services, as defined in 3GPP TS 23.288 [14], are provided by the Application Function (AF).

The following AI/ML services are specified for the AF:

Table 4.1-1: AI/ML Services provided by AF

| Service Name | Description | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- | --- |
| Naf_VFLTraining | This service is provided by an AF acting as VFL client and enables an NF service consumer to request the AF to participate in VFL model training as VFL client and train a local model. | Subscribe | Subscribe / Notify | NWDAF, NEF |
|  |  | Unsubscribe |  |  |
|  |  | Notify |  |  |
|  |  | Request | Request / Response | NWDAF, NEF |
| Naf_VFLInference | This service is provided by AF acting as VFL client and enables an NF service consumer to subscribe/unsubscribe for a VFL inference. | Subscribe | Request / Response | NWDAF |
|  |  | Unsubscribe |  |  |
|  |  | Notify |  |  |
|  |  | Request | Request / Response | NWDAF |
| Naf_Inference | This service is provided by AF acting as VFL server and enables an NF service consumer to subscribe/unsubscribe for a VFL inference. | Subscribe | Subscribe / Notify | NWDAF, NEF |
|  |  | Unsubscribe |  |  |
|  |  | Notify |  |  |
|  |  | Request | Request / Response | NWDAF, NEF |
| Naf_Training | This service is provided by AF acting as VFL server and enables an NF service consumer to subscribe/unsubscribe for a VFL training. | Subscribe | Subscribe / Notify | NWDAF, NEF |
|  |  | Unsubscribe |  |  |
|  |  | Notify |  |  |

# *** Next Change ***

## 5.1	Introduction

The AF offers to other NFs the following services:

-	Naf_VFLInference;

-	Naf_Inference;

-	Naf_VFLTraining

-	Naf_VFLInference

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Naf_VFLTraining | 5.2 | VFL Training Service | TS29530_Naf_VFLTraining.yaml | naf-vfl-train | A.2 |
| Naf_VFLInference | 5.3 | VFL Inference Service | TS29530_Naf_ VFLInference.yaml | naf-vflinference | A.3 |
| Naf_Training | 5.4 | Model training service | TS29530_Naf_Training.yaml | naf-train | A.4 |
| Naf_Inference | 5.5 | Model Inference service | TS29530_Naf_Inference.yaml | naf-inference | A.5 |

# *** Next Change ***

##### 5.3.2.2.2	Subscription for VFL inference event notifications

Figure 5.3.2.2.2-1 shows a scenario where the NF service consumer sends a request to the AF to subscribe for VFL inference event notification(s) (as shown in 3GPP TS 23.288 [14]).

Figure 5.3.2.2.2-1: NF service consumer subscribes to VFL inference notifications

The NF service consumer shall invoke the Naf_VFLInference_Subscribe service operation to request AF VFL client(s) to subscribe to VFL inference event notification(s). The NF service consumer shall send an HTTP POST request with "{apiRoot}/naf-vflinference/<apiVersion>/subscriptions" as Resource URI representing the "AF VFL Inference Subscriptions", as shown in figure 5.3.2.2.2-1, step 1, to create a subscription for an "Individual AF VFL Inference Subscription" according to the information in message body.

The VflInferSub data structure provided in the request body shall include:

-	an URI where to receive the requested notifications as the "notifUri" attribute;

-	a notification correlation identifier assigned by the NF service consumer for the requested notifications as "notifCorrId" attribute; and

-	a description of the subscribed analytics event(s) as the "vflInferAnaSubs" attribute.

and may include:

-	the VFL reporting information as the "reportingReqs" attribute; and

-	the required conditions to apply VFL inference as the "vflInferReq" attribute;

Upon the reception of an HTTP POST request with: "{apiRoot}/naf-vflinference/<apiVersion>/subscriptions" as Resource URI and VflInferSub data structure as request body, the AF shall create a new subscription and store the subscription.

If the AF created an "Individual AF VFL Inference Subscription" resource, the AF shall respond with "201 Created" with the message body containing a representation of the created subscription, as shown in figure 5.3.2.2.2-1, step 2. The AF shall include a Location HTTP header field. The Location header field shall contain the URI of the created subscription i.e. "{apiRoot}/naf-vflinference/<apiVersion>/subscriptions/{subscriptionId}".

If the immediate reporting indication is applicable during the event subscription considering the "reportingReqs" attribute in the "VflInferSub" and "VflInferAnaSub" structures, the AF shall include the intermediate VFL inference results of the subscribed events, if available, as the "vflInferResults" attribute in the HTTP POST response.

If any error occurs when processing the HTTP POST request, the AF shall send an HTTP error response as specified in clause 6.2.7.

# *** Next Change ***

##### 5.3.2.2.3	Update subscription for event notifications

Figure 5.3.2.2.3-1 shows a scenario that the NF service consumer sends an HTTP PUT request to the AF to modify an existing VFL inference subscription (as shown in 3GPP TS 23.288 [14]).

Figure 5.3.2.2.3-1: Modification of VFL inference events subscription information using HTTP PUT

The NF service consumer shall invoke the Naf_VFLInference_Subscribe service operation to modify an existing VFL inference subscription. The NF service consumer shall send an HTTP PUT request with: "{apiRoot}/naf-vflinference/<apiVersion>/subscriptions/{subscriptionId}" as Resource URI, where "{subscriptionId}" is the subscriptionId of the existing VFL inference subscription to be modified, to update an "Individual VFL Inference Subscription" according to the information in the message body. The VflInferSub data structure provided in the request body shall include the same contents as described in clause 5.3.2.2.2.

Upon receipt of an HTTP PUT request with: "{apiRoot}/naf-vflinference/<apiVersion>/subscriptions/{subscriptionId}" as Resource URI and VflInferSub data type as request body, if the request is successfully processed and accepted, the AF shall:

-	modify the concerned subscription; and

-	store the subscription.

If the AF successfully processed and accepted the received HTTP PUT request, the AF shall update an "Individual VFL Inference Subscription" resource, and shall respond with:

-	HTTP "204 No Content" response (as shown in figure 5.3.2.2.3-1, step 2a); or

-	HTTP "200 OK" response (as shown in figure 5.3.2.2.3-1, step 2b) with a response body containing a representation of the updated subscription in the VflInferSub data type.

If the immediate reporting indication is applicable during the event subscription update considering the "reportingReqs" attribute in the "VflInferSub" and "VflInferAnaSub" structures, the AF shall include the reports of the subscribed events, if available, as the "vflInferResults" attribute in the HTTP PUT response.

If any error occurs when processing the HTTP PUT request, the AF shall send an HTTP error response as specified in clause 6.2.7.

If the AF determines that the received HTTP PUT request needs to be redirected, the AF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

# *** Next Change ***

##### 5.3.2.2.4	Partial update subscription for event notifications

Figure 5.3.2.2.4-1 shows a scenario that the NF service consumer sends an HTTP PATCH request to the AF to partial modify an existing VFL inference subscription (as shown in 3GPP TS 23.288 [14]).

Figure 5.3.2.2.4-1: Partial modification of VFL inference subscription information using HTTP PATCH

The NF service consumer shall invoke the Naf_VFLInference_Subscribe service operation to partial modify an existing VFL inference subscription. The NF service consumer shall send an HTTP PATCH request with: "{apiRoot}/naf-vflinference/<apiVersion>/subscriptions/{subscriptionId}" as Resource URI, where "{subscriptionId}" is the subscriptionId of the existing VFL inference subscription to be modified, to update an "Individual VFL Inference Subscription" according to the information in the message body.

Upon receipt of an HTTP PATCH request with: "{apiRoot}/naf-vflinference/<apiVersion>/subscriptions/{subscriptionId}" as Resource URI and VflInferSubPatch data type as request body, if the request is successfully processed and accepted, the AF shall:

-	partial modify the concerned subscription; and

-	store the subscription.

If the AF successfully processed and accepted the received HTTP PATCH request, the AF shall partial update an "Individual VFL Inference Subscription" resource, and shall respond with:

-	HTTP "204 No Content" response (as shown in figure 5.3.2.2.4-1, step 2a); or

-	HTTP "200 OK" response (as shown in figure 5.3.2.2.4-1, step 2b) with a response body containing a representation of the updated subscription in the VflInferSubPatch data type.

If the immediate reporting indication is applicable during the event subscription update considering the "reportingReqs" attribute within the in the "VflInferSub" and "VflInferAnaSub" structures on the event subscription or the event subscription update, the AF shall include the reports of the subscribed events, if available, as the "vflInferResults" attribute in the HTTP PATCH response.

If any error occurs when processing the HTTP PATCH request, the AF shall send an HTTP error response as specified in clause 6.2.7.

If the AF determines that the received HTTP PATCH request needs to be redirected, the AF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

# *** Next Change ***

#### 6.2.3.1	Resource Structure

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.2.3.1-1 depicts the resource URIs structure for the Naf_VFLInference API.

Figure 6.2.3.1-1: Resource URI structure of the Naf_VFLInference API

Table 6.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| VFL Inference Subscriptions | /subscriptions | POST | Creates a new Individual VFL Inference Subscription resource. |
| Individual VFL Inference Subscription | /subscriptions/{subscriptionId} | GET | Retrieve an existing Individual VFL Inference Subscription identified by subresource {subscriptionId}. |
|  |  | PUT | Updates an existing Individual VFL Inference Subscription identified by subresource {subscriptionId}. |
|  |  | PATCH | Modifies an existing Individual VFL Inference Subscription identified by subresource {subscriptionId}. |
|  |  | DELETE | Deletes an Individual VFL Inference Subscription identified by subresource {subscriptionId}. |

# *** Next Change ***

#### 6.2.3.2	Resource: VFL Inference Subscriptions

# *** Next Change ***

##### 6.2.3.2.1	Description

The VFL Inference Subscriptions resource represents all VFL Inference subscriptions to the Naf_VFLInference service at a given AF. The resource allows an NF service consumer to create a new Individual VFL Inference Subscription resource.

# *** Next Change ***

###### 6.2.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.2.3.2.3.1-1.

Table 6.2.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.3.2.3.1-3.

Table 6.2.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VflInferSub | M | 1 | Creates a new Individual VFL Inference Subscription resource. |

Table 6.2.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VflInferSub | M | 1 | 201 Created | The creation of an Individual VFL Inference Subscription resource is confirmed and a representation of that resource is returned. |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/naf-vflinference/<apiVersion>/subscriptions/{subscriptionId} |

# *** Next Change ***

#### 6.2.3.3	Resource: Individual VFL Inference Subscription

# *** Next Change ***

##### 6.2.3.3.1	Description

The Individual VFL Inference Subscription resource represents a single VFL inference subscription to the Naf_VFLInference service at a given AF.

# *** Next Change ***

###### 6.2.3.3.3.1	GET

The GET method allows an NF service consumer to retrieve an existing "Individual VFL Inference Subscription" resource managed by the AF.

This method shall support the URI query parameters specified in table 6.2.3.3.3.1-1.

Table 6.2.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.1-2 and the response data structures and response codes specified in table 6.2.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VflInferSub | M | 1 | 200 OK | Successful case. The requested "Individual VFL Inference Subscription" resource is returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.2.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

Table 6.2.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target AF (service) instance towards which the request is redirected. |

# *** Next Change ***

###### 6.2.3.3.3.2	PUT

This method shall support the URI query parameters specified in table 6.2.3.3.3.2-1.

Table 6.2.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.2-2 and the response data structures and response codes specified in table 6.2.3.3.3.2-3.

Table 6.2.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VflInferSub | M | 1 | Parameters to replace a subscription to VFL Subscription resource. |

Table 6.2.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VflInferSub | M | 1 | 200 OK | The Individual VFL Subscription resource was modified successfully, and a representation of that resource is returned. |
| n/a |  |  | 204 No Content | The Individual VFL Subscription resource was modified successfully. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.2.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the request is redirected |

Table 6.2.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the request is redirected |

# *** Next Change ***

###### 6.2.3.3.3.3	PATCH

This method shall support the URI query parameters specified in table 6.2.3.3.3.3-1.

Table 6.2.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.3-2 and the response data structures and response codes specified in table 6.2.3.3.3.3-3.

Table 6.2.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VflInferSubPatch | M | 1 | Partial update of parameters to a subscription to VFL Inference Subscription resource. |

Table 6.2.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VflInferSub | M | 1 | 200 OK | The Individual VFL Inference Subscription resource was partial modified successfully and a representation of that resource is returned. |
| n/a |  |  | 204 No Content | The Individual VFL Inference Subscription resource was partial modified successfully. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.2.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the request is redirected |

Table 6.2.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the request is redirected |

# *** Next Change ***

###### 6.2.3.3.3.4	DELETE

This method shall support the URI query parameters specified in table 6.2.3.3.3.4-1.

Table 6.2.3.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.4-2 and the response data structures and response codes specified in table 6.2.3.3.3.4-3.

Table 6.2.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |

Table 6.2.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case: The Individual VFL Subscription resource matching the subscriptionId was deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.2.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.2.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AF (service) instance towards which the request is redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the request is redirected. |

# *** Next Change ***

##### 6.2.5.2.1	Description

The VFL Inference Event Notification is used by the AF to report one or several observed VFL Inference Events to a NF service consumer that has subscribed to such Notifications via the Individual VFL Inference Subscription Resource.

# *** Next Change ***

##### 6.2.5.2.2	Operation Definition

Callback URI: {notifUri}

The operation shall support the callback URI variables defined in table 6.2.5.2.2-1, the request data structures specified in table 6.2.5.2.2-2 and the response data structure and response codes specified in table 6.2.5.2.2-3.

Table 6.2.5.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notifUri | Uri | The Notification Uri as assigned within the Individual VFL Inference Subscription. |

Table 6.2.5.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VflInferNotif | M | 1 | Provides Information about observed events. |

Table 6.2.5.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [4]). |  |  |  |  |  |

Table 6.2.5.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the notification request is redirected. |

Table 6.2.5.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Contains the identifier of the target NF (service) instance towards which the notification request is redirected |

# *** Next Change ***

##### 6.2.6.2.2	Type VflInferSub

Table 6.2.6.2.2-1: Definition of type VflInferSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifCorrId | string | M | 1 | The value of Notification Correlation ID in the corresponding notification. |  |
| notifUri | Uri | M | 1 | URI at which the NF service consumer requests to receive notifications. |  |
| reportingReqs | ReportingInformation | O | 0..1 | Reporting requirement information of the VFL inference subscription. |  |
| suppFeats | SupportedFeatures | C | 0..1 | List of Supported features used as described in clause 6.2.8. It shall be supplied by NF service consumer in the POST requests that request the creation of a VFL Subscriptions resource and shall be supplied by the AF in the reply of corresponding request. |  |
| vflInferAnaSubs | array(VflInferAnaSub) | M | 1..N | Contains the VFL inference subscription information for the subscribed analytics ID(s). (NOTE) |  |
| vflInferReq | VflInferReq | O | 0..1 | Represents required conditions to apply VFL inference. |  |
| vflInferResults | array(VflInferResult) | O | 1..N | Represents intermediate VFL inference results. (NOTE) |  |
| NOTE: The "intGroupIds" and "supis" attributes of the data type are not applicable for the AF services if the AF is an untrusted AF. The "exterGroupIds" and "gpsis" attributes of the data type are not applicable for the AF services if the AF is a trusted AF. |  |  |  |  |  |

# *** Next Change ***

##### 6.2.6.2.3	Type VflInferSubPatch

Table 6.2.6.2.3-1: Definition of type VflInferSubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | O | 0..1 | URI at which the NF service consumer requests to receive notifications. |  |
| notifCorrId | string | O | 0..1 | The value of Notification Correlation ID in the corresponding notification. |  |
| reportingReqs | ReportingInformation | O | 0..1 | Reporting requirements of the VFL inference subscription. |  |
| vflInferAnaSubs | array(VflInferAnaSub) | O | 1..N | Contains the VFL inference subscription information for the subscribed analytics ID(s). (NOTE) |  |
| vflInferReq | VflInferReq | O | 0..1 | Represents required conditions to apply VFL inference. |  |
| NOTE: The "intGroupIds" and "supis" attributes of the data type are not applicable for the AF services if the AF is an untrusted AF. The "exterGroupIds" and "gpsis" attributes of the data type are not applicable for the AF services if the AF is a trusted AF. |  |  |  |  |  |

# *** Next Change ***

# A.3	Naf_VFLInference API

openapi: 3.0.0

info:

title: Naf_VFLInference

version: 1.0.0-alpha.1

description: |

Naf_VflInference API Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.530 V0.1.0; 5G System; Application Function Artificial Intelligence/Machine

Learning (AI/ML) Services

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.530/

servers:

- url: '{apiRoot}/naf-vflinference/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- naf-vflinference

paths:

/subscriptions:

post:

summary: Create a new Individual VFL Inference Subscription resource.

operationId: CreateVFLInferenceSubcription

tags:

- Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VflInferSub'

responses:

'201':

description: Create a new Individual VFL Inference Subscription resource.

content:

application/json:

schema:

$ref: '#/components/schemas/VflInferSub'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the

structure

{apiRoot}/naf-vflinference/v1/subscriptions/{subscriptionId}.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflInferNotif'

responses:

'204':

description: No Content, Notification was succesfull

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: String identifying a subscription to the Naf_VFLInference Service.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual VFL Inference Subscription resource.

operationId: GetIndVFLTrainSubsc

tags:

- Individual VFL Inference Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual VFL Inference Subscription resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/VflInferSub'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Update an existing Individual VFL Inference Subscription

operationId: UpdateVFLInferenceSubcription

tags:

- Individual VFL Inference Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VflInferSub'

responses:

'200':

description: >

The Individual VFL Inference Subscription resource was modified

successfully and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/VflInferSub'

'204':

description: >

The Individual VFL Inference Subscription resource was modified successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Partial update an existing Individual VFL Inference Subscription

operationId: PartialUpdateVFLInferenceSubcription

tags:

- Individual VFL Inference Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/VflInferSubPatch'

responses:

'200':

description: >

The Individual VFL Inference Subscription resource was partial modified successfully

and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/VflInferSub'

'204':

description: >

The Individual VFL Inference Subscription resource was partial modified

successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual VFL Inference Subscription.

operationId: DeleteVFLInferenceSubcription

tags:

- Individual VFL Inference Subscription (Document)

responses:

'204':

description: >

No Content. The Individual VFL Inference Subscription matching the

subscriptionId was deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

naf-vflinference: Access to the Naf_VflInference API

schemas:

VflInferSub:

description: Represents a VFL Inference subscription.

type: object

properties:

notifCorrId:

type: string

description: >

String identifying the Notification Correlation ID in the corresponding

notification.

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

reportingReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

suppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

vflInferAnaSubs:

type: array

items:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflInferAnaSub'

minItems: 1

description: >

Contains VFL inference subscription information for the subscribed

analytics ID(s).

vflInferReq:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflInferReq'

vflInferResults:

type: array

items:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflInferResult'

minItems: 1

description: Represents intermediate VFL Inference result.

required:

- notifUri

- notifCorrId

- vflInferAnaSubs

VflInferSubPatch:

description: >

Represents parameters to request the modification of a VFL Inference

subscription.

type: object

properties:

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorrId:

type: string

reportingReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

vflInferAnaSubs:

type: array

items:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflInferAnaSub'

minItems: 1

description: >

Contains updated VFL inference subscription information for the subscribed

analytics ID(s).

vflInferReq:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflInferReq'

#

# ENUMERATIONS DATA TYPES

#