3GPP TSG CT WG3 Meeting #143	C3-254258

# Sophia-Antipolis, France, 13 – 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.525 | CR | 0406 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to UE policies for supporting V2X/A2X Capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, eV2XARC, SBIProtoc19 |  |  |  |  |  | Date: |  |  | 2025-9-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The V2XP includes the V2XP over PC5 and over Uu interface, and the current UE policy only allows providing the V2X capability over PC5 interface from UE side. However, in TS 23.287, it indicated, the UE may support: 4.4.1 UE In addition to the functions defined in TS 23.501 [6], the UE may support the following functions: - Report the V2X Capability and PC5 Capability for V2X to 5GC over N1 reference point. In addition, in 23.287, it specifies: 4.1 General concept There are two modes of operation for V2X communication, namely V2X communication over PC5 reference point and V2X communication over Uu reference point. These two operation modes may be used by a UE independently for transmission and reception. V2X communications over PC5 reference point are supported by LTE and/or NR. V2X communications over Uu reference point are supported by E-UTRA connected to 5GC and/or NR connected to 5GC: - For E-UTRA connected to 5GC, V2X communication over Uu reference point is only unicast. - For NR connected to 5GC, V2X communication over Uu reference point can be unicast and/or multicast/broadcast as defined in TS 23.247 [28]. An RSU is not an architectural entity, but an implementation option. This is achieved by collocating a V2X application logic/server with some entities of the 3GPP system, as shown in examples in Annex B. Therefore, the UE can indicate it supports V2X over PC5 and/or over Uu interface. In TS 24.501, clause 5.5.1.2.2, when UE register the capability for v2x capability, in addition to the pc5capability, there is also another encode to support general v2xcapability. If the UE supports V2X as specified in 3GPP TS 24.587 [19B], the UE shall set the V2X bit to "V2X supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports V2X communication over E-UTRA-PC5 as specified in 3GPP TS 24.587 [19B], the UE shall set the V2XCEPC5 bit to "V2X communication over E-UTRA-PC5 supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports V2X communication over NR-PC5 as specified in 3GPP TS 24.587 [19B], the UE shall set the V2XCNPC5 bit to "V2X communication over NR-PC5 supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. In TS24.587, further specified both v2x communication over Pc5 and Uu interface. Therefore, the UE shall set the V2X bit to "V2X supported" is used by a UE supporting V2X over PC5, V2X over Uu, or both. And then the AMF should be able to indicate such capability the PCF as well during UE policy creation and update. In 29.525, it also specifies that: ##### 4.2.2.2.4 Vehicle-to-Everything Policy (V2XP) V2XP includes the V2XP over PC5 and over Uu interfaces. The V2XP over PC5 are defined in clause 5.2.3 of 3GPP TS 24.587 [24] and the corresponding encoding is defined in clause 5.3.1 of 3GPP TS 24.588 [25]. The V2XP over Uu are defined in clause 5.2.4 of 3GPP TS 24.587 [24] and the corresponding encoding is defined in clause 5.3.2 of 3GPP TS 24.588 [25]. But the handling for indicating the V2X capability of Uu interface is not covered and not aligned with the stage 2 requirements. The H-PCF is the one who derives the V2XP/A2XP for the roaming and non-roaming case. A clarification can be done to align with the specification, such as clause 4.2.4.6. The Pc5Capability is not applicable to A2X. The description of Pc5Capability is incorrect. The description of the A2XCapability is incorrect in OpenAPI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the supported V2X capability provisioning for Uu interface. Update the policyAssociationRequest and PolicyAssocaitionUpdateRequest to be able to include the V2X capability for Uu interface. Add feature dependency. Introduce new attribute to cover both the PC5 and Uu case to be backward compatible solution. Add the clarification for H-PCF deriving the V2XP/A2XP for roaming and non-roaming. Correct the description for Pc5Capability. Correct the description to A2XCapability in OpenAPI. Update the OpenAPI accordingly. Table format has been updated to aligned the starting and ending size. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The V2XP over the Uu interface is not supported and SA2 requirements are not fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1, 4.2.2.2.1.2, 4.2.3.1, 4.2.4.5, 4.2.4.8, 5.6.1,5.6.2.3, 5.6.2.4, 5.6.3.5, 5.6.3.14, 5.8, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible corrections to the following APIs: TS29525_Npcf_UEPolicyControl.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

#### 4.2.2.1	General

The procedure in the present clause is applicable in the following cases:

-	UE performs initial registration to the network, as defined in clause 5.5.1.2.2 of 3GPP TS 24.501 [15];

-	UE performs a mobility registration, if the UE operating in single-registration mode performs inter-system change from S1 mode to N1 mode, as defined in clause 5.5.1.3.2 of 3GPP TS 24.501 [15], and there is no existing UE Policy Association between AMF and PCF for this UE;

-	the AMF is relocated (between the different AMF sets) and the new AMF selects a new PCF. The procedure for the case where the AMF is relocated and the new AMF selects the old PCF is defined in clause 4.2.3.1; and

-	when the UE Policy Association establishment is controled by the UE Policy Association Indicator provided by the UDM, when the indicator is set to enabled.

To support the delivery of URSP in EPS, the procedure in the present clause is also applicable when:

-	When the UE triggers a BEARER RESOURCE MODIFICATION REQUEST message with a UE policy container IE after the UE performs ePCO capability negotiation during PDN connection establishment procedure (during the Initial Attach with default PDN connection establishment or during the first PDN connection establishment or during PDN connection modification with or without QoS update or during new PDN connection establishment when no other existing PDN connection indicates support of URPS provisioning in EPS) as defined in 3GPP TS 24.301 [36], and both, the UE and the network support URSP provisioning in EPS PCO; and

-	5GS to EPS handover or 5GS to EPS Idle Mode mobility (both referred as 5GS to EPS mobility in the present document) as defined in 3GPP TS 24.501 [15] and if the UE and at least one of the PDN connection(s) supports URSP delivery in EPS as specified in 3GPP TS 29.512 [31].

The creation of a UE policy association only applies for normally registered UEs, i.e. it does not apply for emergency-registered UEs.

Figure 4.2.2.1-1 illustrates the procedure used for the creation of a policy association.

Figure 4.2.2.1-1: Creation of a UE policy association

NOTE 1:	For the roaming scenario, the PCF represents the V-PCF, if the NF service consumer is an AMF, and the PCF represents the H-PCF, if the NF service consumer is a V-PCF.

During UE registration or AMF reallocation with a new selected PCF, the AMF triggers the establishment of the UE Policy Association as follows:

i.	if the AMF received from the UDM the UE Policy Association Indicator set to enabled, the AMF shall establish a UE policy association with the (V-)PCF, in case there is no existing UE policy association for the UE;

ii.	if the AMF received from the UDM the UE Policy Association Indicator set to disabled, the AMF shall not establish the UE policy association with the (V-)PCF;

iii.	 if the AMF does not receive from the UDM the UE Policy Association Indicator:

-	if the AMF obtains from the UE a UE policy delivery protocol message as defined in Annex D of 3GPP TS 24.501 [15] and/or the authorized PC5 capability for 5G ProSe, and/or the authorized PC5 capability for V2X communications and/or A2X communications, and/or the authorized PC5 capability for Ranging/SL, the AMF shall establish the UE Policy Association with the (V-)PCF, in case there is no existing UE policy association for the UE; otherwise the AMF may establish a UE Policy Association with the (V-)PCF based on AMF local configuration.

NOTE 2:	The indication of whether the UE Policy Association is allowed by UDM subscription is delivered by the UDM to the NF service consumer within the Access and Mobility Subscription Data Retrieval service operation as described in 3GPP TS 29.503 [43].

NOTE 3:	In the roaming scenario, the visited AMF's local configuration can indicate whether UE Policy delivery is needed based on the roaming agreement with the home PLMN of the UE. The AMF’s local configuration takes precedence over the UE Policy Association Indicator, if received from the HPLMN.

If the AMF receives from the UDM the UE Policy Association Indicator, the AMF shall store the UE policy container received from the UE in the UE context, if received.

If the UE Policy Association Indicator changes from disabled to enabled and the PCF has not previously rejected or terminated the UE Policy Association, the AMF shall immediately establish the UE Policy Association and shall use the UE policy container stored in the UE context, if available. The (H-)(V-)PCF provides to the UE all the applicable UE Policies.

NOTE 4:	The AMF can still decide to initiate the establishment of the UE Policy Association based on local policies, even if the PCF had previously rejected or terminated the UE Policy Association.

NOTE 5:	The UE applies the received UE Policies for the PDU session(s) that were established before the enablement of the UE Policy Association as specified in 3GPP TS 24.526 [16].

During UE Initial Attach with default PDN connection or the establishment of the first PDN connection in EPS or a new PDN connection when no other existing PDN connection indicates the support of URSP provisioning in EPS, if the UE and the SMF+PGW support URSP provisioning in EPS PCO, the PCF for the PDU session supports the URSP provisioning in EPS as defined in 3GPP TS 29.512 [31], the PCF for a PDU session associated with the SMF+PGW-C serving the PDN connection obtains from the UE a UE policy container in a Npcf_SMPolicyControl_Update procedure triggered by a bearer resource modification procedure as described in 3GPP TS 29.512 [31]. Then, if the "EpsUrsp" feature described in clause 5.8 is supported, the PCF for a PDU session shall establish a UE policy association with the (V-)PCF for the UE for the delivery of URSP only.

During 5GS to EPS mobility with N26, and if the "EpsUrsp" feature described in clause 5.8 is supported, the PCF for the PDU session determines whether 5GS to EPS mobility applies based on the received RAT and/or Access-Type change event as described in 3GPP TS 29.512 [31]. Then, for non-roaming and Home Routed roaming scenarios, the PCF for a PDU session shall determine whether the UE supports URSP provisioning in EPS as described in 3GPP TS 29.512 [31], and if supported, shall establish a UE policy association with the PCF for the UE that is handling the UE policy association with the source AMF. For LBO roaming scenarios, the V-PCF for the PDU session determines based on local configuration whether to establish a UE Policy Association towards the V-PCF for the UE.

NOTE 6:	The PCF for the PDU session discovers the address of the PCF for the UE handling the UE policy association with the source AMF by querying the BSF as described in 3GPP TS 29.521 [35].

NOTE 7:	If during the 5GS to EPS mobility there are more than one PCF for the PDU session maintaining PDN connections for the UE, every PCF for the PDU session establishes a UE Policy Association towards the PCF for the UE. In LBO scenarios, the V-PCF for the UE will handle only one UE Policy Association towards the H-PCF for the UE.

To establish a UE policy association with the PCF, the NF service consumer (e.g. AMF) shall send an HTTP POST request with "{apiRoot}/npcf-ue-policy-control/v1/policies" as Resource URI and the PolicyAssociationRequest data structure as request body, which shall include:

-	the Notification URI encoded as "notificationUri" attribute;

-	the SUPI encoded as "supi" attribute; and

-	the features supported by the NF service consumer encoded as "suppFeat" attribute,

shall also include, when available:

-	the GPSI encoded as "gpsi" attribute;

-	the Access type encoded as "accessType" attribute;

-	the Permanent Equipment Identifier (PEI) encoded as "pei" attribute;

-	the User Location Information encoded as "userLoc" attribute;

-	the UE Time Zone encoded as "timeZone" attribute;

-	the identifier of the serving network (the PLMN Identifier or the SNPN Identifier), encoded as "servingPlmn" attribute;

NOTE 8:	The SNPN Identifier consists of the PLMN Identifier and the NID.

NOTE 9:	For Indirect Network Sharing and the interaction between the hosting operator network and the participating operator network case, the identifier of the serving network is set to the selected PLMN ID during the UE Policy Association establishment procedure, as specified in clause 5.18.1 of 3GPP TS 23.501 [2].

-	the RAT type encoded as "ratType" attribute;

-	the received UE policy delivery protocol message defined in Annex D of 3GPP TS 24.501 [15] encoded as "uePolReq" attribute;

-	for the roaming scenario, if the NF service consumer is an AMF, the H-PCF ID encoded as "hPcfId" attribute, and if the "EnhEstRoaming" feature is supported, the H-PCF URI encoded as the "hPcfUri" attribute and the H-PCF Set ID encoded as "hPcfSetId" attribute;

-	the Internal Group Identifier(s) encoded as "groupIds" attribute;

-	the PC5 capability for V2X encoded as "pc5Capab" attribute if the "V2X" feature defined in clause 5.8 is supported;

-	the V2X capability encoded as "v2xCapab" attribute if the "V2X_Ext" feature defined in clause 5.8 is supported;

-	the 5G ProSe capability within the "proSeCapab" attribute, if the "ProSe" feature defined in clause 5.8 is supported;

-	the Ranging/SL capability within the "rangSlCapab" attribute, if the "Ranging_SL" feature defined in clause 5.8 is supported;

-	if the NF service consumer is an AMF, the GUAMI encoded as "guami" attribute;

-	if the NF service consumer is an AMF, the serving AMF Id encoded as "servingNfId" attribute;

NOTE 10:	If the PCF received the "servingNfId" attribute, the PCF can use the Nnrf_NFDiscovery Service specified in 3GPP TS 29.510 [13] to retrieve the NF profile of the Namf_Communication service available in the indicated AMF instance Id.

-	if the NF service consumer is an AMF and the "SliceAwareANDSP" feature is supported:

-	if the UE indicated the support of slice-based N3IWF and/or TNGF selection as specified in 3GPP TS 24.501 [15], the AMF may provide information about these UE indications within the "sliceN3gNodeSelCap" attribute;

-	if the AMF has determined that the UE has selected a non-3gpp access node (i.e. TNGF or N3IWF) that is not compatible with the allowed S-NSSAI(s), and the UE indicated the support of slice-based N3IWF and/or TNGF selection as specified in 3GPP TS 24.501 [15], the wrongly selected type of non-3gpp access node encoded as "n3gNodeReSel" attribute, and, in the roaming case, also the Configured NSSAI for the serving PLMN encoded as "confSnssais" attribute;

-	if the NF service consumer is an AMF, the Satellite Backhaul Category encoded as "satBackhaulCategory" attribute, if the "EnSatBackhaulCategoryChg" feature defined in clause 5.8 is supported;

-	if the NF service consumer is the PCF for the PDU session, and the "EpsUrsp" feature defined in clause 5.8 is supported, the indication that the trigger for the UE Policy Association Establishment is the 5GS to EPS mobility scenario encoded as the "5gsToEpsMob" attribute;

-	for the roaming scenario, if the NF service consumer is an AMF and the "NssaiChange" feature is supported, the Configured NSSAI for the serving PLMN encoded as "confSnssais" attribute and optionally the mapped each S-NSSAI value of home network corresponding to the configured S-NSSAI values in the serving PLMN encoded as "mappedHomeSnssai" attribute within the "confSnssais" attribute;

-	the A2X capability encoded as "a2xCapab" attribute if the "A2X" feature defined in clause 5.8 is supported;

-	if the feature "AccessChange" is supported, the NF service consumer shall include:

a)	the "accessTypes" attribute indicating registration in the 3GPP access, in the non-3GPP access, or in both 3GPP and non-3GPP access, if available; and

b)	the RAT type entry corresponding to the 3GPP access and/or the RAT type entry corresponding to the non-3GPP access encoded in the "ratTypes" attribute, if available.

NOTE 11:	If the feature "AccessChange" is not supported or it is not known yet whether it is supported in the PCF, the NF service consumer can also provide the "accessType" attribute and the "ratType" attribute, if available, with one available access type and RAT type.

NOTE 12:	When the UE is simultaneously connected to the 5G Core Network of a PLMN/SNPN over a 3GPP access and a non-3GPP access, the UE is served by the same AMF, as specified in 3GPP TS 23.501 [2]. In this case, the UE Policy Association contains both, 3GPP and non-3GPP accesses.
When the UE is simultaneously connected to 5G Core Network over 3GPP access and non-3GPP access in different PLMN(s)/SNPN(s), the UE is served by different AMFs. In this case, there can be two UE Policy Associations, each with the corresponding access type.

-	for the roaming scenario, if the NF service consumer is a V-PCF and the "VPLMNSpecificURSP" feature is supported, the AF guidance on VPLMN-specific URSP rules related information, if applicable, within the "vpsUePolGuidance" attribute, that shall contain for each related AF:

a.	the AF guidance on VPLMN-Specific URSP rules within the "urspGuidance" attribute; and

b.	if the AF requested to the VPLMN notifications about the delivery of UE Policies, the "deliveryEvents" attribute including the "SUCCESS_UE_POL_DEL_SP" and/or "UNSUCCESS_UE_POL_DEL_SP" and/or if feature "ExtDeliveryOutcome" is supported, "PARTLY_UNSUCC_UE_POL_DEL_SP" and/or "UNSUCCESS_PCF_SERVICE_AUTHORIZATION" events; and

-	for the roaming scenario, if the NF service consumer is an AMF, and the "VPLMNSpecificURSP" feature is supported, LBO information within the "lboRoamInfo" attribute.

and may include:

-	if the NF service consumer is an AMF, the name of a service produced by the AMF that expects to receive information via the Npcf_UEPolicyControl_UpdateNotify service operation encoded as "serviceName" attribute;

-	if the NF service consumer is an AMF, the alternate or backup IPv4 Address(es) where to send Notifications encoded as "altNotifIpv4Addrs" attribute;

-	if the NF service consumer is an AMF, the alternate or backup IPv6 Address(es) where to send Notifications encoded as "altNotifIpv6Addrs" attribute;

-	if the NF service consumer is an AMF, the alternate or backup FQDN(s) where to send Notifications encoded as "altNotifFqdns" attribute;

-	if the NF service consumer is an AMF, the alternate or backup FQDN(s) where to send Notifications encoded as "altNotifFqdns" attribute.

Upon the reception of the HTTP POST request,

-	the (V-)(H-)PCF shall assign a UE policy association ID;

-	for the roaming scenario and based on operator policy, the V-PCF (as the NF service consumer) should send to the H-PCF a request for the Creation of a UE policy association as described in the present clause;

-	the (V-)(H-)PCF shall determine the applicable UE policy as detailed in clause 4.2.2.2. For the V-PCF, any policy received from the H-PCF in the reply to the possible request for the Creation of a policy association should be taken into consideration;

-	if the (V-)PCF determines that UE policy needs to be provisioned, it shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the UE policy according to clause 4.2.2.2 and as follows:

(i)	the (V-)PCF shall subscribe to the AMF to notifications on N1 messages for UE Policy Delivery Results using the Namf_Communication_N1N2MessageSubscribe service operation;

(ii)	the (V-)PCF shall send the determined UE policy (e.g. ANDSP, URSP, V2XP, A2XP, ProSeP, RSLPP) using Namf_Communication_N1N2MessageTransfer service operation(s); and

(iii) the (V-)PCF shall be prepared to receive UE Policy Delivery Results from the AMF and/or subsequent UE policy requests (e.g. for V2XP and/or A2XP and/or ProSeP and/or RSLPP) within the Namf_Communication_N1MessageNotify service operation. For the V-PCF, if the received UE Policy Delivery results relate to UE policy sections provided by the H-PCF, the V-PCF shall use the Npcf_UEPolicyControl_Update Service Operation defined in clause 4.2.3 to send those UE Policy Delivery results to the H-PCF;

-	if the UE indicates the support of V2X communications over PC5 reference point and the "V2X" feature is supported, the (H-)PCF shall determine the applicable V2XP, as detailed in clause 4.2.2.2.1.2, and V2X N2 PC5 policy, as detailed in clause 4.2.2.3 and based on the operator's policy;

-	if the UE indicates the support of 5G ProSe and the "ProSe" feature is supported, the (H-)PCF shall determine the applicable ProSeP, as detailed in clause 4.2.2.2.1.3, and 5G ProSe N2 PC5 policy, as detailed in clause 4.2.2.4 and based on the operator's policy;

-	if the UE indicates the support of Ranging/SL and the "Ranging_SL" feature is supported, the (H-)PCF shall determine the applicable RSLPP, as detailed in clause 4.2.2.2.1.5, and Ranging/SL N2 PC5 policy, as detailed in clause 4.2.2.7, and based on the operator's policy;

-	if the PCF determines that N2 PC5 policy (e.g., for V2X communications, for 5G ProSe, for Ranging/SL) needs to be provisioned, including the case of the V-PCF when receiving the N2 PC5 policy from the H-PCF, the PCF shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the N2 PC5 policy according to clause 4.2.2.3 and/or clause 4.2.2.4;

-	if the UE indicates support for URSP provisionng in EPS, the "EpsUrsp" feature is supported, and the (V-)PCF determines that UE policy needs to be provisioned via a PCF for a PDU session, the (V-)PCF shall select a UE Policy Association and shall provision the UE policy according to clause 4.2.2.2 and as follows:

(i)	the (V-)PCF shall send a UE policy container with the determined URSP using Npcf_UEPolicyControl_Create response service operation(s); and

(ii) the (V-)PCF shall be prepared to receive UE Policy Delivery Results from the PCF for a PDU session. The PCF for a PDU session shall use the Npcf_UEPolicyControl_Update service operation defined in clause 4.2.3 to send those UE Policy Delivery results to the (V-)PCF;

-	if the UE indicates the support of A2X communications over PC5 reference point and the "A2X" feature is supported, the (H-)PCF shall determine the applicable A2XP, as detailed in clause 4.2.2.2.1.4, and A2X N2 PC5 policy, as detailed in clause 4.2.2.5 and based on the operator's policy;

for the successful case, the (V-)(H-)PCF shall send a HTTP "201 Created" response with the URI for the created resource in the "Location" header field.

NOTE 13:	The assigned policy association ID is part of the URI for the created resource and is thus associated with the SUPI.

and the PolicyAssociation data type as response body, including:

-	mandatorily, the negotiated supported features encoded as "suppFeat" attribute;

-	optionally, the information provided by the NF service consumer when requesting the creation of this policy association encoded as "request" attribute;

-	optionally, for the H-PCF as service producer communicating with the V-PCF, UE policy (see clause 4.2.2.2) encoded as "uePolicy" attribute;

-	optionally, for the H-PCF as service producer communicating with the V-PCF, N2 PC5 policy (see clause 4.2.2.3 and/or clause 4.2.2.4 and/or clause 4.2.2.5 and/or clause 4.2.2.6) encoded as "n2Pc5Pol" attribute (for V2X communications) and/or "n2Pc5PolA2x" attribute (for A2X communications) and/or "n2Pc5ProSePol" attribute (for 5G ProSe) and/or "n2Pc5RsppPol" attribute (for Ranging/SL);

-	optionally, for the H-PCF as service producer communicating with the V-PCF, and when the feature "UECapabilityIndication" is supported, if the H-PCF did not receive from the UE information about ANDSP support and the information is available and reliable in the UDR (see clause 4.2.2.2.1.1), the ANDSP support indication retrieved from UDR encoded as "andspInd" attribute;

-	optionally, for the (V-)PCF communicating with the AMF, and if the "URSPEnforcement" feature is supported, the request to the AMF to be notified about the PDU session established/terminated events by providing the PCF for the UE callback information within the "pcfUeInfo" attribute, and the DNN and S-NSSAI combination of the concerned PDU session(s) within the "matchPdus" attribute.

-	optionally, one or several of the following Policy Control Request Trigger(s) encoded as "triggers" attribute (see clause 4.2.3.2):

a)	Location change (tracking area);

b)	Change of UE presence in PRA;

c)	Change of PLMN, if the "PlmnChange" feature is supported;

d)	Change of UE connectivity state, if the "ConnectivityStateChange" feature is supported;

e)	URSP rule enforcement information, if the "URSPEnforcement" feature is supported;

f)	Change of Satellite Backhaul Category, if the "EnSatBackhaulCategoryChg" feature is supported;

g)	Change of Access Type and RAT Type, if the "AccessChange" feature is supported;

h)	LBO information change, applicable to roaming scenarios, if the "VPLMNSpecificURSP" feature is supported and the NF service consumer is an AMF; and

i)	Change of Configured NSSAI, in roaming scenarios, if the "NssaiChange" feature is supported and the NF service consumer is the AMF;

-	if the Policy Control Request Trigger "Change of UE presence in PRA" is provided, the presence reporting areas for which reporting is required encoded as "pras" attribute;

-	if the Policy Control Request Trigger "LBO information change" is provided, optionally, the DNNs(s) and S-NSSAI(s) for which LBO information is required encoded as "pduSessions" attribute;

-	if the NF service consumer is an AMF and the "SLAMUP" feature is supported, based on the operator policies the H-PCF indicates that the AMF should select the same CHF that is selected by the H-PCF for a UE, the charging address(es) information encoded in the "chfInfo" attribute.

-	for the roaming scenario, if the NF service consumer is a V-PCF and the "SLAMUP" feature is supported, based on the operator policies the H-PCF interacts with V-PCF to indicate that the AMF should select the same H-CHF that is selected by the H-PCF for a UE, the charging address(es) information encoded in the "chfInfo" attribute.

NOTE 14:If the PCF uses a Presence Reporting Area identifier referring to a Set of Core Network predefined Presence Reporting Areas as defined in 3GPP TS 23.501 [2], the PCF includes the identifier of this Presence Reporting Area set within the "praId" attribute.

-	if the "SliceAwareANDSP" feature is supported, the PCF received the "n3gNodeReSel" attribute and the PCF has successfully delivered to the UE the ANDSP/WLANSP with the slice selection information for the corresponding non-3gpp node, the indication of the successful UE configuration by providing the "andspDelInd" attribute with the value "CONFIGURED". The PCF may delay the indication of the configuration result to a subsequent Npcf_UEPolicyControl_UpdateNotify request, as described in clause 4.2.4.2.

-	if errors occur when processing the HTTP POST request, the (V-)(H-)PCF shall apply error handling procedures as specified in clause 5.7 and according to the following provisions:

-	if the user information received within the "supi" attribute is unknown, the (V-)(H-)PCF shall reject the request and include in an HTTP "400 Bad Request" response message the "cause" attribute of the ProblemDetails data structure set to "USER_UNKNOWN"; and

-	if the (V-)(H-)PCF is, due to incomplete, erroneous or missing information in the request, not able to provision a UE policy decision, the (V-)(H-)PCF may reject the request and include in an HTTP "400 Bad Request" response message the "cause" attribute of the ProblemDetails data structure set to "ERROR_REQUEST_PARAMETERS".

If the (V-)PCF received a GUAMI, the (V-)PCF may subscribe to GUAMI changes using the AMFStatusChange service operation of the Namf_Communication service specified in 3GPP TS 29.518 [14], and it may use the Nnrf_NFDiscovery Service specified in 3GPP TS 29.510 [13] (using the obtained GUAMI and possibly service name) to query the other AMFs within the AMF (service) set.

When the "SliceAwareANDSP" feature is supported, and the AMF receives the "andspDelInd" attribute, the AMF, based on operator's policies, may reject the UE Registration request, and may provide a valid target N3IWF/TNGF within the Registration Reject message as specified in clause 5.5.1.3.5 of 3GPP TS 24.501 [15]. In this case, the AMF terminates the UE Policy Association as described in clause 4.2.5 (if the UE is not registered over 3GPP access).

# *** Next Change ***

###### 4.2.2.2.1.2	Provisioning of Vehicle-to-Everything Policy

When the UE registers to the network, if the AMF receives from the UE the PC5 capability for V2X communications in the Registration Request message, the UE is authorized to use V2X service based on the UE's subscription information and the "V2X" feature is supported, the AMF further reports to the PCF the PC5 capability for V2X communications within the "pc5Capab" attribute and/or if the "V2X_Ext" feature is supported, the V2X capability within the "v2xCapab" attribute as defined in clause 4.2.2.1. The PCF may determine the V2XP over PC5 interface and/or V2XP based on the received UE's PC5 capability and/or if the "V2X_Ext" feature is supported, the received V2X capability for V2X, the Service specific parameter information retrieved from UE's Application Data in the UDR as defined in clause 6.2.15 of 3GPP TS 29.519 [17] and the operator's policy.

After UE registration, if the UE supports V2X communication and it does not have valid V2XP, the UE includes the "UE POLICY PROVISIONING REQUEST" message as defined in 3GPP TS 24.587 [24] during the NAS transport procedure. The PCF may reject the request by sending back a "UE POLICY PROVISIONING REJECT" message as defined in clause 7.2.2 of 3GPP TS 24.587 [24] or provision the policy, as defined in clause 4.2.2.2.1, based on the service specific parameter information retrieved from UE's Application Data in the UDR as defined in clause 6.2.15 of 3GPP TS 29.519 [17] and the operator's policy.

For both scenarios mentioned above, in the roaming case, the H-PCF may include the V2XP within the "uePolicy" attribute in the policy association create or update response to the V-PCF and in the policy association update request initiated by the H-PCF.

In the roaming or non-roaming case, the (V-)PCF shall use the Namf_Communication_N1N2MessageTransfer service operation defined in clause 5.2.2.3.1 of 3GPP TS 29.518 [14] to send the V2XP to the UE.

# *** Next Change ***

#### 4.2.3.1	General

The procedure in the present clause is applicable when the NF service consumer modifies an existing UE policy association (including the case where the AMF is relocated and the new AMF selects to maintain the policy association with the old PCF and to update the Notification URI).

Figure 4.2.3.1-1 illustrates the update of a policy association.

Figure 4.2.3.1-1: Update of a UE policy association

NOTE 1:	For the roaming case, the PCF represents the V-PCF if the NF service consumer is an AMF and the PCF represents the H-PCF if the NF service consumer is a V-PCF.

The AMF, as NF service consumer, invokes this procedure when a subscribed policy control request trigger (see clause 4.2.3.2) occurs. When a policy control request trigger that requires the subscription as defined in table 5.6.3.3-1 (e.g. LOC_CH trigger) occurs, the NF service consumer (AMF) shall only invoke this procedure if the PCF has explicitly subscribed to that event trigger. When a policy control request trigger that does not require the subscription as defined in table 5.6.3.3-1 (e.g. GROUP_ID_LIST_CHG trigger) occurs, the NF service consumer (AMF) shall always invoke the procedure.

NOTE 2:	The AMF uses the Namf_Communication_N1MessageNotify service operation specified in 3GPP TS 29.518 [14] to send to the V-PCF a "MANAGE UE POLICY COMPLETE" message or a "MANAGE UE POLICY COMMAND REJECT" message, as defined in Annex D.5 of 3GPP TS 24.501 [15] or a "UE POLICY PROVISIONING REQUEST" message as defined in clause 7.2.1.1 of 3GPP TS 24.587 [24].

If an AMF as NF service consumer knows by implementation specific means that the UE context has been transferred to an AMF with another GUAMI within the AMF set, it may also invoke this procedure to update the Notification URI.

NOTE 3:	Either the old or the new AMF can invoke this procedure.

During the AMF relocation, if the new AMF received the resource URI of the individual UE Policy from the old AMF and selects the old PCF, the new AMF shall also invoke this procedure to update the Notification URI. The new AMF may also update the alternate or backup IP addresses, and if service discovery via NRF applies, the AMF Id. If the feature "FeatureRenegotiation" is supported, the new AMF may perform feature renegotiation, as described in clause 4.2.3.4.

NOTE 4:	During inter-AMF mobility, the N1N2 Individual Subscription context is transferred from the source AMF to the target AMF as specified in 3GPP TS 29.518 [14]. When the target AMF determines to reuse the UE Policy Association indicated by the source AMF, the PCF can keep the N1N2 Individual Subscription context and, for subsequent interactions, replace in the request URI the {apiRoot} of the N1N2 Individual Subscription resource with the one of the target AMF.

The V-PCF, as NF service consumer, invokes this procedure when a policy control request trigger (see clause 4.2.3.2) occurs. When a policy control request trigger that does not require the subscription as defined in table 5.6.3.3-1 (e.g. UE_POLICY trigger) occurs, the V-PCF shall always invoke the procedure. When a policy control request trigger that requires the subscription as defined in table 5.6.3.3-1 (e.g. LOC_CH trigger) occurs, the V-PCF shall only invoke this procedure if the H-PCF has subscribed to that event trigger.

To request policies (e.g. policy control request trigger(s) is/are met) from the PCF, to update the Notification URI, to renegotiate features, to update the trace control configuration or to request the termination of trace, the NF Service Consumer shall request the update of the associated UE Policy Association by providing the relevant parameters about the UE context in an HTTP POST request with "{apiRoot}/npcf-ue-policy-control/v1/policies/{polAssoId}/update" as Resource URI and the PolicyAssociationUpdateRequest data structure as request body that shall include:

-	at least one of the following:

1.	a new Notification URI encoded in the "notificationUri" attribute;

2.	observed Policy Control Request Trigger(s) (see clause 4.2.3.2) encoded as "triggers" attribute;

3.	if a UE location change occurred, the UE location encoded as "userLoc" attribute;

4.	if a "MANAGE UE POLICY COMPLETE" message or a "MANAGE UE POLICY COMMAND REJECT" message of the UE policy delivery protocol defined in Annex D of 3GPP TS 24.501 [15] has been received by the V-PCF as NF service consumer, and at least parts of the contents relate to UPSIs of the HPLMN, the parts of that message that relate to UPSIs of the HPLMN encoded as "uePolDelResult" attribute;

5.	if the Policy Control Request Trigger "Change of UE presence in PRA" is provided, the current presence status of the UE for the presence reporting areas for which reporting was requested, if not previously provided, or the presence reporting areas for which reporting was requested and the status has changed encoded as "praStatuses" attribute;

NOTE 5:		If the PCF included the identifer of a Core Network predefined Presence Reporting Area Set within the "praId" attribute during the subscription to changes of UE presence in PRA, the AMF only provides the presence reporting area information corresponding to the concerned individual Presence Reporting Area Identifier(s) within the Set. The "praId" attribute within each returned "PresenceInfo" data type hence includes the identifier of the concerned individual Presence Reporting Area.

6.	if the NF service consumer is an AMF, for AMF relocation scenarios, if available, alternate or backup IPv4 Address(es) where to send Notifications encoded as "altNotifIpv4Addrs" attribute;

7.	if the NF service consumer is an AMF, for AMF relocation scenarios, if available, alternate or backup IPv6 Address(es) where to send Notifications encoded as "altNotifIpv6Addrs" attribute;

8.		if the NF service consumer is an AMF, for AMF relocation scenarios, if available, alternate or backup FQDN(s) where to send Notifications encoded as "altNotifFqdns" attribute;

9.	for AMF relocation scenarios, the GUAMI encoded as "guami" attribute;

NOTE 6:	An alternate NF service consumer than the one that requested the generation of the subscription resource can send the request. For instance, an AMF as service consumer can change;

10.		if the NF service consumer is an AMF, for AMF relocation scenarios, the new serving AMF Id encoded in the "servingNfId" attribute;

NOTE 7:	If the PCF received the "servingNfId" attribute, the PCF can use the Nnrf_NFDiscovery Service specified in 3GPP TS 29.510 [13] to retrieve the NF profile of the Namf_Communication service available in the indicated AMF instance Id.

11.	if a UE PLMN change occurred and the "PlmnChange" feature defined in clause 5.8 is supported, the PLMN Identifier or the SNPN Identifier of the new serving network encoded as "plmnId" attribute;

NOTE 8:	The SNPN Identifier consists of the PLMN Identifier and the NID.

NOTE 9:	When the UE moves between PLMNs, the trigger reports changes of equivalent PLMNs.

NOTE 10:	Mobility between non-equivalent SNPNs, and between SNPN and PLMN is not supported. When the UE is operating in SNPN access mode, the trigger reports changes of equivalent SNPNs.

12.	if a "UE POLICY PROVISIONING REQUEST" message defined in clause 7.2.1.1 of 3GPP TS 24.587 [24] has been received by the V-PCF as NF service consumer and respectively the "V2X" feature and/or the "A2X" feature and/or the "ProSe" feature and/or the "Ranging_SL" feature defined in clause 5.8 is/are supported, the message encoded as "uePolReq" attribute;

13.	if a UE Internal Group Identifier(s) change occurred and the "GroupIdListChange" feature defined in clause 5.8 is supported, the Internal Group Identifier(s) of the served UE encoded as "groupIds" attribute;

14.	if a change of PC5 capablity for 5G ProSe occurred and the "ProSe" feature defined in clause 5.8 is supported, the PC5 capability for 5G ProSe encoded as "proSeCapab" attribute;

14a.	if a change of the Ranging/SL Capability occurred and the "Ranging_SL" feature defined in clause 5.8 is supported, the Ranging/SL Capability encoded as "rangSlCapab" attribute;

14b.	if a change of A2X capablities for 5G A2X occurred and the "FeatureRenegotiation" and the "V2X" features defined in clause 5.8 are supported, the A2X capablities for A2X encoded as "a2xCapab" attribute;

14c.	if a change of V2X capablity for 5G V2X occurred and the "FeatureRenegotiation" and the "V2X_Ext" features defined in clause 5.8 are supported, the V2X capablity for V2X encoded as "v2xCapab" attribute;

14d.	if a change of V2X PC5 capablity for 5G V2X occurred and the "FeatureRenegotiation" and the "V2X" features defined in clause 5.8 are supported, the V2X PC5 capablity for V2X encoded as "pc5Capab" attribute;

15.	if a change of the connectivity state of the UE occurred and the "ConnectivityStateChange" feature defined in clause 5.8 is supported, the connectivity state of the served UE encoded as "connectState" attribute;

16.	when a response with HTTP status code 4xx or 5xx as defined in clause 5.2.2.3.1.2 of 3GPP TS 29.518 [14] or a N1N2 Transfer Failure Notification as defined in clause 5.2.2.3.2 of 3GPP TS 29.518 [14] is received by the V-PCF after provisioning the UE policy by invoking the Namf_Communication_N1N2MessageTransfer service operation to the AMF, this UE policy transfer failure notification encoded as "uePolTransFailNotif" attribute;

17.		if the NF service consumer is an AMF, the "SliceAwareANDSP" feature is supported, and the "NON_3GPP_NODE_RESELECTION" trigger is reported within the "triggers" attribute, the wrongly selected type of non-3gpp access node encoded as "n3gNodeReSel" attribute, and, in the roaming case, also the Configured NSSAI for the serving PLMN encoded as "confSnssais" attribute;

18.		if satellite backhaul category change occurred and the "EnSatBackhaulCategoryChg" feature defined in clause 5.8 is supported, the satellite backhaul category or non-satellite backhaul encoded as "satBackhaulCategory" attribute;

19.	for the roaming scenario, if the NF service consumer is an AMF, Configured NSSAI change occurred and the "NssaiChange" feature is supported, the Configured NSSAI for the serving PLMN encoded as "confSnssais" attribute and optionally the mapped each S-NSSAI value of home network corresponding to the configured S-NSSAI values in the serving PLMN encoded as "mappedHomeSnssai" attribute within the "confSnssais" attribute;

20	for the roaming scenario, if the NF service consumer is a V-PCF, the "URSPEnforcement" feature is supported, and the "URSP_ENF_INFO" policy control request trigger is met, the URSP rule enforcement information within the "urspEnfRep" attribute;

21.	for the roaming scenario, if the NF service consumer is a V-PCF the "VPLMNSpecificURSP" feature is supported, the new/modified/deleted AF guidance on VPLMN-specific URSP rules related information within the "vpsUePolGuidance" attribute, that shall contain for each related AF:

a.	the AF guidance on VPLMN-Specific URSP rules within the "urspGuidance" attribute, if the AF updated/provided this information; and/or

b.	if the AF requested to the VPLMN notifications about the delivery of UE Policies or the update of the subscription to notification information previously provided, the "deliveryEvents" attribute including the"SUCCESS_UE_POL_DEL_SP" and/or "UNSUCCESS_UE_POL_DEL_SP" and/or if feature "ExtDeliveryOutcome" is supported, "PARTLY_UNSUCC_UE_POL_DEL_SP" and/or "UNSUCCESS_PCF_SERVICE_AUTHORIZATION" events;

22.	for the roaming scenario, if the NF service consumer is an AMF, the "VPLMNSpecificURSP" feature is supported and the "LBO_INFO_CH" policy control request trigger is met, the LBO roaming information within the "lboRoamInfo" attribute; and/or

23.		if an access type change occurred and the "AccessChange" feature defined in clause 5.8 is supported, the access type(s) where the UE is registered encoded within the "accessTypes" attribute and the corresponding RAT Type(s), if available, in the "ratTypes" attribute.

Upon the reception of the HTTP POST request:

-	if the PCF is a V-PCF and the V-PCF has an established policy association with the H-PCF, the V-PCF shall determine based on the contents of a potentially received "uePolDelResult" attribute to be sent to the H-PCF (see above) and requested event triggers of the H-PCF whether to send as the NF service consumer towards the H-PCF a request for the update of the policy association as described in the present clause;

-	the (V-)(H-)PCF shall determine the applicable UE policy based on the contents of the received HTTP POST request, the UE Policy Sections stored in UDR, local policy and, for the H-PCF, taking into consideration the information received within the UE policy delivery protocol encoded in the "uePolReq" attribute, if available, and for the V-PCF, taking into consideration any policy received from the H-PCF encoded in the "uePolicy" attribute in the reply to the possible request for the update of the associated policy association. When the "ProSe" feature is supported, the H-PCF shall determine the applicable ProSeP based on the received PC5 capability for 5G ProSe. When the UE disables a 5G ProSe capability the PCF may stop updating the corresponding ProSeP, and when the UE enables a 5G ProSe capability the PCF may update the corresponding ProSeP;

-	if the UE indicated the support of A2X communications over PC5 reference point, "A2X" feature is supported, and for the H-PCF, if the UE POLICY PROVISIONING REQUEST message was included in the "uePolReq" attribute, the (H-)PCF shall determine the applicable A2XP and A2X N2 PC5 policy as detailed in clauses 4.2.2.2.1.4 and 4.2.2.5, based on the operator's policy;

-	if the UE indicates the support of 5G ProSe communications over PC5 reference point, the "ProSe" feature is supported, and for the H-PCF, if the UE POLICY PROVISIONING REQUEST message with the requested 5G ProSe policies was included in the "uePolReq" attribute, the (H-)PCF shall determine the applicable ProSeP and 5G ProSe N2 PC5 policy, as detailed in clauses 4.2.2.2.1.3 and 4.2.2.4, based on the operator's policy;

-	if the UE indicated the support of V2X communications over PC5 reference point, "V2X" feature is supported, and for the H-PCF, if the UE POLICY PROVISIONING REQUEST message was included in the "uePolReq" attribute, the (H-)PCF shall determine the applicable V2XP and V2X N2 PC5 policy as detailed in clauses 4.2.2.2.1.2 and 4.2.2.3, based on the operator's policy;

-	if the UE indicated the support of Ranging/SL over the PC5 reference point, the "Ranging_SL" feature is supported, and for the H-PCF, if the UE POLICY PROVISIONING REQUEST message was included in the "uePolReq" attribute, the (H-)PCF shall determine the applicable RSLPP and Ranging/SL N2 PC5 policy as detailed in clauses 4.2.2.2.1.5 and 4.2.2.6 based on the operator's policy;

-	for the succesfull case, the (V-)(H-)PCF shall send a HTTP "200 OK" response with the PolicyUpdate data type as response body with the possibly updated of UE policy (for the H-PCF), and/or ProSe N2 PC5 policy (for the H-PCF) as specified in clause 4.2.2.4, N2 PC5 policy for V2X communications and/or A2X communications and/or 5G ProSe (for the H-PCF), as specified in clause 4.2.2.3, and/or the Ranging/SL N2 PC5 policy (for the H-PCF), as specified in clause 4.2.2.6, and/or Policy Control Request Trigger(s) encoded as described in clause 4.2.3.3;

-	if the (V-)PCF determines that UE policy needs to be updated, it shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the UE policy according to clause 4.2.2.2 and as follows:

(i)	the (V-)PCF shall send the determined UE policy using Namf_Communication_N1N2MessageTransfer service operation(s); and

(ii)	the (V-)PCF shall be prepared to receive UE Policy Delivery Results from the AMF within the Namf_Communication_N1MessageNotify service operation, and for the V-PCF, if the received UE Policy Delivery results relate to UE policy sections provided by the H-PCF, the V-PCF shall use the Npcf_UEPolicyControl_Update Service Operation to send those UE Policy Delivery results to the H-PCF; and

NOTE 11:	A PolicyUpdate data structure with only mandatory attribute(s) is included in the "200 OK" response when the PCF decides not to update the policies.

-	if the PCF determines that the V2XP and N2 PC5 policy (e.g. for V2X communications, for 5G ProSe) for V2X communications need to be updated, and for the V-PCF when receiving the updated V2XP and N2 PC5 policy for V2X communications from the H-PCF, it shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the V2XP to the UE and the V2X N2 PC5 policy to NG-RAN according to clauses 4.2.2.2.1.2 and 4.2.2.3;

-	if the PCF determines that the A2XP (e.g. for A2X communications) for A2X communications need to be updated, and for the V-PCF when receiving the updated A2XP and N2 PC5 policy for A2X communications from the H-PCF, it shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the A2XP to the UE and the A2X N2 PC5 policy to NG-RAN according to clauses 4.2.2.2.1.4 and 4.2.2.5;

-	if the PCF determines that ProSeP and 5G ProSe N2 PC5 policy needs to be updated, and for the V-PCF when receiving the updated ProSeP and 5G ProSe N2 PC5 policy from the H-PCF, it shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the ProSeP to the UE and 5G ProSe N2 PC5 policy to NG-RAN according to clauses 4.2.2.2.1.3 and 4.2.2.4;

-	if the PCF determines that RSLPP and Ranging/SL N2 PC5 policy needs to be updated, and for the V-PCF when receiving the updated RSLPP and Ranging/SL N2 PC5 policy from the H-PCF, it shall use the Namf_Communication service specified in 3GPP TS 29.518 [14] to provision the RSLPP to the UE and Ranging/SL N2 PC5 policy to NG-RAN according to clauses 4.2.2.2.1.5 and 4.2.2.6;

-	if the "SliceAwareANDSP" feature is supported, the PCF received the "NON_3GPP_NODE_RESELECTION" trigger, and the PCF has successfully delivered to the UE the ANDSP/WLANSP with the slice selection information for the corresponding non-3gpp node, the indication of the successful UE configuration by providing the "andspDelInd" attribute with the value "CONFIGURED". The PCF may delay the indication of the configuration result to a subsequent Npcf_UEPolicyControl_UpdateNotify request as described in clause 4.2.4.2; and

-	optionally, for the (V-)PCF communicating with the AMF, if the "URSPEnforcement" feature is supported, and if not previously provided, the request to the AMF to be notified about the PDU session established/terminated events by providing the PCF for the UE callback information within the "pcfUeInfo" attribute, and the DNN and S-NSSAI of the concerned PDU session(s) within the "matchPdus" attribute. If previously provided, the (V-)PCF may update the complete list of DNN and S-NSSAI combination(s) of the concerned PDU session(s) within the "matchPdus" attribute and/or update the PCF for the UE callback information within the "pcfUeInfo" attribute.

-	if errors occur when processing the HTTP POST request, the (V-)(H-)PCF shall:

-	send an HTTP error response as specified in clause 5.7; or

-	if the feature "ES3XX" is supported, and the (V-)(H-)PCF determines the received HTTP POST request needs to be redirected, send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [5];

according to the following provisions:

-	if the (V-)(H-)PCF is, due to incomplete, erroneous or missing information in the request not able to provision a UE policy decision, the PCF may reject the request and include in an HTTP "400 Bad Request" response message the "cause" attribute of the ProblemDetails data structure set to "ERROR_REQUEST_PARAMETERS".

If the PCF received a new GUAMI, the PCF may subscribe to GUAMI changes using the AMFStatusChange service operation of the Namf_Communication service specified in 3GPP TS 29.518 [14], and it may use the Nnrf_NFDiscovery Service specified in 3GPP TS 29.510 [13] (using the obtained GUAMI and possibly service name) to query the other AMFs within the AMF set.

When the "SliceAwareANDSP" feature is supported, and the AMF receives the "andspDelInd" attribute with the outcome of the UE configuration with slice aware ANDSP/WLANSP, the AMF proceeds as described in clause 4.2.2.1.

When the "URSPEnforcement" feature is supported and the AMF receives the "matchPdus" attribute, the AMF shall update the affected established PDU session(s), by forwarding the received PCF for the UE callback information for the PDU session(s) matching the new S-NSSAI and DNN combination(s) to the SMF, and removing the previously provided PCF for the UE information for the PDU session(s) matching the removed S-NSSAI and DNN combination(s) from the SMF as defined in 3GPP TS 29.502 [40]. When the AMF receives the "pcfUeInfo" attribute with updated SBA binding indication, the AMF shall apply the updated PCF for the UE callback information to the new PDU sessions only, i.e., already established PDU sessions are not affected.

# *** Next Change ***

#### 4.2.4.5	UE policy provisioning for V2X communication over PC5 and Uu reference points

After the UE policy association establishment and if the "V2X" feature is supported, the (H-)PCF may receive the service specific parameter information notified by the UDR for the change of UE's Application Data as defined in clause 6.3.4 of 3GPP TS 29.519 [17]. In this case:

-	for the roaming case, the H-PCF shall derive the V2XP and provision it to the V-PCF as defined in clause 4.2.4.2; and/or

-	for the roaming and non-roaming case, the (H-)PCF shall derive the V2XP and the (V-)PCF shall use the Namf_Communication Service defined in 3GPP TS 29.518 [14] to send "MANAGE UE POLICY COMMAND" message(s) with the V2XP to the UE via the AMF.

# *** Next Change ***

#### 4.2.4.8	UE policy provisioning for A2X communication over PC5 and A2X communication over Uu reference point

After the UE policy association establishment and if the "A2X" feature is supported, the (H-)PCF may receive the service specific parameter information notified by the UDR for the change of UE's Application Data as defined in clause 6.3.4 of 3GPP TS 29.519 [17]. In this case:

-	for the roaming case, the H-PCF shall derive the A2XP and provision it to the V-PCF as defined in clause 4.2.4.2; and/or

-	for the roaming and non-roaming case, the (H-)PCF shall derive the A2XP and the (V-)PCF shall use the Namf_Communication Service defined in 3GPP TS 29.518 [14] to send "MANAGE UE POLICY COMMAND" message(s) with the A2XP to the UE via the AMF.

# *** Next Change ***

### 5.6.1	General

This clause specifies the application data model supported by the API.

Table 5.6.1-1 specifies the data types defined for the Npcf_UEPolicyControl service based interface protocol.

Table 5.6.1-1: Npcf_UEPolicyControl specific Data Types

| Data type |  | Section defined |  | Description |  |  | Applicability |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A2xCapability |  | 5.6.3.12 |  |  | Indicates the A2X capabilities |  |  | A2X |  |  |
| LboRoamingInformation |  | 5.6.2.10 |  | LBO roaming information for a DNN and S-NSSAI |  |  | VPLMNSpecificURSP |  |  |
| N1N2MessTransferErrorReply |  | 5.6.3.8 |  |  | Error the V-PCF may send to the H-PCF when the V-PCF receives from the AMF an error to the N1N2MessageTransfer request. |  |  | EnErrorHandling |  |  |
| Non3gppAccess |  | 5.6.3.7 |  | Represents a Non-3gpp access node. |  |  | SliceAwareANDSP |  |  |
| Pc5Capability |  | 5.6.3.5 |  | Indicates the specific PC5 RAT(s) which the UE supports for V2X communicationsover PC5 reference point. |  |  | V2X |  |  |
| ProSeCapability |  | 5.6.3.6 |  | Indicates the 5G ProSe capabilities. |  |  | ProSe |  |  |
| PolicyAssociation |  | 5.6.2.2 |  | Description of a policy association that is returned by the PCF when a policy Association is created, or read. |  |  |  |  |  |
| PolicyAssociationReleaseCause |  | 5.6.3.4 |  | The cause why the PCF requests the termination of the policy association. |  |  |  |  |  |
| PolicyAssociationRequest |  | 5.6.2.3 |  | Information that NF service consumer provides when requesting the creation of a policy association. |  |  |  |  |  |
| PolicyAssociationUpdateRequest |  | 5.6.2.4 |  | Information that NF service consumer provides when requesting the update of a policy association. |  |  |  |  |  |
| PolicyStatus |  | 5.6.3.10 |  |  | Represents the configuration status of a UE Policy in the UE. |  |  | SliceAwareANDSP |  |  |
| PolicyUpdate |  | 5.6.2.5 |  | Updated policies that the PCF provides in a notification or in the reply to an Update Request. |  |  |  |  |  |
| RangSLCapability |  | 5.6.3.10 |  |  | Represents the Ranging/SL capabilities. |  |  | Ranging_SL |  |  |
| RequestTrigger |  | 5.6.3.3 |  | Enumeration of possible Request Triggers. |  |  |  |  |  |
| SliceSpecificN3gNodeSelectionCapability |  | 5.6.3.13 |  |  | Represents the UE capabilities with regard to slice-specific non-3gpp node selection. |  |  | SliceAwareANDSP |  |  |
| TerminationNotification |  | 5.6.2.6 |  | Request to terminate a policy Association that the PCF provides in a notification. |  |  |  |  |  |
| UePolicy |  | 5.6.3.2 |  | UE Policies |  |  |  |  |  |
| UePolicyDeliveryResult |  | 5.6.3.2 |  | UE Policy delivery Result |  |  |  |  |  |
| UePolicyNotification |  | 5.6.2.12 |  |  | Contains the delivery outcome of VPLMN-Specific URSP rules |  |  | VPLMNSpecificURSP |  |  |
| UePolicyParameters |  | 5.6.2.9 |  | Contains the service parameters used to guide the VPLMN-specific URSP rule determination. |  |  | VPLMNSpecificURSP |  |  |
| UePolicyRequest |  | 5.6.3.2 |  | Request for UE Policies |  |  |  |  |  |
| UePolicyTransferFailureCause |  | 5.6.4.1 |  |  | UE Policy Transfer Failure Cause |  |  | EnErrorHandling |  |  |
| UePolicyTransferFailureNotification |  | 5.6.2.7 |  | Information that the UE policy is failure to be transferred to the UE. |  |  |  |  |  |
| UrspEnforcementPduSession |  | 5.6.2.11 |  | Represents URSP rule enforcement information for a PDU session. |  |  | URSPEnforcement |  |  |
| UeRequestedValueRep |  | 5.6.2.8 |  | Contains the current applicable values corresponding to the policy control request triggers. |  |  | ImmediateReport |  |  |
| V2xCapability |  | 5.6.3.14 |  | Indicates the V2X capability is supported or not. |  |  | V2X_Ext |  |  |

Table 5.6.1-2 specifies data types re-used by the Npcf_UEPolicyControl service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Npcf_UEPolicyControl service based interface.

Table 5.6.1-2: Npcf_UEPolicyControl re-used Data Types

| Data type |  | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AccessType |  | 3GPP TS 29.571 [11] |  | Represents an Access Type. |  |  |  |
| Bytes |  | 3GPP TS 29.571 [11] |  | String with format "byte". |  |  |  |
| ChargingInformation |  | 3GPP TS 29.512 [31] |  | Contains the charging information. |  | SLAMUP, CHFGroup |  |
| ConfiguredSnssai |  | 3GPP TS 29.531 [34] |  | Contains the configured S-NSSAI and optionally mapped home S-NSSA. |  | SliceAwareANDSP, NssaiChange |  |
| CmState |  | 3GPP TS 29.518 [14] |  | Connectivity state of UE |  | ConnectivityStateChange |  |
| Dnn |  | 3GPP TS 29.571 [11] |  | Represents a DNN. |  | VPLMNSpecificURSP, URSPEnforcement |  |
| Event |  | 3GPP TS 29.522 [41] |  | Subscription to notification about delivery of VPLMN specific URSP rule. |  | VPLMNSpecificURSP |  |
| Fqdn |  | 3GPP TS 29.571 [11] |  | FQDN |  |  |  |
| Gpsi |  | 3GPP TS 29.571 [11] |  | Generic Public Subscription Identifier |  |  |  |
| GroupId |  | 3GPP TS 29.571 [11] |  | Represents a UE Group Identifier |  |  |  |
| Guami |  | 3GPP TS 29.571 [11] |  | Globally Unique AMF Identifier |  |  |  |
| Ipv4Addr |  | 3GPP TS 29.571 [11] |  | Represents an Ipv4 address. |  |  |  |
| Ipv6Addr |  | 3GPP TS 29.571 [11] |  | Represents an Ipv6 address. |  |  |  |
| N1N2MessageTransferCause |  | 3GPP TS 29.518 [14] |  | Contains an error cause for an N1 or N2 message transfer. |  |  |  |
| N2InfoContent |  | 3GPP TS 29.518 [14] |  | Represents a transparent N2 information content to be relayed by AMF. |  | V2X, A2X, ProSe, Ranging_SL |  |
| NfInstanceId |  | 3GPP TS 29.571 [11] |  | Represents an NF instance identifier |  |  |  |
| NfSetId |  | 3GPP TS 29.571 [11] |  | Represents an NF set identifier |  | EnhEstRoaming |  |  |
| PcEventNotification |  | 3GPP TS 29.523 [30] |  | Represents notification about UE Policy Delivery outcome |  | VPLMNSpecificURSP |  |
| PcfUeCallbackInfo |  | 3GPP TS 29.571 [11] |  | Contains the PCF for the UE callback information necessary for the PCF for the PDU session to send Establishment and Termination event. |  | URSPEnforcement |  |
| PduSessionInfo |  | 3GPP TS 29.571 [11] |  | Contains a DNN and SNSSAI combination |  | VPLMNSpecificURSP URSPEnforcement |  |
| PduSessionType |  | 3GPP TS 29.571 [11] |  | Contains the PDU Session Type |  | URSPEnforcement |  |
| Pei |  | 3GPP TS 29.571 [11] |  | Permanent Equipment Identifier |  |  |  |
| PlmnId |  | 3GPP TS 29.571 [11] |  | Represents a PLMN identifier. |  |  |  |
| PlmnIdNid |  | 3GPP TS 29.571 [11] |  | Identifies the network: PLMN Identifier or the SNPN Identifier (the PLMN Identifier and the NID). |  |  |  |
| PresenceInfo |  | 3GPP TS 29.571 [11] |  | Presence reporting area information |  |  |  |
| PresenceInfoRm |  | 3GPP TS 29.571 [11] |  | This data type is defined in the same way as the "PresenceInfo" data type, but with the OpenAPI "nullable: true" property. |  | PresenceInfo |  |  |  |
| ProblemDetails |  | 3GPP TS 29.571 [11] |  | Contains detailed information about an error response. |  |  |  |
| RatType |  | 3GPP TS 29.571 [11] |  | Represents a RAT type. |  |  |  |
| RedirectResponse |  | 3GPP TS 29.571 [11] |  | Contains redirection related information. |  | ES3XX |  |
| ServiceName |  | 3GPP TS 29.510 [13] |  | Name of the service instance. |  |  |  |
| SatelliteBackhaulCategory |  | 3GPP TS 29.571 [11] |  | Indicates the satellite backhaul category or non-satellite backhaul. |  | EnSatBackhaulCategoryChg |  |
| Snssai |  | 3GPP TS 29.571 [11] |  | Represents an S-NSSAI |  | SliceAwareANDSP |  |
| SscMode |  | 3GPP TS 29.571 [11] |  | Service and session continuity mode. |  | URSPEnforcement |  |
| Supi |  | 3GPP TS 29.571 [11] |  | Subscription Permanent Identifier |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [11] |  | Used to negotiate the applicability of the optional features defined in table 5.8-1. |  |  |  |
| TimeZone |  | 3GPP TS 29.571 [11] |  | Represents a time zone. |  |  |  |
| Uinteger |  | 3GPP TS 29.571 [11] |  | Unsigned integer. |  |  |  |
| Uri |  | 3GPP TS 29.571 [11] |  | Represents a URI. |  |  |  |
| UrspEnforcementInfo |  | 3GPP TS 29.512 [31] |  | URSP rule enforcement information as received from the UE. |  | URSPEnforcement |  |
| UrspRuleRequest |  | 3GPP TS 29.522 [41] |  | URSP rule guidance information |  | VPLMNSpecificURSP |  |
| UserLocation |  | 3GPP TS 29.571 [11] |  | Contains User Location information. |  |  |  |

# *** Next Change ***

#### 5.6.2.3	Type PolicyAssociationRequest

Table 5.6.2.3-1: Definition of type PolicyAssociationRequest

| Attribute name |  | Data type |  | P |  | Cardinality | Description |  |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| notificationUri |  | Uri |  | M |  | 1 | Identifies the recipient of Notifications sent by the PCF. |  |  |  |  |
| altNotifIpv4Addrs |  | array(Ipv4Addr) |  | O |  | 1..N | Alternate or backup IPv4 Addess(es) where to send Notifications. |  |  |  |  |
| altNotifIpv6Addrs |  | array(Ipv6Addr) |  | O |  | 1..N | Alternate or backup IPv6 Addess(es) where to send Notifications. |  |  |  |  |
| altNotifFqdns |  | array(Fqdn) |  | O |  | 1..N | Alternate or backup FQDN(s) where to send Notifications. |  |  |  |  |
| supi |  | Supi |  | M |  | 1 | Subscription Permanent Identifier. |  |  |  |  |
| gpsi |  | Gpsi |  | C |  | 0..1 | Generic Public Subscription Identifier. Shall be provided when available. |  |  |  |  |
| accessType |  | AccessType |  | C |  | 0..1 | The Access Type where the served UE is camping. Shall be provided when available. |  |  |  |  |
| accessTypes |  | array(AccessType) |  | C |  | 1..N | The Access Type(s) where the served UE is camping. Shall be provided when available. |  |  | AccessChange |  |
| pei |  | Pei |  | C |  | 0..1 | The Permanent Equipment Identifier of the served UE. Shall be provided when available. |  |  |  |  |
| userLoc |  | UserLocation |  | C |  | 0..1 | The location of the served UE. Shall be provided when available. |  |  |  |  |
| timeZone |  | TimeZone |  | C |  | 0..1 | The time zone of the network where the served UE is camping. Shall be provided when available. |  |  |  |  |
| servingPlmn |  | PlmnIdNid |  | C |  | 0..1 | The serving network (a PLMN or an SNPN) where the served UE is camping. For the SNPN the NID together with the PLMN ID identifies the SNPN. For Indirect Network Sharing and the interaction between the hosting operator network and the participating operator network case, the identifier of the serving network is set to the selected PLMN ID. Shall be provided when available. |  |  |  |  |
| ratType |  | RatType |  | C |  | 0..1 | The RAT Type where the served UE is camping. Shall be provided when available. |  |  |  |  |
| ratTypes |  | array(RatType) |  | C |  | 1..N | The RAT Type(s) where the served UE is camping. Shall be provided when available. |  |  | AccessChange |  |
| groupIds |  | array(GroupId) |  | C |  | 1..N | Internal Group Identifier(s) of the served UE. Shall be provided when available. |  |  |  |  |
| hPcfId |  | NfInstanceId |  | C |  | 0..1 | H-PCF Identifier. Shall be provided by the AMF in roaming scenarios when available. |  |  |  |  |
| hPcfUri | Uri | C | 0..1 |  | H-PCF URI. It shall be provided by the AMF in roaming scenarios, if available. When present, it shall contain the API URI of the Npcf_UEPolicyControl service of the H-PCF ID indicated in the "hPcfId" attribute. The API URI shall take the form specified in clause 5.1. | EnhEstRoaming |  |
| hPcfSetId | NfSetId | C | 0..1 |  | H-PCF Set Identifier of the H-PCF instance indicated in the "hPcfId" attribute. It shall be provided by the AMF in roaming scenarios, if available. | EnhEstRoaming |  |
| uePolReq |  | UePolicyRequest |  | C |  | 0..1 | A request for UE Policies. Shall be provided when the AMF receives an "UE STATE INDICATION" message, as defined in Annex D.5.4 of 3GPP TS 24.501 [15]. |  |  |  |  |
| guami |  | Guami |  | C |  | 0..1 | The Globally Unique AMF Identifier (GUAMI) shall be provided by an AMF as NF service consumer. |  |  |  |  |
| serviceName |  | ServiceName |  | O |  | 0..1 | If the NF service consumer is an AMF, it should provide the name of a service produced by the AMF that makes use of information received within the Npcf_UEPolicyControl_UpdateNotify service operation. |  |  |  |  |
| servingNfId |  | NfInstanceId |  | C |  | 0..1 | If the NF service consumer is an AMF, it shall contain the identifier of the serving AMF. |  |  |  |  |
| pc5Capab |  | Pc5Capability |  | C |  | 0..1 | Indicates the PC5 Capability for V2X communications supported by the UE. It shall be provided when available at the NF service consumer. |  |  | V2X |  |
| v2xCapab |  | V2xCapability |  | C |  | 0..1 | Indicates whether the V2X capability for V2X communications is supported by the UE. It shall be provided when available at the NF service consumer. |  |  | V2X_Ext |  |
| a2xCapab |  | array(A2xCapability) |  | C |  | 1..N | Indicates the A2X Capabilities for A2X communications supported by the UE. It shall be provided when available at the NF service consumer. |  |  | A2X |  |
| proSeCapab |  | array(ProSeCapability) |  | C |  | 1..N | Indicates whether the UE is capable of one or more of the the 5G ProSe Capabilities. This attribute shall be provided when available at the NF service consumer. |  |  | ProSe |  |
| confSnssais |  | array(ConfiguredSnssai) |  | C |  | 1..N | The Configured NSSAI for the serving PLMN, and optionally the mapped S-NSSAI value of home network corresponding to the configured S-NSSAI in the serving PLMN. When the feature SliceAwareANDSP is supported, it shall be provided in the roaming case when available at the NF service consumer and the "n3gNodeReSel" attribute is present. If the feature NssaiChange is supported, it shall be provided in the roaming case. (NOTE 1) |  |  | SliceAwareANDSP, NssaiChange |  |
| n3gNodeReSel |  | Non3gppAccess |  | C |  | 0..1 | A wrongly selected non-3gpp access node. It shall be provided when the UE has selected a non-3gpp access node that is not compatible with the Allowed NSSAI. |  |  | SliceAwareANDSP |  |
| sliceN3gNodeSelCap | SliceSpecificN3gNodeSelectionCapability | O | 0..1 |  | Indicates whether the UE supports N3IWF/TNGF selection based on the slices the UE wishes to use over untrusted/trusted non-3GPP access. | SliceAwareANDSP |  |
| satBackhaulCategory |  | SatelliteBackhaulCategory |  | O |  | 0..1 | Indicates types of the satellite backhaul based on satellite types (when satellite backhaul is used) or non-satellite backhaul (when satellite backhaul is not used). The default value "NON_SATELLITE" shall apply if the attribute is not present. |  |  | EnSatBackhaulCategoryChg |  |
| vpsUePolGuidance |  | map(UePolicyParameters) |  | O |  | 1..N | Contains the service parameter used to guide the VPLMN-specific URSP and may contain the subscription to VPLMN-specific URSP delivery outcome. The key of the map represents the AF request to guide the VPLMN-specific URSP rules. This attribute only applies in roaming and when the V-PCF is the NF service consumer. |  |  | VPLMNSpecificURSP |  |
| lboRoamInfo |  | array(LboRoamingInformation) |  | O |  | 1..N | Contains LBO roaming information for DNN and S-NSSAI combination(s). This attribute only applies in roaming and when the AMF is the NF service consumer. |  |  | VPLMNSpecificURSP |  |
| 5gsToEpsMob |  | boolean |  | O |  | 0..1 | When it is set to true, it indicates the UE Policy Association creation is triggered by a 5GS to EPS mobility scenario. Default value is false. |  |  | EpsUrsp |  |
| suppFeat |  | SupportedFeatures |  | M |  | 1 | Indicates the features supported by the service consumer. |  |  |  |  |
| rangSlCapab |  | array(RangSLCapability) |  | C |  | 1..N | Contains the Ranging/SL related UE capabilities. It shall be provided when available at the NF service consumer. |  |  | Ranging_SL |  |
| NOTE 1: The "mappedHomeSnssai" attribute within the ConfiguredSnssai data type may only be provided if the "NssaiChange" feature is supported. |  |  |  |  |  |  |  |  |  |  |  |

# *** Next Change ***

#### 5.6.2.4	Type PolicyAssociationUpdateRequest

Table 5.6.2.4-1: Definition of type PolicyAssociationUpdateRequest

| Attribute name |  | Data type |  |  | P |  |  | Cardinality |  | Description |  |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| notificationUri |  | Uri |  |  | O |  |  | 0..1 |  | Identifies the recipient of Notifications sent by the PCF. |  |  |  |  |
| altNotifIpv4Addrs |  | array(Ipv4Addr) |  |  | O |  |  | 1..N |  | Alternate or backup IPv4 Address(es) where to send Notifications. |  |  |  |  |
| altNotifIpv6Addrs |  | array(Ipv6Addr) |  |  | O |  |  | 1..N |  | Alternate or backup IPv6 Address(es) where to send Notifications. |  |  |  |  |
| altNotifFqdns |  | array(Fqdn) |  |  | O |  |  | 1..N |  | Alternate or backup FQDN(s) where to send Notifications. |  |  |  |  |
| triggers |  | array(RequestTrigger) |  |  | C |  |  | 1..N |  | Request Triggers that the NF service consumer observes. |  |  |  |  |
| praStatuses |  | map(PresenceInfo) |  |  | C |  |  | 1..N |  | If the Trigger "PRA_CH" is reported, the UE presence status for tracking area for which changes of the UE presence occurred shall be provided. The "praId" attribute within the PresenceInfo data type shall also be the key of the map. The "presenceState" attribute within the PresenceInfo data type shall be supplied. The "additionalPraId" attribute within the PresenceInfo data type shall not be supplied. The "praId" attribute within the PresenceInfo data type shall include the identifier of an individual presence reporting area. |  |  |  |  |
| userLoc |  | UserLocation |  |  | C |  |  | 0..1 |  | The location of the served UE shall be provided for trigger "LOC_CH". |  |  |  |  |
| uePolDelResult |  | UePolicyDeliveryResult |  |  | C |  |  | 0..1 |  | UE Policy Delivery Result. Shall be provided together with trigger "UE_POLICY" when a "MANAGE UE POLICY COMPLETE" message or a "MANAGE UE POLICY COMMAND REJECT" message, as defined in Annex D.5 of 3GPP TS 24.501 [15], has been received by the V-PCF and is being forwarded to the H-PCF. |  |  |  |  |
| uePolTransFailNotif |  | UePolicyTransferFailureNotification |  |  | C |  |  | 0..1 |  | The UE policy transfer failure notification. Shall be the provided together with trigger "UE_POLICY" when a response with HTTP status code 4xx or 5xx as defined in clause 5.2.2.3.1.2 of 3GPP TS 29.518 [14] or a N1N2 Transfer Failure Notification as defined in clause 5.2.2.3.2 of 3GPP TS 29.518 [14] is received after the V-PCF provisioned the UE policy by invoking the Namf_Communication_N1N2MessageTransfer service operation to the AMF and is notifying the H-PCF. |  |  |  |  |
| uePolReq |  | UePolicyRequest |  |  | C |  |  | 0..1 |  | A request for UE Policies. Shall be provided together with trigger "UE_POLICY" when the V-PCF receives an "UE POLICY PROVISIONING REQUEST" message, as defined in clause 7.2.1.1 of 3GPP TS 24.587 [24], if the "V2X" feature is supported, and/or when the V-PCF receives an "UE POLICY PROVISIONING REQUEST" message for 5G ProSe, as defined in clause 10.4.1 of 3GPP TS 24.554 [28], if the "ProSe" feature is supported and/or when the V-PCF receives an "UE POLICY PROVISIONING REQUEST" message for A2X, as defined 3GPP TS 24.577 [32], if the "A2X" feature is supported and/or when the V-PCF receives an "UE POLICY PROVISIONING REQUEST" message for Ranging/SL, as defined 3GPP TS 24.514 [42], if the "Ranging_SL" feature is supported.. |  |  | V2X, A2X, ProSe, Ranging_SL |  |
| guami |  | Guami |  |  | C |  |  | 0..1 |  | The Globally Unique AMF Identifier (GUAMI) shall be provided by an AMF as NF service consumer during the AMF relocation. |  |  |  |  |
| servingNfId |  | NfInstanceId |  |  | C |  |  | 0..1 |  | It shall contain the identifier of the new AMF during the AMF relocation. |  |  |  |  |
| plmnId |  | PlmnIdNid |  |  | C |  |  | 0..1 |  | The serving network identity (a PLMN or an SNPN) of the served UE shall be provided for trigger "PLMN_CH". |  |  | PlmnChange |  |
| connectState |  | CmState |  |  | C |  |  | 0..1 |  | The connectivity state of the served UE shall be provided for trigger "CON_STATE_CH". |  |  | ConnectivityStateChange |  |
| groupIds |  | array(GroupId) |  |  | C |  |  | 1..N |  | Internal Group Identifier(s) of the served UE. Shall be provided for trigger "GROUP_ID_LIST_CHG". |  |  | GroupIdListChange |  |
| pc5Capab |  | Pc5Capability |  |  | C |  |  | 0..1 |  | Indicates the PC5 Capability for V2X communications supported by the UE. It shall be provided when available at the NF service consumer. It shall be included by the target AMF only in inter-AMF mobility scenarios and for trigger "FEAT_RENEG". It requires that the "V2X" feature is supported. |  |  | FeatureRenegotiation |  |
| v2xCapab |  | V2xCapability |  |  | C |  |  | 0..1 |  | Indicates whether the V2X Capability for V2X communications is supported by the UE. It shall be provided when available at the NF service consumer. It shall be included by the target AMF only in inter-AMF mobility scenarios and for trigger "FEAT_RENEG". It requires that the "V2X_Ext" feature is supported. |  |  | FeatureRenegotiation |  |
| a2xCapab |  | array(A2xCapability) |  |  | C |  |  | 1..N |  | Indicates the A2X capabilities supported by the UE. It shall be provided when available at the NF service consumer. It shall be included by the target AMF only in inter-AMF mobility scenarios and for trigger "FEAT_RENEG". It requires that the "A2X" feature is supported. |  |  | FeatureRenegotiation |  |
| proSeCapab |  | array(ProSeCapability) |  |  | O |  |  | 1..N |  | Indicates whether the UE is capable of one or more of the 5G ProSe Capabilities. |  |  | ProSe |  |
| confSnssais |  | array(ConfiguredSnssai) |  |  | C |  |  | 1..N |  | The Configured NSSAI for the serving PLMN, and optionally the mapped S-NSSAI value of home network corresponding to the configured S-NSSAI in the serving PLMN. It shall be provided in case of roaming for trigger "CONF_NSSAI_CH" or for trigger "NON_3GPP_NODE_RESELECTION". (NOTE) |  |  | SliceAwareANDSP, NssaiChange |  |
| n3gNodeReSel |  | Non3gppAccess |  |  | C |  |  | 0..1 |  | A wrongly selected non-3gpp access node. It shall be provided when available at the NF service consumer and the "NON_3GPP_NODE_RESELECTION" trigger is reported within the "triggers" attribute. |  |  | SliceAwareANDSP |  |
| sliceN3gNodeSelCap |  | SliceSpecificN3gNodeSelectionCapability |  |  | O |  |  | 0..1 |  | Indicates whether the UE supports N3IWF/TNGF selection based on the slices the UE wishes to use over untrusted/trusted non-3GPP access. It may be included by the target AMF only in inter-AMF mobility scenarios and for trigger "FEAT_RENEG". It requires that the "SliceAwareANDSP" feature is supported. |  |  | FeatureRenegotiation |  |
| satBackhaulCategory |  | SatelliteBackhaulCategory |  |  | C |  |  | 0..1 |  | Indicates types of the satellite backhaul based on satellite types (when satellite backhaul is used) or non-satellite backhaul (when satellite backhaul is not used). It shall be provided for trigger "SAT_CATEGORY_CHG". |  |  | EnSatBackhaulCategoryChg |  |
| urspEnfRep |  | map(UrspEnforcementPduSession) |  |  | C |  |  | 1..N |  | Represents information about the enforced URSP rule(s) in one or more PDU sessions for the affected UE. The key of the map is a character string that represents an integer value (it may correspond with a PDU session identifier). It shall be present when the notified policy control request trigger is "URSP_ENF_INFO". |  |  | URSPEnforcement |  |
| vpsUePolGuidance |  | map(UePolicyParameters) |  |  | O |  |  | 1..N |  | Contains the service parameter used to guide the VPLMN-specific URSP rule determination and may contain the subscription to VPLMN-specific URSP delivery outcome. The key of the map represents the AF request to guide VPLMN-specific URSP rules. This attribute only applies in roaming and when the V-PCF is the NF service consumer. |  |  | VPLMNSpecificURSP |  |
| lboRoamInfo |  | array(LboRoamingInformation) |  |  | O |  |  | 1..N |  | Contains LBO roaming information for a DNN and S-NSSAI combination(s). This attribute only applies in roaming and when the AMF is the NF service consumer. |  |  | VPLMNSpecificURSP |  |
| accessTypes |  | array(AccessType) |  |  | C |  |  | 1..N |  | The Access Type(s) where the served UE is camping. It shall be provided for trigger "ACCESS_TYPE_CH" when the access type(s) changes or when the access type(s) is initially reported as consequence of the provisioning of the trigger. |  |  | AccessChange |  |
| ratTypes |  | array(RatType) |  |  | C |  |  | 1..N |  | The RAT Type(s), if available, for the reported "accessTypes" where the served UE is camping. It shall be provided, if available, for trigger "ACCESS_TYPE_CH" when the access type(s) changes or when the access type(s) is initially reported as consequence of the provisioning of the trigger. |  |  | AccessChange |  |
| suppFeat |  | SupportedFeatures |  |  | C |  |  | 0..1 |  | Indicates the features supported by the NF service consumer. It shall be included by the target AMF in inter-AMF mobility scenarios for trigger "FEAT_RENEG". |  |  | FeatureRenegotiation |  |
| rangSlCapab |  | array(RangSLCapability) |  |  | O |  |  | 1..N |  | Contains the Ranging/SL related UE capabilities. |  |  | Ranging_SL |  |
| NOTE: The "mappedHomeSnssai" attribute within the ConfiguredSnssai data type may only be provided if the "NssaiChange" feature is supported. |  |  |  |  |  |  |  |  |

# *** Next Change ***

#### 5.6.3.5	Enumeration: Pc5Capability

The enumeration Pc5Capability indicates the specific PC5 RAT(s) which the UE supports for V2X communication over PC5 reference point. It shall comply with the provisions defined in table 5.6.3.5-1.

Table 5.6.3.5-1: Enumeration Pc5Capability

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| LTE_PC5 | This value is used to indicate that the UE supports PC5 LTE RAT for V2X communication over PC5 reference point. |  |
| NR_PC5 | This value is used to indicate that the UE supports PC5 NR RAT for V2X communication over PC5 reference point. |  |
| LTE_NR_PC5 | This value is used to indicate that the UE supports both PC5 LTE and NR RAT for V2X communication over PC5 reference point. |  |

# *** Next Change ***

#### 5.6.3.14	Enumeration: V2xCapability

The enumeration V2XCapability indicates the V2X capabilities the UE supports for V2X communication. It shall comply with the provisions defined in table 5.6.3.14-1.

Table 5.6.3.14-1: Enumeration V2xCapability

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SUPPORTED | This value is used to indicate that the UE supports V2X communication over PC5 and/or Uu reference point. |  |
| NOT_SUPPORTED | This value is used to indicate that the UE does not support V2X communication over PC5 and/or Uu reference point. |  |

# *** Next Change ***

## 5.8	Feature negotiation

The optional features in table 5.8-1 are defined for the Npcf_UEPolicyControl API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [5].

Table 5.8-1: Supported Features

| Feature number |  |  | Feature Name |  |  |  | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | PendingTransaction |  |  |  | This feature indicates support for the race condition handling as defined in 3GPP TS 29.513 [7]. |  |  |  |  |
| 2 |  |  | PlmnChange |  |  |  | This feature indicates support for the change of PLMN trigger handling. |  |  |  |  |
| 3 |  |  | ConnectivityStateChange |  |  |  | This feature indicates support for the UE connectivity state change trigger handling. |  |  |  |  |
| 4 |  |  |  | V2X |  |  |  | This feature indicates support for the UE policy provisioning and N2 information provisioning for V2X communications. |  |  |  |
| 5 |  |  |  | GroupIdListChange |  |  |  | This feature indicates the support for the notification of changes in the list of internal group identifiers. |  |  |  |
| 6 |  |  |  | ImmediateReport |  |  |  | This feature indicates the support of the current applicable values report corresponding to the policy control request triggers for policy update notification. |  |  |  |
| 7 |  |  |  | ErrorResponse |  |  |  | This feature indicates support for "404 Not Found" error response code for policy update notification between AMF and (V-)PCF. |  |  |  |
| 8 |  |  |  | ES3XX |  |  |  | Extended Support for 3xx redirections. This feature indicates the support of redirection for any service operation, according to Stateless NF procedures as specified in clauses 6.5.3.2 and 6.5.3.3 of 3GPP TS 29.500 [5] and according to HTTP redirection principles for indirect communication, as specified in clause 6.10.9 of 3GPP TS 29.500 [5]. |  |  |  |
| 9 |  |  |  | ProSe |  |  |  | This feature indicates support of UE policy and N2 information provisioning for 5G ProSe. |  |  |  |
| 10 |  |  |  | FeatureRenegotiation |  |  |  | This feature indicates the support of feature renegotiation during the update of a policy association triggered by UE mobility with AMF change. |  |  |  |
| 11 |  |  |  | SliceAwareANDSP |  |  |  | This feature indicates the support of ANDSP/WLANSP policies that consider the slices supported by the UE. |  |  |  |
| 12 |  |  |  | EpsUrsp |  |  |  | This feature indicates support of URSP provisioning in EPS and is only applicable in the case of of 5GC and EPC interworking. |  |  |  |
| 13 |  |  |  | EnSatBackhaulCategoryChg |  |  |  | This feature indicates the support of notification of a change between different satellite backhaul categories, or dynamic satellite backhaul categories, or between satellite backhaul and non-satellite backhaul. |  |  |  |
| 14 |  |  |  | UECapabilityIndication |  |  |  | This feature indicates the support of the provisioning by the H-PCF to the V-PCF of the UE Capability for UE Policy, when the UE Capability is not received from the UE and the information is available and reliable in the UDR. |  |  |  |
| 15 |  |  |  | A2X |  |  |  | This feature indicates support of A2X communications. |  |  |  |
| 16 |  |  |  | NssaiChange |  |  |  | This feature indicates support for the change of Configured NSSAI trigger handling. |  |  |  |
| 17 |  |  |  | ProSe_Ph2 |  |  |  | This feature indicates the support of UE policy and N2 information provisioning for 5G ProSe UE-to-UE Relay function. This feature requires that the "ProSe" feature is also supported. |  |  |  |
| 18 |  |  |  | PresenceInfo |  |  |  | The feature indicates the support of policy update to remove the existing presence reporting areas entry. |  |  |  |
| 19 |  |  |  | URSPEnforcement |  |  |  | This feature indicates the support of the report of URSP rule enforcement information by the V-PCF to the H-PCF. |  |  |  |
| 20 |  |  |  | VPLMNSpecificURSP |  |  |  | This feature indicates the support of AF guidance on VPLMN-specific URSP rules. It requires the support of NssaiChange feature. |  |  |  |
| 21 |  |  |  | Ranging_SL |  |  |  | This feature indicates the support of the ranging and sidelink positioning functionality. The following functionalities are supported: - Support for the UE policy provisioning and N2 information provisioning for Ranging and sidelink positioning. |  |  |  |
| 22 |  |  |  | AccessChange |  |  |  | This feature indicates the support of the reporting of an access type and RAT type changes, the addition of an access type and RAT type or the removal of an existing access type and RAT type. |  |  |  |
| 23 |  |  |  | EnErrorHandling |  |  |  | This feature indicates the support of the indication from the V-PCF to the H-PCF of the received AMF error response to the UE Policy Delivery transfer request. |  |  |  |
| 24 |  |  |  | SLAMUP |  |  |  | This feature indicates the support of the provisioning to the AMF of the CHF information of the CHF selected by the PCF for UE policy. |  |  |  |
| 25 |  |  |  | EnhEstRoaming |  |  |  | The feature enhanced UE Policy Establishment procedure in roaming scenarios indicates the provisioning of the AMF to the V-PCF of the H-PCF Set Id and H-PCF URI of the selected H-PCF ID. |  |  |  |
| 26 |  |  |  | ProSe_Ph3 |  |  |  | This feature indicates the support of the second set of enhancements to the 5G ProSe functionality. The following sub-functionalities are supported: - Support UE policy and N2 information provisioning for Multi-hop UE-to-Network Relay. - Support UE policy and N2 information provisioning for Layer-3 Multi-hop UE-to-UE Relay. This feature requires that the "ProSe_Ph2" feature is also supported. |  |  |  |
| 27 |  |  |  | CHFGroup |  |  |  | This feature indicates the support of the CHF Group ID handling for the discovery of the CHF. |  |  |  |
| 28 |  |  |  | ExtDeliveryOutcome |  |  |  | Indicates the support of notifications about the unsuccessfully executed UE policy outcome together with the partially unsuccessful UE policy delivery event related to the invocation of AF provisioned service parameters. This feature requires the support of VPLMNSpecificURSP feature. |  |  |  |
| 29 |  |  |  | V2X_Ext |  |  |  | This feature indicates support of V2X capability. |  |  |  |

# *** Next Change ***

# A.2	Npcf_UEPolicyControl API

openapi: 3.0.0

info:

version: 1.4.0-alpha.4

title: Npcf_UEPolicyControl

description: |

UE Policy Control Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.525 V19.4.0; 5G System; UE Policy Control Service.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.525/'

servers:

- url: '{apiRoot}/npcf-ue-policy-control/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- npcf-ue-policy-control

paths:

/policies:

post:

operationId: CreateIndividualUEPolicyAssociation

summary: Create individual UE policy association.

tags:

- UE Policy Associations (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyAssociationRequest'

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyAssociation'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/npcf-ue-policy-control/v1/policies/{polAssoId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

policyUpdateNotification:

'{$request.body#/notificationUri}/update':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyUpdate'

responses:

'200':

description: >

OK. The current applicable values corresponding to the policy control request

trigger is reported

content:

application/json:

schema:

$ref: '#/components/schemas/UeRequestedValueRep'

'204':

description: No Content, Notification was successful

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

policyAssocitionTerminationRequestNotification:

'{$request.body#/notificationUri}/terminate':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TerminationNotification'

responses:

'204':

description: No Content, Notification was successful

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policies/{polAssoId}:

get:

operationId: ReadIndividualUEPolicyAssociation

summary: Read individual UE policy association.

tags:

- Individual UE Policy Association (Document)

parameters:

- name: polAssoId

in: path

description: Identifier of a policy association

required: true

schema:

type: string

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyAssociation'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

operationId: DeleteIndividualUEPolicyAssociation

summary: Delete individual UE policy association.

tags:

- Individual UE Policy Association (Document)

parameters:

- name: polAssoId

in: path

description: Identifier of a policy association

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was successfully deleted

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policies/{polAssoId}/update:

post:

operationId: ReportObservedEventTriggersForIndividualUEPolicyAssociation

summary: >

Report observed event triggers and possibly obtain updated policies for an individual UE

policy association.

tags:

- Individual UE Policy Association (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyAssociationUpdateRequest'

parameters:

- name: polAssoId

in: path

description: Identifier of a policy association

required: true

schema:

type: string

responses:

'200':

description: OK. Updated policies are returned

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyUpdate'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

npcf-ue-policy-control: Access to the Npcf_UEPolicyControl API

schemas:

PolicyAssociation:

description: >

Contains the description of a policy association that is returned by the PCF when a policy

Association is created, updated, or read.

type: object

properties:

request:

$ref: '#/components/schemas/PolicyAssociationRequest'

uePolicy:

$ref: '#/components/schemas/UePolicy'

n2Pc5Pol:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

n2Pc5PolA2x:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

n2Pc5ProSePol:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

triggers:

type: array

items:

$ref: '#/components/schemas/RequestTrigger'

minItems: 1

description: >

Request Triggers that the PCF subscribes.

pras:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 1

description: >

Contains the presence reporting area(s) for which reporting was requested.

The praId attribute within the PresenceInfo data type is the key of the map.

andspDelInd:

$ref: '#/components/schemas/PolicyStatus'

andspInd:

description: >

Indication of UE support of ANDSP. When set to true, it indicates the UE supports ANDSP,

when set to false it indicates the UE does not support ANDSP.

type: boolean

pduSessions:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionInfo'

minItems: 1

description: Combination of DNN and S-NSSAIs for which LBO information is requested.

chfInfo:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/ChargingInformation'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

n2Pc5RsppPol:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

pcfUeInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PcfUeCallbackInfo'

matchPdus:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionInfo'

minItems: 1

required:

- suppFeat

PolicyAssociationRequest:

description: >

Represents information that the NF service consumer provides when requesting the creation of

a policy association.

type: object

properties:

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

altNotifIpv4Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

minItems: 1

description: Alternate or backup IPv4 Address(es) where to send Notifications.

altNotifIpv6Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

minItems: 1

description: Alternate or backup IPv6 Address(es) where to send Notifications.

altNotifFqdns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

minItems: 1

description: Alternate or backup FQDN(s) where to send Notifications.

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

accessType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

accessTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

minItems: 1

description: >

The Access Type(s) where the served UE is camping.

It shall be provided, if available, for trigger "ACCESS_TYPE_CH.

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

userLoc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

timeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

servingPlmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

ratTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

minItems: 1

description: >

The RAT Type(s), if available, for the reported "accessTypes" where the served UE is

camping. It shall be provided, if available, for trigger "ACCESS_TYPE_CH.

groupIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

minItems: 1

hPcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

hPcfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

hPcfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

uePolReq:

$ref: '#/components/schemas/UePolicyRequest'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

serviceName:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/ServiceName'

servingNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pc5Capab:

$ref: '#/components/schemas/Pc5Capability'

v2xCapab:

$ref: '#/components/schemas/V2xCapability'

a2xCapab:

type: array

items:

$ref: '#/components/schemas/A2xCapability'

minItems: 1

proSeCapab:

type: array

items:

$ref: '#/components/schemas/ProSeCapability'

minItems: 1

confSnssais:

type: array

items:

$ref: 'TS29531_Nnssf_NSSelection.yaml#/components/schemas/ConfiguredSnssai'

minItems: 1

description: >

The Configured NSSAI for the serving PLMN, and the mapped S-NSSAI value of home

network corresponding to the configured S-NSSAI in the serving PLMN.

n3gNodeReSel:

$ref: '#/components/schemas/Non3gppAccess'

sliceN3gNodeSelCap:

$ref: '#/components/schemas/SliceSpecificN3gNodeSelectionCapability'

satBackhaulCategory:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

5gsToEpsMob:

type: boolean

description: >

It indicates the UE Policy Association is triggered by a 5GS to EPS mobility

scenario.

vpsUePolGuidance:

type: object

additionalProperties:

$ref: '#/components/schemas/UePolicyParameters'

minProperties: 1

description: >

Contains the service parameter used to guide the VPLMN-specific URSP and may contain

the subscription to VPLMN-specific URSP delivery outcome.

The key of the map represents the AF request to guide VPLMN-specific URSP rules.

This attribute only applies in roaming and when the V-PCF is the NF service consumer.

lboRoamInfo:

type: array

items:

$ref: '#/components/schemas/LboRoamingInformation'

minItems: 1

description: >

Contains LBO roaming information for DNN and S-NSSAI combination(s).

This attribute only applies in roaming and when the AMF is the NF service consumer.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

rangSlCapab:

type: array

items:

$ref: '#/components/schemas/RangSLCapability'

minItems: 1

required:

- notificationUri

- suppFeat

- supi

PolicyAssociationUpdateRequest:

description: >

Represents Information that the NF service consumer provides when requesting the update of

a policy association.

type: object

properties:

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

altNotifIpv4Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

minItems: 1

description: Alternate or backup IPv4 Address(es) where to send Notifications.

altNotifIpv6Addrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

minItems: 1

description: Alternate or backup IPv6 Address(es) where to send Notifications.

altNotifFqdns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

minItems: 1

description: Alternate or backup FQDN(s) where to send Notifications.

triggers:

type: array

items:

$ref: '#/components/schemas/RequestTrigger'

minItems: 1

description: Request Triggers that the NF service consumer observes.

praStatuses:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

description: >

Contains the UE presence status for tracking area for which changes of the UE presence

occurred. The praId attribute within the PresenceInfo data type is the key of the map.

minProperties: 1

userLoc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uePolDelResult:

$ref: '#/components/schemas/UePolicyDeliveryResult'

uePolTransFailNotif:

$ref: '#/components/schemas/UePolicyTransferFailureNotification'

uePolReq:

$ref: '#/components/schemas/UePolicyRequest'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

servingNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

connectState:

$ref: 'TS29518_Namf_EventExposure.yaml#/components/schemas/CmState'

groupIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

minItems: 1

pc5Capab:

$ref: '#/components/schemas/Pc5Capability'

v2xCapab:

$ref: '#/components/schemas/V2xCapability'

a2xCapab:

type: array

items:

$ref: '#/components/schemas/A2xCapability'

minItems: 1

proSeCapab:

type: array

items:

$ref: '#/components/schemas/ProSeCapability'

minItems: 1

confSnssais:

type: array

items:

$ref: 'TS29531_Nnssf_NSSelection.yaml#/components/schemas/ConfiguredSnssai'

minItems: 1

description: >

The Configured NSSAI for the serving PLMN, and the mapped S-NSSAI value of home

network corresponding to the configured S-NSSAI in the serving PLMN.

n3gNodeReSel:

$ref: '#/components/schemas/Non3gppAccess'

sliceN3gNodeSelCap:

$ref: '#/components/schemas/SliceSpecificN3gNodeSelectionCapability'

satBackhaulCategory:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

urspEnfRep:

type: object

additionalProperties:

$ref: '#/components/schemas/UrspEnforcementPduSession'

description: >

Contains information about the enforced URSP rule(s) in one or more PDU sessions.

The key of the map is a character string that represents an integer value.

minProperties: 1

vpsUePolGuidance:

type: object

additionalProperties:

$ref: '#/components/schemas/UePolicyParameters'

minProperties: 1

description: >

Contains the service parameter used to guide the VPLMN-specific URSP and may contain

the subscription to VPLMN-specific URSP delivery outcome.

The key of the map represents the AF request to guide VPLMN-specific URSP rules.

This attribute only applies in roaming and when the V-PCF is the NF service consumer.

nullable: true

lboRoamInfo:

type: array

items:

$ref: '#/components/schemas/LboRoamingInformation'

minItems: 1

description: >

Contains LBO roaming information for DNN and S-NSSAI combination(s).

This attribute only applies in roaming and when the AMF is the NF service consumer.

accessTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

minItems: 1

description: >

The Access Type(s) where the served UE is camping.

It shall be provided, if available, for trigger "ACCESS_TYPE_CH.

ratTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

minItems: 1

description: >

The RAT Type(s), if available, for the reported "accessTypes" where the served UE is

camping. It shall be provided, if available, for trigger "ACCESS_TYPE_CH.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

rangSlCapab:

type: array

items:

$ref: '#/components/schemas/RangSLCapability'

minItems: 1

description: >

Contains the Ranging/SL related UE capabilities.

PolicyUpdate:

description: >

Represents updated policies that the PCF provides in a notification or in the reply to an

Update Request.

type: object

properties:

resourceUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

uePolicy:

$ref: '#/components/schemas/UePolicy'

n2Pc5Pol:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

n2Pc5PolA2x:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

n2Pc5ProSePol:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

triggers:

type: array

items:

$ref: '#/components/schemas/RequestTrigger'

minItems: 1

nullable: true

description: >

Request Triggers that the PCF subscribes.

pras:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfoRm'

description: >

Contains the presence reporting area(s) for which reporting was requested.

The praId attribute within the PresenceInfoRm data type is the key of the map.

minProperties: 1

nullable: true

andspDelInd:

$ref: '#/components/schemas/PolicyStatus'

delivReport:

type: object

additionalProperties:

$ref: '#/components/schemas/UePolicyNotification'

minProperties: 1

description: >

Contains the delivery outcome of the VPLMN-specific URSP.

The key of the map represents the AF request of the corresponding subscription, i.e. its

value shall match the key that was previously provided by the V-PCF in the

vpsUePolGuidance attribute.

This attribute only applies in roaming and when the V-PCF is the NF service consumer.

pduSessions:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionInfo'

minItems: 1

description: >

Combination of DNN and S-NSSAIs for which LBO information is requested.

nullable: true

pcfUeInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PcfUeCallbackInfo'

matchPdus:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionInfo'

minItems: 1

nullable: true

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

n2Pc5RsppPol:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N2InfoContent'

required:

- resourceUri

TerminationNotification:

description: >

Represents a request to terminate a policy association that the PCF provides in a

notification.

type: object

properties:

resourceUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

cause:

$ref: '#/components/schemas/PolicyAssociationReleaseCause'

required:

- resourceUri

- cause

UePolicyTransferFailureNotification:

description: >

Represents information on the failure of a UE policy transfer to the UE because the UE is

not reachable.

type: object

properties:

cause:

$ref: '#/components/schemas/UePolicyTransferFailureCause'

retryAfter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

ptis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minItems: 1

description: >

This contains a list of PTI assigned by the H-PCF corresponding to the UE policy(s)

which could not be transferred by the AMF.

required:

- cause

- ptis

UeRequestedValueRep:

description: >

Contains the current applicable values corresponding to the policy control request triggers.

type: object

properties:

userLoc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

praStatuses:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 1

description: >

Contains the UE presence statuses for tracking areas. The praId attribute within the

PresenceInfo data type is the key of the map.

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

connectState:

$ref: 'TS29518_Namf_EventExposure.yaml#/components/schemas/CmState'

confSnssais:

type: array

items:

$ref: 'TS29531_Nnssf_NSSelection.yaml#/components/schemas/ConfiguredSnssai'

minItems: 1

description: >

The Configured NSSAI for the serving PLMN, and the mapped S-NSSAI value of home

network corresponding to the configured S-NSSAI in the serving PLMN.

satBackhaulCategory:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

urspEnfRep:

type: object

additionalProperties:

$ref: '#/components/schemas/UrspEnforcementPduSession'

description: >

Contains information about the enforced URSP rule(s) in one or more PDU sessions.

The key of the map is a character string that represents an integer value.

minProperties: 1

lboRoamInfo:

type: array

items:

$ref: '#/components/schemas/LboRoamingInformation'

minItems: 1

description: >

Contains LBO roaming information for DNN and S-NSSAI combination(s).

accessTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

minItems: 1

description: >

The Access Type(s) where the served UE is camping.

It shall be provided, if available, for trigger "ACCESS_TYPE_CH.

ratTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

minItems: 1

description: >

The RAT Type(s), if available, for the reported "accessTypes" where the served UE is

camping. It shall be provided, if available, for trigger "ACCESS_TYPE_CH.

UePolicyParameters:

description: >

Contains the service parameters used to guide the VPLMN-specific URSP rule determination.

type: object

properties:

urspGuidance:

type: array

items:

$ref: 'TS29522_ServiceParameter.yaml#/components/schemas/UrspRuleRequest'

minItems: 1

description: >

Contains the service parameter used to guide the VPLMN-specific URSP.

deliveryEvents:

type: array

items:

$ref: 'TS29522_ServiceParameter.yaml#/components/schemas/Event'

minItems: 1

description: >

AF subscribed event(s) notifications related to AF provisioned guidance

for VPLMN-specific URSP rules.

nullable: true

nullable: true

LboRoamingInformation:

description: >

Contains LBO roaming information for a DNN and S-NSSAI.

type: object

properties:

lboRoamAllowed:

type: boolean

description: >

Indicates whether LBO for the DNN and S-NSSAI is allowed when roaming.

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

required:

- dnn

- snssai

UrspEnforcementPduSession:

description: >

Represents URSP rule enforcement information for a PDU session.

type: object

required:

- urspEnfInfo

properties:

urspEnfInfo:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/UrspEnforcementInfo'

sscMode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SscMode'

ueReqDnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

ueReqPduSessionType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

UePolicyNotification:

description: >

Contains the delivery outcome of VPLMN-specific URSP rules.

type: object

required:

- eventNotifs

properties:

eventNotifs:

type: array

items:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/PcEventNotification'

minItems: 1

description: >

Represents the events to be reported according to the subscription to notifications

of VPLMN-specific URSP delivery outcome events.

UePolicy:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

UePolicyDeliveryResult:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

UePolicyRequest:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

RequestTrigger:

anyOf:

- type: string

enum:

- LOC_CH

- PRA_CH

- UE_POLICY

- PLMN_CH

- CON_STATE_CH

- GROUP_ID_LIST_CHG

- UE_CAP_CH

- SAT_CATEGORY_CHG

- NON_3GPP_NODE_RESELECTION

- CONF_NSSAI_CH

- LBO_INFO_CH

- FEAT_RENEG

- URSP_ENF_INFO

- ACCESS_TYPE_CH

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the possible request triggers.

Possible values are:

- LOC_CH: Location change (tracking area). The tracking area of the UE has changed.

- PRA_CH: Change of UE presence in PRA. The AMF reports the current presence status

of the UE in a Presence Reporting Area, and notifies that the UE enters/leaves the

Presence Reporting Area.

- UE_POLICY: A MANAGE UE POLICY COMPLETE message or a MANAGE UE POLICY COMMAND REJECT

message, as defined in Annex D.5 of 3GPP TS 24.501 or a "UE POLICY PROVISIONING REQUEST"

message, as defined in clause 7.2.1.1 of 3GPP TS 24.587, has been received by the AMF

and is being forwarded.

- PLMN_CH: PLMN change. the serving PLMN of UE has changed.

- CON_STATE_CH: Connectivity state change: the connectivity state of UE has changed.

- GROUP_ID_LIST_CHG: UE Internal Group Identifier(s) has changed. This policy

control request

trigger does not require a subscription.

- UE_CAP_CH: UE Capabilities change: the UE provided 5G ProSe capabilities have changed.

This policy control request trigger does not require subscription.

- SAT_CATEGORY_CHG: Indicates that the AMF has detected a change between different satellite

category, or non-satellite backhaul.

- NON_3GPP_NODE_RESELECTION: The UE has connected to a wrong non-3GPP access node that

does not match its subscribed S-NSSAI(s). This policy control request trigger does not

require a subscription.

- CONF_NSSAI_CH: Configured NSSAI change. Indicates that the configured NSSAI has changed.

- LBO_INFO_CH: LBO information change. The AMF reports LBO roaming allowed or not allowed

for the requested DNN(s) and S-NSSAI(s). This policy control request trigger only applies

in roaming scenarios when the NF service consumer is the AMF.

- FEAT_RENEG: The NF service consumer notifies that the target AMF is requesting feature

re-negotiation.

- URSP_ENF_INFO: The V-PCF has received URSP rule enforcement information about the enforced

URSP rule(s) in one or more PDU sessions. This trigger applies in roaming scenarios and

to the V-PCF.

- ACCESS_TYPE_CH: Access Type change. The registered access type and RAT type

has changed, an access type and RAT type is added or removed.

PolicyAssociationReleaseCause:

anyOf:

- type: string

enum:

- UNSPECIFIED

- UE_SUBSCRIPTION

- INSUFFICIENT_RES

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the cause why the PCF requests the policy association termination.

Possible values are:

- UNSPECIFIED: This value is used for unspecified reasons.

- UE_SUBSCRIPTION: This value is used to indicate that the policy association needs to be

terminated because the subscription of UE has changed (e.g. was removed).

- INSUFFICIENT_RES: This value is used to indicate that the server is overloaded and needs

to abort the policy association.

Pc5Capability:

anyOf:

- type: string

enum:

- LTE_PC5

- NR_PC5

- LTE_NR_PC5

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the specific PC5 RAT(s) which the UE supports for V2X communications over

PC5 reference point.

Possible values are:

- LTE_PC5: This value is used to indicate that UE supports PC5 LTE RAT for V2X

communications over the PC5 reference point

- NR_PC5: This value is used to indicate that UE supports PC5 NR RAT for V2X communications

over the PC5 reference point.

- LTE_NR_PC5: This value is used to indicate that UE supports both PC5 LTE and NR RAT for

V2X communications over the PC5 reference point.

ProSeCapability:

anyOf:

- type: string

enum:

- PROSE_DD

- PROSE_DC

- PROSE_L2_U2N_RELAY

- PROSE_L3_U2N_RELAY

- PROSE_L2_REMOTE_UE

- PROSE_L3_REMOTE_UE

- PROSE_L2_U2U_RELAY

- PROSE_L3_U2U_RELAY

- PROSE_L2_END_UE

- PROSE_L3_END_UE

- PROSE_MH_L2_U2N_RELAY

- PROSE_MH_L3_U2N_RELAY

- PROSE_MH_L2_REMOTE_UE

- PROSE_MH_L3_REMOTE_UE

- PROSE_MH_L2_INTERMEDIATE_UE

- PROSE_MH_L3_INTERMEDIATE_UE

- PROSE_MH_L3_U2U_RELAY

- PROSE_MH_L3_END_UE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

the content defined in the present version of this API.

description: |

Represents the 5G ProSe capabilities.

Possible values are:

- PROSE_DD: This value is used to indicate that 5G ProSe Direct Discovery is supported

by the UE.

- PROSE_DC: This value is used to indicate that 5G ProSe Direct Communication is supported

by the UE.

- PROSE_L2_U2N_RELAY: This value is used to indicate that Layer-2 5G ProSe UE-to-Network

Relay is supported by the UE.

- PROSE_L3_U2N_RELAY: This value is used to indicate that Layer-3 5G ProSe UE-to-Network

Relay is supported by the UE.

- PROSE_L2_REMOTE_UE: This value is used to indicate that Layer-2 5G ProSe Remote UE is

supported by the UE.

- PROSE_L3_REMOTE_UE: This value is used to indicate that Layer-3 5G ProSe Remote UE is

supported by the UE.

- PROSE_L2_U2U_RELAY: This value is used to indicate that Layer-2 5G ProSe UE-to-UE

Relay is supported by the UE.

- PROSE_L3_U2U_RELAY: This value is used to indicate that Layer-3 5G ProSe UE-to-UE

Relay is supported by the UE.

- PROSE_L2_END_UE: This value is used to indicate that Layer-2 5G ProSe End UE is

supported by the UE.

- PROSE_L3_END_UE: This value is used to indicate that Layer-3 5G ProSe End UE is

supported by the UE.

- PROSE_MH_L2_U2N_RELAY: Indicates that the UE supports acting as a 5G ProSe Layer-2

UE-to-Network Relay UE supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay.

- PROSE_MH_L3_U2N_RELAY: Indicates that the UE supports acting as a 5G ProSe Layer-3

UE-to-Network Relay UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay.

- PROSE_MH_L2_REMOTE_UE: Indicates that the UE supports acting as a 5G ProSe Layer-2 Remote

UE supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay.

- PROSE_MH_L3_REMOTE_UE: Indicates that the UE supports acting as a 5G ProSe Layer-3 Remote

UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay.

- PROSE_MH_L2_INTERMEDIATE_UE: Indicates that the UE supports acting as a 5G ProSe Layer-2

Intermediate UE-to-Network Relay supporting 5G ProSe Layer-2 multi-hop UE-to-Network

Relay.

- PROSE_MH_L3_INTERMEDIATE_UE: Indicates that the UE supports acting as a 5G ProSe Layer-3

Intermediate UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network

Relay.

- PROSE_MH_L3_U2U_RELAY: Indicates that the UE supports acting as a 5G ProSe Layer-3

UE-to-UE Relay UE supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay.

- PROSE_MH_L3_END_UE: Indicates that the UE supports acting as a 5G ProSe Layer-3 End UE

supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay.

Non3gppAccess:

anyOf:

- type: string

enum:

- N3IWF

- TNGF

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents a non-3gpp access node.

Possible values are:

- N3IWF: Non-3gpp Interworking Function.

- TNGF: Trusted Non-3gpp Gateway Function.

N1N2MessTransferErrorReply:

anyOf:

- type: string

enum:

- UE_NOT_REACHABLE

- UNSPECIFIED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents an N1N2 Message Transfer error.

Possible values are:

- UE_NOT_REACHABLE: The UE is not reachable for paging.

- UNSPECIFIED: Unspecified error.

RangSLCapability:

anyOf:

- type: string

enum:

- PC5_RANGING_SL

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

but is not used to encode content defined in the present version of this API.

description: |

Indicates the Ranging and Sidelink Capability.

Possible values are:

- PC5_RANGING_SL: Indicates that the PC5 Capability for Ranging and Sidelink is supported

by the UE.

PolicyStatus:

anyOf:

- type: string

enum:

- CONFIGURED

- NOT_CONFIGURED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the configuration status of a UE Policy in the UE.

Possible values are:

- CONFIGURED: The UE Policy is configured in the UE.

- NOT_CONFIGURED: The UE Policy is not configured in the UE.

A2xCapability:

anyOf:

- type: string

enum:

- EUTRA_PC5

- NR_PC5

- UU

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the A2X capabilities the UE supports for A2X communication.

Possible values are:

- EUTRA_PC5: This value is used to indicate that the UE supports PC5 EUTRA RAT for A2X

communications over the PC5 reference point

- NR_PC5: This value is used to indicate that the UE supports PC5 NR RAT for A2X

communications over the PC5 reference point.

- UU: This value is used to indicate that UE supports A2X communications over the Uu

reference point.

V2xCapability:

anyOf:

- type: string

enum:

- SUPPORTED

- NOT_SUPPORTED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the V2X capabilities the UE supports for V2X communication.

Possible values are:

- SUPPORTED: This value is used to indicate that the UE supports V2X communications

over the PC5 and/or reference point

- NOT_SUPPORTED: This value is used to indicate that the UE does not support V2X

communication over PC5 and/or Uu reference point.

SliceSpecificN3gNodeSelectionCapability:

anyOf:

- type: string

enum:

- ONLY_N3IWF_SS_SEL

- ONLY_TNGF_SS_SEL

- TNGF_N3IWF_SS_SEL

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the UE capabilities with regard to slice-specific non-3gpp node selection.

Possible values are:

- ONLY_N3IWF_SS_SEL: Indicates that the UE supports N3IWF selection based on the slices

the UE wishes to use over untrusted non-3GPP access.

- ONLY_TNGF_SS_SEL: Indicates that the UE supports TNGF selection based on the slices

the UE wishes to use over trusted non-3GPP access.

- TNGF_N3IWF_SS_SEL: Indicates that the UE supports N3IWF selection based on the slices

the UE wishes to use over untrusted non-3GPP access and TNGF selection based on the

slices the UE wishes to use over trusted non-3GPP access.

#

UePolicyTransferFailureCause:

description: UE Policy Transfer Failure Cause.

anyOf:

- $ref: 'TS29518_Namf_Communication.yaml#/components/schemas/N1N2MessageTransferCause'

- $ref: '#/components/schemas/N1N2MessTransferErrorReply'

# *** End of Changes ***