3GPP TSG CT WG3 Meeting #143	C3-254269

# Sophia-Antipolis, France, 13th – 17th October, 2025

Source:	Nokia

Title:	Service operation description corrections for the AF VFL APIs

Spec:	3GPP TS 29.530

Agenda item:	19.39 (AIML_CN)

Document for:	Agreement

1. Introduction

There are some mistakes in the service operation descriptions of TS 29.530, including wrong attribute lists and data type names, presence conditions, and more.

2. Reason for Change

Correct the service operation descriptions of TS 29.530.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.530 V 1.0.0.

* * * * First change * * * *

##### 5.2.2.4.2	VFL Training Notification

Figure 5.2.2.4.2-1 depicts a scenario where the AF sends a request to notify a previously subscribed service consumer on VFL Training report(s) (see also clause 6.2H of 3GPP°TS°23.288°[14]).

Figure 5.2.2.4.2-1: Procedure for VFL Training Notification

1.	In order to notify a previously subscribed service consumer on VFL Training report(s), the AF shall send an HTTP POST request to the NF service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the NF service consumer during the creation/update of the corresponding VFL Training Subscription using the procedures defined in clauses 5.2.2.2, and the request body including the VflTrainingNotify data structure.

2a.	Upon success, the NF service consumer shall respond to the AF with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.7.

* * * * Next change * * * *

##### 5.3.2.2.4	Partial update subscription for event notifications

Figure 5.3.2.2.4-1 shows a scenario that the NF service consumer sends an HTTP PATCH request to the AF to partial modify an existing VFL inference subscription (as shown in 3GPP TS 23.288 [14]).

Figure 5.3.2.2.4-1: Partial modification of VFL inference subscription information using HTTP PATCH

The NF service consumer shall invoke the Naf_VFLInference_Subscribe service operation to partial modify an existing VFL inference subscription. The NF service consumer shall send an HTTP PATCH request with: "{apiRoot}/naf-vflinference/<apiVersion>/subscriptions/{subscriptionId}" as Resource URI, where "{subscriptionId}" is the subscriptionId of the existing VFL inference subscription to be modified, to update an "Individual AF VFL Inference Subscription" according to the information in the message body.

Upon receipt of an HTTP PATCH request with: "{apiRoot}/naf-vflinference/<apiVersion>/subscriptions/{subscriptionId}" as Resource URI and VflInferSubPatch data type as request body, if the request is successfully processed and accepted, the AF shall:

-	partial modify the concerned subscription; and

-	store the subscription.

If the AF successfully processed and accepted the received HTTP PATCH request, the AF shall partially update an "Individual AF VFL Inference Subscription" resource, and shall respond with:

-	HTTP "204 No Content" response (as shown in figure 5.3.2.2.4-1, step 2a); or

-	HTTP "200 OK" response (as shown in figure 5.3.2.2.4-1, step 2b) with a response body containing a representation of the updated subscription in the VflInferSub data type.

If the immediate reporting indication in the "immRep" attribute within the within the "ReportingInformation" structure in the "vflReportInfo" attribute sets to "true" during the event subscription, the AF shall include the reports of the subscribed events, if available, as the "vflInferResults" attribute in the HTTP PATCH response.

If any error occurs when processing the HTTP PATCH request, the AF shall send an HTTP error response as specified in clause 6.2.7.

If the AF determines that the received HTTP PATCH request needs to be redirected, the AF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

* * * * Next change * * * *

##### 5.3.2.4.2	Notification about subscribed event

Figure 5.3.2.4.2-1 shows a scenario where the AF sends a request to the NF Service Consumer to notify for VFL inference event notifications (see also 3GPP TS 23.288 [14]).

Figure 5.3.2.4.2-1: AF notifies the subscribed VFL inference event

The AF shall invoke the Naf_VFLInference_Notify service operation to notify about a subscribed VFL inference event. The AF shall send an HTTP POST request with "{notifUri}" received in the Naf_VFLInference_Subscribe service operation as Resource URI, as shown in figure 5.3.2.4.2-1, step 1. The VflInferNotif data structure provided in the request body that shall include:

-	a notification correlation identifier as "notifCorreId" attribute;

-	description of the notified event as "vflInferResults " attribute;

Upon the reception of an HTTP POST request, if the NF service consumer successfully processed and accepted the received HTTP POST request, the NF Service Consumer shall store the notification and respond with HTTP "204 No Content" status code.

If the NF service consumer determines the received HTTP POST request needs to be redirected, the NF service consumer shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

If errors occur when processing the HTTP POST request, the NF service consumer shall send an HTTP error response as specified in clause 6.2.7.

* * * * Next change * * * *

##### 5.5.2.2.2	Inference Subscription Creation

Figure 5.5.2.2.2-1 a scenario where an NF service consumer sends a request to the AF to request the creation of an Inference Subscription.

Figure 5.5.2.2.2-1: Procedure for Inference subscription

In order to subscribe to Inference event notifications, the NF service consumer shall send an HTTP POST request to the AF targeting the URI of the "Inference Subscriptions" collection resource, with the request body including the InferEventSubsc data structure as defined in clause 5.4.6.4.2.

Upon the reception of an HTTP POST request with: "{apiRoot}/naf-inference/<apiVersion>/subscriptions" as Resource URI and InferEventSubsc data structure as request body:

-	If the VFL server is a trusted AF:

-	If no VFL model is already trained, the VFL server initiates VFL Training by sending Nnwdaf_VFLTraining_Subscribe towards NWDAF(s) acting as VFL Client(s).

-	If VFL model is already trained, the VFL server may decide to initiate the VFL inference procedure towards the selected NWDAF VFL client(s) by sending Nnwdaf_VFLInference_Subscribe.

-	If the VFL server is an untrusted AF:

-	If no VFL model is already trained, the VFL server initiates VFL Training by sending for each NWDAF acting as VFL client an Nnef_VFLTraining_Subscribe request towards NEF indicating the identity of the NWDAF acting as VFL Client.

-	If VFL model is already trained, the VFL server may decide to initiate the VFL inference procedure. For each selected NWDAF VFL client the VFL server sends an Nnef_VFLInference_Subscribe request to the NEF indicating the identity of the NWDAF acting as VFL Client.

If no NWDAF VFL Client(s) are selected, the VFL server may generate the VFL inference results based only on its local trained ML model associated with the determined VFL correlation ID.

Upon success, the AF shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual Inference Subscription" resource within the InferEventsSubsc data structure, and an HTTP "Location" header field containing the URI of the created resource.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.4.7.

##### 5.5.2.2.3	Inference Subscription Update

Figure 5.5.2.2.3-1 illustrates the modification of an existing subscription.

Figure 5.5.2.2.3-1: Modification of an existing subscription

In order to request the update of an existing inference subscription, the NF service consumer shall send an HTTP PUT/PATCH request to the AF, targeting the URI of the corresponding "Individual Inference Subscription" resource, with the request body including either:

-	the updated representation of the resource within the InferEventsSubsc data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the InferEventsSubscPatch data structure, in case the HTTP PATCH method is used.

Upon receipt of the HTTP PUT/PATCH request from the NF service consumer the AF shall:-

if the VFL server is a trusted AF and there are NWDAF VFL subscriptions associated to the AF inference subscription, the VFL updates those NWDAF VFL inference subscriptions by sending an Nnwdaf_VFLInference_Subscribe request towards each applicable NWDAF VFL client(s);

-	if the VFL server is an untrusted AF and there are NWDAF VFL subscriptions associated to the AF inference subscription, the VFL updates those NWDAF VFL inference subscriptions by sending for each applicable NWDAF client an Nnef_VFLInference_Subscribe request to the NEF indicating the identity of the NWDAF VFL Client;

Upon success, the AF shall update the targeted "Individual Inference Subscription" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Inference Subscription" resource within the InferEventsSubsc data structure; or

-	an HTTP "204 No Content" status code.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 6.4.7.

* * * * End of changes * * * *