3GPP TSG-CT WG3 Meeting #144	C3-255084

# Dallas, US, 17-21, November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.561 | CR | 0207 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Lowercase naming for HTTP custom headers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Section 4.2 of RFC 9114 requires all HTTP field names to be lowercased prior to HTTP/3 encoding: "Characters in field names MUST be converted to lowercase prior to their encoding. A request or response containing uppercase characters in field names MUST be treated as malformed." The current uppercase naming of “3gpp-Connect-Req-Inband-Info” header and “3gpp-Connect-Resp-Inband-Info” may lead to non-compliance and implementation errors even though the current specification is aligned with TS 29.500 clause 5.2.3 conventions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Explanatory NOTEs added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non-compliance with RFC 9114, potentially leading to malformed HTTP header names and interoperability failures in implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

### 22.3.2	Sending Media Related Information using proxying UDP in HTTP Datagram

To establish a connect-UDP tunnel as defined in IETF RFC 9298 [64], the HTTP client in the UPF shall include in the HTTP/3 CONNECT request:

a)	the :protocol pseudo-header set to connect-udp; and

b)	the 3gpp-Connect-Req-Inband-Info header defined in Annex A.3 with the intended-inband-purpose parameter set to "MRI".

EXAMPLE 1:	3gpp-Connect-Req-Inband-Info header in HTTP CONNECT request message for the format that includes the media related information in the HTTP Datagram payload:

3gpp-Connect-Req-Inband-Info: intended-inband-purpose=MRI

NOTE 1:	The above example follows HTTP/1.1 encoding conventions. As specified in section 4.2 of IETF RFC 9114 [65], field names are required to be normalised to lowercase prior to HTTP/3 encoding. The colon and optional whitespace characters between the field name and field value are not encoded.

Upon successfully establishing the connection, the HTTP server (AS proxy) shall return a 2XX response including the 3gpp-Connect-Resp-Inband-Info header defined in Annex A.3 with:

a)	the inband-purpose parameter set to "MRI" indicating that the AS will send Media Related Information as defined in clause 22.2, in HTTP Datagrams; and

b)	the context-id parameter set to a non-zero odd value registered by the AS for the Context ID that will be used in HTTP Datagrams carrying Media Related Information.

EXAMPLE 2: 	3gpp-Connect-Resp-Inband-Info header in HTTP CONNECT response message indicating successfully established connection with the registered Context ID (decimal) value 7:

3gpp-Connect-Resp-Inband-Info: inband-purpose=MRI; context-id=0b111

NOTE 2:	The above example follows HTTP/1.1 encoding conventions. As specified in section 4.2 of IETF RFC 9114 [65], field names are required to be normalised to lowercase prior to HTTP/3 encoding. The colon and optional whitespace characters between the field name and field value are not encoded.

Upon receipt of the successful 2XX response as per IETF RFC 9298 [64], the UPF shall consider that the UDP tunnel is established to receive HTTP Datagrams carrying Media Related Information with the AS registered Context ID, received in the 3gpp-Connect-Resp-Inband-Info header.

If the HTTP client in the UPF chooses to reuse the QUIC connection to the HTTP server for multiple UEs, the UPF shall open a different QUIC stream for the HTTP/3 CONNECT request issued for each UE.

The HTTP client in the UPF may use different QUIC connections to the HTTP server for multiple UEs, each connection with a separate TLS security context as per IETF RFC 9114 [65].

When the Media Related Information is encapsulated in HTTP Datagrams of the connect-UDP tunnel, then the HTTP server (AS proxy) shall encode the HTTP Datagram payload format as below:

a)	Context ID with format defined in IETF RFC 9298 [64], set to an odd number, derived as per the above procedure; and

b)	a payload containing:

1)	The length of the Media Related Information, encoded as one octet and indicating the length of the MRI in number of bytes.

2)	Media Related Information with the format as defined in clause 22.2; and

3)	UDP proxying payload.

Upon receipt of the HTTP Datagram with the Context ID registered as the value indicated in the 3gpp-Connect-Resp-Inband-Info header, the UPF shall extract the Media Related Information from the HTTP Datagram payload for DL transmission in the GTP-U header as defined in 3GPP TS 23.501 [2].

NOTE 3:	The UDP proxying payload of the HTTP Datagram payload contains application data which is end-to-end encrypted between the UE and the Application Server.

# * * * * End of Changes * * * *