3GPP TSG-CT WG3 Meeting #144	C3-255268

# Dallas ,United States, 17 - 21 November 2025

Source:	Nokia

Title:	Pseudo-CR on correction on AIMLES_ContextTransfer API

Spec:	3GPP TS 29.482 v1.2.0

Agenda item:	19.41

Document for:	Agreement

1. Introduction

<Introduction part (optional)>

2. Reason for Change

Various corrections are needed in the Open API definition of AIMLES_ContextTransfer API, as follows:

- It should be transfer instead of request/transfer in the paths object, the attribute names should be aligned with agreed data model.

3. Conclusions

This pCR proposes to correct all of the above issues.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.482 v1.2.0.

* * * First Change * * *

# A.2	AIMLES_ContextTransfer API

openapi: 3.0.0

info:

title: AIMLE Context Transfer Service

version: 1.0.0-alpha.4

description: |

AIMLE Context Trnaser Information Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.482 v1.2.0; Service Enabler Architecture Layer for Verticals (SEAL); Artificial

Intelligence Machine Learning Enablement (AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.482/

servers:

- url: '{apiRoot}/aimles-ct/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/transfer:

post:

summary: Request AIMLE context information transfer.

operationId: Transfer

tags:

- AIMLE Context Transfer Request

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TransReq'

responses:

'204':

description: >

No Content. The AIMLE Context Transfer request is successfully received and

processed.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

TransReq:

description: >

Represents the AIMLE Context Transfer request.

type: object

properties:

clientId:

$ref: '#/components/schemas/AimleClientId'

currServerId:

type: string

prevServerIds:

type: array

items:

type: string

minItems: 1

servStatus:

$ref: '#/components/schemas/AimleServStatus'

servRes:

type: string

servApp:

$ref: '#/components/schemas/AimleServApp'

mlContextInfo:

$ref: '#/components/schemas/MLContextInformation'

required:

- clientId

AimleServStatus:

description: >

Represents the AIMLE service operations status related information.

type: object

properties:

servOpStatus:

$ref: '#/components/schemas/AimleServOpStatus'

servOpCompPer:

type: integer

minimum: 0

maximum: 100

required:

- servOpStatus

AimleServApp:

description: >

Represents the AIMLE service applicability information.

type: object

properties:

serviceArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

operationPipeline:

type: string

MLContextInformation:

description: >

Represents the context information related to the ML operation.

type: object

properties:

valServId:

type: string

mlModelInfo:

$ref: 'TS29482_AIMLES_MLModelRetrieval.yaml#/components/schemas/MLModelDetail'

mlTask:

$ref: 'TS24560_Aimles_AIMLEClientRegistration.yaml#/components/schemas/AimlOperation'

dataSetInfo:

$ref: 'TS24560_Aimlec_MLModTngCapEva.yaml#/components/schemas/DataSetRequirements

trainOpStatus:

$ref: '#/components/schemas/AimleServOpStatus'

splitTaskInfo:

$ref: 'TS29482_AIMLES_SplitOpEvent.yaml#/components/schemas/SplitOpProfile'

required:

- mlModelInfo

# SIMPLE DATA TYPES

#

AimleClientId:

description: Represents the unique identifier of a AIMLE Client.

type: string

#

# ENUMERATIONS

#

AimleServOpStatus:

anyOf:

- type: string

enum:

- ACTIVE

- PAUSED

- COMPLETED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the AIMLE service operations status.

Possible values are:

- ACTIVE: Indicates that the status of the AIML operations at the AIMLE Client is active.

- PAUSED: Indicates that the status of the AIML operations at the AIMLE Client is paused.

- COMPLETED: Indicates that the status of the AIML operations at the AIMLE Client is

completed.

* * * End of Change * * *