3GPP TSG-CT WG3 Meeting #144	C3-255341

# Dallas, United States, 17 - 21 November 2025

Source:	China Mobile

Title:	Pseudo-CR on MMTel_DCAppCall OpenAPI

Spec:	3GPP TS 29.392 v1.0.0

Agenda item:	19.62

Document for:	Decision

1. Introduction

According to stage 2 definition, it is proposed to define the MMTel_DCAppCall OpenAPI..

2. Reason for Change

Define the MMTel_DCAppCall OpenAPI for MMTel Enabler Server services.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.392 v1.0.0.

* * * First Change * * * *

A.3	MMTel_DCAppCall API

And so on if there are more than two services supported by the NF.

openapi: 3.0.0

info:

title: MMTel_DCAppCall API

version: 1.0.0

description: |

MMTel_DCAppCall Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

contact:

name: 3GPP

url: https://www.3gpp.org

license:

name: 3GPP

url: https://www.3gpp.org/ftp/Specs/archive/

externalDocs:

description: 3GPP TS 29.392 V1.0.0; MMTel Enabler Server Services; Stage 3

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.392/

servers:

- url: '{apiRoot}/mmtel-dcappcall/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

tags:

- name: DC Call Sessions

description: Operations for establishing Calls with Data Channel capability

- name: A2P DC Media

description: Operations for adding A2P Data Channel media to existing IMS sessions

- name: Notifications

description: Callback operations for DC resource notifications

paths:

/dccalls:

post:

tags:

- DC Call Sessions

summary: Establish Call with Data Channel capability

operationId: MMTel_DCAppCall_DCCallReq

description: This operation is used by the service consumer to request to establish a Call with Data Channel capability

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DcCallReq'

description: DC Call establishment request

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/DcCallResp'

headers:

Location:

description: Contains the URI of the newly created resource

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

/a2p-dcmedia:

post:

tags:

- A2P DC Media

summary: Add A2P Data Channel media to existing IMS session

operationId: MMTel_DCAppCall_UpdateDCMedia

description: This operation is used by the service consumer to request to add an A2P Data Channel media to an existing IMS session

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DcMediaUpdateReq'

description: A2P Data Channel media update request

responses:

'200':

description: OK

content:

application/json:

schema:

$ref: '#/components/schemas/DcMediaUpdateResp'

headers:

Location:

description: Contains the URI of the updated media resource

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

/notifications/dc-resource:

post:

tags:

- Notifications

summary: DC Resource Information Notification

operationId: MMTel_DCAppCall_Notify

description: This operation is used by the MMTel Enabler Server to notify the service consumer about any update of DC resource information

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DcResourceNotify'

description: DC resource information notification

responses:

'204':

description: No Content - Notification successfully received

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'410':

$ref: 'TS29122_CommonData.yaml#/components/responses/410'

components:

schemas:

DcCallReq:

type: object

required:

- originatingId

- terminatingId

- notificationInfo

properties:

originatingId:

type: string

description: The identifier of the caller

terminatingId:

type: string

description: The identifier of the callee

mediaInfo:

type: string

description: Identifier of one or more media type(s)

enum:

- audio

- video

- audio_and_video

dcMediaInfo:

type: boolean

description: Identifier of whether DC media is expected to be used in this call

appProfileRequested:

type: string

description: The DC application profile expected to be used in this call

notificationInfo:

$ref: 'TS29122_CommonData.yaml#/components/schemas/CallbackReference'

DcCallResp:

type: object

required:

- callResult

- originatingId

- terminatingId

properties:

callResult:

type: string

description: Indication if the Call establishment is success or failure

enum:

- SUCCESS

- FAILED

sessionId:

type: string

description: The identifier of the call session

failureCause:

type: string

description: The reason for failure

originatingId:

type: string

description: The identifier of the caller

terminatingId:

type: string

description: The identifier of the callee

mediaInfo:

type: string

description: Identifier of one or more media type(s)

enum:

- audio

- video

- audio_and_video

dcMediaInfo:

type: boolean

description: Identifier of whether DC media is expected to be used in this call

appProfileRequested:

type: string

description: The DC application profile expected to be used in this call

DcMediaUpdateReq:

type: object

required:

- dcAppId

- sessionId

- notificationAddress

properties:

dcAppId:

type: string

description: The identifier of the DC application

sessionId:

type: string

description: The session identifier of the session on which MMTel Enabler Server must add the new media

mediaResourceInfo:

type: object

description: The media information to be passed via the Data Channel

properties:

mediaType:

type: string

enum:

- AUDIO

- VIDEO

- DATA_CHANNEL

codec:

type: string

description: Codec information for the media

bandwidth:

type: integer

description: Required bandwidth in kbps

direction:

type: string

enum:

- SEND_ONLY

- RECV_ONLY

- SEND_RECV

appProfileRequested:

type: string

description: The DC application profile expected to be used in this call

notificationAddress:

type: string

format: uri

description: The address where the call related notification is sent to

a2pType:

type: string

description: Type of A2P communication

enum:

- PUSH_NOTIFICATION

- INTERACTIVE_SESSION

- BACKGROUND_DATA

DcMediaUpdateResp:

type: object

required:

- sessionId

- result

properties:

sessionId:

type: string

description: The session identifier of the session

result:

type: string

description: Indicates the Notification of Application of DC media info (A2P) request was successful or failure

enum:

- SUCCESS

- FAILED

cause:

type: string

description: Indicates the cause of request failure

mdc2Endpoint:

type: string

description: MDC2 endpoint address for the data channel

format: uri

DcResourceNotify:

type: object

required:

- sessionId

- notificationType

properties:

sessionId:

type: string

description: The session identifier of the session on which MMTel Enabler Server must add the new media

notificationType:

type: string

description: Type of notification

enum:

- DC_RESOURCE_AVAILABLE

- DC_RESOURCE_UPDATED

- DC_RESOURCE_RELEASED

mediaResourceInfo:

type: object

description: The media information to be passed via the Data Channel

properties:

mdc2ResourceInfo:

type: object

description: MDC2 resource information

availableBandwidth:

type: integer

description: Available bandwidth in kbps

supportedCodecs:

type: array

items:

type: string

description: List of supported codecs

appProfileRequested:

type: string

description: The DC application profile expected to be used in this call

timestamp:

type: string

format: date-time

description: Time when the notification was generated

resourceStatus:

type: string

description: Status of the DC resource

enum:

- ACTIVE

- INACTIVE

- SUSPENDED

- TERMINATED

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

* * * Next Change * * * *

<Proposed change in revision marks>

* * * End of Changes * * * *