3GPP TSG-CT WG3 Meeting #144	C3-255346

# Dallas, United States, 17 - 21 November 2025					(revision of C3-255281)

# Title:	Reply on LS to 3GPP CT1 and CT3 on Reserved QoS Rule Precedence Values

# Response to:	LS to 3GPP CT1 and CT3 on Reserved QoS Rule Precedence Values (C3-255021, C3-255022)

# Release:	Rel-19

# Work Item:	-

Source:	CT3

To:	GSMA Terminal Steering Group, CT1

Cc:

Contact Person:

Name:	Bhaskar Paul

E-mail Address:	bhaskar.paul@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1. Overall Description:

CT3 thanks GSMA’s Terminal Steering Group (TSG) for their LS on Reserved QoS Rule Precedence Values (C3-255021).

CT3 also thanks CT1 for their LS on Reserved QoS Rule Precedence Values (C3-255022).

CT3 discussed the issues mentioned in the respective received LS(s) and has not reached any agreement to correct/clarify in our CT3 TS 29.512 as because of the following reasons:

- The value 80 (decimal) is reserved is mentioned in CT1 TS 24.501 that is referred to in clause 4.2.6.5.7 of CT3 TS 29.512 as below:

NOTE 2:	While the UE applies a standardized value for the precedence of all UE derived QoS rules, PCC rules precedence values can vary and PCF configuration has to ensure that there is a large enough value range for the precedence of PCC rules corresponding to UE derived QoS rules. To avoid that the precedence of network provided QoS rules need to be changed when Reflective QoS is activated and filters are overlapping, the PCF will take the standardized value for the precedence of UE derived QoS rules into account and will setting the precedence value of PCC rules subject to Reflective QoS to a value in the range from 70 to 99 (decimal), as specified in 3GPP TS 24.501 [20], subclause 6.2.5.1.1.3.

- As the Precedence value for PccRule QoS Flows defined in 3GPP TS 29.512 Table 5.6.2.6-1 refers to reflective QoS in the NOTE 4 as below:

NOTE 4:	The "precedence" attribute is used to specify the precedence of the PCC rule among all PCC rules associated with the PDU session. It includes an integer value in the range from 0 to 255 (decimal). The higher the value of the "precedence" attribute, the lower the precedence of that PCC rule is. For the PCC rules subject to Reflective QoS. The precedence value range from 70 to 99 (decimal) shall be used for the PCC rules subject to Reflective QoS.

The standardization value 80 is not explicitly mentioned (i.e. the value 80 shall not be used when setting the precedence attribute for the purpose of Signalled QoS in the PCC rule ) in this NOTE 4 because it is clarified in clause 4.2.6.5.7 of TS 29.512 (see above NOTE 2).

2. Actions:

To GSMA’s Terminal Steering Group (TSG).

ACTION: 	CT3 kindly asks GSMA’s Terminal Steering Group (TSG) to take into consideration of above analysis.

To CT1.

ACTION: 	CT3 kindly asks CT1 to take into consideration of above analysis.

3. Date of Next CT3 Meetings:

CT3 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C3.htm