3GPP TSG-CT WG3 Meeting #144	C3-255546

# Dallas ,United States, 17 - 21 November 2025					(revision of C3-255391 was 5264)

Source:	Nokia, Ericsson

Title:	Pseudo-CR on update and correction on Open API definition of AIMLES_MLModelRetrieval

Spec:	3GPP TS 29.482 v1.2.0

Agenda item:	19.41

Document for:	Agreement

1. Introduction

<Introduction part (optional)>

2. Reason for Change

Update and correction of Open API definition of AIMLES_MLModelRetrieval based on agreed data model.

3. Conclusions

Update and correction of Open API definition of AIMLES_MLModelRetrieval based on agreed data model.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.482 v1.2.0

* * * First Change * * *

# A.11	AIMLES_MLModelRetrieval API

openapi: 3.0.0

info:

title: AIMLES_MLModelRetrieval

description: |

API for AIMLES ML Model Retrieval Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.0-alpha.3"

externalDocs:

description: >

3GPP TS 29.482 v1.2.0; Artificial Intelligence Machine Learning Enablement

(AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

servers:

- url: '{apiRoot}/aimles-mlmr/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/retrieve:

post:

summary: Enables a service consumer to send AIMLE ML Model retrieval request to the AIMLE server.

operationId: RetrieveMLModel

tags:

- Retrieve

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MLMdlRetReq'

responses:

'200':

description: OK (The requested location information)

content:

application/json:

schema:

$ref: '#/components/schemas/MLMdlRetRsp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions:

post:

summary: Request the creation of a Individual AIMLE ML Model Retrieval Subscription.

operationId: CreateAimleMLModelRetrievalSubscription

tags:

- AIMLE ML Model Retrieval Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MLMdlRetSub'

responses:

'201':

description: >

The requested Individual AIMLE ML Model Retrieval Subscription is successfully

created and a representation of the created resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MLMdlRetSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

summary: Notify on the requested data.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MlMdlRetNotif'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the Individual AIMLE ML Model Retrieval Subscription resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing the Individual AIMLE ML Model Retrieval Subscription resource.

operationId: GetIndAimleMLModelRetrievalSubscription

tags:

- Individual AIMLE ML Model Retrieval Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual AIMLE ML Model Retrieval Subscription resource

shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MLMdlRetSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual AIMLE ML Model Retrieval Subscription resource.

operationId: UpdateIndAimleMLModelRetrievalSubscription

tags:

- Individual AIMLE ML Model Retrieval Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MLMdlRetSub'

responses:

'200':

description: >

OK. The Individual AIMLE ML Model Retrieval Subscription resource is

successfully updated and a representation of the updated resource shall be returned in

the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MLMdlRetSub'

'204':

description: >

No Content. The Individual AIMLE ML Model Retrieval Subscription resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual AIMLE ML Model Retrieval Subscription resource.

operationId: ModifyIndAimleMLModelRetrievalSubscription

tags:

- Individual AIMLE ML Model Retrieval Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/MLMdlRetSubPatch'

responses:

'200':

description: >

OK. The Individual AIMLE ML Model Retrieval Subscription resource is

successfully modified and a representation of the updated resource shall be returned in

the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MLMdlRetSub'

'204':

description: >

No Content. The Individual AIMLE ML Model Retrieval Subscription resource is

successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Remove the Individual AIMLE ML Model Retrieval Subscription.

operationId: UnsubscribeAimleMLModelRetrievalSubscription

tags:

- Individual AIMLE ML Model Retrieval Subscription (Document).

responses:

'204':

description: >

The individual AIMLE ML Model Retrieval Subscription resource matching the

subscriptionId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

MLMdlRetSub:

description: Represents the AIMLE ML Model Retrieval subscription information.

type: object

properties:

requestorId:

type: string

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

modelSelCrit:

$ref: 'TS29482_MLR_MLModelManagement.yaml#/components/schemas/MLModel'

expTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- requestorId

- notifUri

MLMdlRetSubPatch:

description: Represents the requested modifications to the AIMLE ML Model Retrieval subscription information.

type: object

properties:

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

modelSelCrit:

$ref: 'TS29482_MLR_MLModelManagement.yaml#/components/schemas/MLModel'

expTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

MlMdlRetNotif:

description: Represents the AIMLE ML Model Retrieval notification.

type: object

properties:

subId:

type: string

mlModels:

type: array

items:

$ref: '#/components/schemas/MLModelDetail'

minItems: 1

required:

- subId

MLMdlRetReq:

description: Represents the AIMLE ML Model Retrieval request information.

type: object

properties:

requestorId:

type: string

modelSelCrit:

$ref: 'TS29482_MLR_MLModelManagement.yaml#/components/schemas/MLModel'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

MLMdlRetRsp:

description: Represents the AIMLE ML Model Retrieval response information.

type: object

properties:

mlModels:

type: array

items:

$ref: '#/components/schemas/MLModelDetail'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- mlModels

MLModelDetail:

description: Represents the ML Model information.

type: object

properties:

mlModelId:

type: string

mlMdlEndpoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

mlModel:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Bytes'

required:

- mlModelId

oneOf:

- required: [mlMdlEndpoint]

- required: [mlModel]

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

* * * End of Changes * * *