3GPP TSG CT WG3 Meeting #144	C3-255xxx

# Dallas, US, 17th – 21st November, 2025														was C3-255213

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.569 | CR | 0011 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Defining the AIoT message size related restrictions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  | 2025-11-20 |
| Category: | B |  |  |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  | As clarified by RAN2 in their LS in R2-2507932, the maximum size of an AIoT NAS message (and consequently the maximum size of AIoT application data) that can be transported by AS is 119 octets in the R2D direction and 125 octets in the D2R direction. This result can be obtained/concluded from the corresponding message specification in 3GPP TS 38.391. C1-257026 to TS 24.369 is submitted to the parallel CT1#158 meeting to align the maximum AIoT NAS message size, with the maximum allowed AIoT application data length set to 99 for both read and write commands. The necessary alignments to the application data length that can be set by the AF/NEF should hence be specified by CT3. |  |  |  |  |  |  |
| Summary of change: |  |  | This CR proposes to: - Address the above-detailed issues. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | - The above-detailed issues are not addressed and the definition of the related Ambient IoT requirements/provisions is not completed. |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.3.2, 6.1.6.2.4, 6.1.7.3 |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  | This CR does not impact the OpenAPI descriptions of the APIs. |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 9113: "HTTP/2".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 9457: "Problem Details for HTTP APIs".

[14]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things; Stage 2".

[15]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[16]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[17]	3GPP TS 29.122: "T8 reference point for Northbound Application Programming Interfaces (APIs)".

[18]	3GPP TS 24.369: "Ambient IoT Non-Access-Stratum (AIoT NAS) protocol for 5G System (5GS); Stage 3".

* * * * Next changes * * * *

##### 5.2.2.3.2	AIoT Command Request

Figure 5.2.2.3.2-1 depicts a scenario where an NF service consumer requests to perform an AIoT Command operation to the AIOTF (see also clause 6.2.3 of 3GPP TS 23.369 [14]).

Figure 5.2.2.3.2-1: AIoT Command Request

1.	In order to request to perform an AIoT Command operation, the NF service consumer shall send an HTTP POST request message to the AIOTF targeting the URI of the corresponding custom operation (i.e., "CommandRequest"), with the request body containing the CommandReq data structure.

2a.	Upon reception of the Command request from the NF service consumer:

-	the AIOTF may perform the AF authorization for AIoT Services procedure as defined in clauses 5.6 and 6.2.3 of 3GPP TS 23.369 [14]; and

-	if the AF authorization for AIoT Services procedure is successful and upon successful processing of the request, the AIOTF shall respond to the NF service consumer with an HTTP "200 OK" status code to indicate that the AIoT Command request is successfully received and processed, with the response body containing AIoT Command related information within the CommandResp data structure.

2b.	On failure, the AIOTF shall take proper error handling actions, as specified in clause 6.1.7, and respond to the NF service consumer with an appropriate error status code. In particular:

-	if AF authorization for AIoT Services procedure is not successful, the AIOTF shall reject the request with an HTTP "403 Forbidden" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "AF_NOT_AUTHORIZED" application error;

-	if the target(s) of the AIoT Command request (e.g., target AIoT device(s), filtering information) is/are not supported and/or not allowed, the AIOTF shall reject the request with an HTTP "403 Forbidden" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "AIOT_TARGETS_ERROR" application error;

-	if the provided length of application data is too long (e.g., above the allowed maximum value), the AIOTF shall reject the request with an HTTP "403 Forbidden" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "APP_DATA_TOO_LONG" application error; and

-	if the AIOTF fails to process the AIoT Command request, the AIOTF shall reject the request with an HTTP "500 Internal Server Error" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "UNSPECIFIED_FAILURE" application error.

* * * * Next changes * * * *

##### 6.1.6.2.4	CommandReq

Table 6.1.6.2.4-1: Definition of type CommandReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Contains the identifier of the AF that triggered the request. |  |
| commandType | CommandType | M | 1 | Contains the type of the requested command. |  |
| targetArea | AiotArea | C | 0..1 | Contains the target area within which the requested Command operation shall apply. (NOTE 1) |  |
| targetDevices | AIoTDevices | C | 0..1 | Contains the target AIoT device(s) related information. (NOTE 1) |  |
| numDevices | Uinteger | O | 0..1 | Contains the approximative number of the targeted AIoT device(s). |  |
| msgSize | Uinteger | O | 0..1 | Contains the approximative message size in units of Bytes. This attribute may be present only if the "commandType" attribute is set to "READ". |  |
| offset | Uinteger | C | 0..1 | Contains the offset, expressed in units of bytes. This attribute shall be present only if the "commandType" attribute is set to "READ" or "WRITE": - If the "commandType" attribute is set to "READ", this attribute contains the offset from which to read the application data. - If the "commandType" attribute is set to "WRITE", this attribute contains the offset from which to write the application data. |  |
| length | Uinteger | C | 0..1 | Contains the length of application data, expressed in units of bytes (i.e., byte length). This attribute shall be present only if the "commandType" attribute is set to "READ" or "WRITE": - If the "commandType" attribute is set to "READ", this attribute contains the length of application data to read. - If the "commandType" attribute is set to "WRITE", this attribute contains the length of application data to write. (NOTE 2) |  |
| data | Bytes | C | 0..1 | Contains the application data to write. This attribute shall be present only if the "commandType" attribute is set to "WRITE". |  |
| notifUri | Uri | M | 1 | Contains the URI via which the AIoT Command operation related notifications shall be delivered. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.1.8. This attribute shall be present only when feature negotiation is required. |  |
| NOTE 1: At least one of these attributes shall be present. NOTE 2: The maximum value that this attribute can contain shall be as specified in clause 7.2.4 of 3GPP TS 24.369 [18]. |  |  |  |  |  |

* * * * Next changes * * * *

#### 6.1.7.3	Application Errors

The application errors defined for the Naiotf_AIoT service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| AF_NOT_AUTHORIZED | 403 Forbidden | The request for AIoT services is rejected because the AF is not authorized for the requested AIoT Services. |  |
| AIOT_TARGETS_ERROR | 403 Forbidden | The request for AIoT services is rejected because the target(s) of the AIoT request (e.g., target AIoT device(s), filtering information) is/are not supported and/or not allowed. |  |
| INVALID_AGGR_TIME_INVERTAVAL | 403 Forbidden | The AIoT Inventory request is rejected because the provided aggregation time interval is invalid. |  |
| APP_DATA_TOO_LONG | 403 Forbidden | The AIoT Command request is rejected because the provided length of application data is too long (e.g., above the allowed maximum value). |  |
| UNSPECIFIED_FAILURE | 500 Internal Server Error | The request for AIoT services is rejected because the AIOTF failed to process it for an unspecified reason. |  |

* * * * End of changes * * * *