3GPP TSG CT WG3 Meeting #144	C3-255xxx

# Dallas, US, 17th – 21st November, 2025														was C3-255225

Source:	Huawei

Title:	Pseudo-CR on updates and corrections to the Data Model clauses

Spec:	3GPP TS 29.566

Agenda item:	19.60 (EnergySys)

Document for:	Agreement

1. Introduction

The following issues still need to be addressed in the data model of the Neif_EventExposure API:

-	The reply LS from SA2 in S2-259467 and the related agreed CR#6341 to TS 23.501 in S2-259466, in which it is clarified that the time period to be used for threshold-based reporting is a time duration and the time window does not apply to threshold-based reporting (i.e., only applicable hence to periodic reporting).

-	The key of the map-encoded "eventsSubscSets" attribute should rather be set to the value of the "subscSetIdevent" attribute, not the "event" attribute of the corresponding map value encoded using the EnergyEeSubscSet data structure. This way, it is possible to subscribe to the same event with different reporting requirements.

-	Various additional corrections to the data model definition.

2. Reason for Change

Update the new TS to apply the necessary updates/corrections to fix the above-detailed issues.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.566 V 1.2.0.

* * * * Start of changes * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Neif_EventExposure service-based interface protocol.

Table 6.1.6.1-1: Neif_EventExposure specific Data Type

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| EnergyEeEvent | 6.1.6.3.3 | Represents the Energy Exposure event. |  |
| EnergyEeNotif | 6.1.6.2.4 | Represents the Energy Event Exposure Notification. |  |
| EnergyEeReport | 6.1.6.2.6 | Represents the Energy Event Exposure report. |  |
| EnergyEeSubsc | 6.1.6.2.2 | Represents the Energy Event Exposure Subscription. |  |
| EnergyEeSubscPatch | 6.1.6.2.3 | Represents the requested modifications to an Energy Event Exposure Subscription. |  |
| EnergyEeSubscSet | 6.1.6.2.5 | Represents the Energy Event Exposure Subscription set. |  |

Table 6.1.6.1-2 specifies data types re-used by the Neif_EventExposure service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Neif_EventExposure service-based interface.

Table 6.1.6.1-2: Neif_EventExposure re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ApplicationId | 3GPP TS 29.571 [14] | Represents the application identifier. |  |
| DateTime | 3GPP TS 29.571 [14] | Represents a date and a time. |  |
| Dnn | 3GPP TS 29.571 [14] | Represents a DNN. |  |
| DurationSec | 3GPP TS 29.571 [14] | Represents a time duration in units of seconds. |  |
| EnergyInfo | 3GPP TS 29.122 [17] | Represents the Energy consumption information. |  |
| FlowDescription | 3GPP TS 29.514 [16] | Represents an IP flow description. |  |
| Gpsi | 3GPP TS 29.571 [14] | Represents a GPSI. |  |
| RedirectResponse | 3GPP TS 29.571 [14] | Represents redirection related information. |  |
| ReportingInformation | 3GPP TS 29.523 [18] | Represents the reporting requirements. |  |
| Snssai | 3GPP TS 29.571 [14] | Represents an S-NSSAI. |  |
| Supi | 3GPP TS 29.571 [14] | Represents a SUPI. |  |
| SupportedFeatures | 3GPP TS 29.571 [14] | Represents the list of supported feature(s) and is used to negotiate the applicability of optional features. |  |
| TimeWindow | 3GPP TS 29.122 [17] | Represents a time window. |  |
| Uinteger | 3GPP TS 29.571 [14] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.571 [14] | Represents a URI. |  |

* * * * Next changes * * * *

##### 6.1.6.2.2	Type: EnergyEeSubsc

Table 6.1.6.2.2-1: Definition of type EnergyEeSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | M | 1 | Contains the URI via which Energy consumption information related event(s) reporting shall be delivered. |  |
| eventsSubscSets | map(EnergyEeSubscSet) | M | 1..N | Contains the set(s) of Energy Event Exposure subscription related details. The key of the map shall be set to the value of the "subscSetId" attribute of the corresponding map value encoded using the EnergyEeSubscSet data structure. |  |
| repReqs | ReportingInformation | O | 0..1 | Contains the reporting requirements of the subscription. |  |
| reports | array(EnergyEeReport) | C | 1..N | Contains the Energy Event Exposure report(s). This attribute shall be present only when immediate reporting was requested in the corresponding subscription request and there are available report(s). |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features used as defined in clause 6.1.8. This attribute shall be present only when feature negotiation is required. |  |

* * * * Next changes * * * *

##### 6.1.6.2.3	Type: EnergyEeSubscPatch

Table 6.1.6.2.3-1: Definition of type EnergyEeSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | O | 0..1 | Contains the updated URI via which Energy consumption information related event(s) reporting shall be delivered. |  |
| eventsSubscSets | maps(EnergyEeSubscSet) | O | 1..N | Contains the set(s) of Energy Event Exposure subscription related details. The key of the map shall be set to the value of the "subscSetId" attribute of the corresponding map value encoded using the EnergyEeSubscSet data structure. |  |
| repReqs | ReportingInformation | O | 0..1 | Contains the reporting requirements of the subscription. |  |

* * * * Next changes * * * *

##### 6.1.6.2.5	Type: EnergyEeSubscSet

Table 6.1.6.2.5-1: Definition of type  EnergyEeSubscSet

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | EnergyEeEvent | M | 1 | Contains the subscribed Energy related event. |  |
| subscSetId | string | M | 1 | Contains the identifier of the Energy event exposure subscription set. |  |
| supi | Supi | C | 0..1 | Contains the target SUPI. (NOTE 1) |  |
| gpsi | Gpsi | C | 0..1 | Contains the target GPSI. (NOTE 1) |  |
| dnn | Dnn | C | 0..1 | Contains the target DNN. (NOTE 2) |  |
| snssai | Snssai | C | 0..1 | Contains the target S-NSSAI. (NOTE 2, NOTE 4) |  |
| appId | ApplicationId | C | 0..1 | Contains the identifier of the target application. (NOTE 3) |  |
| flowDescs | array(FlowDescription) | C | 1..N | Contains the service flow description information for the target application. (NOTE 3) |  |
| repTimeWin | TimeWindow | O | 0..1 | Contains the reporting time window, i.e., the start time and end time during which data collection for the reporting shall take place. This attribute shall be present only in case time window based reporting is requested. (NOTE 6, NOTE 7) |  |
| enrgRepThres | EnergyInfo | O | 0..1 | Contains the reporting thresholds for the Energy consumption information event exposure. This attribute shall be present only in case threshold-based reporting is requested. (NOTE 7) |  |
| repPeriodThres | DurationSec | O | 0..1 | Contains the reporting period, i.e., the time period during which the thresholds provided within the "enrgRepThres" shall be crossed to trigger the reporting. This attribute may be present only in case threshold-based reporting is requested, i.e., in case the "enrgRepThres" is also present. |  |
| repReqs | ReportingInformation | O | 0..1 | Contains the reporting requirements of the subscription set. The "repPeriod" attribute within this attribute shall be present only in case periodic reporting is requested. (NOTE 5, NOTE 7) |  |
| NOTE 1: These attributes are mutually exclusive and only one of them shall be present. NOTE 2: If the value of the "event" attribute is set to "PDU_SESSION_ENERGY" or "SERVICE_FLOW_ENERGY", then at least one of these attributes shall be present. NOTE 3: If the value of the "event" attribute is set to "SERVICE_FLOW_ENERGY", then only one of these attributes shall be present. NOTE 4: if the value of the "event" attribute is set to "UE_SNSSAI_ENERGY", then this attribute shall be present. NOTE 5: When this attribute is present, the event reporting requirements provided within this attribute shall take precedence over the common events reporting requirements provided within the "repReqs" attribute of the parent EnergyEeSubsc data structure. NOTE 6: In this Release of the specification, the values of both the start time and end time within the "repTimeWin" attribute shall be in the future. NOTE 7: These attributes and the "repPeriod" attribute within the "repReqs" attribute are mutually exclusive. |  |  |  |  |  |

* * * * Next changes * * * *

##### 6.1.6.2.6	Type: EnergyEeReport

Table 6.1.6.2.6-1: Definition of type EnergyEeReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | EnergyEeEvent | M | 1 | Contains the reported Energy related event. |  |
| subscSetId | string | M | 1 | Contains the identifier of the Energy event exposure subscription set to which this report is related. |  |
| timeStamp | DateTime | M | 1 | Contains the time at which the reported event within the "event" attribute is observed. |  |
| timeWindow | TimeWindow | O | 0..1 | Contains the time window representing the data collection period. |  |
| energyInfo | EnergyInfo | O | 0..1 | Contains the energy consumption information report. |  |

* * * * Next changes * * * *

##### 6.1.6.3.3	Enumeration: EnergyEeEvent

The enumeration EnergyEeEvent represents the Energy Exposure event. It shall comply with the provisions defined in table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration EnergyEeEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UE_ENERGY | Indicates that the Energy Exposure event is the energy consumed by a UE. |  |
| PDU_SESSION_ENERGY | Indicates that the Energy Exposure event is the energy consumed by a PDU session of a UE. |  |
| SERVICE_FLOW_ENERGY | Indicates that the Energy Exposure event is the energy consumed by an application or service data flow of an application traffic of a UE. |  |
| UE_SNSSAI_ENERGY | Indicates that the Energy Exposure event is the energy consumed by a UE in an S-NSSAI. |  |

* * * * End of changes * * * *