3GPP TSG-CT WG3 Meeting #144	C3-255569

# Dallas, USA, 17-21 November 2025

Title:	LS on Signalling Storm Analytics

Release:	Release 19

Work Item:	AIML_CN

Source:	CT3

To:	SA2

Cc:	CT4

Contact person:	Shiramizu Takashi

takashi.shiramizu.gx@nttdocomo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

TS 23.288 details the input and output of timer information as a necessary component of the signalling storm mitigation strategy (e.g., in clause 6.22). Furthermore, TS 29.520 has already defined the specific timer information types, including the back-off timer (timerType: BACKOFF_TIMER), for the signalling storm analytics information provided to the NF consumers.

When this signalling storm analytics information is consumed by e.g., an SMF, applying a back-off timer requires explicit knowledge of the affected session context, namely the DNN and S-NSSAI. The lack of a defined structure to carry these congestion control parameters alongside the BACKOFF_TIMER information prevents the SMF from accurately implementing the mitigation actions.

CT3 has discussed the Stage 3 implementation of timer information association to the DNN and/or S-NSSAI for both input (data collection from SMF/AMF) and output (analytics delivery from NWDAF) concerning timer information.

However, CT3 could not reach consensus on whether this timer information requires the DNN and/or S-NSSAI association.

Question: CT3 requires confirmation from SA2 on whether the DNN and/or S-NSSAI association is required for the input and output timer information if the timer information is SM NAS related timer information (e.g. back-off).

CT3 kindly asks SA2 to provide answer for the above question.

2	Actions

To SA2

ACTION: 	CT3 kindly asks SA2 to provide answer for the above question.

3		Dates of next TSG CT WG 3 meetings

CT3#145	09-13 February 2026	India

CT3#146	13-17 April 2026 		Malta