3GPP TSG-CT WG3 Meeting #144	C3-255309

# Dallas, United States, 17 - 21 November 2025

Source:	China Mobile

Title:	Pseudo-CR on Miscellaneous corrections for MMTel Enabler Server

Spec:	3GPP TS 29.392 v1.0.0

Agenda item:	19.62

Document for:	Approval

1. Introduction

This pCR introduce Miscellaneous corrections for MMTel Enabler Server service.

2. Reason for Change

There are some errors in the specification, e.g., the guidance still exists, uncorrect reference to the TSs, etc. These errors need to be corrected.

3. Conclusions

Correct the errors listed above.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.392 v1.0.0.

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

4	Overview

3GPP TS 23.392 [6] has specified the application layer architecture, architectural requirements, procedures, information flows, in order to support the Application enablement for MMTel Service, mainly including DC Application management, DC Application downloading control, MMTel service usage and Multiple call control handling.

The present document specifies the APIs needed to support MMTel Services for interworking between the MMTel Enabler Server and the Controlling Application Server or Application Server, including the following functionalities:

1.	Server-side functionality with the DC application configuration, update, deletion and information query, provided by the MMTel Enabler Server over the MMTel-2 interface.

2.	Server-side functionality to provide capabilities to application providers/Vertical service providers to use MMTel services over the MMTel-3 interface.

Figure 4-1 shows the reference model of the MMTel Application Enabler Layer, with a focus on the MMTel Enabler Server:

Figure 4-1: MMTel Application Enabler Layer functional model

# *** Next Change ***

5.2.1	Service Description

The MMTel_DCAppManagement Service as defined in clause 8.2 in 3GPP TS 23.392 [6], is provided by the MMTel Enabler Server.

This service:

-	allows Controlling Application Server invokes services provided by a MMTel Enabler Server to config DC application and profile to the MMTel Enabler Server;

-	allows Controlling Application Server invokes services provided by a MMTel Enabler Server to update DC application and profile to the MMTel Enabler Server.

-	allows Controlling Application Server invokes services provided by a MMTel Enabler Server to delete DC application.

-	allows Controlling Application Server invokes services provided by a MMTel Enabler Server to obtain detail DC application profile information on the MMTel Enabler Server.

# *** Next Change ***

5.2.2.1	Introduction

The service operations defined for MMTel_DCAppManagement Service is shown in the table 5.2.2.1-1

Table 5.2.2.1-1: Operations of the MMTel_DCAppManagement Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| MMTel_DCAppManagement_Configure | This service operation is used by the service consumer to configure DC application and profile to the MMTel Enabler Server. | e.g., Controlling Application Server |
| MMTel_DCAppManagement_Update | This service operation is used by Controlling Application Server to update an existing DC application profile information or DC application. | e.g., Controlling Application Server |
| MMTel_DCAppManagement_Delete | This service operation is used by the service consumer to delete an existing DC application. | e.g., Controlling Application Server |
| MMTel_DCAppManagement_Retrieval | This service operation is used by the service consumer to retrieve the details of an existing DC application profile information. | e.g., Controlling Application Server |

# *** Next Change ***

5.2.2.2.2	DC Application and Profile Configuration

Figure 5.2.2.2.2-1 depicts a scenario where a Controlling Application Server sends a request to the MMTel Enabler Server to configure DC application and profile (see also clause 8.2.2 of 3GPP TS 23.392 [6]).

Figure 5.2.2.2.2-1: Procedure for DC Application and Profile Configuration

1.	In order to configure DC application and profile, the Controlling Application Server shall send an HTTP POST request (i.e., custom operation "Configure") to the MMTel Enabler Server targeting the "DC APP" collection resource, with the request body including the DcAppConfigReq data structure.

2a.	Upon success, the MMTel Enabler Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "DC APP" resource and potentially additional information within the DcAppConfigResp data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.7.

# *** Next Change ***

5.2.2.3.2	DC Application and Profile Update

Figure 5.2.2.3.2-1 depicts a scenario where a Controlling Application Server sends a request to the MMTel Enabler Server to update existing DC application and profile (see also clause 8.2.3 of 3GPP TS 23.392 [6]).

Figure 5.2.2.3.2-1: Procedure for DC Application and Profile Update

1.	In order to update existing DC application or DC application profile related information, the Controlling Application Server shall send an HTTP POST request (i.e., custom operation "Update") to the MMTel Enabler Server targeting the corresponding "DC APP" resource, with the request body including the DcAppUpdateReq data structure.

2a.	Upon success, the MMTel Enabler Server shall respond with an HTTP "200 OK" status code with the response body containing a representation of the updated "DC APP" resource within the DcAppStatResp data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.7.

# *** Next Change ***

5.2.2.4.2	DC Application and Profile Deletion

Figure 5.2.2.4.2-1 depicts a scenario where a Controlling Application Server sends a request to the MMTel Enabler Server to delete existing DC application and profile (see also clause 8.2.4 of 3GPP TS 23.392 [6]).

Figure 5.2.2.4.2-1: Procedure for DC Application and Profile Deletion

1.	In order to configure DC application and profile, the Controlling Application Server shall send an HTTP POST request (i.e., custom operation "Delete") to the MMTel Enabler Server targeting the "DC APP" collection resource, with the request body including the DcAppIdReq data structure;

2.	Upon success, the MMTel Enabler Server shall respond with an HTTP "200 OK" status code with the response body containing a representation of the updated "DC APP" resource within the DcAppStatResp data structure.

3.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.7.

# *** Next Change ***

5.2.2.5.2	DC Application and Profile Retrieval

Figure 5.2.2.5.2-1 depicts a scenario where a Controlling Application Server sends a request to the MMTel Enabler Server to retrieve DC application profile related information (see also clause 8.2.5 of 3GPP TS 23.392 [6]).

Figure 5.2.2.5.2-1: Procedure for DC Application and Profile Retrieval

1.	In order to retrieve existing DC application profile related information, the Controlling Application Server shall send an HTTP POST request(i.e., custom operation "Retrieval") to the MMTel Enabler Server targeting the "DC APP" collection resource, with the request body including the DcAppIdReq data structure.

2.	Upon success, the MMTel Enabler Server shall respond with an HTTP "200 OK" status code with the response body containing a representation of the retrievaled "DC APP" resource within the DcAppIdResp data structure.

3.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP GET response body, as specified in clause 6.1.7.

# *** Next Change ***

5.3.1	Service Description

The MMTel_ DCAppCall Service as defined in clause 8.4 in 3GPP TS 23.392 [6], is provided by the MMTel Enabler Server.

This service:

-	allows Application Server invokes the service provided by a MMTel Enabler Server to establish a Third-Party Call with Data Channel capability;

-	allows Application Server invokes services provided by a MMTel Enabler Server to add an A2P Data Channel to an existing IMS session for establishment of an Application Call with Data Channel capability.

-	allows the MMTel Enabler Server to notify to the Application Server about DC resource information.

# *** Next Change ***

# *** Next Change ***

6.1.1	Introduction

The MMTel_DCAppManagement shall use the MMTel_DCAppManagement API.

The API URI of the MMTel_DCAppManagement shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [2], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

-	The <apiName> shall be "mmtel-dcappmgmt".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

# *** Next Change ***

6.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.3.1-1 depicts the resource URIs structure for the MMTel_DCAppManagement API.

Figure 6.1.3.1-1: Resource URI structure of the MMTel_DCAppManagement API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| DC APP | /dcapps/configure | POST | Configure DC application and DC application profile. |
|  | /dcapps/update | POST | Update existing DC application and DC application profile. |
|  | /dcapps/delete | POST | Delete existing DC application |
|  | /dcapps/retrival | POST | Obtain the DC APP profile related information according to the APPID |

# *** Next Change ***

6.1.3.2.3.1	Void

# *** Next Change ***

6.1.3.2.4.1	Overview

Table 6.1.3.2.4.1-1: Custom operations

| Operation name | Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Configure | /dcapps/configure | POST | Enables to configure DC application and DC application profile. |
| Update | /dcapps/update | POST | Enables to update existing DC application and DC application profile. |
| Delete | /dcapps/delete | POST | Enables to delete existing DC application and DC application profile. |
| Retieval | /dcapps/retrieval | POST | Enables to retrieval existing DC application and DC application profile. |

# *** Next Change ***

6.1.3.2.4.2	Operation: Configure

This custom operation enables to configure DC application and DC application profile.

This operation shall support the request data structures specified in table 6.1.3.2.4.2-1 and the response data structure and response codes specified in table 6.1.3.2.4.2-2.

Table 6.1.3.2.4.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DcAppConfigReq | M | 1 | Represents the DC application and DC application profile configuration information. |

Table 6.1.3.2.4.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DcAppConfigResp | M | 1 | 201 Created | Indicates the successfully configured list of APPID. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codse for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.3.2.4.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

Table 6.1.3.2.4.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

# *** Next Change ***

6.1.3.2.4.3	Operation: Update

This custom operation enables to update existing DC application and DC application profile.

This operation shall support the request data structures specified in table 6.1.3.2.4.3-1 and the response data structure and response codes specified in table 6.1.3.2.4.3-2.

Table 6.1.3.2.4.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DcAppUpdateReq | M | 1 | Represents the DC application and DC application profile information that shall be updated. |

Table 6.1.3.2.4.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DcAppStatResp | M | 1 | 200 OK | Indicates the successfully updated list of APPID. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 6.1.3.2.4.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

Table 6.1.3.2.4.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

# *** Next Change ***

6.1.3.2.4.4	Operation: Delete

This custom operation enables to delete existing DC application.

This operation shall support the request data structures specified in table 6.1.3.2.4.4-1 and the response data structure and response codes specified in table 6.1.3.2.4.4-2.

Table 6.1.3.2.4.4-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DcAppIdReq | M | 1 | Represents the DC application and DC application that shall be deleted. |

Table 6.1.3.2.4.4-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DcAppIdResp | M | 1 | 200 OK | Indicates the successfully deleted list of APPID. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.3.2.4.4-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

Table 6.1.3.2.4.4-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

# *** Next Change ***

6.1.3.2.4.5	Operation: Retrival

This custom operation enables to retrival existing DC application.

This operation shall support the request data structures specified in table 6.1.3.2.4.5-1 and the response data structure and response codes specified in table 6.1.3.2.4.5-2.

Table 6.1.3.2.4.5-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DcAppIdReq | M | 1 | Represents the requested DC application information. |

Table 6.1.3.2.4.5-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DcAppIdResp | M | 1 | 200 OK | The requested DC application profile related information is returned |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.3.2.4.5-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

Table 6.1.3.2.4.5-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

# *** Next Change ***

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined specifically for the MMTel_DCAppManagement API service.

Table 6.1.6.1-1: MMTel_DCAppManagement API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AppLoadPhase | 6.1.6.3.3 | Represents the load phase of the DC application |  |
| Condition | 6.1.6.3.6 | Represents the conditions used by the DC application |  |
| DcAppConfigParameters | 6.1.6.2.3 | Represents the parameters of single-DC applicaion in the configuration request |  |
| DcAppConfigReq | 6.1.6.2.2 | Represents the DC application and profile configuration request |  |
| DcAppConfigResp | 6.1.6.2.4 | Represents the DC application and profile configuration response |  |
| DcAppConfigResponseParameters | 6.2.6.2.5 | Represents the parameters of single-DC applicaion in the configuration response |  |
| DcAppIdReq | 6.1.6.2.10 | Represents the DC application delete request and DC application information retrieval request |  |
| DcAppIdResp | 6.1.6.2.11 | Represents the DC application profile information retrieval response |  |
| DcAppResponseParameters | 6.1.6.2.9 | Represents the parameters of single-DC applicaion in the DC application and profile update response and DC application delete response |  |
| DcAppStatResp | 6.1.6.2.8 | Represents the DC application and profile update response or DC application delete response |  |
| DcAppUpdateReq | 6.1.6.2.6 | Represents the DC application and profile update request |  |
| DcAppUpdateParameters | 6.1.6.2.7 | Represents the parameters of single-DC applicaion in the update response |  |
| Status | 6.1.6.3.5 | Represents the request return status |  |
| SupportScenario | 6.1.6.3.4 | Represents the support scenario of the DC application |  |

Table 6.1.6.1-2 specifies data types re-used by theMMTel_DCAppManagement API service.

Table 6.1.6.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Binary | 3GPP TS 29.122 [3] | Used to represent binary values. |  |
| DateTime | 3GPP TS 29.122 [3] | Used to represent the subscription duration. |  |
| Uri | 3GPP TS 29.122 [3] | Used to indicate the notification URI. |  |

# *** Next Change ***

6.1.6.2.12	Void

# *** Next Change ***

6.1.6.3.6	Enumeration: 	Condition

Table 6.1.6.3.6-1: Definition of type Condition

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| CONDTY | The Data Channel Application is allowed to be used in this condition, e.g. Service area |  |
| CONDVA | The value of the CONFTY. |  |

# *** Next Change ***

# *** Next Change ***

A.2	MMTel_DCAppManagement API

openapi: 3.0.0

info:

title: MMTel DC Application Management Service

version: 1.0.0-alpha.1

description: |

MMTel DC Application Management Service.

© <2025>, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.392 V19.0.0;

Application layer support for MMTel;

MMTel Enabler Server Services;Stage 3.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.392/

servers:

- url: '{apiRoot}/mmtel-dcappmgmt/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/dcapps/configure:

post:

summary: Request the creation of a new DC application management configuration.

operationId: CreateDCAppMgmt

tags:

- DC Application Management Configurations

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DcAppConfigReq'

responses:

'201':

description: >

Created. The DC application and DC application profile configuration is successfully

created and a representation of the created Individual DC application and DC application

profile Configuration shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/DcAppConfigResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/dcapps/update:

post:

summary: Request the updae of a DC application management configuration.

operationId: UpdateDCAppMgmt

tags:

- DC Application Management Configurations Update

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DcAppUpdateReq'

responses:

'200':

description: >

Updated. The DC application and DC application profile configuration is successfully

updated.

content:

application/json:

schema:

$ref: '#/components/schemas/DcAppStatResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/dcapps/delete:

post:

summary: Request the delete of a DC application management configuration.

operationId: DeleteDCAppMgmt

tags:

- DC Application Management Configurations Delete

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DcAppIdReq'

responses:

'200':

description: >

Deleted. The DC application and DC application profile configuration is successfully

deleted.

content:

application/json:

schema:

$ref: '#/components/schemas/DcAppIdResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/dcapps/retrieval:

post:

summary: Request the retrieval of a DC application management configuration.

operationId: RetrievalDCAppMgmt

tags:

- DC Application Management Configurations Retrieval

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DcAppIdReq'

responses:

'200':

description: >

The requested DC application profile related information is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/DcAppIdResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

DcAppConfigReq:

description: >

Represents the DC application and profile configuration request.

type: object

properties:

reqId:

type: string

secCred:

type: string

dcAppNum:

type: integer

dcAppConfigParamList:

type: array

items:

$ref: '#/components/schemas/DcAppConfigParameters'

minItems: 1

required:

- reqId

- dcAppNum

- dcAppConfigParamList

DcAppConfigParameters:

description: >

Represents the parameters of single-DC applicaion in the configuration request.

type: object

properties:

appIndex:

type: string

appName:

type: string

svcType:

type: string

appIconUrl:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

appVer:

type: string

appVal:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

appLoadPh:

$ref: '#/components/schemas/AppLoadPhase'

autoload:

type: boolean

autolaunch:

type: boolean

peerDcReq:

type: boolean

suppScnr:

$ref: '#/components/schemas/SupportScenario'

cond:

$ref: '#/components/schemas/Condition'

qosReq:

type: string

persDataColl:

type: boolean

persDataCollInfoUrl:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

appPkg:

type: binary

required:

- appIndex

DcAppConfigResp:

description: >

Represents the DC application and profile configuration response.

type: object

properties:

dcAppConfigRespList:

type: array

items:

$ref: '#/components/schemas/DcAppConfigResponseParameters'

minItems: 1

required:

- dcAppConfigRespList

DcAppConfigResponseParameters:

description: >

Represents the parameters of single-DC applicaion in the configuration response.

type: object

properties:

appIndex:

type: string

status:

$ref: '#/components/schemas/Status'

appId:

type: string

failureCause:

type: string

required:

- appIndex

- status

DcAppUpdateReq:

description: >

Represents the DC application and profile update request.

type: object

properties:

reqId:

type: string

secCred:

type: string

dcAppNum:

type: integer

dcAppUpdateParamList:

type: array

items:

$ref: '#/components/schemas/DcAppUpdateParameters'

minItems: 1

required:

- reqId

- dcAppNum

- dcAppUpdateParamList

DcAppUpdateParameters:

description: >

Represents the parameters of single-DC applicaion in the update response.

appId:

type: string

appName:

type: string

svcType:

type: string

appIconUrl:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

appVer:

type: string

appVal:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

appLoadPh:

$ref: '#/components/schemas/AppLoadPhase'

autoload:

type: boolean

autolaunch:

type: boolean

peerDcReq:

type: boolean

suppScnr:

$ref: '#/components/schemas/SupportScenario'

cond:

$ref: '#/components/schemas/Condition'

qosReq:

type: string

persDataColl:

type: boolean

persDataCollInfoUrl:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

appPkg:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Binary'

required:

- appId

DcAppStatResp:

description: >

Represents the DC application and profile update response or DC application delete response.

type: object

properties:

dcAppStatRespList:

type: array

items:

$ref: '#/components/schemas/DcAppResponseParameters'

minItems: 1

required:

- dcAppStatRespList

DcAppResponseParameters:

description: >

Represents the parameters of single-DC applicaion in the DC application and

profile update response and DC application delete response.

type: object

properties:

appId:

type: string

status:

$ref: '#/components/schemas/Status'

failureCause:

type: string

required:

- appId

- status

DcAppIdReq:

description: >

Represents the DC application delete request and DC application information

retrieval request.

type: object

properties:

reqId:

type: string

secCred:

type: string

dcAppNum:

type: integer

appIdList:

type: array

items:

type: string

minItems: 1

required:

- reqId

- dcAppNum

- appIdList

DcAppIdResp:

description: >

Represents the DC application profile information retrieval response.

type: object

properties:

status:

$ref: '#/components/schemas/Status'

secCred:

type: string

dcAppInfoList:

type: array

items:

$ref: '#/components/schemas/DcAppUpdateParameters'

minItems: 1

required:

- status

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

AppLoadPhase:

anyOf:

- type: string

enum:

- PRECALL_ONLY

- INCALL

- PRECALL_AND_INCALL

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents the load phase of the DC application.

It complies with the provisions defined in Table 6.1.6.3.3-1 of 3GPP TS 29.392.

Possible values are:

- PRECALL_ONLY: Indicates the Data Channel Application is allowed to be used before

the MMTel call session is established.

- INCALL: Indicates that the Data Channel Application is allowed to be used after

the MMTel call session is established.

- PRECALL_AND_INCALL: Indicateds that the Data Channel Application is allowed to be

used during the entire Precall and incall.

SupportScenario:

anyOf:

- type: string

enum:

- VOICE_CALL_ONLY

- VIDEO_CALL_ONLY

- VOICE_AND_VIDEO_CALL

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents the support scenario of the DC application.

It complies with the provisions defined in Table 6.1.6.3.4-1 of 3GPP TS 29.392.

Possible values are:

- VOICE_CALL_ONLY: Indicates the Data Channel Application can be used if and only

if the corresponding call is a voice call.

- VIDEO_CALL_ONLY: Indicates that the Data Channel Application can be used if

and only if the corresponding call is a video call.

- VOICE_AND_VIDEO_CALL: Indicateds that the Data Channel Application can be used

in both voice call and video call.

Status:

anyOf:

- type: string

enum:

- SUCCESS

- FAILED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents the request return status.

It complies with the provisions defined in Table 6.1.6.3.5-1 of 3GPP TS 29.392.

Possible values are:

- SUCCESS: Indicates that the request is processed successfully.

- FAILED: Indicates that the request fails to be processed.

Condition:

anyOf:

- type: string

enum:

- CONDTY

- CONDVA

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents the conditions used by the DC application.

It complies with the provisions defined in Table 6.1.6.3.6-1 of 3GPP TS 29.392.

Possible values are:

- CONDTY: Indicates the Data Channel Application is allowed to be used in this

condition, e.g. Service area.

- CONDVA: Indicates the value of the CONFTY.

# *** Next Change ***

Annex B (informative):
Withdrawn API versions

B.1	General

This Annex lists withdrawn API versions of the APIs defined in the present specification. 3GPP TS 29.501 [5] clause 4.3.1.6 describes the withdrawal of API versions.

# *** Next Change ***

B.2	MMTel_DCAppManagement API

The API versions listed in table B.2-1 are withdrawn for the MMTel_DCAppManagement API.

Table B.2-1: Withdrawn API versions of the MMTel_DCAppManagement service

| API version number | Remarks |
| --- | --- |

# *** Next Change ***

B.3	MMTel_DCAppCall API

The API versions listed in table B.2-1 are withdrawn for the MMTel_DCAppCall API.

Table B.3-1: Withdrawn API versions of the MMTel_DCAppCall service

| API version number | Remarks |
| --- | --- |

# *** Next Change ***

B.4	MMTel_CallControlHandling API

The API versions listed in table B.2-1 are withdrawn for the MMTel_CallControlHandling API.

Table B.4-1: Withdrawn API versions of the MMTel_CallControlHandling service

| API version number | Remarks |
| --- | --- |

*** End of Changes ***