3GPP TSG-CT WG3 Meeting #144	C3-255336r1

# Dallas, United States, 17 - 21 November 2025

Source:	China Mobile

Title:	Pseudo-CR on DCAppCall and Call Control services

Spec:	3GPP TS 29.392 v1.0.0

Agenda item:	19.62

Document for:	Decision

1. Introduction

According to stage 2 definition, it is proposed to define the DCAppCall services, including service functions of establishing DC call, either as Application call or ThirdParty call specified in clause 8.4 in 3GPP TS 23.392, and update/notification of DC media of existing session. In addition, the call control service function defined in clause 8.5 in 3GPP TS 23.392 currently re-using API defined in TS 23.228 and is not expected included in this phase, such clauses is going to be removed.

2. Reason for Change

Define the DCAppCall services following stage 2 definition.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.392 v1.0.0.

* * * First Change * * * *

5.3.2.2	MMTel_DCAppCall_DCCallReq

5.3.2.2.1	General

This service operation is used by the service consumer to request the MMTel Enabler Server to establish a call with Data Channel capability. It supports two scenarios:

-	Application Call Scenario: A non-IMS application initiates a call with a single UE, specified in TS 23.392 clause 8.4.2;

-	Third-Party Call Scenario: Application Server triggers a call between two UEs, ,specified in TS 23.392 clause 8.4.3.

The following procedures are supported by the "MMTel_DCAppCall_DCCallReq" service operation:

-	allows Application Server invokes the service provided by a MMTel Enabler Server to establish a Data Channel capability enabled call;

-	allows Application Server invokes services provided by a MMTel Enabler Server to add an A2P Data Channel to an existing IMS session for establishment of a Data Channel capability enabled call.

-	allows the MMTel Enabler Server to notify to the Application Server about DC resource information.

5.3.2.2.2	DC Call Establishment

Figure 5.3.2.2.2-1 depicts a scenario where an Application Server sends a request to the MMTel Enabler Server to establish a Data Channel capability enabled call.

Figure 5.3.2.2.2-1: Procedure for DC Application Call Establishment

To establish a call with Data Channel capability, the Application Server shall send an HTTP POST request to the MMTel Enabler Server as specified in clause 8.4.2.2 or 8.4.3.2 of 3GPP TS 23.392[x] with the request body including the DCCallReq data structure as specified in 8.4.3.3.1i of 3GPP TS 23.392[x].

Upon reception of the HTTP POST request from the Application Server, the MMTel Enabler Server shall:

- Verify the service consumer’s identity and authorization (including verifying additional OMA Third-Party Call API authorization);

- if the service consumer is authorized to establish a DC call, the MMTel Enabler Server respond with an HTTP "200 OK " status code with the response body including the DCCallResp data structure as specified in 8.4.3.3.2 of 3GPP TS 23.392[x].

- On failure, the appropriate HTTP status code indicating the error shall be returned as specified in clause 6.2.7.

5.3.2.3	MMTel_DCAppCall_UpdateDCMedia

5.3.2.3.1	General

This service operation is used by the service consumer to request the MMTel Enabler Server to add an A2P Data Channel media to an existing IMS session.

5.2.2.3.2	DC Media Update

Figure 5.3.2.3.2-1 depicts the scenario where an Application Server sends a request to the MMTel Enabler Server to add A2P Data Channel media to an existing IMS session.

Figure 5.2.2.3.2-1: Procedure for DC Media Update

To add A2P Data Channel media, the Application Server shall send an HTTP POST request to the MMTel Enabler Server as specified in clause 8.4.4 of 3GPP TS 23.392[x], with the request body including the DCMediaUpdateReq data structure.

Upon reception of the HTTP POST request from the Application Server, the MMTel Enabler Server shall:

- Verify the service consumer’s identity and authorization;

- if the service consumer is authorized, the MMTel Enabler Server respond with an HTTP "200 OK" status code with the response body including DCMediaUpdateResp data structure

- On failure, the appropriate HTTP status code indicating the error shall be returned as specified in clause 6.2.6.

5.2.2.4	MMTel_DCAppCall_Notify

5.2.2.4.1	General

This service operation is used by the MMTel Enabler Server to notify the service consumer about DC media information updates.

5.2.2.4.2	DC Media Notification

Figure 5.3.2.4.2-1 depicts a scenario where the MMTel Enabler Server sends a notification to the Application Server about DC resource information (see also clause 8.4.4.1 of 3GPP TS 23.392 [6]).

Figure 5.2.2.4.2-1: Procedure for DC Application and Profile Deletion

1.	When DC resource information changes, the MMTel Enabler Server shall send an HTTP POST request to the notification endpoint provided by the Application Server, with the request body containing the updated DC resource information within the DCMediaNotifyReq data structure.

2.	The Application Server shall respond with an HTTP "200 OK" status code upon successful receipt of the notification with the response body including DCMediaNotifyResp data structure.

3.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information as specified in clause 6.2.7.

* * * Next Change * * * *

* * * End of Changes * * * *