3GPP TSG-CT WG3 Meeting #144	C3-255340r2

# Dallas, United States, 17 - 21 November 2025

Source:	China Mobile

Title:	Pseudo-CR on MMTel_DCAppCall API

Spec:	3GPP TS 29.392 v1.0.0

Agenda item:	19.62

Document for:	Decision

1. Introduction

According to stage 2 definition, it is proposed to define the MMTel_DCAppCall API, including operations of establishing DC call and updating/notifying DC media, corresponding to proposed service function .

2. Reason for Change

Define the MMTel_DCAppCall API (TS 29.392) for MMTel Enabler Server services.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.392 v1.0.0.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.122: "T8 reference point for Northbound Application Programming Interfaces (APIs)".

[3]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[4]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[5]	3GPP TR 21.900: "Technical Specification Group working methods".

[6]	3GPP TS 23.392: "Application enablement aspects for MMTel".

[7]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[8]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[9]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[10]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[11]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[y]	OMA-TS-REST_NetAPI_ThirdPartyCall-V1_0-20130212-C: "RESTful Network API Framework for Third Party Call ".

* * * First Change * * * *

6.2	MMTel_DCAppCall API

6.2.1	Introduction

The MMTel_DCAppCall shall use the MMTel_DCAppCall API.

The API URI of the MMTel_DCAppCall shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [x], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

-	The <apiName> shall be "mmtel-dcappcall".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

6.2.2	Usage of HTTP and common API related aspects

The provisions of clause 5.2 of 3GPP TS 29.122 [2] shall apply for the MMTel_DCAppCall API.

6.2.3	Resources

There are no resources defined for this API in this release of the specification.

6.2.4	Custom Operations without associated resources

6.2.4.1	Overview

Figure 6.2.4.1-1 depicts the resource URIs structure for the MMTel_DCAppCall API.

Figure 6.2.4.1-1: Resource URI structure of the MMTel_DCAppCall API

Table 6.2.4.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the MMTel_DCAppCall API.

Table 6.2.4.1-1: Resources and methods overview

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {apiRoot}/mmtel-dcappcall/<apiVersion>/dccall | POST | Request of AS to establish a DC capability enabled call to a given MMTel Enabler Server. |
| {apiRoot}/mmtel-dcappcall/<apiVersion>/update-dc-media | POST | Request of AS to update DC media of an application call to a given MMTel Enabler Server. |

6.2.4.2	Operation: update-dc-media

6.2.4.2.1	Description

This operation is used by the Application Server to request a call enabled by DC capabilities to a given MMTel Enabler Server.

6.2.4.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in Table 6.2.4.2.2-1 and Table 6.2.4.2.2-2.

Table 6.2.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DCMediaUpdateReq | M | 1 | Represents the requested DC media update information. |

Table 6.2.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DCMediaUpdateResp | M | 1 | 200 OK | The requested update information of DC media is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.4.2.2-: Headers supported by the 307 method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

Table 6.2.4.2.2-5: Headers supported by the 308 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

6.2.4.3	Operation: dccall

6.2.4.3.1	Description

This operation is used by the Application Server to request a call enabled by DC capabilities to a given MMTel Enabler Server.

6.2.4.3.2	Operation Definition

This operation shall support the response data structures and response codes specified in Table 6.2.4.3.2-1 and Table 6.2.4.3.2-2.

Table 6.2.4.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DcCallReq | M | 1 | Represents the requested DC Call. |

Table 6.2.4.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DcCallResp | M | 1 | 200 OK | The requested DC Call information is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.4.3.2-3: Headers supported by the 307 method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

Table 6.2.4.3.2-5: Headers supported by the 308 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

6.2.5	Notifications

The MMTel_DCAppCall API supports notifications from the MMTel Enabler Server to the Application Server for asynchronous event reporting. Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [24].

Table 6.2.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| DC Media Notification | {notificationURI} | POST | Notify about DC Media changes from MMTel Enabler Server. |

6.2.5.1	DC Media Notification

6.2.5.1.1	Description

The DC Media Notification is used by the MMTel Enabler Server to notify DC Media change to the Application Server.

6.2.5.1.2	Target URI

The Callback URI "{notificationURI}" shall be used with the callback URI variables defined in table 6.2.5.1.2-1.

Table 6.2.5.1.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notificationInfo | String formatted as URI with the Callback Uri |

6.2.5.1.3	Standard Methods

6.2.5.1.3.1	POST

This method shall support the request data structures specified in table 6.2.5.1.3.1-1 and the response data structures and response codes specified in table 6.2.5.1.3.1-2.

Table 6.2.5.1.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DcMediaNotifyReq | M | 1 | Provides information about subscribed DC Media. |

Table 6.2.5.1.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DcMediaNotifyResp |  |  | 200 OK | The receipt of the Notification is acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [24] shall also apply. |  |  |  |  |

Table 6.2.5.1.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

Table 6.2.5.1.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

6.2.6	Data Model

6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined specifically for the MMTel_DCAppCall API service.

Table 6.2.6.1-1: MMTel_DCAppCall API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| DcCallReq | 6.2.6.2.2 | Represents the DC Call establishment request |  |
| DcCallResp | 6.2.6.2.3 | Represents the DC Call establishment response |  |
| DcMediaUpdateReq | 6.2.6.2.4 | Represents the DC media update request |  |
| DcMediaUpdateResp | 6.2.6.2.5 | Represents the DC media update response |  |
| DcMediaNotifyReq | 6.2.6.2.6 | Represents the DC media update notification request |  |
| DcMediaNotifyResp | 6.2.6.2.6 | Represents the DC media update notification response |  |

Table 6.2.6.1-2 specifies data types re-used by the MMTel_DCAppCall API service.

Table 6.2.6.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DcAppUpdateParameters | 6.1.6.2.7 | Parameters of DC application profile |  |
| Uri | 3GPP TS 29.571 [7] | Represents a URI. |  |

6.2.6.2	Structured data types

6.2.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.2.6.2.2	Type: DCCallReq

Table 6.2.6.2.2: Definition of type DCCallReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| originatingId | Uri | C | 0..1 | The identifier of the caller, corresponding to participant address in OMA Third Party Call API[y], which could be ‘sip’ URI, ‘tel’ URI or ‘acr’ URI, this IE is required if the callType is P2P. |  |
| terminatingId | Uri | M | 1 | The identifier of the callee, corresponding to participant address in OMA Third Party Call API[y], which could be ‘sip’ URI, ‘tel’ URI or ‘acr’ URI. |  |
| mediaInfo | Array(string) | O | 1..N | Identifier of one or more media type(s), corresponding to the mediaInfo of Call |  |
| dcMediaInfo | boolean | O | 0..1 | Identifier of whether DC media is expected to be used. |  |
| appProfileRequested | DcAppUpdateParameters | O | 0..1 | The DC application profile expected to be used. |  |
| notificationInfo | Uri | M | 1 | The address where call related notification is sent, corresponding to callbackReference in OMA Third Party Call API. |  |
| callType | string | M | 1 | Indicate if the call is established as A2P or P2P call. |  |

6.2.6.2.3	Type: DCCallResp

Table 6.2.6.2.3-1: Definition of type DCCallResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| callResult | string | M | 1 | Indication if the Call establishment is success or failure. |  |
| sessionId | string | O | 0..1 | The identifier of the call session, corresponding to callSessionId in OMA Third Party Call API |  |
| failureCause | string | O | 0..1 | The reason for failure, corresponding to callSessionId in OMA Third Party Call API |  |
| originatingId | string | C | 1 | The identifier of the caller, corresponding to participant address in OMA Third Party Call API[y], which could be ‘sip’ URI, ‘tel’ URI or ‘acr’ URI, this IE is required if the callType is P2P. |  |
| terminatingId | string | M | 1 | The identifier of the callee, corresponding to participant address in OMA Third Party Call API[y], which could be ‘sip’ URI, ‘tel’ URI or ‘acr’ URI. |  |
| mediaInfo | Array(string) | O | 0..1 | Identifier of one or more media type(s) |  |
| dcMediaInfo | string | O | 0..1 | Identifier of whether DC media is expected to be used |  |
| appProfileRequested | DcAppUpdateParameters | O | 0..1 | The DC application profile expected to be used |  |
| callType | string | M | 1 | Indicate if the call is established as A2P or P2P call. |  |

6.2.6.2.4	Type: DCMediaUpdateReq

Table 6.2.6.2.4-1: Definition of type DCMediaUpdateReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dcAppId | string | M | 1 | The identifier of the DC application |  |
| sessionId | string | M | 1 | The session identifier for media update |  |
| mediaResourceInfo | object | O | 0..1 | The media information to be passed via the Data Channel |  |
| appProfileRequested | DcAppUpdateParameters | O | 0..1 | The DC application profile expected to be used |  |
| notificationInfo | string | M | 1 | The address where call related notification is sent to |  |
| mediaDirection | string | M | 0..1 | Type of DC communication, e.g. A2P or P2A. |  |

6.2.6.2.5	Type: DCMediaUpdateResp

Table 6.2.6.2.5-1: Definition of type DCMediaUpdateResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sessionId | string | M | 1 | The session identifier of the session |  |
| result | Status | M | 1 | Indicates if the request was successful or failed |  |
| cause | string | O | 0..1 | Indicates the cause of request failure |  |

6.2.6.2.6	Type: DCMediaNotifyReq

Table 6.2.6.2.6-1: Definition of type DCMediaNotifyReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sessionId | string | M | 1 | The session identifier of the session |  |
| mediaResourceInfo | object | M | 1 | The media information to be passed via the Data Channel |  |
| mediaDirection | string | M | 0..1 | Type of DC communication, e.g. A2P or P2A. |  |

6.2.6.2.7	Type: DCMediaNotifyResp

Table 6.2.6.2.7-1: Definition of type DCMediaNotifyResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sessionId | string | M | 1 | The session identifier of the session |  |
| result | Status | M | 1 | Indicates if the request was successful or failed |  |
| cause | string | O | 0..1 | Indicates the cause of request failure |  |

6.2.6.3	Simple data types and enumerations

6.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.2.6.3.2	Simple data types

None.

6.2.7	Error Handling

6.2.7.1	General

For the MMTel_DCAppCall API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [2]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [2] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [2].

In addition, the requirements in the following clauses are applicable for the MMTel_DCAppCall API.

6.2.7.2	Protocol Errors

No specific protocol errors for the MMTel_DCAppCall API are specified.

6.2.7.3	Application Errors

The application errors defined for the MMTel_DCAppCall API are listed in Table 6.2.7.3-1.

Table 6.2.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

6.2.8	Feature negotiation

The optional features in table 6.2.8-1 are defined for the MMTel_DCAppCall API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [2].

Table 6.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

6.2.9	Security

The provisions of clause 6 of 3GPP TS 29.122 [2] shall apply for the MMTel_DCAppCall API.

* * * End of Changes * * * *