3GPP TSG-CT WG3 Meeting #144	C3-255341r3

# Dallas, United States, 17 - 21 November 2025

Source:	China Mobile

Title:	Pseudo-CR on MMTel_DCAppCall OpenAPI

Spec:	3GPP TS 29.392 v1.0.0

Agenda item:	19.62

Document for:	Decision

1. Introduction

According to stage 2 definition, it is proposed to define the MMTel_DCAppCall OpenAPI..

2. Reason for Change

Define the MMTel_DCAppCall OpenAPI for MMTel Enabler Server services.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.392 v1.0.0.

* * * First Change * * * *

A.3	MMTel_DCAppCall API

openapi: 3.0.0

info:

version: 1.0.0-alpha.1

title: MMTel_DCAppCall

description: |

MMTel Enabler Server DC Application Call Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.392 V1.0.0; Application layer support for MMTel; MMTel Enabler Server Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.392/

servers:

- url: '{apiRoot}/mmtel-dcappcall/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials:

- mmtel-dcappcall

paths:

/dccall:

post:

summary: Request to Establish a call with DC capability.

tags:

- Custom Operation (No Resource)

operationId: createDcCall

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DcCallReq'

responses:

'200':

description: >

Call establishment successful

content:

application/json:

schema:

$ref: '#/components/schemas/DcCallResp'

'307':

description: Temporary Redirect

headers:

Location:

description: Resource URI of the alternative MMTel Enabler Server

schema:

type: string

format: uri

content: {}

'308':

description: Permanent Redirect

headers:

Location:

description: Resource URI of the alternative MMTel Enabler Server

schema:

type: string

format: uri

content: {}

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

DcMediaNotification:

'{$request.body#/notificationInfo}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DcMediaNotifyReq'

responses:

'200':

description: >

Call establishment successful

content:

application/json:

schema:

$ref: '#/components/schemas/DcMediaNotifyResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/update-dc-media:

post:

summary: Update DC media of an existing session.

tags:

- Custom Operation (No Resource)

operationId: updateDcMedia

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DcMediaUpdateReq'

responses:

'200':

description: >

Call establishment successful

content:

application/json:

schema:

$ref: '#/components/schemas/DcMediaUpdateResp'

'307':

description: Temporary Redirect

headers:

Location:

description: Resource URI of the alternative MMTel Enabler Server

schema:

type: string

format: uri

content: {}

'308':

description: Permanent Redirect

headers:

Location:

description: Resource URI of the alternative MMTel Enabler Server

schema:

type: string

format: uri

content: {}

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

# Reference to Uri type from 3GPP TS 29.571

Uri:

type: string

format: uri

description: URI compliant with RFC 3986, supporting sip/tel/acr formats (mapping to OMA Third Party Call API)

# DC application profile parameters

DcAppUpdateParameters:

type: object

description: DC application profile parameters (defined in TS 29.392 clause 6.1.6.2.7)

properties:

dcAppProfileId:

type: string

description: Unique identifier of the DC application profile

dcQosRequirements:

type: object

description: QoS requirements for DC media (e.g., bandwidth, latency)

properties:

bandwidthUplink:

type: integer

description: Uplink bandwidth (Kbps)

bandwidthDownlink:

type: integer

description: Downlink bandwidth (Kbps)

# DC call establishment request

DcCallReq:

type: object

required:

- terminatingId

- notificationInfo

- callType

properties:

originatingId:

$ref: '#/components/schemas/Uri'

terminatingId:

$ref: '#/components/schemas/Uri'

mediaInfo:

type: array

items:

type: string

description: Identifier of one or more media types (e.g., Audio/Video/Data), maps to OMA mediaInfo

dcMediaInfo:

type: boolean

description: Indicator of whether DC media is to be used

appProfileRequested:

$ref: '#/components/schemas/DcAppUpdateParameters'

notificationInfo:

$ref: '#/components/schemas/Uri'

callType:

type: string

description: Call type(A2P;P2P)

# DC call establishment response

DcCallResp:

type: object

required:

- callResult

- terminatingId

- callType

properties:

callResult:

type: string

enum: [SUCCESS, FAILED]

description: Result of call establishment

sessionId:

type: string

description: Call session identifier, maps to callSessionId in OMA Third Party Call API

failureCause:

type: string

description: Cause of call establishment failure

originatingId:

$ref: '#/components/schemas/Uri'

terminatingId:

$ref: '#/components/schemas/Uri'

mediaInfo:

type: array

items:

type: string

description: Negotiated media types

dcMediaInfo:

type: string

description: DC media activation status

appProfileRequested:

$ref: '#/components/schemas/DcAppUpdateParameters'

callType:

type: string

description: Call type

# DC media update request

DcMediaUpdateReq:

type: object

required:

- dcAppId

- sessionId

- notificationInfo

properties:

dcAppId:

type: string

description: Unique identifier of the DC application

sessionId:

type: string

description: Identifier of the session to be updated

mediaResourceInfo:

type: object

description: Media information to be transmitted via the Data Channel

appProfileRequested:

$ref: '#/components/schemas/DcAppUpdateParameters'

notificationInfo:

type: string

description: Address for receiving media update notifications

mediaDirection:

type: string

description: DC communication direction (A2P;P2A)

# DC media update response

DcMediaUpdateResp:

type: object

required:

- sessionId

- result

properties:

sessionId:

type: string

description: Session identifier

result:

type: string

description: Result of the update request

cause:

type: string

description: Cause of update request failure

# DC media notification request

DcMediaNotifyReq:

type: object

required:

- sessionId

- mediaResourceInfo

properties:

sessionId:

type: string

description: Session identifier

mediaResourceInfo:

type: object

description: Media information transmitted via the Data Channel

mediaDirection:

type: string

description: DC communication direction

# DC media notification response

DcMediaNotifyResp:

type: object

required:

- sessionId

- result

properties:

sessionId:

type: string

description: Session identifier

result:

type: string

description: Result of notification receipt

cause:

type: string

description: Cause of notification processing failure

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

AdcType:

description: The type of the data channel media direction.

anyOf:

- type: string

enum:

- A2P

- P2A

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

* * * End of Changes * * * *