3GPP TSG-CT WG3 Meeting #144	C3-255343r1

# Dallas, United States, 17 - 21 November 2025

Source:	China Mobile

Title:	Pseudo-CR on CallControlHandling API

Spec:	3GPP TS 29.392 v1.0.0

Agenda item:	19.62

Document for:	Decision

1. Introduction

According to stage 2 definition, it is proposed to define the CallControlHandling API..

2. Reason for Change

Define the CallControlHandling API for MMTel Enabler Server services.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.392 v1.0.0.

* * * First Change * * * *

<Proposed change in revision marks>

6.3	MMTel_CallEvent Service API

6.3.1	Introduction

The MMTel_CallEvent service shall use the MMTel_CallEvent API.

The API URI of the MMTel_CallEvent API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [x1], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [x1].

-	The <apiName> shall be "mmtel-callevent".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.3.3.

6.3.2	Usage of HTTP and common API related aspects

The provisions of clause 5.2 of 3GPP TS 29.122 [x6] shall apply for the MMTel_CallEvent API.

6.3.3	Resources

6.3.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.3.3.1-1 depicts the resource URIs structure for the MMTel_CallEvent service API.

Figure 6.3.3.1-1: Resource URI structure of the MMTel_CallEvent API

Table 6.3.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.3.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Session Event Subscriptions (Collection) | /session-event-subscriptions | POST | This is a pseudo resource. |

6.3.3.2	Resource: Session Event Subscriptions

6.3.3.2.1	Description

6.3.3.2.2	Resource Definition

Resource URI: {apiRoot}/mmtel-callevent/<apiVersion>/session-event-subscriptions

This resource shall support the resource URI variables defined in table 6.3.3.2.2-1.

Table 6.3.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.3.1 |

6.3.3.2.3	Resource Standard Methods

6.3.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.3.3.2.3.1-1.

Table 6.3.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.2.3.1-2 and the response data structures and response codes specified in table 6.3.3.2.3.1-3.

Table 6.3.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| Any |  |  |  |

Table 6.3.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |
| NOTE: The mandatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

6.3.3.2.4	Resource Custom Operations

None.

6.3.4	Custom Operations without associated resources

None in this release of the specification.

6.3.5	Notifications

6.3.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [x2] and clause 4.6.2.3 of 3GPP TS 29.501 [x1].

Table 6.3.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Session Event Notification | {SessionEventNotificationUri} | POST | Report the session event observed. |

6.3.5.2	Session Event Notification

6.3.5.2.1	Description

The Session Event Notification is used by the MMTel Enabler Server to report the received Session Event to the Application Server.

6.3.5.2.2	Target URI

The Callback URI "{SessionEventNotificationUri}" shall be used with the callback URI variables defined in table 6.3.5.2.2-1.

Table 6.3.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| SessionEventNotificationUri | The notification URI of the Service Consumer (i.e., Application Server) to receive the session events. In this release, the SessionEventNotificationUri of the Service Consumer is locally configured in the MMTel Enabler Server or discovered by the MMTel Enabler Server via NRF (in the default notification subscription with the "DC_SESSION_EVENT_NOTIFICATION" notification type). |

6.3.5.2.3	Standard Methods

6.3.5.2.3.1	POST

This method shall support the request data structures specified in table 6.3.5.2.3.1-1 and the response data structures and response codes specified in table 6.3.5.2.3.1-1.

Table 6.3.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SessionEventNotification | M | 1 | The session event notification to the Application Server. |

Table 6.3.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | This case represents a successful notification of the event. |
| ProblemDetails | O | 0..1 | 404 Not Found | Indicates the session event notification has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND, e.g. if the Application Server does not serve this service user; - NOTIFICATION_URI_NOT_FOUND, if the Application Server considers the "SessionEventNotificationUri" is not recognized. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [x2] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.30.9.1 of 3GPP TS 29.500 [x2]. |  |  |  |  |

Table 6.3.5.2.3-3: Headers supported by the 307 Response Code on this endpoint

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to the endpoint of the NF service consumer instance to which the request should be sent. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.30.9.1 in 3GPP TS 29.500 [x2]. |

Table 6.3.5.2.3-4: Headers supported by the 308 Response Code on this endpoint

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to the endpoint of the NF service consumer instance to which the request should be sent. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.30.9.1 in 3GPP TS 29.500 [x2]. |

6.3.6	Data Model

6.3.6.3	General

This clause specifies the application data model supported by the API.

Table 6.3.6.3-1 specifies the data types defined for the MMTel_CallEvent API.

Table 6.3.6.3-1: MMTel_CallEvent specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |

Table 6.3.6.3-2 specifies data types re-used by the MMTel_CallEvent API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the MMTel_CallEvent service based interface.

Table 6.3.6.3-2: MMTel_CallEvent re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| SessionEventNotification | 3GPP TS 29.175 [x5] | Represents the IMS Session Notification. |

6.3.6.2	Structured data types

No specific structures for the MMTel_CallEvent service are specified in this clause.

6.3.7	Error Handling

6.3.7.1	General

For the MMTel_CallEvent API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [x6]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [x6] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [x6].

In addition, the requirements in the following clauses are applicable for the MMTel_CallEvent API.

6.3.7.2	Protocol Errors

No specific protocol errors for the MMTel_CallEvent API are specified.

6.3.7.3	Application Errors

The application errors defined for the MMTel_CallEvent API are listed in Table 6.3.7.3-1.

Table 6.3.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

6.3.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the MMTel_CallEvent API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [x6].

Table 6.3.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

6.3.9	Security

The provisions of clause 6 of 3GPP TS 29.122 [x6] shall apply for the MMTel_CallEvent API.

6.4	MMTel_CallControl API

6.4.1	Introduction

The Nnef_MMTel_CallControl service shall use the MMTel_CallControl API.

The API URI of the MMTel_CallControl API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [x6], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [x6].

-	The <apiName> shall be "mmtel-callcontrol".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [x6].

6.4.2	Usage of HTTP and common API related aspects

The provisions of clause 5.2 of 3GPP TS 29.122 [x6] shall apply for the MMTel_CallEvent API.

6.4.3	Resources

6.4.3.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 6.4.3.1-1 and the resources and HTTP methods used for the MMTel_CallControl API.

Figure 6.4.3.1-1: Resource URI structure of the MMTel_CallControl API

Table 6.4.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.4.3.1-1: Resources and methods overview

| Resource name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| IMS Sessions | /ims-sessions | POST | Create a new IMS Session at the MMTel Enabler Server. |
| Individual IMS Session | /ims-sessions/{sessionId} | PUT | Update an existing "Individual IMS Session" resource. |
|  |  | PATCH | Modify an existing "Individual IMS Session" resource. |
|  |  | DELETE | Delete an existing "Individual IMS Session" resource. |

6.4.3.2	Resource: IMS Sessions

6.4.3.2.1	Description

This resource represents the collection of IMS Sessions managed by the MMTel Enabler Server.

The resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [x1]).

6.4.3.2.2	Resource Definition

Resource URI: {apiRoot}/mmtel-callcontrol/<apiVersion>/ims-sessions

This resource shall support the resource URI variables defined in table 6.4.3.2.2-1.

Table 6.4.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.4.1. |

6.4.3.2.3	Resource Standard Methods

6.4.3.2.3.1	POST

The HTTP POST method enables an AF to request the creation of a new IMS Session at the MMTel Enabler Server.

This method shall support the URI query parameters specified in table 6.4.3.2.3.1-1.

Table 6.4.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.2.3.1-2 and the response data structures and response codes specified in table 6.4.3.2.3.1-3.

Table 6.4.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ImsSession | M | 1 | Contains the parameters to request the creation of a new IMS Session. |

Table 6.4.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsSession | M | 1 | 201 Created | Successful response. The "Individual IMS Session" resource is successfully created and a representation of the created resource is returned in the response body. The URI of the created resource shall be returned in an HTTP "Location" header. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [x6] shall also apply. NOTE 2: Failure cases are described in clause 6.4.7. |  |  |  |  |

Table 6.4.3.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/mmtel-callcontrol/<apiVersion>/ims-sessions/{sessionId} |

6.4.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

6.4.3.3	Resource: Individual IMS Session

6.4.3.3.1	Description

This resource represents an "Individual IMS Session" resource managed by the MMTel Enabler Server.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [x1]).

6.4.3.3.2	Resource Definition

Resource URI: {apiRoot}/mmtel-callcontrol/<apiVersion>/ims-sessions/{sessionId}

This resource shall support the resource URI variables defined in table 6.4.3.3.2-1.

Table 6.4.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.4.1. |
| sessionId | string | Represents the identifier of the "Individual IMS Session" resource. |

6.4.3.3.3	Resource Standard Methods

6.4.3.3.3.1	PUT

The HTTP PUT method enables an AF to request the update of an existing "Individual IMS Session" resource at the MMTel Enabler Server.

This method shall support the URI query parameters specified in table 6.4.3.3.3.1-1.

Table 6.4.3.3.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.1-2 and the response data structures and response codes specified in table 6.4.3.3.3.1-3.

Table 6.4.3.3.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ImsSession | M | 1 | Represents the updated "Individual IMS Session" resource representation. |

Table 6.4.3.3.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |  |
| --- | --- | --- | --- | --- | --- |
| ImsSession | M | 1 | 200 OK | Successful response. The "Individual IMS Session" resource is successfully updated and a representation of the updated resource is returned in the response body. |  |
| n/a |  |  | 204 No Content | Successful response. The "Individual IMS Session" resource is successfully updated and no content is returned in the response body. |  |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [x6]. |  |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [x6]. |  |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |  |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [x6] shall also apply. NOTE 2: Failure cases are described in clause 6.4.7. |  |  |  |  |

Table 6.4.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative MMTel Enabler Server. |

Table 6.4.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative MMTel Enabler Server. |

6.4.3.3.3.2	PATCH

The HTTP PATCH method enables an AF to request the modification of an existing "Individual IMS Session" resource at the MMTel Enabler Server.

This method shall support the URI query parameters specified in table 6.4.3.3.3.2-1.

Table 6.4.3.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.2-2 and the response data structures and response codes specified in table 6.4.3.3.3.2-3.

Table 6.4.3.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ImsSessionPatch | M | 1 | Represents the requested modifications to the "Individual IMS Session" resource. |

Table 6.4.3.3.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsSession | M | 1 | 200 OK | Successful response. The "Individual IMS Session" resource is successfully modified and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful response. The "Individual IMS Session" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [x6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [x6]. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [x6] shall also apply. NOTE 2: Failure cases are described in clause 6.4.7. |  |  |  |  |

Table 6.4.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located on an alternative MMTel Enabler Server. |

Table 6.4.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located on an alternative MMTel Enabler Server. |

6.4.3.3.3.3	DELETE

The HTTP DELETE method enables an AF to request the deletion of an existing "Individual IMS Session" resource at the MMTel Enabler Server.

This method shall support the URI query parameters specified in table 6.4.3.3.3.3-1.

Table 6.4.3.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.3-2 and the response data structures and response codes specified in table 6.4.3.3.3.3-3.

Table 6.4.3.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.4.3.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No content | Successful response. The "Individual IMS Session" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [x6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [x6]. |
| NOTE 1: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [x6] shall also apply. |  |  |  |  |

Table 6.4.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located on an alternative MMTel Enabler Server. |

Table 6.4.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located on an alternative MMTel Enabler Server. |

6.4.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

6.4.4	Custom Operations without associated resources

There are no resource custom operations defined for this resource in this release of the specification.

6.4.5	Notifications

#### 6.4.5.1	General

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [x6].

Table 6.4.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| IMS Session Notification | {notifUri} | POST | Enable the MMTel Enabler Server to notify a previously subscribed AF on IMS Session related event(s). |

#### 6.4.5.2	IMS Session Notification

##### 6.4.5.2.1	Description

The IMS Session Notification is used by the MMTel Enabler Server to report IMS session related event(s) to a previously subscribed AF.

##### 6.4.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 5.43.4.2.2-1.

Table 6.4.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Represents the callback URI encoded as a string formatted as a URI. |

##### 6.4.5.2.3	Operation Definition

This method shall support the request data structures specified in table 6.4.5.2.3-1 and the response data structures and response codes specified in table 6.4.5.2.3-2.

Table 6.4.5.2.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ImsSessionEventNotification | M | 1 | Contains the IMS Session Notification. |

Table 6.4.5.2.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The IMS Session Notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [x6]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [x6]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [x6] shall also apply. |  |  |  |  |

Table 6.4.5.2.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 6.4.5.2.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

6.4.6	Data Model

6.4.6.1	General

This clause specifies the application data model supported by the API.

Table 6.4.6.1-1 specifies the data types defined for the MMTel_CallControl service-based interface protocol.

Table 6.4.6.1-1: MMTel_CallControl specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| ImsSession | 6.4.6.2.2 | Represent an IMS Session. |  |
| ImsSessionPatch | 6.4.6.2.3 | Represents the requested modifications to an IMS Session. |  |

Table 6.4.6.1-2 specifies data types re-used by the MMTel_CallControl API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the MMTel_CallControl service based interface.

Table 6.4.6.1-2: MMTel_CallControl re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ImsSessionEventNotification | 3GPP TS 29.175 [x5] | Represents the IMS Session Notification. |  |
| ImsSession | 3GPP TS 29.522 [x7] | Represent an IMS Session. |  |
| ImsSessionPatch | 3GPP TS 29.522 [x7] | Represents the requested modifications to an IMS Session. |  |

6.4.6.2	Structured data types

6.4.6.2.1	Introduction

This clause defines the structured data types to be used in resource representations.

6.4.6.3	Simple data types and enumerations

6.4.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.4.6.3.2	Simple data types

The simple data types defined in table 6.4.6.3.2-1 shall be supported.

Table 6.4.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

6.4.7	Error Handling

6.4.7.1	General

For the MMTel_CallEvent API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [x6]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [x6] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [x6].

In addition, the requirements in the following clauses are applicable for the MMTel_CallEvent API.

6.4.7.2	Protocol Errors

No specific protocol errors for the MMTel_CallEvent API are specified.

6.4.7.3	Application Errors

The application errors defined for the MMTel_CallEvent API are listed in Table 6.4.7.3-1.

Table 6.3.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

6.4.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the MMTel_CallEvent API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [x6].

Table 6.4.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

6.4.9	Security

The provisions of clause 6 of 3GPP TS 29.122 [x6] shall apply for the MMTel_CallEvent API.

* * * End of Changes * * * *