3GPP TSG-CT WG3 Meeting #144	C3-255400

# Dallas, US, 17-21, November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.514 | CR | 0810 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | expTranInd attribute name and Boolean handling update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The wrong attribute name mentioned for the expedited data transfer with reflective QoS indication in the service description. The nullable information is missing for expTranInd attribute is missing in MediaComponentRm. TS 23.502 clause 5.2.6.9.2, 5.2.6.9.5 specifies Data Burst Size Marking Support Indication as described in clause 6.1.3.27.5 of TS 23.503 [20], Time to Next Burst Support Indication as described in clause 6.1.3.27.5 of TS 23.503 [20], Data Burst Size Marking Support Indication and Time to Next burst support indication can be reset to false as well as per stage 2. This change has to be updated in stage 3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | expTranInd attribute name and the related formatting is updated. Data Burst Size Marking Support Indication and Time to Next burst support indication are updated for the boolean value handling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The wrong atribute name and the nullable information for expedited data transfer with reflective QoS indication is not defined in stage 3. Stage 3 is not aligned with Stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3.47, 5.6.2.7, 5.6.2.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backwards compatible new feature to the OpenAPI descriptions of the following APIs: TS29514_Npcf_PolicyAuthorization.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

#### 4.2.3.47	Provisioning of dynamically changing traffic characteristics

If the "TrafficCharChange" feature is supported, this procedure is used by the NF service consumer to indicate that the Data Burst Size is marked in the header of the protocol identified by the DL Protocol Description.

The NF service consumer may modify the previously provided Data Burst Size Marking Indication by providing the updated attribute(s) in the "datBurstSizeInd" and "protoDescDl" within the "medComponents" map contained in the HTTP PATCH request message described in clause 4.2.3.2.

The PCF determines the policies for the identification and marking of the Data Burst Size and provisions the PCC rule as described in 3GPP TS 29.512 [8].

If the "TrafficCharChange" feature is supported, this procedure is used by the NF service consumer to indicate that TTNB is marked in the header of the protocol identified by the DL Protocol Description.

The NF service consumer may modify the previously provided TTNB Indication by providing the updated attribute(s) in the " timetoNextBurstInd" and "protoDescDl" within the "medComponents" map contained in the HTTP PATCH request message described in clause 4.2.3.2.

The PCF determines the policies for the identification of the TTNB and provisions the PCC rule as described in 3GPP TS 29.512 [8].

If the "TrafficCharChange" feature is supported, this procedure is used by the NF service consumer to request expedited data transfer with reflective QoS to expedite the data transfer of larger payload for Non-GBR service flow(s) of XR application.

The NF service consumer may provision the expedited data transfer with reflective QoS indication by providing the updated attribute(s) in the "expTranInd" and/or "protoDescDl" within the "medComponents" map contained in the HTTP PATCH request message described in clause 4.2.3.2.

The PCF determines the policies for the expedited data transfer with reflective QoS and provisions the PCC rule as described in 3GPP TS 29.512 [8].

The PCF shall reply to the NF service consumer as described in clause 4.2.2.2.

# * * * * Next change * * * *

#### 5.6.2.7	Type MediaComponent

Table 5.6.2.7-1: Definition of type MediaComponent

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| afAppId |  | AfAppId |  | O |  | 0..1 |  | Contains information that identifies the particular service the AF session belongs to. |  |  |  |
| afRoutReq |  | AfRoutingRequirement |  | O |  | 0..1 |  | Indicates the AF traffic routing requirements. |  | InfluenceOnTrafficRouting |  |
| afSfcReq |  | AfSfcRequirement |  | O |  | 0..1 |  | Indicates the AF requirements on steering traffic to a pre-configured chain of service functions on N6-LAN. |  | SFC |  |
| afHdrReq |  | AfHeaderHandlingControlInfo |  | O |  | 0..1 |  | Indicates the AF handling of payload headers requirements. |  | HeaderHandling |  |
| qosReference |  | string |  | O |  | 0..1 |  | Identifies a pre-defined QoS information. |  | AuthorizationWithRequiredQoS |  |
| altSerReqs |  | array(string) |  | O |  | 1..N |  | Ordered list of alternative service requirements that include a set of QoS references. The lower the index of the array for a given entry, the higher the priority.(NOTE 1) |  | AuthorizationWithRequiredQoS |  |
| altSerReqsData |  | array(AlternativeServiceRequirementsData) |  | O |  | 1..N |  | Ordered list of alternative service requirements that include individual QoS parameter sets. The lower the index of the array for a given entry, the higher the priority. (NOTE 1) |  | AltSerReqsWithIndQoS |  |
| disUeNotif |  | boolean |  | O |  | 0..1 |  | Indicates to disable QoS flow parameters signalling to the UE when the SMF is notified by the NG-RAN of changes in the fulfilled QoS situation when it is included and set to "true". The fulfilled situation is either the QoS profile or an Alternative QoS Profile. The default value "false" shall apply, if the attribute is not present and has not been supplied previously. |  | DisableUENotification |  |
| contVer |  | ContentVersion |  | O |  | 0..1 |  | Represents the content version of a media component. |  | MediaComponentVersioning |  |
| desMaxLatency |  | Float |  | O |  | 0..1 |  | Indicates a maximum desirable transport level packet latency in milliseconds. |  | FLUS, QoSHint |  |
| desMaxLoss |  | Float |  | O |  | 0..1 |  | Indicates the maximum desirable transport level packet loss rate in percent (without "%" sign). |  | FLUS, QoSHint |  |
| flusId |  | string |  | O |  | 0..1 |  | Indicates that the media component is used for FLUS media. It is derived from the media level attribute "a=label:" (see IETF RFC 4574 [50]) obtained from the SDP body. It contains the string after "a=label:" starting with "flus" and may be followed by more characters as described in 3GPP TS 26.238 [51]. |  | FLUS |  |
| medCompN |  | integer |  | M |  | 1 |  | Identifies the media component number, and it contains the ordinal number of the media component. |  |  |  |
| medSubComps |  | map(MediaSubComponent) |  | O |  | 1..N |  | Contains the requested bitrate and filters for the set of service data flows identified by their common flow identifier. The key of the map is the attribute "fNum". (NOTE 3) |  |  |  |
| medType |  | MediaType |  | O |  | 0..1 |  | Indicates the media type of the service. |  |  |  |
| marBwUl |  | BitRate |  | O |  | 0..1 |  | Maximum requested bandwidth for the Uplink. |  |  |  |
| marBwDl |  | BitRate |  | O |  | 0..1 |  | Maximum requested bandwidth for the Downlink. |  |  |  |
| maxPacketLossRateDl |  | PacketLossRateRm |  | O |  | 0..1 |  | Indicates the downlink maximum rate for lost packets that can be tolerated for the service data flow. |  | CHEM |  |
| maxPacketLossRateUl |  | PacketLossRateRm |  | O |  | 0..1 |  | Indicates the uplink maximum rate for lost packets that can be tolerated for the service data flow. |  | CHEM |  |
| maxSuppBwDl |  | BitRate |  | O |  | 0..1 |  | Maximum supported bandwidth for the Downlink. |  | IMS_SBI |  |
| maxSuppBwUl |  | BitRate |  | O |  | 0..1 |  | Maximum supported bandwidth for the Uplink. |  | IMS_SBI |  |
| minDesBwDl |  | BitRate |  | O |  | 0..1 |  | Minimum desired bandwidth for the Downlink. |  | IMS_SBI |  |
| minDesBwUl |  | BitRate |  | O |  | 0..1 |  | Minimum desired bandwidth for the Uplink. |  | IMS_SBI |  |
| mirBwUl |  | BitRate |  | O |  | 0..1 |  | Minimum requested bandwidth for the Uplink. |  |  |  |
| mirBwDl |  | BitRate |  | O |  | 0..1 |  | Minimum requested bandwidth for the Downlink. |  |  |  |
| fStatus |  | FlowStatus |  | O |  | 0..1 |  | Indicates whether the status of the service data flows is enabled, or disabled. |  |  |  |
| preemptCap |  | PreemptionCapability |  | O |  | 0..1 |  | Defines whether the media flow may get resources that were already assigned to another media flow with a lower priority level. It may be included together with "prioSharingInd" for ARP decision. |  | MCPTT-Preemption |  |
| preemptVuln |  | PreemptionVulnerability |  | O |  | 0..1 |  | Defines whether the media flow may lose the resources assigned to it in order to admit a media flow with higher priority level. It may be included together with "prioSharingInd" for ARP decision. |  | MCPTT-Preemption |  |
| prioSharingInd |  | PrioritySharingIndicator |  | O |  | 0..1 |  | Indicates that the media flow is allowed to use the same ARP as media flows belonging to other "Individual Application Session Context" resources bound to the same PDU session. |  | PrioritySharing |  |
| resPrio |  | ReservPriority |  | O |  | 0..1 |  | Indicates the reservation priority. |  |  |  |
| rrBw |  | BitRate |  | O |  | 0..1 |  | Indicates the maximum required bandwidth in bits per second for RTCP receiver reports within the session component as specified in IETF RFC 3556 [37]. The bandwidth contains all the overhead coming from the IP-layer and the layers above, i.e. IP, UDP and RTCP. |  | IMS_SBI |  |
| rsBw |  | BitRate |  | O |  | 0..1 |  | Indicates the maximum required bandwidth in bits per second for RTCP sender reports within the session component as specified in IETF RFC 3556 [37]. The bandwidth contains all the overhead coming from the IP-layer and the layers above, i.e. IP, UDP and RTCP. |  | IMS_SBI |  |
| sharingKeyDl |  | Uint32 |  | O |  | 0..1 |  | Identifies which media components share resources in the downlink direction. If resource sharing applies between media components across "Individual Application Session Context" resources for the same PDU session, the same value of the "sharingKeyDl" attribute shall be used. If resource sharing does not apply among media components across "Individual Application Session Context" resources for the same PDU session, a different value for the "sharingKeyDl" attribute shall be used. |  | ResourceSharing |  |
| sharingKeyUl |  | Uint32 |  | O |  | 0..1 |  | Identifies which media components share resources in the uplink direction. If resource sharing applies between media components across "Individual Application Session Context" resources for the same PDU session, the same value of the "sharingKeyUl" attribute shall be used. If resource sharing does not apply among media components across "Individual Application Session Context" resources for the same PDU session, a different value for the "sharingKeyUl" attribute shall be used. |  | ResourceSharing |  |
| codecs |  | array(CodecData) |  | O |  | 1..2 |  | Indicates the codec data. |  |  |  |
| tsnQos |  | TsnQoSContainer |  | O |  | 0..1 |  | Transports QoS parameters for TSC traffic. |  | TimeSensitiveNetworking |  |
| tscaiInputUl |  | TscaiInputContainer |  | O |  | 0..1 |  | Transports TSCAI input parameters for TSC traffic at the ingress interface of the DS-TT/UE (uplink flow direction). (NOTE 2) |  | TimeSensitiveNetworking |  |
| tscaiInputDl |  | TscaiInputContainer |  | O |  | 0..1 |  | Transports TSCAI input parameters for TSC traffic at the ingress of the NW-TT (downlink flow direction). (NOTE 2) |  | TimeSensitiveNetworking |  |
| tscaiTimeDom |  | Uinteger |  | O |  | 0..1 |  | Indicates the (g)PTP domain that the (TSN)AF is located in. |  | TimeSensitiveCommunication |  |
| capBatAdaptation |  | boolean |  | O |  | 0..1 |  | Indicates the capability for AF to adjust the burst sending time, when it is supported and set to "true". The default value is "false" if omitted. (NOTE 2) |  | EnTSCAC |  |
| rTLatencyInd |  | boolean |  | O |  | 0..1 |  | Indicates the service data flow needs to meet the Round-Trip (RT) latency requirement of the service, when it is included and set to "true". The default value is "false" if omitted. (NOTE 4, NOTE 5) |  | RTLatency |  |
| pdb |  | PacketDelBudget |  | O |  | 0..1 |  | Indicates an upper bound for the time that a packet may be delayed between the UE and the PSA UPF. |  | RTLatency |  |
| rTLatencyIndCorreId |  | RttFlowReference |  | O |  | 0..1 |  | Identifies which Media Components contribute to the RT Latency requirement for two service data flows. (NOTE 4) |  | RTLatency |  |
| pduSetQosDl |  | PduSetQosPara |  | O |  | 0..1 |  | PDU Set QoS parameter(s) for the downlink direction. |  | PDUSetHandling |  |
| pduSetQosUl |  | PduSetQosPara |  | O |  | 0..1 |  | PDU Set QoS parameter(s) for the uplink direction. |  | PDUSetHandling |  |
| protoDescDl |  | ProtocolDescription |  | O |  | 0..1 |  | Downlink Protocol description for PDU Set identification, the detection of the end of data burst indication, the detection of the Data Burst Size marking indication, TTNB indication and/or indication of whether MoQ or UDP-option is used to carry media related information. |  | PDUSetHandling PowerSaving TrafficCharChange OnPathN6MediaInfo |  |
| protoDescUl |  | ProtocolDescription |  | O |  | 0..1 |  | Uplink Protocol description for PDU Set identification in UPF. |  | PDUSetHandling |  |
| periodUl |  | DurationMilliSec |  | O |  | 0..1 |  | Indicates the time period between the start of the two data bursts in units of milliseconds in Uplink direction. |  | PowerSaving |  |
| periodDl |  | DurationMilliSec |  | O |  | 0..1 |  | Indicates the time period between the start of the two data bursts in units of milliseconds in Downlink direction. |  | PowerSaving |  |
| l4sInd |  | UplinkDownlinkSupport |  | O |  | 0..1 |  | Indicates whether ECN marking for L4S support is supported for the UL, the DL or both, UL and DL. (NOTE 3) |  | L4S |  |
| datBurstSizeInd |  | boolean |  | O |  | 0..1 |  | Indicates the Data Burst Size marking for the DL service data flow is supported when it is included and set to "true". The default value is "false" if omitted. |  | TrafficCharChange |  |
| timetoNextBurstInd |  | boolean |  | O |  | 0..1 |  | Indicates the Time to Next Burst for the DL service data flow is supported, when it is included and set to "true". The default value is "false" if omitted. |  | TrafficCharChange |  |
| onPathN6SigInfo |  | OnPathN6SigInfo |  | O |  | 0..1 |  | Contains the on-path N6 signaling information for delivering media related information. |  | OnPathN6MediaInfo |  |
| expTranInd |  | boolean |  | O |  | 0..1 |  | Expedited Transfer Indication for the downlink traffic to enable expedited data transfer with reflective QoS for the Non-GBR service data flow. - "true": the expedited data transfer of larger payload for XR application is enabled for the flow. - "false": the expedited data transfer of larger payload for XR application is not enabled for the flow. The default value is "false" if omitted. |  | TrafficCharChange |  |
| NOTE 1: The attributes "altSerReqs" and "altSerReqsData" are mutually exclusive. Of the two, only the attribute "altSerReqs" may be provided if the attribute "qosReference" is provided, while only the attribute "altSerReqsData" may be provided if the attribute "qosReference" is not provided. NOTE 2: The "burstArrivalTimeWnd" attribute, within the "tscaiInputUl" and/or "tscaiInputDl" attributes, and the "capBatAdaptation attribute are mutually exclusive. NOTE 3: Within the MediaComponent entry, the NF service consumer may include either the indication of L4S support within the "l4sInd" attribute or the request for congestion measurements within the "evSubsc" attribute included in one or more entries of the "medSubComps" attribute, but the indication of L4S and the subscription to congestion monitoring shall not be provided simultaneously. NOTE 4: The "rTLatencyInd" attribute and the "rTLatencyIndCorreId" attribute are mutually exclusive. NOTE 5: If more than one "medSubComps" attributes are present, the PCF selects the media subcomponent and derive the PCC rule for RT latency control. |  |  |  |  |  |  |  |  |  |  |  |

All IP flows within a "MediaSubComponent" data type are permanently disabled by supplying "FlowStatus" data type with a deletion indication.

Bandwidth information and the "fStatus" attribute provided within the MediaComponent applies to all those IP flows within the media component, for which no corresponding information is being provided within the "medSubComps" attribute. As defined in 3GPP TS 29.513 [7], the bandwidth information within the media component level "marBwUl" and "marBwDl" attributes applies separately to each media subcomponent except for media subcomponents with a "flowUsage" attribute with the value "RTCP". The mapping of bandwidth information for RTCP media subcomponent is defined in 3GPP TS 29.513 [7] clause 7.3.3.

# * * * * Next change * * * *

#### 5.6.2.26	Type MediaComponentRm

This data type is defined in the same way as the "MediaComponent" data type, but:

-	with the OpenAPI "nullable: true" property; and

-	the removable attributes "afRoutReq" is defined with the removable data type "AfRoutingRequirementRm"; "maxPacketLossRateDl" and "maxPacketLossRateUl" are defined with the removable data type "PacketLossRateRm"; "medSubComps" is defined with the removable data type "MediaSubComponentRm"; "preemptCap" is defined with the removable data type "PreemptionCapabilityRm"; "preemptVuln" is defined with the removable data type "PreemptionVulnerabilityRm"; "marBwDl", "marBwUl", "minDesBwDl", "minDesBwUl", "mirBwDl", "mirBwUl", "maxSuppBwDl", "maxSuppBwUl", "rrBw", "rsBw" are defined with the removable data type "BitRateRm"; "sharingKeyDl" and "sharingKeyUl" are defined with the removable data types "Uint32Rm", "tsnQos" is defined with the removable data type "TsnQosContainerRm"; "pduSetQosDl" and "pduSetQosUl" are defined with the removable data type "pduSetQosParaRm"; "desMaxLatency" and "desMaxLoss" are defined with the removable data type "FloatRm"; "protoDescDl" and "protoDescUl" are defined with the removable data type "ProtocolDescriptionRm"; the "afSfcReq" attributo with the removable data type "AfSfcRequirement", the removable attribute "pdb" with the removable data type "PacketDelBudgetRm", the removable attribute "rTLatencyIndCorreId" with the removable data type "RttFlowReferenceRm", the removable attribute "afHdrReq" with the removable data type "AfHeaderHandlingControlInfo"; the removable attributes "periodUl" and "periodDl" are defined with the removable data type "DurationMilliSecRm"; the removable attribute "onPathN6SigInfo" with the removable data type "OnPathN6SigInfo"; and

-	the removable attributes "flusId", "qosReference", "altSerReqs", "altSerReqsData", "rTLatencyInd", "datBurstSizeInd" and "timetoNextBurstInd" are defined with the property "nullable: true" in the OpenAPI.

Table 5.6.2.26-1: Definition of type MediaComponentRm

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| afAppId |  | AfAppId |  | O |  | 0..1 |  | Contains information that identifies the particular service the AF session belongs to. |  |  |  |
| afRoutReq |  | AfRoutingRequirementRm |  | O |  | 0..1 |  | Indicates the AF traffic routing requirements. |  | InfluenceOnTrafficRouting |  |
| afSfcReq |  | AfSfcRequirement |  | O |  | 0..1 |  | Indicates the AF requirements on steering traffic to a pre-configured chain of service functions on N6-LAN. |  | SFC |  |
| afHdrReq |  | AfHeaderHandlingControlInfo |  | O |  | 0..1 |  | Indicates the AF handling of payload headers requirements. |  | HeaderHandling |  |
| qosReference |  | string |  | O |  | 0..1 |  | Identifies a pre-defined QoS information. |  | AuthorizationWithRequiredQoS |  |
| altSerReqs |  | array(string) |  | O |  | 1..N |  | Ordered list of alternative service requirements that include a set of QoS references. The lower the index of the array for a given entry, the higher the priority. (NOTE 1) |  | AuthorizationWithRequiredQoS |  |
| altSerReqsData |  | array(AlternativeServiceRequirementsData) |  | O |  | 1..N |  | Ordered list of alternative service requirements that include individual QoS parameter sets. The lower the index of the array for a given entry, the higher the priority. (NOTE 1) |  | AltSerReqsWithIndQoS |  |
| disUeNotif |  | boolean |  | O |  | 0..1 |  | Indicates to disable QoS flow parameters signalling to the UE when the SMF is notified by the NG-RAN of changes in the fulfilled QoS situation when it is included and set to "true". The fulfilled situation is either the QoS profile or an Alternative QoS Profile. The default value "false" shall apply, if the attribute is not present and has not been supplied previously. |  | DisableUENotification |  |
| contVer |  | ContentVersion |  | O |  | 0..1 |  | Represents the content version of a media component. |  | MediaComponentVersioning |  |
| desMaxLatency |  | FloatRm |  | O |  | 0..1 |  | Indicates a maximum desirable transport level packet latency in milliseconds. |  | FLUS, QoSHint |  |
| desMaxLoss |  | FloatRm |  | O |  | 0..1 |  | Indicates the maximum desirable transport level packet loss rate in percent (without "%" sign). |  | FLUS, QoSHint |  |
| flusId |  | string |  | O |  | 0..1 |  | Indicates that the media component is used for FLUS media. It is derived from the media level attribute "a=label:" (see IETF RFC 4574 [50]) obtained from the SDP body. It contains the string after "a=label:" starting with "flus" and may be followed by more characters as described in 3GPP TS 26.238 [51]. |  | FLUS |  |
| maxPacketLossRateDl |  | PacketLossRateRm |  | O |  | 0..1 |  | Indicates the downlink maximum rate for lost packets that can be tolerated for the service data flow. |  | CHEM |  |
| maxPacketLossRateUl |  | PacketLossRateRm |  | O |  | 0..1 |  | Indicates the uplink maximum rate for lost packets that can be tolerated for the service data flow. |  | CHEM |  |
| medCompN |  | integer |  | M |  | 1 |  | Identifies the media component number, and it contains the ordinal number of the media component. |  |  |  |
| medSubComps |  | map(MediaSubComponentRm) |  | O |  | 1..N |  | Contains the requested bitrate and filters for the set of service data flows identified by their common flow identifier. The key of the map is the attribute "fNum". (NOTE 3) |  |  |  |
| medType |  | MediaType |  | O |  | 0..1 |  | Indicates the media type of the service. |  |  |  |
| marBwUl |  | BitRateRm |  | O |  | 0..1 |  | Maximum requested bandwidth for the Uplink. |  |  |  |
| marBwDl |  | BitRateRm |  | O |  | 0..1 |  | Maximum requested bandwidth for the Downlink. |  |  |  |
| maxSuppBwDl |  | BitRateRm |  | O |  | 0..1 |  | Maximum supported bandwidth for the Downlink. |  | IMS_SBI |  |
| maxSuppBwUl |  | BitRateRm |  | O |  | 0..1 |  | Maximum supported bandwidth for the Uplink. |  | IMS_SBI |  |
| minDesBwDl |  | BitRateRm |  | O |  | 0..1 |  | Minimum desired bandwidth for the Downlink. |  | IMS_SBI |  |
| minDesBwUl |  | BitRateRm |  | O |  | 0..1 |  | Minimum desired bandwidth for the Uplink. |  | IMS_SBI |  |
| mirBwUl |  | BitRateRm |  | O |  | 0..1 |  | Minimum requested bandwidth for the Uplink. |  |  |  |
| mirBwDl |  | BitRateRm |  | O |  | 0..1 |  | Minimum requested bandwidth for the Downlink. |  |  |  |
| fStatus |  | FlowStatus |  | O |  | 0..1 |  | Indicates whether the status of the service data flows is enabled, or disabled. |  |  |  |
| preemptCap |  | PreemptionCapabilityRm |  | O |  | 0..1 |  | Defines whether the media flow may get resources that were already assigned to another media flow with a lower priority level. |  | MCPTT-Preemption |  |
| preemptVuln |  | PreemptionVulnerabilityRm |  | O |  | 0..1 |  | Defines whether the media flow may lose the resources assigned to it in order to admit a media flow with higher priority level. |  | MCPTT-Preemption |  |
| prioSharingInd |  | PrioritySharingIndicator |  | O |  | 0..1 |  | Indicates that the media flow is allowed to use the same ARP as media flows belonging to other "Individual Application Session Context" resources bound to the same PDU session. |  | PrioritySharing |  |
| resPrio |  | ReservPriority |  | O |  | 0..1 |  | Indicates the reservation priority. |  |  |  |
| rrBw |  | BitRateRm |  | O |  | 0..1 |  | Indicates the maximum required bandwidth in bits per second for RTCP receiver reports within the session component as specified in IETF RFC 3556 [37]. The bandwidth contains all the overhead coming from the IP-layer and the layers above, i.e. IP, UDP and RTCP. |  | IMS_SBI |  |
| rsBw |  | BitRateRm |  | O |  | 0..1 |  | Indicates the maximum required bandwidth in bits per second for RTCP sender reports within the session component as specified in IETF RFC 3556 [37]. The bandwidth contains all the overhead coming from the IP-layer and the layers above, i.e. IP, UDP and RTCP. |  | IMS_SBI |  |
| codecs |  | array(CodecData) |  | O |  | 1..2 |  | Indicates the codec data. |  |  |  |
| sharingKeyDl |  | Uint32Rm |  | O |  | 0..1 |  | Identifies which media components share resources in the downlink direction. If resource sharing applies between media components across "Individual Application Session Context" resources for the same PDU session, the same value of the "sharingKeyDl" attribute shall be used. If resource sharing does not apply among media components across "Individual Application Session Context" resources for the same PDU session, a different value for the "sharingKeyDl" attribute shall be used. If resource sharing does no longer apply for this media component, the "sharingKeyDl" attribute shall be set to "null". |  | ResourceSharing |  |
| sharingKeyUl |  | Uint32Rm |  | O |  | 0..1 |  | Identifies which media components share resources in the uplink direction. If resource sharing applies between media components across "Individual Application Session Context" resources for the same PDU session, the same value of the "sharingKeyUl" attribute shall be used. If resource sharing does not apply among media components across "Individual Application Session Context" resources for the same PDU session, a different value for the "sharingKeyUl" attribute shall be used. If resource sharing does no longer apply for this media component, the "sharingKeyUl" attribute shall be set to "null". |  | ResourceSharing |  |
| tsnQos |  | TsnQoSContainerRm |  | O |  | 0..1 |  | Transports QoS parameters for TSC traffic. |  | TimeSensitiveNetworking |  |
| tscaiInputUl |  | TscaiInputContainer |  | O |  | 0..1 |  | Transports TSCAI input parameters for TSC traffic at the ingress interface of the DS-TT/UE (uplink flow direction). |  | TimeSensitiveNetworking |  |
| tscaiInputDl |  | TscaiInputContainer |  | O |  | 0..1 |  | Transports TSCAI input parameters for TSC traffic at the ingress of the NW-TT (downlink flow direction). |  | TimeSensitiveNetworking |  |
| tscaiTimeDom |  | Uinteger |  | O |  | 0..1 |  | Indicates the (g)PTP domain that the (TSN)AF is located in. |  | TimeSensitiveCommunication |  |
| capBatAdaptation |  | boolean |  | O |  | 0..1 |  | Indicates the capability for AF to adjust the burst sending time, when it is supported and set to "true". (NOTE 2) |  | EnTSCAC |  |
| rTLatencyInd |  | boolean |  | O |  | 0..1 |  | Indicates the service data flow needs to meet the Round-Trip (RT) latency requirement of the service, when it is included and set to "true". |  | RTLatency |  |
| pdb |  | PacketDelBudgetRm |  | O |  | 0..1 |  | Indicates an upper bound for the time that a packet may be delayed between the UE and the PSA UPF. |  | RTLatency |  |
| rTLatencyIndCorreId |  | RttFlowReferenceRm |  | O |  | 0..1 |  | Identifies which Media Components contribute to the RT Latency requirement for two service data flows. |  | RTLatency |  |
| pduSetQosDl |  | PduSetQosParaRm |  | O |  | 0..1 |  | PDU Set QoS parameter(s) for the downlink direction. |  | PDUSetHandling |  |
| pduSetQosUl |  | PduSetQosParaRm |  | O |  | 0..1 |  | PDU Set QoS parameter(s) for the uplink direction. |  | PDUSetHandling |  |
| protoDescDl |  | ProtocolDescriptionRm |  | O |  | 0..1 |  | Downlink Protocol description for PDU Set identification, the detection of the end of data burst indication, the detection of the Data Burst Size marking indication, TTNB indication, and/or indication of whether MoQ or UDP-option is used to carry media related information. |  | PDUSetHandling PowerSaving TrafficCharChange OnPathN6MediaInfo |  |
| protoDescUl |  | ProtocolDescriptionRm |  | O |  | 0..1 |  | Uplink Protocol description for PDU Set identification in UE. |  | PDUSetHandling |  |
| periodUl |  | DurationMilliSecRm |  | O |  | 0..1 |  | Indicates the time period between the start of the two data bursts in units of milliseconds in Uplink direction. |  | PowerSaving |  |
| periodDl |  | DurationMilliSecRm |  | O |  | 0..1 |  | Indicates the time period between the start of the two data bursts in units of milliseconds in Downlink direction. |  | PowerSaving |  |
| l4sInd |  | UplinkDownlinkSupport |  | O |  | 0..1 |  | When provided, it represents an explicit indication of whether ECN marking for L4S support is supported for the UL, the DL or both, UL and DL. It may be present when the media component is initially provided. (NOTE 3) |  | L4S |  |
| datBurstSizeInd |  | boolean |  | O |  | 0..1 |  | Indicates the Data Burst Size marking for the DL service data flow is supported when it is included and set to "true". |  | TrafficCharChange |  |
| timetoNextBurstInd |  | boolean |  | O |  | 0..1 |  | Indicates the Time to Next Burst for the DL service data flow is supported, when it is included and set to "true". |  | TrafficCharChange |  |
| onPathN6SigInfo |  | OnPathN6SigInfo |  | O |  | 0..1 |  | Contains the on-path N6 signaling information for delivering media related information. |  | OnPathN6MediaInfo |  |
| expTranInd |  | boolean |  | O |  | 0..1 |  | Expedited Transfer Indication for the downlink traffic to enable expedited data transfer with reflective QoS for the Non-GBR service data flow. - "true": the expedited data transfer of larger payload for XR application is enabled for the flow. - "false": the expedited data transfer of larger payload for XR application is not enabled for the flow. |  | TrafficCharChange |  |
| NOTE 1: The attributes "altSerReqs" and "altSerReqsData" are mutually exclusive. NOTE 2: The "burstArrivalTimeWnd" attribute, within the "tscaiInputUl" and/or "tscaiInputDl" attributes, and the "capBatAdaptation" attribute are mutually exclusive. NOTE 3: Within a MediaComponentRm entry, the NF service consumer may include either the indication of L4S support within the "l4sInd" attribute or the request for congestion measurements within the "evSubsc" attribute included in one or more entries of the "medSubComps" attribute. A MediaComponent entry within the Individual Application Session Context resource shall not contain simultaneously both, the indication of L4S support and the subscription to congestion monitoring. |  |  |  |  |  |  |  |  |  |  |  |

# * * * * End of Changes * * * *