3GPP TSG-CT WG4 Meeting #131	C4-254155

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.564 | CR | 0152 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of UE NAT Mapping Information event |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The UE NAT Mapping Information event is used to inform the NEF of one or more Public UE IP address(es) and port(s) corresponding to a Private UE IP address. Based on the types of event notifications for this event type listed in clause of 5.2.2.3.1, For the event " UE_NAT_MAPPING_INFO ", this service operation is used by the UPF to send the following types of event notifications: - one-time immediate report, when in the subscription the event reporting mode is set to "ONE_TIME" and the indication for immediate reporting is set to true; - event triggered notification on the change of the information for (NATed) Public UE IP addresses and/or port(s) assigned to the PDU Session; - if no UE NAT mapping exists yet, the UPF sends an empty array to the consumer as part of the UPF event subscription response if the immediate reporting flag is included, otherwise the empty array is sent in the event subscription notify. it is unnessary to send any remaining data when the the subscription is terminated due to released N4 session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The remainingDataReports shall not be presnt when the type IE included in UpfEvent of subscrition requestion is set to "UE_NAT_MAPPING_INFO". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary messages may be triggered due to misuse of the attribute of remainingDataReports. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.3.6, 6.1.6.2.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR doesn't introduce any backward compatible correction to openAPI files. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.2.1.3.6	UE NAT Mapping Information

Table 5.2.1.3.6-1: UE NAT Mapping Information event

| Description | This event provides one or more Public UE IP address(es) and port(s) mapped to a Private UE IP address of a PDU session to a target remote end IP address, for a UPF that is deployed with NAT functionality. |
| --- | --- |
| Subscription type | Subscription to UPF |
| Subscription inputs to UPF | Required: - (5GC) Private UE IP address. - Public IPv4 or IPv6 address of the remote end - UPF event consumer notification URI - Notification correlation ID - immediate reporting indication Optional: - DNN - S-NSSAI - Port number of the remote end - IP domain |
| Report type | One-Time Report Continuous (event triggered) Report |

* * * Next Change * * * *

##### 6.1.6.2.13	Type: UpfEvent

Table 6.1.6.2.13-1: Definition of type UpfEvent

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| type | EventType | M | 1 | Describes the UPF event type to be reported |  |
| immediateFlag | boolean | O | 0..1 | Indicates if an immediate event report containing the currently available value / status of the event is requested. The report contains the value / status of the event currently available at the UPF at the time of the subscription. This IE shall be present and set to true if the type IE is set to "UE_NAT_MAPPING_INFO". It may be present otherwise. The default value is false. |  |
| measurementTypes | array(MeasurementType) | C | 1..N | This IE shall be present if the type IE is set to "USER_DATA_USAGE_MEASURES". When present, this IE shall indicate the types of requested measurements. |  |
| appIds | array(ApplicationId) | O | 1..N | Contains the application identifiers. (NOTE 1, NOTE 2) |  |
| trafficFilters | array(FlowInformation) | O | 1..N | Identifies IP or Ethernet packet filters. (NOTE 1, NOTE 2) |  |
| granularityOfMeasurement | GranularityOfMeasurement | O | 0..1 | Indicates the granularity of measurement. (NOTE 2) |  |
| reportingSuggestionInfo | ReportingSuggestionInformation | C | 0..1 | The IE should be present if the event notification can be delayed, i.e. it is delay tolerant. |  |
| remoteIpv4Addr | Ipv4Addr | C | 0..1 | Either this or the remoteIpv6Addr IE shall be present if the type IE is set to "UE_NAT_MAPPING_INFO". When present, this IE shall indicate the remote end IPv4 address for which the NAT mapping is requested. |  |
| remoteIpv6Addr | Ipv6Addr | C | 0..1 | Either this or the remoteIpv4Addr IE shall be present if the type IE is set to "UE_NAT_MAPPING_INFO". When present, this IE shall indicate the remote end IPv6 address for which the NAT mapping is requested. |  |
| remotePortNumber | Uint16 | O | 0..1 | This IE may be present if the type IE is set to "UE_NAT_MAPPING_INFO". When present, this IE shall identify the Port number of the remote end for which the NAT mapping is requested. |  |
| ipDomain | String | C | 0..1 | This IE shall be present if the type IE is set to "UE_NAT_MAPPING_INFO" and the DNN is configured with overlapping UE private address range(s). When present, this IE shall indicate the IP domain of the UE IP address. |  |
| remainingDataReports | RemainingDataReports | O | 0..1 | This IE may be present when subscribing to an event. When present, this IE shall indicate how to handle the data collected for the related event by the source UPF and not yet reported to the NF service consumer, when the subscription is terminated (e.g. when the corresponding N4 session is released). This IE shall not be presnt when the type IE is set to "UE_NAT_MAPPING_INFO". |  |
| skipReportInstruction | SkipReportingInstruction | O | 0..1 | This IE may be present when subscribing to an event. When present, it shall include the conditions under which the UPF shall skip sending event reports, e.g. when event reports would only contain null measurements, or measurements below a specific threshold, or outside the reporting period(s). | SEER |
| inclRatType | boolean | O | 0..1 | The IE may be present and set to true if the NF service consumer requests the UPF to include the RAT type in the corresponding event reports (i.e. in NotificationItem). true: the RAT type on which the UE is camping when the measurement is performed shall be included in the NotificationItem and multiple NotificationItems corresponding to the measurement reports for the same event but measured over different RAT type during a reporting period may be included in the report. Presence of this IE with false value shall be prohibited. |  |
| ratTypeList | array(RatType) | O | 1..N | The IE may be present if the inclRatType is set to true. When present, it contains a list of RatType(s) identifying a list of RAT types for which the NF service consumer require to receive the event reports from the PDU session(s) of the UE(s) camping on these RAT type(s). |  |
| NOTE 1: Either the appIds IE or the trafficFilters IE may be present, not both. NOTE 2: If the appIds or trafficFilters is provided, the granularityOfMeasurement shall not be set to "PER_SESSION". If neither appIds nor trafficFilters is provided, the granularityOfMeasurement may be set to "PER_SESSION", "PER_APPLICATION" or "PER_FLOW" to request the UPF to provide measurements with the corresponding granularity. |  |  |  |  |  |

* * * End of Change * * * *