3GPP TSG-CT WG4 Meeting #131	C4-254158

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.380 | CR | 0133 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the PCRF/PCF-based restoration solution for EPC/5GC NF failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IMS_RES-CT |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the clause 7.2.3 solution of "P-CSCF receiving EPC/5GC NF failure notification from PCRF/PCF", it requires: 4d. If the EPC/5GC NF failure is detected in 4a, 4b or 4c, the PCRF/PCF may store the failure status of the CN failure of the same PGW-C/SMF for future usage e.g. to avoid triggering Gx RAR message frequently. If any request or response with success code is received from the same PGW-C/SMF, the PCRF/PCF will update the status of the PGW-C/SMF. However, in step 4a, the condition in yellow highlight doesn't clearly indicate how to inform the P-CSCF of service unavailability. If the P-CSCF is not informed, the restoration procedure will not be triggered. 4a. The PCRF/PCF decides to monitor the status of CN nodes. If the PCRF/PCF has previously stored the failure status of the CN failure for the same PGW-C/SMF and the status is still valid, step 4a-4d are skipped. Thus it is proposed to complete the behaviour in this case to require the PCRF/PCF to include the service unavailability indication in the Rx Authorization-Authentication-Answer (AAA) or the Npcf_PolicyAuthorization_Create response. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Require the PCRF/PCF to inform the P-CSCF if the failure status of the CN failure has been stored in the PCRF/PCF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The restoration procedure will not be triggered when the failure status of the CN failure has been stored in the PCRF/PCF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.2.3.2	Description

Figure 7.2.3.2-1: P-CSCF receiving EPC/5GC NF failure notification from PCRF/PCF

1.	The terminating I/S-CSCF receives a SIP message for a destination UE and forwards the SIP message to this called UE's terminating P-CSCF.

2.	When the P-CSCF (acting as the AF) determines that session related media requires PCC supervision, or the access network information (i.e. user location and/or user timezone information) is required, the P-CSCF in terminating network sends a Rx Authorization-Authentication-Request (AAR) to the PCRF as specified in 3GPP TS 29.214 [7] or invokes the Npcf_PolicyAuthorization_Create service operation to the PCF as specified in 3GPP TS 29.513 [35].

3.	The PCRF/PCF returns the Rx Authorization-Authentication-Answer (AAA) or the Npcf_PolicyAuthorization_Create response to the P-CSCF.

4a.	The PCRF/PCF decides to monitor the status of CN nodes. If the PCRF/PCF has previously stored the failure status of the CN failure for the same PGW-C/SMF and the status is still valid, step 4b-4d are skipped and the PCRF/PCF includes the service unavailability indication in the Rx Authorization-Authentication-Answer (AAA) or the Npcf_PolicyAuthorization_Create response as decribed in step 3.

NOTE:	The PCRF/PCF decides to monitor the status of CN nodes based on the operator's configuration. Frequent CN node failure detection need be avoided. For example, the PCRF/PCF is configured to count the number of Rx sessions associated with the same PGW-C/SMF or the PCRF/PCF triggers the detection periodically.

4b.	PCRF/PCF sends a Gx Re-Auth-Request (RAR) or invokes the Npcf_SMPolicyControl_UpdateNotify service operation to the PGW-C/SMF.

4c.	If the PCRF/PCF receives no response from the PGW-C/SMF, it may determine the service unavailability of the PGW-C/SMF.

If the PGW-C/SMF detects some errors in the user plane, it may send a response to the PCRF/PCF with service unavailability indication which is considered as the case of EPC/5GC NF failure.

4d.	If the EPC/5GC NF failure is detected in 4a, 4b or 4c, the PCRF/PCF may store the failure status of the CN failure of the same PGW-C/SMF for future usage e.g. to avoid triggering Gx RAR message frequently. If any request or response with success code is received from the same PGW-C/SMF, the PCRF/PCF will update the status of the PGW-C/SMF.

5.	If the EPC/5GC NF failure is detected in steps 4b-4d, the PCRF/PCF sends Rx Abort-Session-Request (ASR) or invokes the Npcf_PolicyAuthorization_Notify service operation to the P-CSCF including the service unavailability indication.

6.	Based on this indication, P-CSCF decides to send a response with an error code (i.e. the UE registration data is not present) to the S-CSCF or not to send any response.

When the S-CSCF receives the error code (i.e. the UE registration data is not present) or no response, it considers the case as the called UE's terminating P-CSCF is not able to process this request. Then the steps 4-8 of the HSS based P-CSCF restoration procedure as described in clause 5.4.2.1 are executed.

* * * End of Change * * * *