3GPP TSG-CT WG4 Meeting #131	C4-254182

Sophia-Antipolis; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 1268 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Expiry Time Re-negotiation with AMF Change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When new AMF receives EE subscriptions created by UDM in the old AMF, this new AMF may require an expiry time shorter than the expiry time currently agreed. If that happens AMF shall notify the new expiry time to UDM so the EE subscription duration is updated in all involved nodes and UDM can inform NEF and other subscribed NFs about new expiry time of the EE subscription as agreed in C4-253454. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to update current Namf_EventExposure Notify operation to include the new expiryTime in the AmfEventNotification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If AMF is subscribed with a shorter expiry time and UDM/NEF/AF is not informed, the AMF subscription can expire while the EE subscription is still active and will not be renewed, which means AMF events happening after the new AMF expiry time will not be notified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.4.1, 6.2.5.2.1, 6.2.6.2.4, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR… CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

##### 5.3.2.4.1	General

The Notify service operation is invoked by the AMF, to send a notification, towards the notification URI, when certain event included in the subscription has taken place.

The AMF shall use the HTTP method POST, using the notification URI received in the subscription creation as specified in clause 5.3.2.2.2, including e.g. the subscription ID, Event ID(s) for which event has happened, notification correlation ID provided by the NF service consumer at the time of event subscription, to send a notification. See Figure 5.3.2.4.1-1.

Additionally, the Notify service operation shall also be invoked by the AMF, when:

-	there is a change of AMF during UE mobility procedures, if the subscription Id changes (i.e. Registration procedures and Handover procedures), or the expiry time in the new AMF is shorter than requested expiry time, or

-	the subscription is terminated by the AMF, if event subscription termination notification is requested by the NF consumer.

Figure 5.3.2.4.1-1 Notify

1.	The AMF shall send a POST request to send a notification.

2a.	On success, "204 No content" shall be returned by the NF Service Consumer.

2b.	On failure or redirection, the appropriate HTTP status code (e.g. "403 Forbidden") indicating the error shall be returned and appropriate additional error information should be returned.

When the AMF received the following response code (and application error), the AMF should consider the subscription is no longer valid and terminate the subscription:

-	"400 Bad Request" with application error "RESOURCE_CONTEXT_NOT_FOUND"

-	"404 Not Found"

When AMF terminates the subscription in above scenarios, if the subscription is created by the NF consumer on behalf of another NF (e.g. the UDM subscribes to the AMF on behalf of the NEF) and notification of event subscription termination is requested by the NF consumer, the AMF supporting the 'STEN" feature shall send a notification to the NF consumer (e.g. the UDM) to report the termination of the subscription.

# *** Next Change ***

##### 6.2.5.2.1	Notification Definition

Call-back URI: {callbackUri}

Call-back URI is provided by NF Service Consumer during creation of the subscription. If the notification is to inform the change of subscription ID or expiry time is shorter than requested expiry time and if the "subsChangeNotifyUri" was provided in the AmfEventSubscription, then this callback URI shall be the "subsChangeNotifyUri" provided in the AmfEventSubscription. Otherwise, this callback URI shall be the "eventNotifyUri" provided in the AmfEventSubscription.

# *** Next Change ***

##### 6.2.6.2.4	Type: AmfEventNotification

Table 6.2.6.2.4-1: Definition of type AmfEventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifyCorrelationId | string | C | 0..1 | This IE shall be included if the notification is not for informing creation of a new subscription Id. This IE shall also be included if the notification is for informing the creation of a new subscription Id and the corresponding event subscription did not contain subsChangeNotifyCorrelationId attribute (see clause 6.2.6.2.2). When present, this IE shall indicate the notification correlation Id provided by the NF service consumer during event subscription. This parameter can be useful if the NF service consumer uses a common call-back URI for multiple subscriptions. |  |
| subsChangeNotifyCorrelationId | string | C | 0..1 | This IE shall be included if the notification is for informing the creation of a new subscription Id at the AMF or for informing about new expiry time shorter than requested expiry time and the corresponding event subscription contains the subsChangeNotifyCorrelationId attribute (see clause 6.2.6.2.2). When present, this IE shall be set to the value of the subsChangeNotifyCorrelationId provided during subscription (see clause 6.2.6.2.2). |  |
| reportList | array(AmfEventReport) | C | 1..N | This IE shall be present if a event is reported. When present, this IE represents the event reports to be delivered. |  |
| eventSubsSyncInfo | AmfEventSubsSyncInfo | C | 0..1 | This IE may be present for AMF to initiate event subscription synchronization with UDM during UE mobility procedures. When present, this IE shall contain the information for event subscription synchronization, including all active event subscriptions specificially targeting the UE. | ESSYNC |
| amfId | NfInstanceId | C | 0..1 | This IE shall be present for an AMF set level bulk subscription. When present, it shall be set to the NF instance ID of the AMF instance sending the event report. See clause 5.3.1.4.3. | ASETSUB |
| expiry | DateTime | O | 0..1 | This IE shall be included, if, based on operator policy and taking into account the expiry time requested for the subscription, the AMF needs to update the expiry time. When present, this IE shall represent the UTC time after which the subscribed event(s) shall stop generating report and the subscription becomes invalid. |  |

# *** Next Change ***

# A.3	Namf_EventExposure API

openapi: 3.0.0

…

AmfEventNotification:

description: Data within a AMF Event Notification request

type: object

properties:

notifyCorrelationId:

type: string

subsChangeNotifyCorrelationId:

type: string

reportList:

type: array

items:

$ref: '#/components/schemas/AmfEventReport'

minItems: 1

eventSubsSyncInfo:

$ref: '#/components/schemas/AmfEventSubsSyncInfo'

amfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

…

# *** End of Changes ***