3GPP TSG-CT WG4 Meeting #133	C4-260150

# Goa, India; 09th – 13th February 2026

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.506 | CR | 0005 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update AiotDeviceProfileData to align with 29369 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | TSG CT WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2026-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The T-ID handling information is included in AIOT Device Profile Data as specified in clause 5.4.3 of 3GPP TS 33.369. In 3GPP TS 29.369, the T-ID handling information is included in Data Type "AiotDevProfileData". However, in 3GPP TS 29.506, the T-ID handling information is missing from Data Type "AiotDeviceProfileData". The AIOT Device Profile Data stored in ADM and UDR is inconsistent, which may lead to service interuption. So, this CR propose to eliminate the inconsistency by removing the data type definition in TS 29.506 and reusing the data type defined in TS29.369. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. change the AiotDeviceProfileData to AiotDevProfileData in GET method. 2. remove the data type definition of AiotDeviceProfileData and reuse the AiotDevProfileData from TS 29.369. 3. update the OpenAPI accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AIOT Device Profile Data stored in ADM and UDR is inconsistent, which may lead to service interuption. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.3.1, 5.5.1, 5.5.2.2, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the following APIs: TS29506_Aiot_Data.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

##### 5.2.3.3.1	GET

This method shall support the URI query parameters specified in table 5.2.3.3.1-1.

Table 5.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.3.3.1-2 and the response data structures and response codes specified in table 5.2.3.3.1-3.

Table 5.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AiotDevProfileData | M | 1 | 200 OK | Upon success, a response body containing AIoT Device Profile Data shall be returned. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - DATA_NOT_FOUND |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

==============Next change==============

### 5.5.1	General

This clause specifies the application data model supported by the API.

Table 5.5.1-1 specifies the data types defined for the Nudr_DataRepository for AIoT data service-based interface protocol.

Table 5.5.1-1: Nudr_DataRepository specific Data Types for AIoT data

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AiotDeviceProfileDataPatch | 5.5.2.3 | AIoT Device Profile Data for patch operation |  |

Table 5.5.1-2 specifies data types re-used by the Nudr_DataRepository for AIoT Data service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudr_DataRepository service based interface for Ambient IoT data.

Table 5.5.1-2: Nudr_DataRepository re-used Data Types for AIoT data

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AiotDevPermId | 3GPP TS 29.571 [18] | AIoT device permanent identifier |  |
| AiotDevProfileData | 3GPP TS 29.369 [19] | AIoT Device Profile Data |  |
| LastKnownAiotfInfo | 3GPP TS 29.369 [19] | Last Known AIoTF Information |  |
| Tid | 3GPP TS 29.369 [19] | Temporary ID (T-ID) |  |
| PatchResult | 3GPP TS 29.571 [18] |  |  |
| ProblemDetails | 3GPP TS 29.571 [18] | Used in error responses to provide more detailed information about an error. |  |
| SupportedFeatures | 3GPP TS 29.571 [18] | see 3GPP TS 29.500 [4] clause 6.6 |  |
| AfAuthorizationData | 3GPP TS 29.369 [19] | AF Authorization Data |  |
| AfId | 3GPP TS 29.369 [19] | AF ID |  |

==============Next change==============

==============Next change==============

#### 5.5.2.2	Void

==============Next change==============

# A.2	Nudr_DataRepository API for AIoT data API

For the purpose of referencing entities in the Open API file defined in this Annex, it shall be assumed that this Open API file is contained in a physical file named "TS29506_Aiot_Device_Profile_Data.yaml".

openapi: 3.0.0

info:

version: '-'

title: Unified Data Repository Service API file for Ambient IoT data

description: |

The API version is defined in 3GPP TS 29.504

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.506 V19.1.0; 5G System; Usage of the Unified Data Repository Service for AIoT Data

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.506/'

[…]

paths:

/aiot-data/aiot-device-profile-data/{aiotDevPermId}:

parameters:

- name: aiotDevPermId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

get:

summary: Retrieves the AIoT device profile data for an AIoT device permanent identifier

operationId: ReadAiotDeviceProfileData

tags:

- AiotDeviceProfileData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:aiot-data:aiot-device-profile-data

- nudr-dr:aiot-data:aiot-device-profile-data:aiotDevPermId:read

responses:

'200':

description: >

Upon success, a response body containing AIoT device profile data shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/AiotDevProfileData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify the AIoT device profile data for an AIoT device permanent identifier.

operationId: UpdateAiotDeviceProfileData

tags:

- AiotDeviceProfileData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:aiot-data:aiot-device-profile-data

- nudr-dr:aiot-data:aiot-device-profile-data:aiotDevPermId:modify

parameters:

- name: supported-features

in: query

description: Supported Features

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/AiotDeviceProfileDataPatch'

responses:

'200':

description: Expected response to a valid request.

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'204':

description: No content. Response to successful modification.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

[…]

# STRUCTURED TYPES

==============End of change==============