3GPP TSG-CT WG4 Meeting #131	C4-254258

# Sophia Antipolis, France; 13th – 17th October 2025	(was C4-254214)

Source:	Ericsson

Title:	Solution #2 drawbacks

Spec:	3GPP TR 29.867 v0.2.0

Agenda item:	21.4

Document for:	Approval

1. Introduction

Solution #2 in TR 29.867 is proposing the detection of a P-CSCF failure to avoid massive IMS Registrations in other interfaces (e.g. Rx) where SIP signaling traffic is not exchanged. However, an Rx interface failure does not preclude other interfaces carrying SIP traffic might be failing.

Furthermore, as it is described today in TR 29.867, the detection of a P-CSCF failure affects all users registered in this P-CSCF, sending notifications to all of them. Depending on the P-CSCF recovery time and the IMS re-registration timer used, the number of IMS Registration Retransmissions can be much lower than the number of notifications sent to the UEs.

As an example, if an operator providing IMS voice services has a P-CSCF serving 1 million UEs with a re-registration timer set to 1 hour and the P-CSCF recovery time lasts 15 minutes, 250k IMS Registrations will be currently evenly retried. However, with the proposed solution #2, in the same situation, 2 million notifications will be sent, which will imply sending notifications to UEs whose IMS Re-registration might not be even affected by the failure time.

2. Reason for Change

Some drawbacks related to solution#2 were missing.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 29.867.

* * * First Change * * * *

### 6.2.4	Cons

The cons of the solution:

-	Requires extending IE in PCO/ePCO.

-	The SIP Client of the UE needs to be modified (e.g. SIP registration suppression).

-	This solution describes several methods to detect that the P-CSCF is down. If all methods are used, in combination, the determination of the P-CSCF failure might be reasonably accurate; however, if just one method is used, the conclusion might not be correct; for example, detecting a failure solely by monitoring the Rx interface (as described in clause 6.2.1.2.1) does not imply that IMS Registrations are failing, on one hand P-CSCF may not be down and on the other hand, SIP signalling may follow a different routing path.

-	A bulk failure notification procedure, as described in clause 6.2.1.3, implies sending individual notifications for all users currently registered in this P-CSCF. Depending on the P-CSCF recovery time, the signalling generated for all these notifications can be much higher than the number of retries of IMS Registrations while the P-CSCF is not yet recovered which might not affect all users registered in this P-CSCF.

* * * End of Changes * * * *