3GPP TSG-CT WG4 Meeting #132	C4-255324

# Dallas, US; 17th – 21st November 2025	(was C4-255111)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.503 | CR | 1523 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Old PEI in Nudm_EE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Verizon, HPE, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc19 |  |  |  |  |  | Date: |  |  | 2025-10-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When there is a change on the association between SUPI and PEI, it is useful for fraud prevention to know, not only the new PEI that a given UE (SUPI) is using, but also the previous PEI used by this subscriber. Similarly, it is also helpful to receive the timestamp when the previous PEI was received and stored by UDM. A typical use case could be to detect when a SIM swap occurs between enterprise and consumer devices. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the old PEI (and timestamp) to the ChangeOfSupiPeiAssociationReport data type. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SUPI-PEI association event does not provide information about the previous PEI associated with a given SUPI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.6.2.7, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible new features with impacts on the following APIs: - TS29503_Nudm_EE.yaml - TS29520_Nnwdaf_AnalyticsInfo.yaml - TS29520_Nnwdaf_DataManagement.yaml - TS29520_Nnwdaf_RoamingData.yaml - TS29574_Ndccf_DataManagement.yaml - TS29575_Nadrf_DataManagement.yaml - TS29576_Nmfaf_3caDataManagement.yaml - TS29576_Nmfaf_ContextManagement.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * First Change * * * *

##### 6.4.6.2.7	Type: ChangeOfSupiPeiAssociationReport

Table 6.4.6.2.7-1: Definition of type ChangeOfSupiPeiAssociationReport

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| newPei | Pei | M | 1 | The new PEI |
| previousPei | Pei | O | 0..1 | The previous PEI, if available, prior to the reception of the new PEI by UDM |
| previousPeiTimestamp | DateTime | O | 0..1 | Timestamp when the previous PEI was received and stored by UDM. |

# * * * Next Change * * * *

# A.5	Nudm_EE API

(... text not shown for clarity ...)

ReachabilityForSmsReport:

type: object

required:

- smsfAccessType

properties:

smsfAccessType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

maxAvailabilityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

ChangeOfSupiPeiAssociationReport:

type: object

required:

- newPei

properties:

newPei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

previousPei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

previousPeiTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

RoamingStatusReport:

type: object

required:

- roaming

- newServingPlmn

properties:

roaming:

type: boolean

newServingPlmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

accessType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

purged:

type: boolean

enum:

- true

(... text not shown for clarity ...)

# * * * End of Changes * * * *