3GPP TSG-CT WG4 Meeting #132	C4-255354

Dallas, US; 17th – 21st November 2025 	was C4-255093

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0385 | rev | 1 | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transfer UE UP Positioning capabilities in LocContextData |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP TSG CT WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 24.501 specifies that the UE shall indicate its user plane positioning capabilities in the 5GMM capability IE of the REGISTRATION REQUEST message. TS 23.273 further specifies that the AMF shall convey these UE user plane positioning capabilities to the LMF in the DetermineLocation service operation to enable appropriate user plane configuration and LCS context handling. However, TS 29.572 currently does not provide a mechanism to transfer the UE’s user plane positioning capabilities when location contexts are handed over from a source LMF to a target LMF (e.g., during LMF relocation or AMF change). As a result, the target LMF may not be aware of the UE’s supported user plane capabilities, leading to inconsistent or incorrect handling of ongoing LCS-UPP connections |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an optional ueUpPosCaps IE in LocContextData to convey the UE’s user-plane positioning capabilities. Require the source LMF to include ueUpPosCaps when transferring context to a target LMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 29.572 remains inconsistent with TS 24.501 and TS 23.273, risking incorrect or incomplete behaviour in handover / LMF relocation scenarios and causing potential interoperability issues between implementations. The target LMF does not able to determine whether the UE supports the LCS-UPP capability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.5.2, 6.1.6.2.30, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to the following APIs: TS29572_Nlmf_Location.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

##### 5.2.2.5.2	Transfer Location Context

This procedure allows a NF service consumer (e.g. the old LMF) to transfer location context information for periodic or triggered location for a target UE (see clause 6.4 and clause 6.7.2 of 3GPP TS 23.273 [19]). The NF service consumer discovers the service URI of the new LMF by performing a discovery via NRF using:

-	the identification of the LMF received (from an AMF) during an earlier Namf_Communication_N1MessageNotify service operation to the consumer NF;

otherwise (if not available),

-	the identification of an LMF locally provisioned in the consumer NF.

Figure 5.2.2.5.2-1: LocationContextTransfer Request

1.	The NF Service Consumer shall send an HTTP POST request to the Custom operation URI ("/location-context-transfer") on the Service URI discovered as described above.

The request body shall include an AMF identity, Deferred location type, Deferred location parameters, Notification Target Address (H-GMLC callback URI), Notification Correlation ID (LDR reference), an embedded event report message and may include an event reporting status, UE location information, scheduled location time and LOS/NLOS measurement indication, or UE user plane positioning capabilities.

If an N1 message is received from the UE with Control Plane CIoT 5GS Optimisation, the request body shall include an indication of Control Plane CIoT 5GS Optimisation.

If the location context information for periodic or triggered location for a target UE includes the endpoint address of the location reporting over user plane, the NF Service Consumer may include the cumulative event report timer, or the maximum number of location reports over user plane.

If the location context(s) being transferred are associated with the LCS-UPP connection between the UE and the source LMF, the source LMF shall include the lcsUppExistInd IE with the value true for the associated location context(s). If the event report message from the UE is not available for the location context(s) being transferred, the NF Service consumer shall set the event class of the eventReportMessage IE to the value "DUMMY" in the LocContextData object and the target LMF shall ignore the received eventReportMessage IE with event class set to the value "DUMMY".

2a.	On success, "204 No content" shall be returned by the LMF.

2b.	On failure or redirection, one of the HTTP status codes listed in Table 6.1.4.4.2-2 shall be returned. For a 4xx/5xx response, the message body should contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.4.4.2-2.

==============Next change==============

##### 6.1.6.2.30	Type: LocContextData

Table 6.1.6.2.30-1: Definition of type LocContextData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| amfId | NfInstanceId | M | 1 | Indicates the AMF Instance serving the UE. LMF shall use the AMF Instance to forward LCS related N1/N2 messages to the UE/RAN. |
| locationQoS | LocationQoS | C | 0..1 | This IE shall contain the location QoS if available. |
| supportedGADShapes | array(SupportedGADShapes) | C | 1..N | This IE shall contain the supported GAD shapes if available. |
| supi | Supi | C | 0..1 | This IE shall contain the SUPI if available. |
| gpsi | Gpsi | C | 0..1 | This IE shall contain the GPSI if available. |
| ldrType | LdrType | M | 1 | The type of LDR |
| hgmlcCallBackURI | Uri | M | 1 | Callback URI of the H-GMLC |
| ldrReference | LdrReference | M | 1 | LDR Reference |
| periodicEventInfo | PeriodicEventInfo | C | 0..1 | Information for periodic event reporting. (NOTE) |
| areaEventInfo | AreaEventInfo | C | 0..1 | Information for area event reporting. (NOTE) |
| motionEventInfo | MotionEventInfo | C | 0..1 | Information for motion event reporting. (NOTE) |
| eventReportMessage | EventReportMessage | M | 1 | Contains an embedded event report. When the event report message from the UE is not available for the location context, e.g. during LCS-UPP connection modification procedure, the EventClass of this IE shall be set to "DUMMY" and the target LMF shall ignore the received IE with EventClass set to the value "DUMMY". |
| eventReportingStatus | EventReportingStatus | O | 0..1 | Status of event reporting |
| ueLocationInfo | UELocationInfo | O | 0..1 | Location information for the target UE |
| cIoT5GSOptimisation | boolean | C | 0..1 | This IE shall be present if it was received from AMF. When present, it shall be set as follows: - true: Control Plane CIoT 5GS Optimisation was used and no signalling or data is currently pending for the UE at the AMF. - false (default): Control Plane CIoT 5GS Optimisation was not used or signalling or data is currently pending for the UE at the AMF. |
| ecgi | Ecgi | C | 0..1 | When present, this IE shall indicate the identifier of the E-UTRAN cell serving the UE. This IE shall be present if it was received from AMF. |
| ncgi | Ncgi | C | 0..1 | When present, this IE shall indicate the identifier of the NR cell serving the UE. This IE shall be present if it was received from AMF |
| guami | Guami | C | 0..1 | This IE shall be present if it was received from AMF. When present, it shall contain the GUAMI serving the UE. |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.9 is supported. |
| uePositioningCap | UePositioningCapabilities | O | 0..1 | When present, this IE shall indicate the positioning capabilities supported by the UE. |
| ueUpPosCaps | array(UeUpPositioningCapabilities) | C | 1..N | This IE shall be present if the source LMF has received the UE user plane positioning capabilities. The IE shall contain the supported UE user plane positioning capabilities. |
| scheduledLocTime | DateTime | O | 0..1 | When present, this IE shall contain the scheduled time (in UTC) that the UE needs to be located. |
| indoorOutdoorInd | IndoorOutdoorInd | O | 0..1 | When present, this IE shall indicate whether the UE is indoor or outdoor. |
| losNlosMeasureInd | LosNlosMeasureInd | O | 0..1 | When present, this IE shall indicate whether LOS measurement or NLOS measurement is used. |
| upCumEvtRptCriteria | UpCumEvtRptCriteria | O | 0..1 | When present, this IE shall include the cumulative event report timer or the maximum number of location reporting over user plane or both. |
| lcsUppExistInd | boolean | C | 0..1 | This IE shall be present with the value true, if the location context being transferred in this requested is associated with the LCS-UPP connection between the UE and the source LMF. Presence of this IE with the value false shall be prohibited. |
| NOTE: At least one of periodicEventInfo, areaEventInfo or motionEventInfo shall be present in the LocContextData structure. |  |  |  |  |

==============Next change==============

# A.2	Nlmf_Location API

openapi: 3.0.0

info:

version: '1.3.4'

title: 'LMF Location'

description: |

LMF Location Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.572 V18.10.0; 5G System; Location Management Services; Stage 3

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.572/'

servers:

- url: '{apiRoot}/nlmf-loc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nlmf-loc

[Skipped for clarity]

LocContextData:

description: Information within Transfer Location Context Request.

type: object

required:

- amfId

- ldrType

- hgmlcCallBackURI

- ldrReference

- eventReportMessage

properties:

amfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

locationQoS:

$ref: '#/components/schemas/LocationQoS'

supportedGADShapes:

type: array

items:

$ref: '#/components/schemas/SupportedGADShapes'

minItems: 1

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

addLocContextList:

type: array

items:

$ref: '#/components/schemas/AddLocationContext'

minItems: 1

ldrType:

$ref: '#/components/schemas/LdrType'

hgmlcCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ldrReference:

$ref: '#/components/schemas/LdrReference'

periodicEventInfo:

$ref: '#/components/schemas/PeriodicEventInfo'

areaEventInfo:

$ref: '#/components/schemas/AreaEventInfo'

motionEventInfo:

$ref: '#/components/schemas/MotionEventInfo'

eventReportMessage:

$ref: '#/components/schemas/EventReportMessage'

eventReportingStatus:

$ref: '#/components/schemas/EventReportingStatus'

ueLocationInfo:

$ref: '#/components/schemas/UELocationInfo'

cIoT5GSOptimisation:

type: boolean

default: false

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

uePositioningCap:

$ref: '#/components/schemas/UePositioningCapabilities'

ueUpPosCaps:

type: array

items:

$ref: '#/components/schemas/UeUpPositioningCapabilities'

minItems: 1

scheduledLocTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

indoorOutdoorInd:

$ref: '#/components/schemas/IndoorOutdoorInd'

losNlosMeasureInd:

$ref: '#/components/schemas/LosNlosMeasureInd'

upCumEvtRptCriteria:

$ref: 'TS29515_Ngmlc_Location.yaml#/components/schemas/UpCumEvtRptCriteria'

lcsUppExistInd:

type: boolean

enum:

- true

[Skipped for clarity]

==============End of change==============