33GPP TSG-CT WG4 Meeting #133	C4-260357

# Goa, India; 09th – 13th February 2026

# Title:	Reply LS on the support for Analytics for Redundant Transmission Experience

# Release:	Release 19

# Work Item:	TEI19

Source:	CT4

To:	SA2

Cc:	CT3

Contact Person:

Name:	Frank Yong Yang

Tel. Number:

E-mail Address:	frank.yong.yang@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None.

1. Overall Description:

CT4 discussed the support of the Analytics for Redundant Transmission Experience for redundant PDU sessions.

Analytics for Redundant Transmission Experience can be used by PCF to determine if the UE shall activate a redundant PDU session as specified in clause 6.13.1 of 3GPP TS 23.288.

If the Analytics ID "Redundant Transmission Experience" is subscribed, the PCF will receive the output data "Table 6.13.3-1: Redundant Transmission Experience statistics" and "Table 6.13.3-2: Redundant Transmission Experience predictions" as described in clause 6.13.3 of TS 23.288 from the NWDAF.

Redundant Transmission Experience statistics and Redundant Transmission Experience predictions would include the measurement information on the average and variance, e.g. for E2E UL/DL packet delay, for both cases when the Redundant Transmission Status is activated and when the Redundant Transmission Status is NOT activated.

Then, based on the difference e.g. for the E2E UL/DL packet delay measure result, the PCF may decide to request the UE to activate a redundant PDU session subsequently, e.g. if the value for the case when redundant transmission is on is significantly smaller than when redundant transmission is NOT activated, i.e. redundant transmission is useful.

Therefore, the NWDAF needs to know whether the measurement reported from the UPF is associated with a redundant PDU session or not.

CT4 discussed two possible solutions:

1. 	Requiring the SMF to include the UE IP address in the SMF event report sent to the NWDAF for the RED_TRANS_EXP event, so the NWDAF can use the UE IP address to correlate the event report sent from the UPF to know if the QoS measurement is for a redundant PDU session; or

2. 	Enabling the SMF to populate the PDU Session Pair ID and/or RSN to the UPF, so that the UPF can include the same in the UPF QoS monitoring event report to the NWDAF.

2. Actions:

To SA2 group.

ACTION: 	CT4 kindly asks SA2 group to provide the feedback for the issue and the corresponding solutions, and update their specification if necessary.

3. Date of Next CT4 Meetings:

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm