3GPP TSG-CT WG4 Meeting #133	C4-260372

Goa, India; 09th – 13th February 2026		(revision of C4-260319)

# Source:	Samsung

# Title:	New WID on Providing per-subscriber Allowed MAC addresses from UDM

# Document for:	Approval

# Agenda Item:	20.2.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	CT aspects of Providing per-subscriber Allowed MAC addresses from UDM

Acronym:	TEI20_MACSUB-CT

Unique identifier:	1110003

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| TEI20_MACSUB | SA2 | 1090008 | Providing per-subscriber Allowed MAC addresses from UDM |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

The support for allowed MAC addresses handling in 5GS for Ethernet PDU Sessions is currently rather limited. As

specified in TS 23.501, clause 5.6.6 and 5.6.10.2:

- 	There is support for a DN-AAA Server to provide a list of allowed MAC addresses to SMF (for a maximum of 16 MAC addresses). This allows the UPF to allow packets to/from a PDU Session only if it has one of the allowed MAC addresses. Ethernet frames with other MAC addresses will be dropped.

As the DN-AAA server is invoked only when secondary authentication and authorization is enabled for the corresponding DNN using which UE has triggered PDU session or based on SMF policies which may require DN authorization without Secondary authentication/authorization, support for SMF/UPF to allow/block MAC addresses with different values per subscriber is thus not available. An appropriate place for per-subscriber MAC address handling information is in SM subscription data in UDM/UDR as was done for TEI19_VLANSUB for per-subscriber VLAN handling and the impact will be limited to SMF and UDM/UDR.

SA#109 approved SP-251213 as new WID “Providing per-subscriber Allowed MAC addresses from UDM” (TEI20_MACSUB) for SA2 normative work. Upon SA#110 approved related CRs and reported completion of this WID, the scope of the WID is confirmed accordingly.

Considering above, impacts on CT aspects of protocols and interfaces under CT WG4 responsibilities are foreseen and the related work in CT WG4 should be carried out within Rel-20.

# 4	Objective

The objective of this work item is to specify the stage 3 aspects to Provide per-subscriber Allowed MAC addresses from UDM according to stage 2 requirements agreed under the stage 2 work item TEI20_MACSUB.

The following impacts on 3GPP CT working groups are identified:

CT4

-	Impacts on adding allowed MAC address and handling information in the Session Management Subscription data (data needed for PDU Session Establishment).

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 29.503 | Impacts on adding allowed MAC address and handling information in the Session Management Subscription data (data needed for PDU Session Establishment). | TSG#115 (March, 2027) | CT4 |
| 29.244 | Impacts on adding allowed MAC address and handling information in the Session Management Subscription data | TSG#115 (March, 2027) | CT4 |

# 6	Work item Rapporteur(s)

Nivedya Parambath Sasi, nivedya.ps@samsung.com

# 7	Work item leadership

CT4

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

Samsung

CEWiT

Oracle

Ericsson

Tejas Networks Limited

InterDigital