3GPP TSG RAN Meeting #109												RP-251973

Beijing, China, September 15-18, 2025

Agenda Item:	[8.2.1.3]

Source:	Robert Bosch GmbH

Title:	Text Proposal for TR 38.914 for Architecture Requirements of Local Networks in 6G

Document for:	Discussion/Decision

# Introduction

Many vertical use cases require highly localized wireless communication within systems for e.g., within a vehicle to complement existing communication technologies e.g., Ethernet. Such wireless local networks may bring many benefits, such as lower complexity, higher flexibility, and lower costs for e.g., by simplifying the wiring harness in a car. However, the existing 3GPP communication modes and topologies may not be optimal for such localized communication.  The integration and operation of all system components/devices through the 3GPP infrastructure can be cumbersome or might not be feasible at all if only short-range direct communication within a system is required. Besides, loss of the connection to the 3GPP network may lead to an outage of the system, which is not tolerable for such systems e.g., in a car. The routing of local traffic and management functions through public mobile networks may also lead to unnecessarily higher latency and network load, as well as inefficient usage of available spectrum.

3GPP local networks were approved in SA1 as a 6G use case and the requirements have been already captured in TR 22.870 section 11.9 and 11.10. Local networks are use-case-centric and attached to a mobile network and have partial autonomy to survive during loss of connectivity to the mobile network. The mobile network is envisioned to provide external connectivity for local networks and provide bulk communication resources for local communication between local network devices. The internal traffic of local networks may be realized by intra-local network coordination, i.e. resource management within a local network using the bulk resources granted by the mobile network. This means a local control plane and data plane are required to deploy local networks. The local control plane is configured by the mobile network and a local controller is responsible for the management of it. In the most cases, data is exchanged through local data plane using allocated resources by the local controller.

This contribution presents a Text proposal for TR 38.914 section “5.2 Requirements for architecture and migration” to capture 6G RAN architecture support for local networks.

======================Start of the changes========================

## 5.2	Requirements for Architecture and Migration

5.2.X Local Networks RAN Architecture

5.2.X.1 Local control plane of local networks

NOTE 1: A local network has a local control plane, controlled by a local network device with gNB functionalities, which configures and manages other UEs in the local network. The required configuration for local connectivity can be done by mobile network gNB.

5.2.X.2 Local data plane of local networks

NOTE 2: Devices in a local network need to communicate locally and the user traffic does not leave the local network.

5.2.X.3 Radio resource management aspects of local networks

NOTE 3: Mobile network gNB needs to allocate required resources for local communication in a local network in bulk. Further allocation of resource can be done by a local network controller.

# ========== End of Changes ==========