3GPP TSG RAN meeting #109                                                 RP-252033

Beijing, China, Sep 15-18, 2025

Title: 	Views on Rel-20 UE RF topics for 5G-A

Source: 	ZTE Corporation,Sanechips

Agenda item:	9.1.4.1

Document for:	Discussion

Introduction

In the RAN#108 meeting, there were lots of discussions on Rel-20 RAN4 package and the discussion outcomes are endorsed in the way forward on RAN4 R20 package [1]. In this contribution, we would like to share some further views on Rel-20 UE RF topics for 5G-A.

Discussion

- 2.1. UE RF enhancement

- 2.1.1 ZTE’s view

The high level views on the companies’ Rel-20 UE RF proposals so far are summarized in the following table.

The high-level scope of the WI on UE RF centric evolution (the exact scope and WID draft will be discussed and decided in RAN#109): - FR1 HPUE enhancement - Further consider/discuss until RAN#109 on the detailed scope - This work is to focus on the TN application - 6MHz channel bandwidth - It can be prioritized to FDD band <1GHz - Study and, if feasible, reduce UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders, i.e., 64QAM , 256QAM - BS demod scope is included. - The study is performed based on non-AI-based approach at BS receiver as baseline and without RAN1 impact - The details for study is to be discussed until RAN#109 - This work will be started in Q1’26. If the similar scope (The examples include the enhancements enabled by BS advanced receiver and UE Tx signal modifications. It is noted that referring to these examples in this WF should not be deemed as taking these examples as the baseline or starting point in 6G study) is agreed by RAN4 to be studied in 6G, it will be removed from 5G-A package. In this case, it is possible that 6G related conclusion can be applied to 5G. - No RAN1/2 scope is expected - Target 0.5 RF TU in Q4’25 and 1.25 TU (0.75 RF and [0.5] Demod for EVM relaxation and 6MHz) from Q1’26.

For HPUE objective, considering the workload and limited TU for Rel-20 UE RF enhancement, we propose to only focus on single carrier with HPUE operation.

Proposal 1: for the HPUE in Rel-20, we propose to focus on the following objective:

  - Specify the requirements for PC1 single carrier with 4Tx for FWA devices (i.e. 4*26dBm)

    - Example bands: n41, n77/n78, n79, n104

  - Investigate the feasibility of PC1.5 TDD band single carrier with 1Tx for both handheld and FWA device and PC1 TDD band single carrier with 2Tx for FWA device and specify the requirements if the feasibility is confirmed.

    - Example bands: n41, n77/n78, n79, n104

  - Investigate and specify the requirements for PC1.5 FDD band single carrier with 2Tx.

    - Example bands: n28, n1, n3

Proposal 2: for 6MHz objective, propose to use the similar wording as 7MHz by the removal of demod requirement;  NOTE: for 7MHz, there was no single carrier demod requirement defined for it.

Proposal 3: for EVM relaxation objective, we propose to add the following sub-bullets to streamline the discussion in RAN4.

  - The study is performed based on non-AI-based approach at BS receiver as baseline and without RAN1 impact

    - The non-linearity issue from UE transmitter side should be investigated including the realistic PA modelling, memory effect, IQ image, DC leakage, CFR, DPD etc;

    - The type of non-AI based receiver for EVM compensation and the BS implementation complexity;

    - Performance gain for this non-AI based approach compared with the legacy receiver;

- 2.1.2 WID objective

Core part:

- FR1 HPUE enhancements

  - Specify the requirements for PC1 single carrier with 4Tx for FWA devices (i.e. 4*26dBm)

    - Example bands: n41, n77/n78, n79, n104

  - Investigate the feasibility of PC1.5 TDD band single carrier with 1Tx for both handheld and FWA device and PC1 TDD band single carrier with 2Tx for FWA device and specify the requirements if the feasibility is confirmed.

    - Example bands: n41, n77/n78, n79, n104

  - Investigate and specify the requirements for PC1.5 FDD band single carrier with 2Tx.

    - Example bands: n28, n1, n3

- 6MHz channel bandwidth

  - It can be prioritized to FDD band <1GHz

  - Specify the generic core requirements to support 6 MHz NR FR1 channel bandwidth [RAN4]

          - Introduce the channel bandwidth 6MHz

            - Only include 15 kHz SCS

          - Specify the spectrum utilization

          - Specify UE RF requirements for UL and DL

          - Specify BS RF requirements for UL and DL

  - Specify the band specific core requirements to introduce 6 MHz for NR band [nXX]. [RAN4]

          - BS and UE Channel bandwidth per operating band

          - UE reference sensitivity

          - When needed:

            - MPR (relative bandwidth criteria)

            - Additional Maximum Power Reduction (A-MPR), including both for PC3 and PC2.

  - Enable, if feasible, optional UE support of 6 MHz channel bandwidth from Rel-15 onwards in a release independent manner. [RAN2, RAN4]

          - No impact to 38.307 is expected.

  - RAN4 has introduced the new channel bandwidth, RAN2 will introduce the necessary signalling for this channel bandwidth. [RAN2]
Note:	This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN2 will be informed by RAN4 via LS. No additional RAN2 TUs will be requested for this objective.

  - Once RAN4 has introduced the new channel bandwidth, RAN3 will introduce the necessary signalling for this channel bandwidth [RAN3]

Note:This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN3 will be informed by RAN4 via LS. No additional RAN3 TUs will be requested for this objective.

- Study and, if feasible, reduce UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders, i.e., 64QAM , 256QAM

  - BS demod scope is included.

  - The study is performed based on non-AI-based approach at BS receiver as baseline and without RAN1 impact

    - The non-linearity issue from UE transmitter side should be investigated including the realistic PA modelling, memory effect, IQ image, DC leakage, CFR, DPD etc;

    - The type of non-AI based receiver for EVM compensation and the BS implementation complexity;

    - Performance gain for this non-AI based approach compared with the legacy receiver;

  - This work will be started in Q1’26. If the similar scope (The examples include the enhancements enabled by BS advanced receiver and UE Tx signal modifications. It is noted that referring to these examples in this WF should not be deemed as taking these examples as the baseline or starting point in 6G study) is agreed by RAN4 to be studied in 6G, it will be removed from 5G-A package. In this case, it is possible that 6G related conclusion can be applied to 5G.

- No RAN1/2 scope is expected

Perf part:

- Specify necessary BS conformance tests for 6MHz (RAN4)

- Performance requirement for non-AI based receiver which depends on the SI outcome;

Conclusions

In this contribution, we shared some further views on Rel-20 UE RF enhancement based on RAN#108 moderator’s way forward.

Proposal 1: for the HPUE in Rel-20, we propose to focus on the following objective:

  - Specify the requirements for PC1 single carrier with 4Tx for FWA devices (i.e. 4*26dBm)

    - Example bands: n41, n77/n78, n79, n104

  - Investigate the feasibility of PC1.5 TDD band single carrier with 1Tx for both handheld and FWA device and PC1 TDD band single carrier with 2Tx for FWA device and specify the requirements if the feasibility is confirmed.

    - Example bands: n41, n77/n78, n79, n104

  - Investigate and specify the requirements for PC1.5 FDD band single carrier with 2Tx.

    - Example bands: n28, n1, n3

Proposal 2: for 6MHz objective, propose to use the similar wording as 7MHz by the removal of demod requirement;  NOTE: for 7MHz, there was no single carrier demod requirement defined for it.

Proposal 3: for EVM relaxation objective, we propose to add the following sub-bullets to streamline the discussion in RAN4.

  - The study is performed based on non-AI-based approach at BS receiver as baseline and without RAN1 impact

    - The non-linearity issue from UE transmitter side should be investigated including the realistic PA modelling, memory effect, IQ image, DC leakage, CFR, DPD etc;

    - The type of non-AI based receiver for EVM compensation and the BS implementation complexity;

    - Performance gain for this non-AI based approach compared with the legacy receiver;

References

[1] RP-251879,Way forward on RAN4 R20 package, RAN4 Chair (Apple), Approved.