3GPP TSG RAN Meeting #109			RP-252063

Beijing, China, September 15-18, 2025

Agenda item:	8.2.1.4

Title:	Views on 6G Requirements of New and Existing Services

Source:	SK Telecom

Document for:	Discussion

1. Introduction

At RAN#107, the revised SID on 6G requirements has been approved with updating the objectives as follows [1]:

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN - Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting - Energy efficiency and energy saving: both for network and device - Enhanced spectral efficiency - Enhanced overall coverage, focus on cell-edge performance and UL coverage - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band - Target scalable and forward compatible design for diverse device types - Improved spectrum utilization and operations taking into account diverse spectrum allocations - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs - Aim at a harmonized 6G Radio design for TN and NTN, including their integration - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework Note: The detailed time plan for the high-level decisions above will be decided at RAN#108 at the approval of the 6G RAN WG SI and is to be aligned with the RAN SI progress.

In this contribution, we aim at providing our view on 6G requirements which should be taken into account.

2. Discussion

From our perspective, the following requirements for existing services should be considered for 6G RAN study in rel-20 (and also for further normative work). Of course, it should be noted that some of the requirements may possibly be applied to new services as well.

- Improved user experience, TCO reduction, and creation of new revenue streams shall be taken into account for designing 6G radio.

- 6G radio shall be based on the principles that minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

- 6G radio shall support network sharing (i.e. MOCN, MORAN and relevant enhancements if any).

- 6G radio shall support FR1, FR2 and also FR3.

- 6G radio shall be energy-efficient for both network and UE.

- 6G radio shall provide enhanced spectral efficiency compared with baseline.

  - Note: FFS on baseline

- 6G radio shall provide enhanced overall coverage while maintaining performance.

  - Note: For ensuring re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band, proper evaluation should be conducted during SI.

- 6G radio shall support MIMO with providing higher spectral efficiency while ensuring coverage comparable to 5G.

  - The overall framework (inc. CSI reporting mechanism and MIMO codebook) shall be studied for aiming at compactness and efficiency while providing performance.

- 6G radio shall support repeater with improving the positioning accuracy, minimizing the power consumption, enabling separate power control from access link, and enabling to provide the statistics per repeater.

- 6G radio shall aim at providing the functionalities necessary for security, integrity and privacy.

  - Low layer security shall be studied in rel-20 SI.

- Any feature/functionality of 6G radio should be evaluated under practical assumption/configuration.

  - Note: The performance benefit can be over-estimated by evaluation with unrealistic assumption typically not used in commercial network(e.g.,5ms CSI-RS periodicity).

- 6G radio shall support unified access control to enable operators to control the access attempt, from unattended data traffic, in congestion scenarios.

From our perspective, the following requirements for new services should be considered for 6G RAN study in rel-20 (and also for further normative work).

- 6G radio shall support use of AI for efficient/effective RAN operation including performance enhancement, NW optimization, and energy saving.

  - How to ensure secure, efficient, and manageable model transfer and delivery shall be studied in rel-20 SI.

- 6G radio shall support enabling AI applications especially considering the orchestration of computing resources (e.g., xPU) shared by RAN and AI workload.

- 6G radio shall pursue the unified LCM framework for AI/ML that can be universally applied to the use cases, e.g., including collection, management, and distribution of data.

- 6G radio shall support sensing capabilities such as object detection, localization, and tracking, while being tightly integrated with RAN functionalities and AI-driven processing.

  - All sensing modes shall be studied in rel-20 SI, and then 6G radio shall support the mode(s) justified based on the outcome of RAN WG SI.

- 6G radio shall support IDLE and CONNECTED mode mobility between TN and NTN at least with 5G. Additionally, whether/how to support IDLE and CONNECTED mode mobility between TN and NTN with 4G shall be studied in rel-20 SI.

- 6G radio shall support NTN with enhanced capacity through mechanisms such as CA and satellite MIMO.

- 6G radio shall support TN protection from cross‑border satellite signal interference.

- 6G radio shall support IoT/NR-NTN coexistence through a TN harmonized 6G NTN design.

- The requirements in section 2 should be considered for 6G RAN study in rel-20.

3. Conclusion

In this contribution, the following proposals are provided.

- The requirements in section 2 should be considered for 6G RAN study in rel-20.

References

- RP-250810, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, RAN#107.