3GPP TSG RAN Meeting #109   											   RP-252085

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.2

Source:	CMCC

Title:	Discussion on 3GPP energy efficiency requirement

Document for:	Approval

Introduction

In RAN#107, the SID of Study on 6G Scenarios and requirements has been approved [1]. Simultaneously, during the discussion in RAN#106-RAN#108, energy efficiency had been agreed as a test performance requirement for ITU-R (TPR) [2-3], and a key performance indicator (KPI) for 3GPP captured in current version of TR 38.914 [4]:

Agreement in RAN#107 meeting: Proposal 4: Define energy efficiency as a TPR. Agreement in RAN CC#1 and CC#2 meeting: Proposal 2-4: Sustainability is covered by energy efficiency, do not define separate TPR for sustainability.

In this paper, we provide our views on 3GPP energy efficiency requirement, including the definition of energy efficiency and further consideration on 6G power model.

Definition of energy efficiency

In this section, we discuss the definition of energy efficiency in 6G.

In IMT-2020 [5], Energy efficiency is defined as an inspection TPR while current ITU-R WP5D is discussing whether it can be quantitative in IMT-2030. Simultaneously, compare with 5G, 6G:

- Introduces higher frequency range (e.g. 6.425-7.125 GHz), which results in higher requirement on accuracy and power consumption for RF component.

- Introduces larger bandwidth (e.g. up to 200/400 MHz), which results in higher requirement (e.g. FFT size) and power consumption for BB component.

- Introduces larger TRX number (e.g. up to 128/256 TRX), which results more complexity (e.g. beamforming operation in analog/digital domain) and higher power consumption for BB/IF/RF component.

Based on the above analysis, 6G has a higher demand for energy and energy efficiency should be given special attention. Therefore, it is recommended that /energy efficiency should be considered as a quantitative KPI.

Proposal 1: Define energy efficiency as a quantitative KPI in TR 38.914.

Furthermore, ITU-WP5D is discussing the evaluation method for energy efficiency and is considering to define the relative power saving ratio under different load cases. From our point of view, such mechanism is a good starting point for 3GPP and can be further considered from the following aspects:

- The first point is how to calculate the power consumption for the network and UE. Thanks to the hard work during Rel-18/19 NES and Rel-15/16/17/18/19 UE power saving, the comprehensive power models for BS and UE are provided in TR 38.864 [6], TR 38.840 [7], TR 38.869 [8] and TR 38.865 [9], respectively. Such mechanism can be considered as baseline, or other power model should be clarified/reported by the companies, if any.

- The second point is how to ensure the experience while enhancing the energy efficiency. Regarding such issue, we prefer to introduce the constraint on certain metric while evaluate the network EE or Device EE. To be specific:

  - The relative target of certain metric (e.g. threshold value for the metric, or certain performance loss compared with the benchmark) can be set. Constant like certain percent (e.g. 90%) of the UE fulfils the target should be achieved while evaluating the relative power saving for each load case.

  - In addition, since different service may sensitive to different type of metric, it is recommended that the certain metric can be selected according to the service type, for instance:

    - For Vo6G (i.e. VoNR-like) service, the certain metric is latency

    - For burst-buffer service, the certain metric is QoS or UPT

    - For XR service, the certain metric is latency

- The third point is the definition for reference case of relative power saving under UE EE. From our understanding, when NW is fully loaded, a certain UE may not be fully loaded due to there is more than one UE under each BS. Moreover, the load of UE under NW fully loaded case is strongly relevant to the number of UEs under a certain BS, and simply reuse the definition of reference cases under NW EE will cause ambiguity. Therefore, it is better to clarify that the reference case of UE EE is when UE is fully-loaded/has full-buffer service, and it is also necessary to report the number of UE per BS and the traffic model.

Therefore, we propose the following definition for energy efficiency in TR 38.914:

Proposal 2: For energy efficiency, support the following definition to be captured in TR 38.914:

5.1.15 Energy efficiency Energy efficiency is an important metric of sustainability. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving in relation to the traffic characteristics. This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario. The requirement is the relative energy savings between the selected load case(s) and a reference case. For network energy efficiency, the reference case is when network is fully loaded. For UE energy efficiency, the reference case is when UE is fully-loaded/has full-buffer service. Proponents should report the power model used for evaluation (e.g. BS power model in 3GPP TR 38.864, UE power model in 3GPP TR 38.840, TR 38.869 or TR 38.865), the number of UE under each BS and the traffic model. The relative target of certain metric (e.g. threshold value for the metric, or certain performance loss compared with the benchmark) can be set. Constant that certain percent (e.g. 90%) of UE fulfils the target should be achieved while evaluating the relative power saving for each load case. The certain metric can be selected according to the service type: - For Vo6G (i.e. VoNR-like) service, the certain metric is latency - For burst-buffer service, the certain metric is QoS or UPT - For XR service, the certain metric is latency The minimum requirements for energy efficiency are summarized in Tables below. Table 5.1.15-1: The minimum requirements for Network Energy efficiency Test environment(s) **; Selected load case(s); The reference case; Relative power saving (%). N. A; empty load; Fully loaded case *; [X0%]. Indoor Hotspot-IC; empty load, low load, light load and/or medium load *; [X1%, X2%, ...]. Dense Urban-IC; empty load, low load, light load and/or medium load *; [Y1%, Y2%, ...]. Rural-IC; empty load, low load, light load and/or medium load *; [Z1%, Z2%, ...]. *empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation Table 5.1.15-2: The minimum requirements for Device Energy efficiency Test environment(s) **; Selected load case(s); The reference case; Relative power saving (%). N. A; empty load; Device is fully-loaded/has full-buffer service; [A0%]. Indoor Hotspot-IC; empty load, low load, light load and/or medium load *; [A1%, A2%, …]. Dense Urban-IC; empty load, low load, light load and/or medium load *; [B1%, B2%, …]. Rural-IC; empty load, low load, light load and/or medium load *; [C1%, C2%, …].

Further consideration on energy efficiency design between RAN and RAN1

In this section, we discuss further consideration on energy efficiency design between RAN and RAN1.

During the discussion in RAN1#122, several proposals were made for power model and EE in 6G BS and 6G UE [6]:

Agreement Study how to reuse and update reference configurations in TR 38.864 for 6G BS. Agreement Study how/whether to reuse or update the power model in TR 38.864 for evaluating BS power consumption for 6G BS. Agreement - Study metric(s) for UE energy efficiency. - Study metric(s) for BS energy efficiency. Agreement Study reference configurations and power consumption model for 6G UE, considering but not restricted to the following: - TR 38.840 (UEPS), TR38.875 (RedCap), TR38.865 (eRedCap), and TR38.869 (LP-WUS/WUR) for reference configurations - TR 38.840 (UEPS), TR38.875 (RedCap), and TR38.869 (LP-WUS/WUR) for power consumption models Agreement - Study baseline BS setting(s) for evaluating 6G BS EE improvement/impact, considering NR features and 6G BS reference configuration(s) - Study baseline UE setting(s) for evaluating 6G UE EE improvement/impact, considering NR features and 6G UE reference configuration(s)

Based on the existing foundations, we think that RAN plenary can further discuss the amendments for 6G power model and energy efficiency, and give some guidance to RAN1 from the following aspects:

- How to handle the parallel discussion on the definition of energy efficiency between RAN and RAN1. From our understanding, to avoid the ambiguity on EE definitions between RAN and RAN1, it is recommended that RAN can inform RAN1 the current progress on EE definition of RAN 6G SI, so that RAN1 can further work on top of it.

- Design principle on power model for both 6G BS and 6G UE, including:

  - The general structure for different sleep mode, transition time and additional transition energy. Our view is that the definitions in NR can be reused and whether some modifications are needed can be further discussed.

  - The specific values for the above parameters. For our point of view, the values shall be revisited according to the product plan on the potential time point of the commercial use for 6GR (e.g. in the year of 2030).

  - Scaling method of UE power model, e.g. how to jointly consider time/bandwidth/antenna/other aspects.

Proposal 3: RAN plenary further discuss and give some guidance to RAN1 on the design principle of for 6G power model and energy efficiency in 6G from the following aspects:

- How to handle the parallel discussion on the definition of energy efficiency between RAN and RAN1.

- Design principle on power model for both 6G BS and 6G UE, including:

  - General structure for sleep mode, transition time and additional transition energy

  - Settings on specific values of relative power, transition time and additional transition energy

  - Scaling method of UE power model, e.g. how to jointly consider time/bandwidth/antenna/other aspects

Moreover, since currently RAN1 6G SI is discussing the design of 6G BS and 6G UE power model, RAN plenary can wait for RAN1 progress before further discuss the specific value of NW EE and UE EE to avoid the duplicate work.

Proposal 4: RAN plenary wait for RAN1 progress on the design of 6G BS and 6G UE power model, and then discuss the specific value of NW EE and UE EE to avoid the duplicate work.

Proposal

In this paper, we provide our views on contribution, and the following proposals are made:

Proposal 1: Define energy efficiency as a quantitative KPI in TR 38.914.

Proposal 2: For energy efficiency, support the following definition to be captured in TR 38.914:

5.1.15 Energy efficiency Energy efficiency is an important metric of sustainability. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving in relation to the traffic characteristics. This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario. The requirement is the relative energy savings between the selected load case(s) and a reference case. For network energy efficiency, the reference case is when network is fully loaded. For UE energy efficiency, the reference case is when UE is fully-loaded/has full-buffer service. Proponents should report the power model used for evaluation (e.g. BS power model in 3GPP TR 38.864, UE power model in 3GPP TR 38.840, TR 38.869 or TR 38.865), the number of UE under each BS and the traffic model. The relative target of certain metric (e.g. threshold value for the metric, or certain performance loss compared with the benchmark) can be set. Constant that certain percent (e.g. 90%) of UE fulfils the target should be achieved while evaluating the relative power saving for each load case. The certain metric can be selected according to the service type: - For Vo6G (i.e. VoNR-like) service, the certain metric is latency - For burst-buffer service, the certain metric is QoS or UPT - For XR service, the certain metric is latency The minimum requirements for energy efficiency are summarized in Tables below. Table 5.1.15-1: The minimum requirements for Network Energy efficiency Test environment(s) **; Selected load case(s); The reference case; Relative power saving (%). N. A; empty load; Fully loaded case *; [X0%]. Indoor Hotspot-IC; empty load, low load, light load and/or medium load *; [X1%, X2%, ...]. Dense Urban-IC; empty load, low load, light load and/or medium load *; [Y1%, Y2%, ...]. Rural-IC; empty load, low load, light load and/or medium load *; [Z1%, Z2%, ...]. *empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation Table 5.1.15-2: The minimum requirements for Device Energy efficiency Test environment(s) **; Selected load case(s); The reference case; Relative power saving (%). N. A; empty load; Device is fully-loaded/has full-buffer service; [A0%]. Indoor Hotspot-IC; empty load, low load, light load and/or medium load *; [A1%, A2%, …]. Dense Urban-IC; empty load, low load, light load and/or medium load *; [B1%, B2%, …]. Rural-IC; empty load, low load, light load and/or medium load *; [C1%, C2%, …].

Proposal 3: RAN plenary further discuss and give some guidance to RAN1 on the design principle of for 6G power model and energy efficiency in 6G from the following aspects:

- How to handle the parallel discussion on the definition of energy efficiency between RAN and RAN1.

- Design principle on power model for both 6G BS and 6G UE, including:

  - General structure for sleep mode, transition time and additional transition energy

  - Settings on specific values of relative power, transition time and additional transition energy

  - Scaling method of UE power model, e.g. how to jointly consider time/bandwidth/antenna/other aspects

Proposal 4: RAN plenary wait for RAN1 progress on evaluation assumptions of 6G power model, and then discuss the specific value of NW EE and UE EE to avoid the duplicate work.

References

- RP-250810, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, Deutsche Telekom.

- RP-250786, Moderator (CMCC), Moderator's summary for IMT-2030 TPRs.

- RP-251394, Moderator (CMCC), Summary of CC#1 and CC#2 conclusions on IMT-2030 TPRs.

- 3GPP TR 38.914, Study on 6G Scenarios and Requirements.

- Report ITU-R M.2410, Minimum requirements related to technical performance for IMT-2020 radio interface(s).

- Chair notes RAN1#122 v13, Bengaluru, India, Aug 25th – 29th, 2025