3GPP TSG RAN Meeting #109												RP-252129

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.2

Source:	Huawei, HiSilicon

Title:	Consideration on 3GPP requirements and KPIs in addition to ITU-R IMT-2030 TPRs

Document for:	Discussion and decision

Introduction

A RAN-level study on “6G Scenarios and requirements” was approved in RAN#107[1]. Regarding the key performance indicators and relevant key assumptions in the scenario, the objectives of this SID mainly include:

- Investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

- Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services.

- Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate.

- Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least:

  - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting

  - Energy efficiency and energy saving: both for network and device

  - Enhanced spectral efficiency

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

  - Target scalable and forward compatible design for diverse device types

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Furthermore, the KPIs in TR 38.914 [2] are agreed in RAN#108 is in the blew table.

Table 1 KPIs adopted in 38.913 and 38.914 so far

| TR 38.913 | TR 38.914 till RAN#108 |
| --- | --- |
| - Peak data rate. - Peak spectral efficiency - User experienced data rate - 5th percentile user spectral efficiency - Average spectral efficiency - Area traffic capacity - User plane latency - Control plane latency - Connection density - Reliability - Mobility - Mobility interruption time - Bandwidth - Energy efficiency | - Peak data rate. - Peak spectral efficiency - User experienced data rate - 5th percentile user spectral efficiency - Average spectral efficiency - Area traffic capacity - User plane latency - Control plane latency - Connection density - Reliability - Mobility - Mobility interruption time - Bandwidth - Energy efficiency |
|  | - Positioning - Composite requirement - Sensing - AI |

In this contribution, the considerations and views on additional 6G requirements and key performance indicators are discussed. The text proposal to the section of 6G requirements and key performance indicators is also provided.

Consideration on 6G requirements and KPIs in addition to ITU-R IMT-2030

This section mainly discusses additional methodologies to evaluate ITU KPI items, new KPIs and new requirements in addition to ITU-R IMT-2030.

Additional evaluation methodologies for defined KPIs

Additional consideration for Area traffic capacity

In ITU-R Report M.2410 [3] and TR 38.914[2], the area traffic capacity is a measure of how much traffic a network can carry per unit area. It depends on site density, bandwidth and spectrum efficiency. In the case of full buffer traffic and a single layer single band system, it can be expressed as:

Area capacity (bps/m2) = site density (site/m2) × bandwidth (Hz) × spectrum efficiency (bps/Hz/site)

However, full buffer case may not be sufficient for evaluation, which may not completely reflect the traffic in reality. This is why 3GPP defined non-full buffer traffic in TR38.913 [4] and made corresponding UPT / cell throughout evaluation for 5G. The non-full buffer model represents the total traffic throughput served per geographic area (in Mbit/s/m2) and it is seen useful to consider this traffic for 6G. System simulation can be used for detailed evaluation.

Proposal 1: The evaluation of Area traffic capacity should not only consider full buffer traffic, but also non-full buffer traffic with system simulation methodology.

Additional consideration to Composite requirement

According TR 38.838[5], eXtended Reality(XR) is a term for different types of realities and refers to all real-and-virtual combined environments and human-machine interactions generated by computer technology and wearables. The Video-based AI inference task is one of the most important media applications under consideration in immersive communication. So, the quantitative KPI should be considered to reflect 6G system capacity.

According to our papers in [6], different immersive communication services require different composite requirements in terms of different combination of service data rates and latency for DL and UL, respectively. Some representative cases are shown in Table 2.1.2.

Table 2.1.2: Example service use cases of composite requirement

| Examples of composite requirement | data rate (Mbps) | Latency (ms) | Reliability | Number of satisfied users |
| --- | --- | --- | --- | --- |
| Composite Requirement for Immersive Services (UL) |  |  |  |  |
| Case 1NOTE1 | 20Mbps | 15ms | [99%] | [TBD] |
| Case 3NOTE1/NOTE2 | 100Mbps | 15ms | [99%] | [TBD] |
| Composite Requirement for Immersive Services (DL) |  |  |  |  |
| Case 4 NOTE2 | 100Mbps | 10ms | [99%] | [TBD] |
| Case 5 NOTE3 | 500Mbps | 10ms | [99%] | [TBD] |

Note 1: Requirement from Section 6.27 Use case on network-assisted video-based AI inference task offloading for mobile embodied AI in TR 22.870.

Note 2: Requirement from Section 9.1 Use case on immersive gaming in TR 22.870.

Note 3: Requirement from Section 9.7 Use case on holographic telepresence in healthcare in TR 22.870.

Composite requirement refers to total number of devices fulfilling a target immersive communication QoS per unit area (per km2) with a large percentage of UEs being satisfied, where the target QoS is to ensure the user experience by considering different QoS requirements, e.g. combined data rate, packet transmission delay and reliability requirement. By having such a definition, both the requirements for user experience and network capacity can be reflected at the same time, which can serve more advanced services in the future. The number of satisfied users supported for these cases also matter, as the network needs to support multiple users at the same time to meet the composite requirements, the specific values need to be further investigated. The system level simulation needs to be used to evaluate performance corresponding to such a composite requirement.

Proposal 2: The evaluation of composite requirement for immersive communication should be supported by system simulation, with composite requirement of the number of satisfied users, by meeting the different QoS of data rate, latency and reliability per user.

Additional consideration to Energy efficiency for Network and UE

Energy saving is very important for both network and UE sides. The quantitative KPI should be considered to better reflect the gains from 6G. Network energy saving was studied from 3GPP R18, according to TR 38.864 [7], the BS power consumption model and BS power consumption method in TR 38.864 can be starting point to evaluate 6G technologies for FR1 and FR2. UE power saving was studied from 3GPP R16, according to TR 38.840 [8] the UE power consumption model and UE power consumption method in TR 38.840 can be largely reused to evaluate 6G technologies for FR1 and FR2.  For new bands from 7~24GHz, the values in these tables in TR 38.864 (BS) and TR 38.840 (UE) may not be able to be fully reused, which need to be studied and updated accordingly.

Besides, 6GR may involve different UE types with different capabilities to satisfy communication requirement in various use cases. Thus, defining only one power consumption for one band as TR38.840 may be not enough to reflect the real consumption from device perspective, so it is essential to investigate power model for different UE types.

On the other hand, some aspects for energy saving evaluation may need improvement. 5G evaluation on network energy saving is based on the UE perceived throughput (UPT), which requires the UE to be served as soon as possible when the service triggers. This leads to the consequence that the network has to schedule and transmit anytime when the UE has a request, and consumes much power accordingly. In reality as long as the QoS requested by the UE is satisfied, the user experience can be achieved and this does not necessarily require the network to schedule or transmit immediately.

Fig. 2.1.3.1 illustrates two transmission strategy based on UPT (left) and QoS(right), it can be seen that based on user QoS satisfaction criteria, BS can have much longer sleep time, thus save energy consumption without affecting UE’s experience; while transmission strategy based on UPT wastes energy due to many small sporadic transmissions without impacting user’s experience. Not only for NW energy reduction, QoS based transmission is also beneficial for UE energy saving. For example, multiple packets for the same UE can be scheduled within a short duration, as long as the QoS is satisfied, and UE’s energy consumption can be reduced due to longer sleep times.

Figure 2.1.3-1:  comparison between network transmission only for UPT and for QoS satisfaction

The below table summarizes the differences between UPT-based and user QoS satisfaction-based principle, their impacts on energy saving, user’s experience aspect and evaluation result.

Table 2.1.3 Comparison between UPT-based and QoS-based evaluation

|  | Energy saving design considering UPT loss | Energy saving design with QoS guarantee |
| --- | --- | --- |
| Impact on energy saving | Minimizing UPT loss requires network to schedule users’ packets as soon as it arrives, which cannot maximize energy saving gain as both network and UE have to be kept in active mode frequently | Network and UE can keep inactive as much as possible and save energy consumption as long as user’s QoS is satisfied, therefore it is beneficial for network & UE energy saving |
| Impact on user’s experience | UPT represents network average transmission capability, and have no direct relationship with user’s experience. UPT degradation doesn’t necessarily mean user’s experience degradation | user’s experience is not impacted as long as QoS is satisfied |

Thereby, when evaluating the energy consumption, it is more reasonable to consider satisfying the QoS requested by the UE instead of considering throughput/UPT as the target metric. In other words, the network energy saving should be evaluated under the user’s QoS satisfaction.

For UE energy saving, as defined in TR38.840, the evaluation of energy saving uses separate assessment of energy consumption for each traffic type e.g., FTP, IM and VoIP. For a single user, the UE may handle different types of traffic with varying QoS requirements over a long period of time. Such evaluation methodology of energy saving is seen useful to reflect the realistic situation and could also be taken this into account in 6G.

Overall, the system level simulation is used as the evaluation methodology for both network and UE energy saving.

Proposal 3: Energy saving should be a 6G quantitative KPI for both network and UE, and should be evaluated by system simulation methodology.

Proposal 4: The power consumption model/assumptions need to be studied considering:

- BS and UE power consumption model for FR1, FR2 and for frequency range between 7~24GHz.

- Energy saving metric based on QoS satisfaction

Consideration on coverage

Coverage is one important metric for operators to deploy the network with good balance between the performance and cost.

From users’ perspective, it is not sufficient to consider coverage itself as a standalone metric, the cell edge data rate should be considered together to provide consistent user experience everywhere. From operators’ perspective, the coverage of new spectrum, e.g. ~7GHz, should achieve similar range as existing bands, e.g. C-band, which can enable co-site deployment of new spectrum and reduce the cost of building up new sites. In 6G there are diverse device types to meet diverse service requirements. In this case, another aspect which may impact coverage is the different capabilities from diverse device types, UEs with lower capability may result in worse coverage performance.

Therefore, the coverage could be considered as one additional requirement and evaluated with the considerations on cell edge data rate, deployment cost and diverse device types. Link budget methodology can be used for evaluations. However, the coverage gap may vary in different bands due to different propagation loss, and different device type capabilities may also result in different coverage performance, therefore it is more suitable to evaluate coverage and set its target during WG study via link budge evaluation for different channels, further considering diverse device types, instead of setting an absolute value in RAN plenary requirements.

Proposal 5: Coverage shall be considered as a general requirement to support the following:

- ~7GHz shall reach same coverage as existing ~4GHz C-band to enable co-site deployment

- Coverage shall also support certain cell edge data rates to ensure user experience corresponding to diverse device types

- Link budget evaluation methodology is used to address different deployment scenarios

Consideration of additional ISAC capabilities

In ITU-R IMT-2030 discussion, Detection Probability and False Alarm Probability, Horizontal/Vertical Localization Accuracy, Velocity Accuracy and [Sensing Resolution] have been adopted as the items for TPR. In addition to this, KPI for environment reconstruction also needs be considered. In the current TR 22.870 [9], several use cases have described digital twin and environment reconstruction. This has been described in section 7.5 of 22.870 of use case on environment object reconstruction, for static environmental objects, such as outdoor building/city construction and indoor machinery/wall, 3GPP wireless sensing is effective to provide a wealth of low-cost environmental information for both wide-areas and certain areas. Additionally, for monitoring dynamic targets of interest, such as vehicles, this is essential for enhancing environmental perception, especially in scenarios where non-line-of-sight (NLOS) conditions or poor visibility may limit traditional sensing methods. Besides, according to the agreed [10] in 3GPP SA1 August meeting, some segments/parts of objects may be missed at given locations or periods of time but they can be reconstructed back/restored sufficiently in real time, e.g. based on trained AI/ML model(s).

To enable this new commercial service, there is a need to utilize ISAC capabilities and AI capabilities together to construct the environment information, and provide it as a service to the 3rd party. Such information is also useful to improve existing services, such as positioning performance, communication performance and so on as described in [11]. In this case the reconstruction capability shall be defined.

Figure 2.3.1-1:  Ground truth scattering points and sensed scatter points

Different from the detectability, the reconstruction capability refers to the capability of providing sensing service after the preliminary detection of the environmental object presence. Compared with the localization accuracy, the environmental object is of a set of points, and reconstruction accuracy is needed to show the difference between the reconstructed point set and the real point set of the environment objects.

Therefore, Reconstruction accuracy is defined as the closeness of the reconstructed scatter points set to the real scatter points set of sensing target, measured by e.g. Root Mean Square Error (RMSE) and can be defined as the following:

Reconstruction accuracy=1Npi=1Np(PGTi-Pesti)2

where PGT and Pest represents the 3D location of the ground truth points and the estimated points of one environment object, respectively. Np represents the number of points in the set.

System-level and/or link-level simulation are to be performed for the evaluation of reconstruction accuracy.

From service perspective in Section 7.5 of TR22.870 and [11], the 6G shall be able to provide sensing service by detecting the characteristics of individual environmental object. It could observe that positioning accuracy is [0.5m -5m] and sensing resolution is [0.5m] in Section 7.5 of TR22.870.  So, we propose the reconstruction accuracy should be the same with the positioning accuracy as [0.5m-5m], which depends on different reconstructed object.

Proposal 6: Reconstruction accuracy shall be defined as a 6G quantitative KPI for sensing, and the target value should be [submeter level – meter level].

As also discussed in [11], the use case of target detection and tracking should be considered in 6G. As analyzed in [11], 6G shall be able to support the required horizontal localization accuracy is specified as 0.5 and 1 meter for target which is static or moving up to 5km/h, so we propose:

Proposal 7: Horizontal localization accuracy is 1m and velocity accuracy is 1m/s shall be defined as a 6G quantitative KPI for static or moving target up to 5km/h detection use case.

Consideration of additional AI capabilities

As in TR 22.870[9], the system performance optimization using AI in section 6.3 is one of the key AI use cases for 6G. This use case addresses optimization of communication between terminals and network through implementing and using the AI capabilities for the network functions (AI-enabled).  In this case, 6G base stations can further enhance not only for communication but also include sensing, positioning, and other 6G system capabilities. For instance, the prior information could be utilized to do wireless channel prediction and multipath identification for better channel estimation, enhanced beamforming, interference suppression and etc., which can help to enhance system efficiency and user experience.

Proposal 8:  6G RAN shall support AI capabilities to enhance communication performance and user experience.

As also discussed in [12], large AI/ML models such as large language models (LLM), vision language models (VLM), multi-modal language models (MLM), visual language action models (VLA) make AI services developing very fast, and token is widely used for efficient processing for these models. As analyzed in [12], token is consumed rapidly with a huge number everyday as described in [12], whose traffic pattern has very different characteristics as existing services. Therefore, 6G RAN shall provide efficient mechanism to well serve these new AI traffic, to help operators to grab this new business opportunity.

Proposal 9:  6G RAN shall support efficient transmission for traffics relevant for AI applications.

As also discussed in [12] and mentioned in Sec 2.3 in the paper, to support new use cases such as collaborative AI Agents, it requires both ISAC and AI capabilities, e.g. to first collect sufficient data from sensing and then get accurate instruction from AI training and inference. By combing these two capabilities, better quality of services can be provided to applications, e.g. more useful information or more friendly way to exposure the information to the applications. In this case the 6G RAN design shall consider to provide mechanism to jointly utilize AI and sensing capabilities for support new service requirement.

Proposal 10: 6G RAN shall enable efficient utilization of data produced by sensing and AI capabilities to support new services.

Summary

We have discussed the principle to address key performance indicators in 6G. We suggest to define the additional or supplementary evaluation methodologies for ITU-R TPRs, or introduce new KPIs in 6G Immersive communication.

Proposal 1: The evaluation of Area traffic capacity should not only consider full buffer traffic, but also non-full buffer traffic with system simulation methodology.

Proposal 2: The evaluation of composite requirement for immersive communication should be supported by system simulation, with composite requirement of the number of satisfied users, by meeting the different QoS of data rate, latency and reliability per user.

Proposal 3: Energy saving should be a 6G quantitative KPI for both network and UE, and should be evaluated by system simulation methodology.

Proposal 4: The power consumption model/assumptions need to be studied considering:

- BS and UE power consumption model for FR1, FR2 and for frequency range between 7~24GHz.

- Energy saving metric based on QoS satisfaction

Proposal 5: Coverage shall be considered as a general requirement to support the following:

- ~7GHz shall reach same coverage as existing ~4GHz C-band to enable co-site deployment

- Coverage shall also support certain cell edge data rates to ensure user experience corresponding to diverse device types

- Link budget evaluation methodology is used to address different deployment scenarios

Proposal 6: Reconstruction accuracy shall be defined as a 6G quantitative KPI for sensing, and the target value should be [submeter level – meter level].

Proposal 7: Horizontal localization accuracy is 1m and velocity accuracy is 1m/s shall be defined as a 6G quantitative KPI for static or moving target up to 5km/h detection use case.

Proposal 8:  6G RAN shall support AI capabilities to enhance communication performance and user experience.

Proposal 9:  6G RAN shall support efficient transmission for traffics relevant for AI applications.

Proposal 10: 6G RAN shall enable efficient utilization of data produced by sensing and AI capabilities to support new services.

References

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, 3GPP RAN#107, March, 2025.

- TR38.914, Study on 6G Scenarios and Requirements

- ITU-R Report M.2410, Minimum requirements related to technical performance for IMT-2020 radio interface(s)

- TR38.913, Study on scenarios and requirements for next generation access technologies

- TR38.838, Study on XR (Extended Reality) evaluations for NR

- RP-252131, Consideration on supporting immersive communication services for 6G, Huawei, 3GPP RAN#109, September, 2025.

- TR38.864, Study on network energy savings for NR

- TR38.840, Study on User Equipment (UE) power saving in NR

- TR22.870, Study on 6G Use Cases and Service Requirements

- S1-253507, Update of clause 7.5 use case on environment object reconstruction, Huawei, China Mobile, 3GPP SA1#111, August, 2025.

- RP-252133, Consideration on supporting ISAC services for 6G, Huawei, 3GPP RAN#109, September, 2025.

- RP-252132, Consideration on supporting new AI services for 6G, Huawei, 3GPP RAN#109, September, 2025.

- Text proposal for the TR

Proposed wording based on the conclusion of this document.

Area traffic capacity

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time. This metric can be evaluated by two different traffic models: Full buffer model and non-full buffer model。

- By full buffer model: Total traffic throughput served per geographic area (in Mbit/s/m2). The computation of this metric is based on full buffer traffic.

This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g. TRxP (site) density) and bandwidth.

Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through the following equation:

Carea = ρ × W × SEavg

- By non-full buffer model: Total traffic throughput served per geographic area (in Mbit/s/m2). Both the user experienced data rate and the area traffic capacity need to be evaluated at the same time using the same traffic model.

- NOTE1:	Non-full buffer simulations are preferred for the evaluation of this KPI.

In case bandwidth is aggregated across multiple bands, the area traffic capacity will be summed over the bands. In the case of non-full buffer traffic and a single layer single band system, system simulation methodology is used.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirement for area traffic capacity in downlink is TBD Mbit/s/m² in the Indoor Hotspot – IC [test environment].

    - Energy efficiency

Energy efficiency is an important metric of sustainability. This requirement of the candidate RIT/SRIT is characterized by evaluating network and device energy efficiency. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving in relation to the traffic characteristics.

The reference configurations and power consumption model in [TR 38.864] for network energy efficiency evaluation, and reference configurations and power consumption model in [TR38.840(UEPS), TR38.875(RedCap), TR38.865(eRedCap), and TR38.869(LP-WUS/WUR)] for device energy efficiency evaluation.

The requirement is defined as the power consumption (in terms of percentage) for the selected traffic or load case(s). In addition, the metric is evaluated as the power consumption @ QoS satisfaction, where the traffic model is at least defined by date rate and latency budget, system simulation methodology is used.

This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario.

Composite requirement

The composite requirement refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users per [km2 or TRxP].

This requirement is defined as the number of users per [km2 or TRxP] of which at least [90%/95%] are satisfied, where each satisfied user achieves the required packet success probability of transmitting a layer 2/3 packet within a required latency. The latency is the time it takes to deliver a data packet for a given data rate from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum performance requirement of composite requirement is summarized in Table X.

TABLE X Composite requirement

| Test environment | Data rate (Mbits/s) | Latency (ms) | Packet success probability | Number of users ([/km2 or /TRxP]) |
| --- | --- | --- | --- | --- |
| Dense Urban - Immersive Communication | [DL = 100 Mbps/500 Mbps] | [DL=10 ms] | [99%] | TBD |
|  | [UL = 20 Mbps/100 Mbps] | [UL=15 ms] | [99%] | TBD |

Sensing

Sensing reconstruction accuracy

The requirement is a capability to detect and track the characteristics of individual environmental object. The metric for assessing the reconstruction accuracy refers to the closeness of the reconstructed scatter points set to the real scatter points set of sensing target, measured by e.g. Root Mean Square Error (RMSE).

Reconstruction accuracy=1Npi=1Np(PGTi-Pesti)2

where PGT and Pest represents the 3D location of the ground truth points and the estimated points of one environment object, respectively. Np represents the number of points in the set.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The target for sensing reconstruction accuracy should be [0.5-5m].

Link-level or system level evaluation is used as the evaluation methodology

In addition, based on this metric, the following functions can be considered for future evaluation, as

- The sensing functions are not only providing object detection, and can also provide environment awareness etc. The data from sensing can be utilized to improve communication performance, e.g. to reduce overhead and power consumption by reducing measurements.

- The sensing functions can also provide useful information to aware the environment, and identify the NLOS/multipath information better. This can be utilized to improve the positioning performance especially under NLOS conditions.

Editor’s note: The parts for Sensing detectability, Sensing localization accuracy, Sensing velocity accuracy are assumed to be added by IMT-2030 discussion.

AI

6G RAN shall support AI/ML capabilities to enhance communication performance and user experience.

6G RAN shall support efficient transmission for traffics relevant for AI applications, by taking into account of specific characteristics of AI traffics.

6G RAN shall enable efficient utilization of data produced by sensing and AI capabilities to support new services.