3GPP TSG RAN Meeting #109											RP-252134

Beijing, China, Sep 15-18, 2025

Agenda Item:	8.2.1.4

Source:	Huawei, HiSilicon

Title:	Consideration on supporting NTN services for 6G

Document for:	Discussion and decision

Introduction

NTN has been specified in 5G. As an essential complement to the terrestrial network, it is important that NTN also be supported in 6G to complement the coverage and related services where terrestrial network is not available.

In the updated RAN study item [1], the following objective was agreed:

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration

The harmonized radio design would need very clear design targets for NTN, e.g. the coverage requirement from (v)LEO to GSO, the basic service types to be supported, and the corresponding service requirements etc. All these aspects will impact the WG level study on specific technical directions.

In this contribution, the requirements on new and enhanced services for 6G NTN are discussed.

6G NTN coverage considerations

Ubiquitous connectivity can be achieved by utilizing NTN complementing TN to enable seamless global access, even in regions with challenging geographical conditions (e.g., mountains, forests), airspace for aerial operations (e.g., drone operations), as well as the open ocean.

The coverage provided by (V)LEO satellites depends on the deployment stage of the (V)LEO constellation, especially considering phased deployment may be carried out over a few years. In the design of NTN, the different deployment stages may pose different coverage requirements, e.g., the RAN design should enable the satellite operator to provide ubiquitous coverage for the users in the underserved areas not covered by TN irrespective of the (V)LEO constellation deployment stage, as highlighted in the use case on enhanced user experience with sparse LEO satellite deployment [2]. As more satellites are deployed in subsequent deployment stages, the (V)LEO constellation may become denser. Therefore, the coverage requirement of 6G NTN shall be commensurate with the constellation size scaling from sparse (V)LEO satellite constellations to dense (V)LEO satellite constellations.

In 5G NTN, the satellite coverage of GEO is determined based on 10 degrees minimum UE elevation angle, whereas for LEO it is determined based on 30 degrees minimum elevation angle [3], as shown in the figure below.

Figure 2-1 5G NTN satellite footprint/coverage determination based on minimum UE elevation angles.

For 6G NTN, satisfying the ubiquitous coverage requirement may be tied to the constellation size which may impact the coverage area requirement for each satellite. For example, if the constellation is very sparse, there may exist coverage holes, no matter how small the UE elevation angle used to define the satellite footprint. If the coverage area for each satellite is defined based on a minimum UE elevation angle of 30%  per LEO satellite and the satellite coverage of two neighboring satellites do not overlap, then the LEO constellation may be considered as sparse, which is illustrated as the left-hand side figure below. When the constellation size leads to overlapped adjacent satellite coverage based on 30 degrees UE minimum elevation angles, then the constellation deployment may be considered as dense, which is illustrated in the right-hand side figure below. Alternatively, if the satellite coverage of a LEO constellation is defined based on a minimum UE elevation angle of more than 30 degrees such that no coverage gaps are left over a certain region of interest, then the (V)LEO constellation may be considered as dense.  Considering the different deployment stages of the constellation, 6GR design should consider the coverage requirements for (V)LEO constellations ranging from sparse to dense.

Sparse deployement                                                                              Dense deployment

Figure 2-2 Illustration of the 6G sparse deployment and dense deployment.

The UE capability also has some impact on the coverage of LEO constellations. For instance, UEs capable of directional communications can typically follow the direction of the LEO satellite to ensure the best antenna gains. However, for UEs without the capability of automatic beam steering, aligning with the target satellite may not be so obvious. For example, RAN1 assumes the handheld UE in FR1 using omni directional antenna with -5.5dBi gain in every direction in 5G NTN. However, current handheld UE for satellite communication can only provide the assumed antenna gain within a certain direction range. This requires the user to align the handheld UE with the satellite to achieve the assumed antenna gain.  For example, nowadays the current GEO satellite communication requires the user assistance to align the handheld UE with the satellite’s direction. Similar assistance is also needed for the LEO satellite communication. However, unlike the GEO case, where the UE can be easily aligned with the static satellite, the LEO satellite movement makes such alignment quite difficult in practice for the user.  In 6G, it is recommended to allow more flexibility for the user of the handheld device when not being able to achieve accurate alignment between the satellite and the highest antenna gain direction, i.e., 6G NTN system should consider the support of the satellite-misaligned scenario, where the direction of the highest UE antenna gain may be misaligned with the satellite direction.

Besides, in scenarios where the device is located in the user pockets or bags, some extra body loss along with the difficulty to maintain the alignment with the satellite, may lead to a coverage outage for the handheld UE.

6G NTN is targeting at providing a better user experience for the handheld UE by taking into account the special requirements of body loss/NLoS and/or satellite-misaligned scenarios.

Proposal 1: For handheld UE, 6G RAN design shall support NTN services in the underserved area not covered by TN considering both sparse and dense (V)LEO constellation deployments including:

- LoS scenario

- Body loss/NLoS scenario

- Satellite-misaligned scenario.

Basic service types and corresponding requirements for 6G NTN

In LEO, frequent satellite beam switches are inevitable due to the orbital movement and potentially overlapping coverages of the different satellites, especially in massive beam deployments, where a serving satellite beam signal may rapidly degrade and the UE switches to a neighboring satellite beam with better signal quality.  Quasi-fixed beam deployments would help reducing satellite mobility triggered beam switches and enhance service continuity within NTN. Reducing the beam switching overhead and minimizing service interruptions are important design targets when operating with (V)LEO constellations and should therefore be considered as a design target for 6G NTN.

In addition, GNSS resilient operation is studied in 5G NTN Rel-20, which can improve the power consumption from the UE side caused by GNSS information acquire processing. However it does not solve the issue caused by GNSS on large access latency, which can be around second to minute level range. For 6GR NTN, GNSS-free operation can be considered to provide better performance.

Proposal 2: 6G NTN shall support service continuity within NTN, considering (V)LEO constellations ranging from sparse to dense constellations and massive beams per satellite.

Proposal 3: For 6G NTN, GNSS-free operation can be considered in order to reduce the system access latency and power consumption.

Summary

We have discussed the requirements for NTN and propose to adopt the following proposals into TR 38.914 “Requirements of New and Existing Services” section:

Proposal 1: For handheld UE, 6G RAN design should support NTN services in the underserved area not covered by TN considering both sparse and dense (V)LEO constellation deployments including:

- LoS scenario

- Body loss/NLoS scenario

- Satellite-misaligned scenario.

Proposal 2: 6G NTN should support service continuity within NTN, considering (V)LEO constellations ranging from sparse to dense constellations and massive beams per satellite.

Proposal 3: For 6G NTN, GNSS-free operation can be considered in order to reduce the system access latency and power consumption. .

References

- RP-250810, “New SID: Study on 6G Scenarios and requirements”, RAN#107, March 2025.

- 3GPP TR 22.870: “Study on 6G Use Cases and Service Requirements”.

- TR 38.821, “Solutions for NR to support non-terrestrial networks (NTN)”, V16.2.0, March, 2023.