3GPP TSG RAN Meeting #109	RP-252220

Beijing, China, September 15-18, 2025

Agenda item: 	    8.2.1.1

Source: 	China Unicom

Title: 	Discussion on 6G Deployment Scenarios

Document for:	Discussion and Decision

- Introduction

At RAN#107 meeting, the revised SID of Study on 6G scenarios and requirements has been approved [1]. At RAN#108 meeting, the new TR 38.914 was created and captured the latest agreements for 6G deployment scenarios[2].

This contribution provides our views on 6G typical scenarios and the general requirements for 6G scenarios.

- Requirements for 6G Deployment scenarios

  - Indoor Hotspot scenario

Considering the commercial deployment requirements, the operator needs to reuse and upgrade the existing infrastructures from legacy mobile communication systems. Considering the spectrum utilized in the legacy system, e.g. 5G, 4G, it is proposed to update the requirements as below.

- Carrier Frequency: Around 2 GHz (add the new frequency)

- Aggregated system bandwidth: Around 4 GHz: Up to 300 MHz (DL+UL)  ( Considering RAN sharing scenario)

- ISD: For below 7GHz, the ISD requirement should be the same value, e.g. 30~50 meter, considering the practical deployment scenario.

- BS antenna elements:  2T2R and 4T4R

- UE antenna elements:  1T2R as minimum requirement

  - Dense Urban scenario

For dense urban scenario, both the new spectrum and the re-farming spectrum from legacy system should be considered. In dense urban scenario, two layers with flexible frequency configurations are necessary for real network deployment. One layer with lower frequency is providing basic coverage, and the second layer with higher frequency is provided for capacity boosting.

- Carrier Frequency: Around 700MHz

- Layout: Around 7 GHz + Around 2GHz, Around 7 GHz + Around 4GHz, Around 7 GHz + Around 700MHz

- BS antenna elements:  16T16R/64T64R (sub 4GHz), 128T128R/256T256R (higher frequency)

- UE antenna elements:  1T2R, 2T2R, 2T4R

  - Rural scenario

For rural scenario, operator needs to leverage the cell coverage as well as cell capacity. It's related to site planning and radio propagation of different spectrum bands.

- Carrier Frequency: Around 700MHz

- Layout: Around 7 GHz + Around 2GHz, Around 7 GHz + Around 4GHz, Around 7 GHz + Around 700MHz

- BS antenna elements:  2T2R, 4T4R, 8T8R

- UE antenna elements:  1T2R, 2T2R, 2T4R

  - Extreme long distance coverage in low density area

Considering the commercial deployment requirements, the operator needs to reuse and upgrade the existing infrastructures from legacy mobile communication systems. Considering the spectrum utilized in the legacy system, e.g. 5G, 4G, it is proposed to update the requirements as below.

- System Bandwidth: up to 50MHz

- Cell range: 50 KM

Proposal: Discuss and agree with the proposal for the requirements of the above scenarios.

- Conclusion

The observations and proposal of this contribution are as the following.

Proposal: Discuss and agree with the proposal for the requirements of the above scenarios.

- References

- RP-250810, Revised SID: Study on 6G scenarios and requirements, CMCC, Verizon, NTT DOCOMO, Inc., Deutsche Telecom

- TR 38.914 V0.1.0 Study on 6G Scenarios and requirements