3GPP TSG-RAN5 Meeting #108	R5-253972

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5058 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to XR Enh test case 7.1.1.3.18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In NR MAC specification TS 38.321, the new BSR is defined as “Refined Long BSR”, however in the test case prose “Long Refined BSR” is used at several places. - Misc. editorial issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Any instance of “Long Refined BSR” has been corrected into “Refined Long BSR”. - Editorial issues have been fixed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test specification terminology will not be aligned with the core specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.1.1.3.18	Correct Handling of MAC control information / Buffer status / UL data arrive in the UE Tx buffer / Refined BSR

7.1.1.3.18.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state }

ensure that {

when { UL data arrives in the UE transmission buffer for a logical channel in a LCG configured with additionalBS-TableAllowed, and the amount of UL data available for transmission is within the buffer sizes specified in Table 6.1.3.1-3 }

then { UE reports a Refined Long BSR }

}

(2)

with { UE in NR RRC_IDLE state }

ensure that {

when { UL data arrives in the UE transmission buffer for a logical channel in a LCG configured with additionalBS-TableAllowed, and the amount of UL data available for transmission when the MAC PDU containing the BSR is to be built is greater than the largest buffer size specified in Table 6.1.3.1-3 }

then { UE reports a Long BSR }

}

(3)

with { UE in NR RRC_IDLE state }

ensure that {

when { UL data arrives in the UE transmission buffer for a logical channel in a LCG configured with additionalBS-TableAllowed, and the amount of UL data available for transmission when the MAC PDU containing the BSR is to be built is smaller than the smallest buffer size specified in Table 6.1.3.1-3 }

then { UE reports a Short BSR }

}

7.1.1.3.18.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321, clause 5.2.4.6 and TS 38.331, clause 6.3.1 5. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.4.5]

…

For Regular and Periodic BSR, the MAC entity for which logicalChannelGroupIAB-Ext is not configured by upper layers shall:

1>	if for at least one LCG configured with additionalBS-TableAllowed, the amount of UL data available for transmission when the MAC PDU containing the BSR is to be built is within the buffer sizes specified in Table 6.1.3.1-3:

2>	report Refined Long BSR for all LCGs which have data available for transmission;

1>	else:

2>	if more than one LCG has data available for transmission when the MAC PDU containing the BSR is to be built:

3>	report Long BSR for all LCGs which have data available for transmission.

2>	else if one LCG has data available and is configured with additionalBS-TableAllowed and the amount of UL data available for transmission when the MAC PDU containing the BSR is to be built is greater than the largest buffer size specified in Table 6.1.3.1-3:

3>	report Long BSR.

2>	else:

3>	report Short BSR.

[TS 38.321, clause 6.1.3.1]

Buffer Status Report (BSR) MAC CEs consist of either:

-	Short BSR format (fixed size); or

-	Extended Short BSR format (fixed size); or

-	Long BSR format (variable size); or

-	Refined Long BSR format (variable size); or

-	Extended Long BSR format (variable size); or

-	Short Truncated BSR format (fixed size); or

-	Extended Short Truncated BSR format (fixed size); or

-	Long Truncated BSR format (variable size); or

-	Extended Long Truncated BSR format (variable size).

Pre-emptive BSR MAC CE consists of:

-	Pre-emptive BSR format (variable size); or

-	Extended Pre-emptive BSR format (variable size).

The BSR formats are identified by MAC subheaders with LCIDs as specified in Table 6.2.1-2.

The Refined Long BSR format, Extended BSR formats and Pre-emptive BSR format are identified by MAC subheaders with eLCIDs as specified in Table 6.2.1-2b.

The fields in the BSR MAC CE are defined as follows:

-	LCG ID: The Logical Channel Group ID field identifies the group of logical channel(s) whose buffer status is being reported. The length of the field is 3 bits for the case of Short BSR and Short Truncated BSR formats, and 8 bits for the case of Extended Short BSR and Extended Short Truncated BSR formats;

-	LCGi: For the Long BSR format, Refined Long BSR format, Extended Long BSR format, Pre-emptive BSR format, and Extended Pre-emptive BSR format, this field indicates the presence of the Buffer Size field for the logical channel group i. The LCGi field set to 1 indicates that the Buffer Size field for the logical channel group i is reported. The LCGi field set to 0 indicates that the Buffer Size field for the logical channel group i is not reported. For the Long Truncated BSR format and the Extended Long Truncated BSR format, this field indicates whether logical channel group i has data available. The LCGi field set to 1 indicates that logical channel group i has data available. The LCGi field set to 0 indicates that logical channel group i does not have data available;

-	BTi: This field is included only in the Refined Long BSR format. This field is present only if the corresponding LCGi is set to 1; otherwise, this field is reserved and set to 0. If present, this field indicates which buffer size table is used to set the Buffer Size field for the logical channel group i. The BTi field set to 1 indicates that the buffer size table specified in Table 6.1.3.1-3 is used for the logical channel group i. The BTi field set to 0 indicates that the buffer size table specified in Table 6.1.3.1-2 is used for the logical channel group i;

-	Buffer Size: The Buffer Size field identifies the total amount of data available according to the data volume calculation procedure in TSs 38.322 [3] and 38.323 [4] across all logical channels of a logical channel group after the MAC PDU has been built (i.e. after the logical channel prioritization procedure, which may result the value of the Buffer Size field to zero). The amount of data is indicated in number of bytes. The size of the RLC headers and MAC subheaders are not considered in the buffer size computation. The length of this field for the Short BSR format and the Short Truncated BSR format is 5 bits. The length of this field for the Extended Short BSR format and the Extended Short Truncated BSR format is 8 bits. The length of this field for the Long BSR format, the Refined Long BSR format, the Long Truncated BSR format, the Extended Long BSR format, and the Extended Long Truncated format is 8 bits. The values for the 5-bit Buffer Size fields are shown in Table 6.1.3.1-1. The values for the 8-bit Buffer Size fields in the Long BSR format, the Long Truncated BSR format, the Extended Long BSR format, and the Extended Long Truncated format are shown in Table 6.1.3.1-2. For the Refined Long BSR format, if an LCG is configured with additionalBS-TableAllowed and the amount of data for the LCG is within the buffer sizes specified in Table 6.1.3.1-3, the MAC entity shall use the buffer sizes specified in Table 6.1.3.1-3 to set the value of this field; otherwise, the MAC entity shall use Table 6.1.3.1-2 instead. For the Long BSR format, the Refined Long BSR format, the Long Truncated BSR format, the Extended Long BSR format, and the Extended Long Truncated format, the Buffer Size fields are included in ascending order based on the LCGi. For the Long Truncated BSR format and the Extended Long Truncated format the number of Buffer Size fields included is maximised, while not exceeding the number of padding bits. For the Pre-emptive BSR format and the Extended Pre-emptive BSR format, the Buffer Size field identifies the total amount of the data expected to arrive at the IAB-MT of the node where the Pre-emptive BSR/Extended Pre-emptive BSR is triggered and does not include the volume of data currently available in the IAB-MT. Pre-emptive BSR format is identical to the Long BSR format. Extended Pre-emptive BSR format is identical to the Extended Long BSR format.

NOTE 1:	For the Pre-emptive BSR, if configured, the LCGs to be reported, the expected data volume calculation, the exact time to report Pre-emptive BSR and the associated LCH are left to implementation.

NOTE 2:	The mapping of LCGs between the ingress and egress links of an IAB node for purposes of determining expected change in occupancy of IAB-MT buffers (to be reported as Pre-emptive BSR) is left to implementation.

NOTE 3:	The number of the Buffer Size fields in the Long BSR, Extended Long BSR, Long Truncated BSR, and Extended Long Truncated BSR format can be zero.

NOTE 4:	The Extended versions of the BSR formats can only be used by IAB nodes.

...

…

Figure 6.1.3.1-5: Refined Long BSR MAC CE

…

Table 6.1.3.1-3. Refined buffer size levels (in bytes) for 8-bit Buffer Size field

| Index | BS value | Index | BS value | Index | BS Value | Index | BS Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | > 4751 and ≤ 5000 | 64 | ≤ 17584 | 128 | ≤ 61841 | 192 | ≤ 217489 |
| 1 | ≤ 5099 | 65 | ≤ 17933 | 129 | ≤ 63069 | 193 | ≤ 221805 |
| 2 | ≤ 5200 | 66 | ≤ 18289 | 130 | ≤ 64320 | 194 | ≤ 226207 |
| 3 | ≤ 5303 | 67 | ≤ 18652 | 131 | ≤ 65596 | 195 | ≤ 230695 |
| 4 | ≤ 5408 | 68 | ≤ 19022 | 132 | ≤ 66898 | 196 | ≤ 235273 |
| 5 | ≤ 5516 | 69 | ≤ 19399 | 133 | ≤ 68226 | 197 | ≤ 239942 |
| 6 | ≤ 5625 | 70 | ≤ 19784 | 134 | ≤ 69580 | 198 | ≤ 244703 |
| 7 | ≤ 5737 | 71 | ≤ 20177 | 135 | ≤ 70960 | 199 | ≤ 249559 |
| 8 | ≤ 5851 | 72 | ≤ 20577 | 136 | ≤ 72368 | 200 | ≤ 254511 |
| 9 | ≤ 5967 | 73 | ≤ 20985 | 137 | ≤ 73804 | 201 | ≤ 259562 |
| 10 | ≤ 6085 | 74 | ≤ 21402 | 138 | ≤ 75269 | 202 | ≤ 264713 |
| 11 | ≤ 6206 | 75 | ≤ 21827 | 139 | ≤ 76763 | 203 | ≤ 269966 |
| 12 | ≤ 6329 | 76 | ≤ 22260 | 140 | ≤ 78286 | 204 | ≤ 275323 |
| 13 | ≤ 6455 | 77 | ≤ 22702 | 141 | ≤ 79839 | 205 | ≤ 280786 |
| 14 | ≤ 6583 | 78 | ≤ 23152 | 142 | ≤ 81424 | 206 | ≤ 286358 |
| 15 | ≤ 6713 | 79 | ≤ 23611 | 143 | ≤ 83040 | 207 | ≤ 292041 |
| 16 | ≤ 6847 | 80 | ≤ 24080 | 144 | ≤ 84687 | 208 | ≤ 297836 |
| 17 | ≤ 6983 | 81 | ≤ 24558 | 145 | ≤ 86368 | 209 | ≤ 303746 |
| 18 | ≤ 7121 | 82 | ≤ 25045 | 146 | ≤ 88082 | 210 | ≤ 309774 |
| 19 | ≤ 7262 | 83 | ≤ 25542 | 147 | ≤ 89830 | 211 | ≤ 315921 |
| 20 | ≤ 7407 | 84 | ≤ 26049 | 148 | ≤ 91612 | 212 | ≤ 322190 |
| 21 | ≤ 7554 | 85 | ≤ 26566 | 149 | ≤ 93430 | 213 | ≤ 328583 |
| 22 | ≤ 7703 | 86 | ≤ 27093 | 150 | ≤ 95284 | 214 | ≤ 335104 |
| 23 | ≤ 7856 | 87 | ≤ 27631 | 151 | ≤ 97175 | 215 | ≤ 341754 |
| 24 | ≤ 8012 | 88 | ≤ 28179 | 152 | ≤ 99103 | 216 | ≤ 348535 |
| 25 | ≤ 8171 | 89 | ≤ 28738 | 153 | ≤ 101070 | 217 | ≤ 355452 |
| 26 | ≤ 8333 | 90 | ≤ 29309 | 154 | ≤ 103076 | 218 | ≤ 362505 |
| 27 | ≤ 8499 | 91 | ≤ 29890 | 155 | ≤ 105121 | 219 | ≤ 369699 |
| 28 | ≤ 8667 | 92 | ≤ 30483 | 156 | ≤ 107207 | 220 | ≤ 377035 |
| 29 | ≤ 8839 | 93 | ≤ 31088 | 157 | ≤ 109335 | 221 | ≤ 384517 |
| 30 | ≤ 9015 | 94 | ≤ 31705 | 158 | ≤ 111504 | 222 | ≤ 392147 |
| 31 | ≤ 9194 | 95 | ≤ 32334 | 159 | ≤ 113717 | 223 | ≤ 399929 |
| 32 | ≤ 9376 | 96 | ≤ 32976 | 160 | ≤ 115973 | 224 | ≤ 407865 |
| 33 | ≤ 9562 | 97 | ≤ 33630 | 161 | ≤ 118275 | 225 | ≤ 415959 |
| 34 | ≤ 9752 | 98 | ≤ 34298 | 162 | ≤ 120622 | 226 | ≤ 424213 |
| 35 | ≤ 9946 | 99 | ≤ 34978 | 163 | ≤ 123016 | 227 | ≤ 432631 |
| 36 | ≤ 10143 | 100 | ≤ 35672 | 164 | ≤ 125457 | 228 | ≤ 441216 |
| 37 | ≤ 10344 | 101 | ≤ 36380 | 165 | ≤ 127946 | 229 | ≤ 449971 |
| 38 | ≤ 10549 | 102 | ≤ 37102 | 166 | ≤ 130485 | 230 | ≤ 458900 |
| 39 | ≤ 10759 | 103 | ≤ 37839 | 167 | ≤ 133074 | 231 | ≤ 468007 |
| 40 | ≤ 10972 | 104 | ≤ 38589 | 168 | ≤ 135715 | 232 | ≤ 477294 |
| 41 | ≤ 11190 | 105 | ≤ 39355 | 169 | ≤ 138408 | 233 | ≤ 486765 |
| 42 | ≤ 11412 | 106 | ≤ 40136 | 170 | ≤ 141155 | 234 | ≤ 496425 |
| 43 | ≤ 11639 | 107 | ≤ 40933 | 171 | ≤ 143956 | 235 | ≤ 506276 |
| 44 | ≤ 11870 | 108 | ≤ 41745 | 172 | ≤ 146813 | 236 | ≤ 516322 |
| 45 | ≤ 12105 | 109 | ≤ 42573 | 173 | ≤ 149726 | 237 | ≤ 526568 |
| 46 | ≤ 12345 | 110 | ≤ 43418 | 174 | ≤ 152697 | 238 | ≤ 537017 |
| 47 | ≤ 12590 | 111 | ≤ 44280 | 175 | ≤ 155727 | 239 | ≤ 547674 |
| 48 | ≤ 12840 | 112 | ≤ 45158 | 176 | ≤ 158817 | 240 | ≤ 558542 |
| 49 | ≤ 13095 | 113 | ≤ 46055 | 177 | ≤ 161969 | 241 | ≤ 569625 |
| 50 | ≤ 13355 | 114 | ≤ 46968 | 178 | ≤ 165183 | 242 | ≤ 580929 |
| 51 | ≤ 13620 | 115 | ≤ 47900 | 179 | ≤ 168461 | 243 | ≤ 592457 |
| 52 | ≤ 13890 | 116 | ≤ 48851 | 180 | ≤ 171804 | 244 | ≤ 604213 |
| 53 | ≤ 14166 | 117 | ≤ 49820 | 181 | ≤ 175213 | 245 | ≤ 616203 |
| 54 | ≤ 14447 | 118 | ≤ 50809 | 182 | ≤ 178690 | 246 | ≤ 628431 |
| 55 | ≤ 14734 | 119 | ≤ 51817 | 183 | ≤ 182236 | 247 | ≤ 640902 |
| 56 | ≤ 15026 | 120 | ≤ 52846 | 184 | ≤ 185852 | 248 | ≤ 653620 |
| 57 | ≤ 15324 | 121 | ≤ 53894 | 185 | ≤ 189540 | 249 | ≤ 666590 |
| 58 | ≤ 15628 | 122 | ≤ 54964 | 186 | ≤ 193302 | 250 | ≤ 679818 |
| 59 | ≤ 15938 | 123 | ≤ 56054 | 187 | ≤ 197137 | 251 | ≤ 693308 |
| 60 | ≤ 16255 | 124 | ≤ 57167 | 188 | ≤ 201049 | 252 | ≤ 707066 |
| 61 | ≤ 16577 | 125 | ≤ 58301 | 189 | ≤ 205039 | 253 | ≤ 721097 |
| 62 | ≤ 16906 | 126 | ≤ 59458 | 190 | ≤ 209108 | 254 | ≤ 735406 |
| 63 | ≤ 17242 | 127 | ≤ 60638 | 191 | ≤ 213257 | 255 | ≤ 750000 |

[TS 38.331, clause 6.3.1]

accessAllowed2RxXR Indicates if the cells on the frequency support 2Rx XR UEs. If present, 2Rx XR UEs shall consider only these NR frequencies in cell reselection evaluation.

7.1.1.3.18.3	Test description

7.1.1.3.18.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 with the exception of 2 AM DRBs on NR cell configured according to Table 7.1.1.3.18.3.1-1.

Table 7.1.1.3.18.3.1-1: Logical Channel Configuration Settings

| Parameter | Value DRB1 | Value DRB2 |
| --- | --- | --- |
| LogicalChannelIdentity | LCH4(DRB-Identity +3) | LCH5(DRB-Identity +3) |
| Priority | 7 | 6 |
| prioritizedBitRate | 0 kB/s | 0 kB/s |
| logicalChannelGroup | 2 (LCG ID#2) | 1 (LCG ID#1) |
| additionalBS-TableAllowed | 1 | 0 |

7.1.1.3.18.3.2	Test procedure sequence

Table 7.1.1.3.18.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 2 | The SS transmits a MAC PDU containing one RLC SDUs of size 2000 bytes on LCH4 (<4751 bytes) | <-- | MAC PDU (1 RLC SDUs on LCH4) | - | - |
| 3 | SS allocates an UL Grant of 56 bits. (Note 1) | <-- | (UL Grant, 56 bits) | - | - |
| 4 | Check: Does the UE transmit a Short BSR with ‘LCG ID’ field set to ‘2’? (Note 2) | --> | MAC PDU (MAC Short BSR (LCG ID=‘2’,)) | 3 | P |
| 5 | The SS transmits a MAC PDU containing one RLC SDUs of size 50000 bytes on LCH4 (<750000 bytes) | <-- | MAC PDU (1 RLC SDUs on LCH4) | - | - |
| 6 | The SS transmits a MAC PDU containing one RLC SDUs of size 200 bytes on LCH5 (<4751 bytes) |  | MAC PDU (1 RLC SDUs on LCH5) | - | - |
| 7 | SS allocates an UL Grant of 56 bits. (Note 1) | <-- | (UL Grant, 56 bits) | - | - |
| 8 | Check: Does the UE transmit a Refined Long BSR with ‘LCG ID’ field set to ‘2’ and corresponding ‘BT’ field set to ‘1’ & ‘LCG ID’ field set to ‘1’ and corresponding ‘BT’ field set to ‘0’ ? | --> | MAC PDU (MAC Refined Long BSR (LCG ID=‘2’, BT =’1’& LCG ID=’1’, BT=’0’)) | 1 | P |
| 9 | The SS transmits a MAC PDU containing one RLC SDUs of size 30000 bytes on LCH4 (>750000 bytes) | <-- | MAC PDU (1 RLC SDU on LCH4) | - | - |
| 10 | SS allocates an UL Grant of 56 bits. (Note 1) | <-- | (UL Grant, 56 bits) | - | - |
| 11 | Check: Does the UE transmit a Long BSR with ‘LCG ID’ field set to ‘2’ & ‘LCG ID’ field set to ‘1’ ? | --> | MAC PDU (MAC Long BSR (LCG ID=‘2’& LCG ID=’1’)) | 2 | P |
| 12 | SS allocates an UL Grant of 90000 bytes or more. | <-- | (UL Grant) | - | - |
| 13 | The UE transmits a MAC PDU including 4 RLC SDUs looped back corresponding to steps 2, 5, 6 and 9. | --> | MAC PDU (3 MAC sub PDUs from LCH4, 1 MAC sub PDU from LCH5) | - | - |
| 14 | SS transmits an RLC STATUS PDU to acknowledge correctly received data(LCID=’000100’) | <-- | RLC STATUS PDU (ACK_SN=3) | - | - |
| 15 | SS transmits an RLC STATUS PDU to acknowledge correctly received data(LCID=’000101’) | <-- | RLC STATUS PDU (ACK_SN=1) | - | - |
| Note 1: 56 bits enables UE to transmit a MAC PDU with a 1 byte MAC BSR header and a Short BSR (1 byte) or a 2 bytes MAC BSR header and a Long BSR 3 bytes (with 2 LCG configured) or a 3 bytes MAC BSR header and a Refined Long BSR 4 bytes (with 2 LCG configured). |  |  |  |  |  |

7.1.1.3.18.3.3	Specific message contents

None