3GPP TSG-RAN WG5 Meeting #108	R5-253858

Bangaluru, India, 25th – 29th Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0648 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new UE Rx-Tx time difference accuracy test case 15.3.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN#104, WI of Conformance Test Aspects - Expanded and improved NR positioning has been approved, related test cases should be introduced into TS 37.571-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added 15.3.10 UE Rx-Tx time difference measurement accuracy test case in FR2 for Redcap UE without Rx FH |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The release 18 positioning work item will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.3.10 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-3 CR 0177 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

15.3.10	UE Rx-Tx time difference measurement accuracy test case in FR2 for Redcap UE without Rx FH

Editor’s note: This test case is incomplete. The following aspect is either missing or TBD:

- Test tolerance are not added in and analysis is missing

15.3.10.1	Test purpose

The purpose of the test is to verify that the UE Rx-Tx time difference measurement accuracy without RX FH in RRC_CONNECTED state is within the specified limits. This test will verify the requirements in TS 38.133 [50] clause 10.1A.18.2. The test is conducted in AWGN propagation condition in FR2 in standalone scenario when single positioning frequency layer is configured.

15.3.10.2	Test applicability

This test applies to all types of NR redcap UE release 18 onwards that supports Multi-RTT positioning method.

15.3.10.3	Minimum conformance requirements

For UE Rx-Tx time difference measurement performed by 2RX RedCap UE without RX FH, the accuracy requirements corresponding to the PRS bandwidth supported by the RedCap UE for PRS measurement without RX FH in clause 10.1.25.2 shall apply.

The normative reference for this requirement is TS 38.133 [50] clause 10.1A.18.2.

15.3.10.4	Test description

The supported test configuration is listed in Table 15.3.10.4-1.

Table 15.3.10.4-1: Test Configuration

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB and PRS SCS, 100 MHz bandwidth, TDD duplex mode |

15.3.10.4.1	Initial conditions

Test Environment: Normal, as defined in TS 38.508-1 [45] clause 4.1.

Frequencies to be tested: Mid-Range, as defined in TS 38.508-1 [45] clause 4.3.1.

Channel bandwidth to be tested: are specified in Table 15.3.10.4-1.

1.	Connect the SS and AWGN noise sources to the UE antenna connector or antenna connectors as shown in Annex A, Figure A.14.

2.	The general test parameter settings are set up according to Table 15.3.10.5-1.

3.	Propagation conditions are set according to clause 4.14.2.

4.	Message contents are defined in clause 15.3.10.4.3.

5.	There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2).

15.3.10.4.2	Test procedure

Same as defined in clause 15.3.7.4.2.

15.3.10.4.3	Message contents

As defined in clause 15.3.7.4.3, with the following exceptions:

Table 15.3.10.4.3-1 replaces Table 15.3.7.4.3-3, Table 15.3.10.4.3-2 replaces Table 15.3.7.4.3-7, Table 15.3.10.4.3-3 replaces Table 15.3.7.4.3-8 and Table 15.3.10.4.3-4 replaces Table 15.3.7.4.3-5.

Table 15.3.10.4.3-1: SRS-Config

| Derivation Path: TS 38.508-1 [45], Table 4.6.3-182 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-ResourceSetToReleaseList | Not present |  |  |
| srs-ResourceSetToAddModList | Not present |  |  |
| srs-ResourceToReleaseList | Not present |  |  |
| srs-ResourceToAddModList | Not present |  |  |
| tpc-Accumulation | Not present |  |  |
| srs-RequestDCI-1-2-r16 | Not present |  |  |
| srs-RequestDCI-0-2-r16 | Not present |  |  |
| srs-ResourceSetToAddModListDCI-0-2-r16 | Not present |  |  |
| srs-ResourceSetToReleaseListDCI-0-2-r16 | Not present |  |  |
| srs-PosResourceSetToReleaseList-r16 | Not present |  |  |
| srs-PosResourceSetToAddModList-r16 SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16 { | 1 entry |  |  |
| SRS-PosResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| srs-PosResourceSetId-r16 | 0 |  |  |
| srs-PosResourceIdList-r16 SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-PosResourceId-r16 { | 1 entry |  |  |
| SRS-PosResourceId-r16 | 0 | 1 entry |  |
| } |  |  |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| alpha-r16 | alpha0 |  |  |
| p0-r16 | 0 |  |  |
| pathlossReferenceRS-Pos-r16 CHOICE { |  |  |  |
| ssb-IndexServing-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| srs-PosResourceToReleaseList-r16 | Not present |  |  |
| srs-PosResourceToAddModList-r16 SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16 { | 1 entry |  |  |
| SRS-PosResource-r16[1] SEQUENCE { |  | entry 1 |  |
| srs-PosResourceId-r16 | 0 |  |  |
| transmissionComb-r16 CHOICE { |  |  |  |
| n4-r16 SEQUENCE { |  |  |  |
| combOffset-n4-r16 | 0 |  |  |
| cyclicShift-n4-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceMapping-r16 SEQUENCE { |  |  |  |
| startPosition-r16 | 5 |  |  |
| nrofSymbols-r16 | n4 |  |  |
| } |  |  |  |
| freqDomainShift-r16 | 0 |  |  |
| freqHopping-r16 SEQUENCE { |  |  |  |
| c-SRS-r16 | Matches NRB,c Table 15.3.10.5-1 |  |  |
| } |  |  |  |
| groupOrSequenceHopping-r16 | neither |  |  |
| resourceType-r16 CHOICE { |  |  |  |
| periodic-r16 SEQUENCE { |  |  |  |
| periodicityAndOffset-p-r16 CHOICE { |  |  |  |
| sl1280 | 160 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sequenceId-r16 | 0 |  |  |
| spatialRelationInfoPos-r16 | Not present |  |  |
| txHoppingConfig-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| dci-TriggeringPosResourceSetLink-r18 | Not present |  |  |
| } |  |  |  |

Table 15.3.10.4.3-2: NR-DL-PRS-AssistanceData

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ReferenceInfo-r16 SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-DL-PRS-ResourceID-List-r16 | Not present |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 1 entry |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | kHz120 |  |  |
| dl-PRS-ResourceBandwidth-r16 | 11 | 64 PRBs |  |
| dl-PRS-StartPRB-r16 | 0 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA + offsetToCarrier, as defined for the DL frequency of the Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | n4 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 | Cell 1 |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 | Cell 1 |  |  |
| nr-ARFCN-r16 | Cell 1 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| nr-DL-PRS-Info-r16 | As specified in Table 15.3.10.4.3-3 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | Cell 2 |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 | Cell 2 |  |  |
| nr-ARFCN-r16 | Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 23 | About 3 s |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 39 | About 5 s |  |
| nr-DL-PRS-Info-r16 | As specified in Table 15.3.10.4.3-3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-SSB-Config-r16 | Not present |  |  |
| nr-DL-PRS-AggregationInfo-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 15.3.10.4.3-3: NR-DL-PRS-Info

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS-ResourceSet-r16 { | 1 entry |  |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceSetID-r16 | 0 |  |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 CHOICE { |  |  |  |
| scs120-r16 CHOICE { |  |  |  |
| n1280-r16 | 80 |  |  |
| } |  |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 | Not present |  |  |
| dl-PRS-ResourceTimeGap-r16 | s1 |  |  |
| dl-PRS-NumSymbols-r16 | n4 |  |  |
| dl-PRS-MutingOption1-r16 SEQUENCE { |  |  |  |
| dl-prs-MutingBitRepetitionFactor-r16 | Not present |  |  |
| nr-option1-muting-r16 CHOICE { |  |  |  |
| po2-r16 | 10 |  | Cell 1 |
|  | 01 |  | Cell 2 |
| } |  |  |  |
| } |  |  |  |
| dl-PRS-MutingOption2-r16 | Not present |  |  |
| dl-PRS-ResourcePower-r16 | 27 |  |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16 { | 1 entry |  |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceID-r16 | 0 |  |  |
| dl-PRS-SequenceID-r16 | 0 |  | Cell 1 |
|  | 1 |  | Cell 2 |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |
| n4-r16 | 0 |  |  |
| } |  |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 0 |  |  |
| dl-PRS-ResourceSymbolOffset-r16 | 0 |  |  |
| dl-PRS-QCL-Info-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 15.3.10.4.3-4: LPP RequestLocationInformation

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | (0..255) |  |  |
| } |  |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsRequestLocationInformation SEQUENCE { |  |  |  |
| locationInformationType | locationMeasurementsRequired |  |  |
| triggeredReporting | Not present |  |  |
| periodicalReporting | Not present |  |  |
| additionalInformation | onlyReturnInformationRequested |  |  |
| qos SEQUENCE { |  |  |  |
| horizontalAccuracy | Not present |  |  |
| verticalCoordinateRequest | FALSE |  |  |
| verticalAccuracy | Not present |  |  |
| responseTime SEQUENCE { |  |  |  |
| time | See 15.3.10.5 | Result of the response time calculation rounded up to the next second if response time <= 128s. |  |
| responseTimeEarlyFix-r12 | Not present |  | Rel-12 onwards |
| } |  |  |  |
| velocityRequest | FALSE |  |  |
| } |  |  |  |
| environment | Not present |  |  |
| locationCoordinateTypes | Not present |  |  |
| velocityTypes | Not present |  |  |
| } |  |  |  |
| a-gnss-RequestLocationInformation | Not present |  |  |
| otdoa-RequestLocationInformation | Not present |  |  |
| ecid-RequestLocationInformation | Not present |  |  |
| epdu-RequestLocationInformation | Not Present |  |  |
| sensor-RequestLocationInformation-r13 | Not present |  |  |
| tbs-RequestLocationInformation-r13 | Not present |  |  |
| wlan-RequestLocationInformation-r13 | Not present |  |  |
| bt-RequestLocationInformation-r13 | Not present |  |  |
| nr-ECID-RequestLocationInformation-r16 | Not present |  |  |
| nr-Multi-RTT-RequestLocationInformation-r16 SEQUENCE { |  |  |  |
| nr-UE-RxTxTimeDiffMeasurementInfoRequest-r16 | Not present |  |  |
| nr-RequestedMeasurements-r16 | bit 0 = 1 (prsrsrpReq) |  |  |
| nr-AssistanceAvailability-r16 | FALSE |  |  |
| nr-Multi-RTT-ReportConfig-r16 SEQUENCE { |  |  |  |
| maxDL-PRS-RxTxTimeDiffMeasPerTRP-r16 | Not present |  |  |
| timingReportingGranularityFactor-r16 | Not present |  |  |
| } |  |  |  |
| additionalPaths-r16 | Not present |  |  |
| nr-UE-RxTxTEG-Request-r17 | Not present |  |  |
| measureSameDL-PRS-ResourceWith DifferentRxTxTEGs-r17 | Not present |  |  |
| measureSameDL-PRS-ResourceWith DifferentRxTEGs-r17 | Not present |  |  |
| reducedDL-PRS-ProcessingSamples-r1 | Not present |  |  |
| nr-los-nlos-IndicatorRequest-r17 | Not present |  |  |
| additionalPathsExt-r17 | Not present |  |  |
| additionalPathsDL-PRS-RSRP-Request-r17 | Not present |  |  |
| multiMeasInSameReport-r17 | Not present |  |  |
| multiMeasInSameReport-r17 | Not present |  |  |
| nr-DL-PRS-RxHoppingRequest-r18 | Not present |  |  |
| timingReportingGranularityFactorExt-r18 | Not present |  |  |
| nr-DL-PRS-JointMeasurementRequest-r18 | Not present |  |  |
| nr-DL-PRS-MeasurementTimeWindows Config-r18 | Not present |  |  |
| nr-NTN-UE-RxTxMeasurementsRequest-r18 | Not present |  |  |
| } |  |  |  |
| nr-DL-AoD-RequestLocationInformation-r16 | Not present |  |  |
| nr-DL-TDOA-RequestLocationInformation-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

15.3.10.5	Test requirement

FFS