3GPP TSG RAN Meeting #109															RP-252320

Beijing, China, September 15-18, 2025

Agenda item:	8.2.2

Source:	Rakuten Mobile Inc

Title:	Native 3 MHz initial-access profile for low FR1 FDD bands

Document for:	Discussion and Decision

# Introduction

RAN1#122 had following agreement on 6GR study item which basically defer the discussion RAN Plenary.

Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

# Discussion

In some markets, the only practical low-band spectrum for wide-area and deep-indoor coverage is 3 MHz paired FDD. If 6GR initial access were realizable only for cell bandwidths ≥ 5 MHz, operation on 3 MHz would depend on workarounds such as puncturing or off-raster placement, similar to what is done in NR Rel-18 to enable 3 MHz spectrum utilization. These approaches come with coverage-margin penalties, more complex acquisition, and increased implementation overhead. A more robust approach is a native initial-access profile within 3 MHz at 15 kHz SCS on selected low FR1 FDD bands.

We do not propose changes to 30/60 kHz FR1 or 120 kHz FR2 designs, and we do not change any generic “all-UE” minimum-bandwidth assumption. UE impact is minimal (a single capability flag per band). For the RAN1 study, success criteria include: robust SSB detection at high MCL, high RA/SIB1 success at low SINR, and proof that the entire IA chain—including guards—fits inside 3 MHz. RAN4 can assess SEM/ACLR/masks and testability for the chosen bands.

# Conclusions

We have the following proposals for RAN1 to study in the context of 3 MHz initial access.

Proposal 1: RAN1 is invited to agree to study and define a 3 MHz-contained NB-Initial-Access (NB-IA) profile at 15 kHz SCS/FDD for selected low FR1 bands, comprising:

- NB-SSB/NB-PBCH with reduced span for low-SINR detection;

- NB-CORESET#0/PDCCH sufficient for RAR/SIB1 entirely within 3 MHz (no puncturing);

- Measurement & discovery handling on NB-SSB and neighbor search across NB-IA rasters.

Proposal 2: The NB-IA study is band-specific, with no impact to 30/60 kHz FR1 or 120 kHz FR2 designs and no change to the generic “all-UE” minimum-bandwidth assumption.

# References

[1] 	RP-251881, “Study on 6G Radio,” NTT Docomo, June 2025.