3GPP TSG RAN Meeting #109					RP-252656

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.4

Source:	Ericsson, AT&T, Bell Mobility, BT, CHTTL, Deutsche Telekom, FirstNet, Fujitsu, Intel Corporation, KDDI, KT Corp., LG Electronics, NTT DOCOMO, OPPO, Semtech, Sequans, SK Telecom, Sony, Spark NZ, Telstra, Thales, T-Mobile USA, Verizon, Vodafone

Title:	Minimum requirements for Massive Communication for IoT

Document for:	Approval

# 1	Introduction

One of the IMT-2030 usage scenarios is Massive Communication, which is described as follows in [1]:

Massive Communication This usage scenario extends massive Machine Type Communication (mMTC) of IMT-2020 and involves connection of massive number of devices or sensors for a wide range of use cases and applications. Typical use cases include expanded and new applications in smart cities, transportation, logistics, health, energy, environmental monitoring, agriculture, and many other areas such as those requiring a variety of IoT devices without battery or with long-life batteries. This usage scenario would require support of high connection density, and depending on use cases, different data rates, low power consumption, mobility, extended coverage, and high security and reliability.

In this contribution to the RAN study on 6G scenarios and requirements [2][3], we discuss and provide input on the minimum requirements for Massive Communication.

# 2	Discussion

## 2.1	Continued support for IoT devices from Day 1 of 6G

A wide range of Internet-of-Things (IoT) use cases can be successfully served by the low-end 4G/5G based IoT technologies defined in 3GPP (NB-IoT, eMTC, Cat-1/1bis, Cat-4, RedCap, eRedCap). These technologies are expected to live on for a long time, and 6G must be able to coexist with these technologies.

However, eventually 6G should be able to replace the legacy 4G/5G technologies for all significant use cases including IoT use cases. To facilitate a smooth migration to 6G, it is important that IoT use cases are considered already in the first 6G release. Otherwise, the alternative would be to retrofit a non-backwards-compatible IoT solution into 6G at a later stage, which would come with several drawbacks.

## 2.2	Minimum requirements for 6G Massive Communication

Based on the discussion on continued support for diverse device types from Day 1 of 6G (see Section 2.1), the following minimum requirements for Massive Communication can be considered.

### 2.2.1	UE complexity and form factor

The legacy 3GPP technologies optimized for IoT use cases employ at least some of the following main techniques to facilitate UE implementations with low complexity and small form factors.

- Single antenna: NB-IoT, eMTC, Cat-1/1bis, RedCap, and eRedCap UEs can all be implemented with either one or two antennas. The minimum requirement for Massive Communication should also be a single antenna to be able to achieve low enough complexity and small form factor, at least in FR1 bands. Means to restrict the implementation of a single antenna to low-end UEs and to limit their access to specific cells or carriers (in the same way as, e.g., for RedCap) should be considered.

- Modest peak rates: Cat-1/1bis and eRedCap UEs support DL/UL peak rates of 10/5 Mbps and 10/10 Mbps, respectively. Very roughly speaking, NB-IoT and eMTC UEs have an order of magnitude lower peak rates, and Cat-4 and RedCap UEs have an order of magnitude higher peak rates. Too high peak rate should be avoided for complexity and power consumption reasons. Too low peak rates should be avoided for spectral efficiency, latency, and energy consumption reasons. A reasonably high peak rate will be beneficial when somewhat larger amounts of data need to be transferred efficiently, for example during software/firmware upgrade. The minimum required peak rate for Massive Communication may need further discussion, but a reasonable tentative value is 5~10 Mbps.

- Limited bandwidth: Cat-1/1bis, Cat-4, FR1 RedCap, and eRedCap UEs support 20 MHz UE RF bandwidth. NB-IoT and eMTC have 200 kHz and 1.4 MHz UE bandwidths, respectively. To maintain a common air interface for 6G IoT UEs and other 6G UEs (same sync signals, etc.), it is desired that the maximum required UE bandwidth supported by the most low-end UE is not too small, to avoid negative impacts on initial access for other 6G UEs. For this reason, we think a reasonable tentative value is [3-5] MHz, at least for low FR1 bands. The maximum required UE bandwidth could potentially be different for low/mid/high bands. Reducing the UE bandwidth further than this is not expected to provide significant additional complexity reduction. Note that the maximum UE bandwidth does not determine what carrier bandwidths the UE can support. For example, FR1 RedCap UEs have a maximum UE bandwidth of 20 MHz, but they can operate on NR carriers with carrier bandwidths that are much smaller (e.g., 5 MHz) or much larger (e.g., 100 MHz).

- Half-duplex FDD operation: In FDD bands, NB-IoT, eMTC, RedCap, and eRedCap UEs can be implemented with half-duplex operation, meaning that they are not required to transmit and receive at the same time. Other 4G/5G UEs including Cat-1/1bis and Cat-4 only support full-duplex operation, which requires a duplex filter, or possibly multiple duplex filters if the UE supports multiple bands, which can become complex and increase the device cost. It can be troublesome to introduce support for half-duplex FDD retroactively in the standard and in products. Therefore, it is desired that half-duplex FDD operation is properly supported already in the first 6G release.

To avoid the need to introduce non-backwards-compatible solutions in a future 6G release, the design of the first 6G release should support low-end UEs that make use of the above techniques.

### 2.2.2	Latency, coverage, UE energy efficiency, connection density

The legacy 3GPP IoT technologies are also associated with requirements on the following aspects.

- Latency: The CP/UP latency requirements for 6G UEs used for Massive Communication use cases can potentially be relaxed compared to other use cases. If the bandwidth and peak rates are not too small, it should still be possible to achieve, e.g., an end-to-end latency of no more than ~100 ms, allowing an IoT device to set up a connection and send a small packet relatively quickly, or to support a conversational voice service, which can be useful for some IoT services.

- Coverage: Some Massive Communication use cases will require or benefit from coverage extension beyond “normal” 6G coverage, which is yet to be defined. Corresponding deployment scenarios may need to be defined for evaluation purposes. The required coverage extension will probably largely have to rely on repetition of transmissions in both downlink and uplink. It should be possible to reuse parts of the coverage enhancement work done in the later NR releases (e.g., repetition introduced for many signals/channels) and make it generically available to both IoT UEs and other UEs in the first 6G release. The simplest IoT UEs may require some additional repetitions in downlink for coverage recovery due to the single receive antenna.

- UE energy efficiency: There will be Massive Communication use cases in 6G that require long battery lives. The complexity reduction techniques listed in the previous section can help reduce power consumption, and additional power saving techniques such as extended discontinuous reception (eDRX) and wake-up receivers (WUR) are beneficial for UEs that need a long battery life. These additional power saving techniques can be supported by 6G IoT UEs as well as ordinary 6G UEs.

- Connection density: Connection density requirements are expected to be addressed in the IMT-2030 TPR discussion [4][5][6] and are not addressed further in this contribution.

Further discussion is needed regarding potential requirements specific to Massive Communication use cases for the above aspects.

# 3	Text proposal

Based on the discussion above we propose the following text proposal for TR 38.914 [3]:

## 5.4.x	Minimum requirements for Massive Communication

The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication:

- Massive Communication shall be supported in FR1.

- The minimum number of UE receive antennas is 1.

- The minimum PHY peak data rate is [5~10] Mbps in DL and [5~10] Mbps in UL.

- The required UE bandwidth for the lowest-tier UE is [3-5] MHz at least for FDD bands. The UE can support operation in a carrier bandwidth different from the UE bandwidth.

- Half-duplex FDD operation is supported.

- Capabilities of devices for massive communication shall be signaled to the 6G RAN.

- UE energy efficiency and coverage targets are FFS.

- Coexistence with NB-IoT (all deployment modes) and eMTC shall be ensured.

# 4	Conclusion

In this contribution we propose adding the text proposal in section 3 to TR 38.914 clause 5.4.x.

# References

M.2160-0 (11/2023), “Recommendation ITU-R M.2160-0: Framework and overall objectives of the future development of IMT for 2030 and beyond”, ITU-R, November 2023.

RP-251395, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, RAN#108, June 2025.

TR 38.914, “Study on 6G Scenarios and requirements”, RAN.

RP-243325, “Draft Reply to ITUR_WP5D_TEMP_167 = RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, RAN#106, December 2024.

RP-250819, “Moderator's summary for IMT-2030 TPRs”, Moderator (CMCC), RAN#107, March 2025.

RP-251761, “Draft reply LS to ITU-R WP5D =RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, RAN#108, June 2025.