3GPP TSG-RAN Meeting #109	RP-252658

Beijing, China , Sept 15th – 18th, 2025

Source:	Nokia

Title:	Considerations on RAN4-led REL-20 5G-Adv UE RF topics

Agenda item:	9.1.4.1

Document for:	Discussion

Introduction

In the last RAN plenary meeting #108 in Prague, a WF for RAN4 led REL-20 topics in [1] was endorsed. During the meeting a number of agreements were made while some aspects are still open for further discussions. In this paper we discuss these remaining open aspects regarding the UE RF topics for 5G-Adv RAN4 led Rel-20 package.

Discussion

## 2.1	UE RF centric evolution

On the endorsed Way Forward [1] for RAN4-led Rel-20 5G-Adv topics from RAN#108 three topics were identified for UE RF centric evolution:

- FR1 HPUE enhancement

- 6 MHz channel bandwidth

- Reduced UE MPR values with relaxed Tx EVM

It should be noted 6 MHz channel bandwidth item which is currently under UE RF centric evolution will create work on BS RF, RRM and demodulation part.

- We therefore assume that the 6 MHz channel bandwidth would be best handled in a dedicated WI.

As the 6 MHz channel bandwidth will impact UE RF, BS RF, RRM and demod, it would be best handled in a dedicated WI.

For the third item, according to the endorsed WF the work would start in Q1 2026, unless it is agreed to be included in the RAN4 6G work, in which case it would not be in the 5G-A package but a subsequent conclusion in the context of 6G might be applied to 5G. It therefore seems inappropriate to include this item at this stage in the Rel-20 5G-A UE RF WID; it could be added at RAN#110 if by then the RAN4 6G scope has made it clear that this topic will not be included in the 6G study. This would not impact the start date of the work.

The RAN4 5G-A package should include only topics that are immediately implementable and beneficial for the ecosystem. One quite obvious way to do this would be specifying high power classes for existing power amplifier designs, i.e. PC2 and PC3 for FWA, for terrestrial networks. Combining existing PAs brings the additional benefit of making UL-MIMO inherently implementable which would significantly increase the data rate of FWA links which are mainly data pipes.

The work should focus on the single-carrier case; carrier aggregation could be considered later.

Our proposed core part objectives for UE RF enhancements for HPUE are:

High power UE (HPUE) for CA in terrestrial network (TN)

- Define high power PC1.5/PC1 (i.e. 32 dBm, referred to here as PC1bis) UE for TN single carrier in NR FR1

  - PC1bis with 4Tx with 4-layer UL-MIMO for FWA devices.

  - PC1bis with 8Tx with 8-layer UL-MIMO for FWA devices.

    - Redo coexistence studies for FWA UE to see if ACLR can be relaxed from current PC1 37 dBc value which is intended for vehicular use case.

  - PC1.5 FDD with 2Tx for both handheld and FWA devices.

- Specify release independence requirements for the core part in TS 38.307, if needed and feasible.

- Study and specify UE capabilities corresponding to the above HPUE categories

RAN shall consider the above objectives for High power UE (HPUE) for CA in terrestrial network (TN)

Considerations on Basket WIs.

While the spectrum items for Rel-20 in general have not been discussed much, some considerations are needed especially for how RAN4 is intending to handle the Basket WIs. By basket WIs are meant the ones shown in Table 1.

Table 1 Current Rel-19 Basket Work-Items

| Work item | Description |
| --- | --- |
| DC_R19_xBLTE_yBNR | EN-DC and NE-DC band combinations |
| NR_CADC_SUL_R19 | NR CA and NR DC band combinations |
| LTE_CA_R19_xBDL_yBUL | LTE CA |
| HPUE_NR_FR1_bands_R19 | High power UE for FR1 |
| HPUE_LTE_bands_R19 | High power UE for LTE |
| HPUE_DC_LTE_NR_R19 | High power UE for EN-DC and NE-DC |
| HPUE_NR_CADC_SUL_R19 | High power UE for NR CA and NR DC |
| NR_bands_xFeature_R19 | UL-MIMO, 4Rx and 8 RX |
| DL_intrpt_combos_TxSW_R19 | Band combinations supporting no interruption in Tx switching |
| LTE_NR_Simult_RxTx_R19 | Band combinations supporting simultaneous Rx and Tx |
| NR_bands_R19_BWs | New channel bandwidths to existing bands |

In our view some of the current Rel-19 Basket Work-Items may not be needed in Rel-20 but that can be up for further discussion dependent on operator needs. However, in general since Rel-20 is at the end of 5G-Adv, a starting point could be to continue the baskets without too much change, as the focus can then be on how to handle these in 6G. Another thing which RAN4 needs to consider is how the implementation of the use of the band combination database is intended in Rel-20. A suggestion is to include this as an objective in the relevant baskets.

RAN4 may need to consider which basket WIs to continue in Rel-20 and whether any of these need to be revised to reflect new objectives.

With the list of basket WIs, introduction of working procedure of the ETSI MCC band combination database and the expected other spectrum items to come to RAN4 it is important to reserve enough TUs for this spectrum work also.

Emphasis shall be given to the non-negligible spectrum work in RAN4 making sure enough TUs are available.

## 2.7	TU discussion

The overall TU allocation for 5G-Adv topics in Rel-20 needs to take into account the time required for the 6G SI. Great care therefore needs to be taken not to overload the 5G-Adv track, nor to underestimate the TUs required for individual items.

Great care therefore needs to be taken not to overload the 5G-Adv track, nor to underestimate the TUs required for individual items.

Conclusion

Overall, the TU allocation for 5G-Adv topics in Rel-20 needs to take into account the time required for the 6G SI. Great care therefore needs to be taken not to overload the 5G-Adv track, nor to underestimate the TUs required for individual items.

The following Observations and Proposals were made on the specific domains:

Observation 1: It should be noted 6 MHz channel bandwidth item which is currently under UE RF centric evolution will create work on BS RF, RRM and demodulation part.

Proposal 1: As the 6 MHz channel bandwidth will impact UE RF, BS RF, RRM and demod, it would be best handled in a dedicated WI.

Proposal 2: RAN shall consider the above objectives for High power UE (HPUE) for CA in terrestrial network (TN)

Observation 2: RAN4 may need to consider which basket WI to continue in Rel-20 and whether any of these needs to be revised to reflect new objectives.

Proposal 3: Emphasis shall be given to the non-negligible spectrum work in RAN4 making sure enough TUs are available.

Proposal 4: Great care therefore needs to be taken not to overload the 5G-Adv track, nor to underestimate the TUs required for individual items.

- References

- RP-251879; WF for RAN4 led REL-20 topics; RAN4 Chair

- RP-250900, Rel-20 5G-A UE RF and Spectrum, Nokia