3GPP TSG RAN meeting #109										RP-252695

Beijing, China, September 15-18, 2025

## Status Report to TSG

Agenda item:			9.6.4.9

| WI / SI Name | Low NR band carrier aggregation via switching |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_LBCA_Sw |  |  |  |  |
| Unique ID | 1060082 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-251315 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 09/2025 | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 100% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Ivo Maljevic |
|  | Company | TELUS |
|  | Email | ivo.maljevic@telus.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1 #122

General aspects

-	The summary of the maintenance discussion is captured in [11], [12]

-	The summary of the related feature discussion is captured in [29] and in the session notes in [30]

Agreements

-	The following maintenance agreements were achieved:

Agreement - Following TP to Section9, TS38.213 is endorsed in principle 9 UE procedure for reporting control information ## *** Unchanged parts are omitted *** When a UE determines overlapping for PUCCH and/or PUSCH transmissions of the same priority index other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission due to cell DRX operation [11, TS 38.321] or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24 including repetitions if any, - first, the UE resolves the overlapping for PUCCHs with repetitions as described in clause 9.2.6, if any - second, the UE resolves the overlapping for PUCCHs without repetitions as described in clauses 9.2.5 - third, the UE resolves the overlapping for PUSCHs and PUCCHs with repetitions as described in clause 9.2.6 - fourth, the UE resolves the overlapping for PUSCHs and PUCCHs without repetitions as is subsequently described in this clause. ## *** Unchanged parts are omitted *** When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, other than PUCCH transmissions with SL HARQ-ACK reports, before considering limitations for transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24 including repetitions if any, if the UE is provided uci-MuxWithDiffPrio and the timeline conditions in clause 9.2.5 for multiplexing UCI in a PUCCH or a PUSCH are satisfied - first, the UE resolves overlapping for PUCCH and/or PUSCH transmissions of a same priority index as described in clauses 9.2.5 and 9.2.6 - second, the UE resolves the overlapping for PUCCH transmissions of different priority indexes, and ## *** Unchanged parts are omitted *** the UE - transmits the PUCCH or the PUSCH of the larger priority index subject to the limitations for UE transmissions due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 24 and - does not transmit a PUCCH or a PUSCH of smaller priority index When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, other than PUCCH transmissions with SL HARQ-ACK reports, before considering limitations for transmissions including with repetitions, if any, due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24, if the UE is not provided uci-MuxWithDiffPrio, the UE first resolves overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in clauses 9.2.5 and 9.2.6. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where - the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in clauses 9.2.5 and 9.2.6 - any remaining PUCCH and/or PUSCH transmission after overlapping resolution is subjected to the limitations for UE transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24 ## *** Unchanged parts are omitted *** In the remaining of this clause, a UE multiplexes UCIs with same priority index in a PUCCH or a PUSCH before considering limitations for UE transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15, and 17.2 and 24. A PUCCH or a PUSCH is assumed to have a same priority index as a priority index of UCIs a UE multiplexes in the PUCCH or the PUSCH. ## *** Unchanged parts are omitted ***. Agreement - Following TP to Section5.1, TS38.214 is endorsed in principle 5.1 UE procedure for receiving the physical downlink shared channel <Unchanged parts are omitted> If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, or determined as non-active periods of cell DTX, if the serving cell is activated with cell DTX, based on [10, TS 38.321], or not available for PDSCH without a corresponding PDCCH transmission receptions as described in clause 24 of [6, TS 38.213], a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot as specified below. ‒ Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot ‒ Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH. ‒ Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping (even partially) with the survivor PDSCH in step 1 are excluded from Q. ‒ Step 3: Repeat step 1 and 2 until Q is empty or j is equal to the number of unicast/multicast PDSCHs in a slot supported by the UE. <Unchanged parts are omitted>. Agreement For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling of SSB, a UE does not receive the SSB that collides with the LB CA switching pattern - Note: a SSB collides with the LB CA switching pattern if - For SSB in FDD carrier, one or multiple SSB symbols overlaps with symbols configured as either SDL carrier or switching gap - For SSB in SDL carrier, one or multiple SSB symbols overlaps with symbols configured as either FDD carrier or switching gap Conclusion For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support the following restrictions for the semi-static switching pattern - It is RAN1 understanding that at least one SSB in PCell is available every 160ms for meeting existing RAN4 requirements - At least one SSB in SCell is available every 160ms

#### 2.1.2	Remaining Open issues

None

## 2.2	RAN2

#### 2.2.1	Agreements

Stage 2 implementation

-	RAN2 has agreed the CR on the introduction of the feature to the Stage 2 specification in TS38.300 [34]

RRC parameters

-	RAN2 has endorsed the CR on the introduction of the RRC parameters necessary to support the feature to the RRC specification in TS38.331 [35]

-	The endorsed CR is undergoing the RAN2 post-meeting email approval process

#### 2.2.2	Remaining Open issues

None

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #116 – RF (Core)

General aspects

-	The topic summary was prepared in [94]

-	An update of TR 38.768 v1.1.0 was approved in [112]

-	The related feature description was updated in the approved RAN4 feature list in [114]

UE RF aspects

-	The CR introducing the feature to the RF specification in TS38.101-1 was agreed in [97]

-	The text proposals to TR 38.768 were approved in [98], [101] and incorporated into the v1.1.0 update of the TR

-	The following agreements related to the performance part of the work item were captured in [95]:

Issue 1: Downlink reference measurement channels Agreement: - RAN4 can continue to discuss how to specify the DL RMC for this feature as part of the Performance phase of the work item - The following options for the RMC have been proposed: - Option 1: 10ms periodicity with switching period placed in slots 4 and 8 Fixed reference channel for receiver requirements configured for low NR band inter-band carrier aggregation via switching [supportedLowBandSwitching-r19] (SCS 15 kHz, FDD, QPSK 1/3) Parameter; Unit; Value. Channel bandwidth; MHz; 3; 5, 10, 15, 20 (Note 5); 10; 15; 20; 25; 30; 40; 50. Subcarrier spacing; kHz; 15; 15; 15; 15; 15; 15; 15; 15; 15. Subcarrier spacing configuration; 0; 0; 0; 0; 0; 0; 0; 0; 0. Allocated resource blocks; 15; 25; 52; 79; 106; 133; 160; 216; 270. Subcarriers per resource block; 12; 12; 12; 12; 12; 12; 12; 12; 12. Allocated slots per Frame; 8; 8; 8; 8; 8; 8; 8; 8; 8. MCS Index; 4; 4; 4; 4; 4; 4; 4; 4; 4. MCS Table for TBS determination; 64QAM. Modulation; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK. Target Coding Rate; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3. Maximum number of HARQ transmissions; 1; 1; 1; 1; 1; 1; 1; 1; 1. Information Bit Payload per Slot. For Slots 0,1,5,6,7,8; Bits; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A. For Slots 2,3,9; Bits; 984; 1672; 3368; 5120; 6912; 8712; 10504; 14088; 17424. For Slot 4; Bits; 504; 1064; 2600; 4064; 5576; 7088; 8648; 11784; 14808. Transport block CRC; Bits; 16; 16; 16; 24; 24; 24; 24; 24; 24. LDPC base graph; 2; 2; 2; 1; 1; 1; 1; 1; 1. Number of Code Blocks per Slot. For Slots 0,1,5,6,7,8; CBs; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A. For Slots 2,3,9; CBs; 1; 1; 1; 1; 1; 2; 2; 2; 3. For Slot 4; CBs; 1; 1; 1; 1; 1; 2; 2; 2; 3. Binary Channel Bits per Slot. For Slots 0,1,5,6,7,8; Bits; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A. For Slots 2,3,9; Bits; 3240; 5400; 11232; 17064; 22896; 28728; 34560; 46656; 58320. For Slot 4; Bits; 2520; 4200; 8736; 13272; 17808; 22344; 26880; 36288; 45360. Max. Throughput averaged over 1 frame; Mbps; 0.3456; 0.6080; 1.2704; 1.9424; 2.6312; 3.3224; 3.8600; 5.4048; 6.7080. NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: Channel bandwidths 10, 15, and 20 MHz in this column only apply to UEs supporting IE supportOfERedCap-r18 but not eRedCapNotReducedBB-BW-r18.. Fixed reference channel for receiver requirements configured for low NR band inter-band carrier aggregation via switching [supportedLowBandSwitching-r19] (SCS 15 kHz, SDL, QPSK 1/3) Parameter; Unit; Value. Channel bandwidth; MHz; 3; 5, 10, 15, 20 (Note 5); 10; 15; 20; 25; 30; 40; 50. Subcarrier spacing; kHz; 15; 15; 15; 15; 15; 15; 15; 15; 15. Subcarrier spacing configuration; 0; 0; 0; 0; 0; 0; 0; 0; 0. Allocated resource blocks; 15; 25; 52; 79; 106; 133; 160; 216; 270. Subcarriers per resource block; 12; 12; 12; 12; 12; 12; 12; 12; 12. Allocated slots per Frame; 8; 8; 8; 8; 8; 8; 8; 8; 8. MCS Index; 4; 4; 4; 4; 4; 4; 4; 4; 4. MCS Table for TBS determination; 64QAM. Modulation; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK. Target Coding Rate; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3. Maximum number of HARQ transmissions; 1; 1; 1; 1; 1; 1; 1; 1; 1. Information Bit Payload per Slot. For Slots 0,1,2,3,4,9; Bits; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A. For Slots 5,6,7; Bits; 984; 1672; 3368; 5120; 6912; 8712; 10504; 14088; 17424. For Slot 8; 504; 1064; 2600; 4064; 5576; 7088; 8648; 11784; 14808. Transport block CRC; Bits; 16; 16; 16; 24; 24; 24; 24; 24; 24. LDPC base graph; 2; 2; 2; 1; 1; 1; 1; 1; 1. Number of Code Blocks per Slot. For Slots 0,1,2,3,4,9; CBs; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A. For Slots 5,6,7; CBs; 1; 1; 1; 1; 1; 2; 2; 2; 3. For Slot 8; CBs; 1; 1; 1; 1; 1; 2; 2; 2; 3. Binary Channel Bits per Slot. For Slots 0,1,2,3,4,9; Bits; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A. For Slots 5,6,7; Bits; 3240; 5400; 11232; 17064; 22896; 28728; 34560; 46656; 58320. For Slots 8; Bits; 2520; 4200; 8736; 13272; 17808; 22344; 26880; 36288; 45360. Max. Throughput averaged over 1 frame; Mbps; 0.3456; 0.6080; 1.2704; 1.9424; 2.6312; 3.3224; 3.8600; 5.4048; 6.7080. NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: Channel bandwidths 10, 15, and 20 MHz in this column only apply to UEs supporting IE supportOfERedCap-r18 but not eRedCapNotReducedBB-BW-r18... - Option 2: Increase period of RMC to 40 slots and use the LBCA-SwitchingPattern: ‘00011 00110 01100 11001 11100 11001 10011 00110’ Parameter; Unit; Value. Switching Pattern for Low-band CA; LBCA-SwitchingPattern; A bitmap of slots; 00011 00110 01100 11001 11100 11001 10011 00110. Period of Switching Pattern; Slots; 40. Slots on the PCell; A bit value of ‘0’ in LBCA-SwitchingPattern. Slots on the SCell; A bit value of ‘1’ in LBCA-SwitchingPattern. Switch-from DL slot on the PCell/SCell; Slot Location; Mod(i, 20) = 2, 4, 6, 8, 10, 12, 14, 16, 18. The number of Switch-from Slot; 18 slots for each period of 40 slots.. CORESET frequency domain allocation; Full BW. CORESET time domain allocation; 1 OFDM symbol at the begin of each slot. PDSCH mapping type; Type B. PDSCH start symbol index (S); 12 - LBCA-SwitchingGap-Duration-PCelltoSCell Or 12 - LBCA-SwitchingGap-SCelltoPCell. Number of consecutive PDSCH symbols (L); 2. PDSCH PRB bundling; PRBs; 1. Dynamic PRB bundling; false. Overhead value for TBS determination; 0. First DMRS position for Type B PDSCH mapping; PDSCH start symbol index (S). DMRS type; Type 1. Number of additional DMRS; 0. FDM between DMRS and PDSCH; Disable. The number of slots between PDSCH and corresponding HARQ-ACK information (K1 Value); Slots; {1, 2, 5}. Switch-to DL slot on the PCell/SCell; Slot Location; Mod(i, 20) = 3, 5, 7, 9, 11, 13, 15, 17, 19. The number of Switch-from Slot; 18 slots for every period of 40 slots.. CORESET frequency domain allocation; Full BW. CORESET time domain allocation; 1 OFDM symbol at the begin of each slot. PDSCH mapping type; Type B. PDSCH start symbol index (S); 1. Number of consecutive PDSCH symbols (L); 2. PDSCH PRB bundling; PRBs; 1. Dynamic PRB bundling; false. Overhead value for TBS determination; 0. First DMRS position for Type B PDSCH mapping; PDSCH start symbol index (S). DMRS type; Type 1. Number of additional DMRS; 0. FDM between DMRS and PDSCH; Disable. The number of slots between PDSCH and corresponding HARQ-ACK information (K1 Value); Slots; {1, 2, 4}. HARQ process number; 4. Subcarrier spacing; kHz; 15. Subcarrier spacing configuration µ; 0. Subcarriers per resource block; 12. Allocated slots per period of 40 slots on the PCell; Slots; 18. Allocated slots per period of 40 slots on the SCell; Slots; 18. MCS Index; 4. MCS Table for TBS determination; 64QAM / table 5.1.3.1-1: MCS index table 1 for PDSCH in TS 38.214. Modulation; QPSK. Target Coding Rate R x [1024]; 308. Maximum number of HARQ transmissions; 1. Channel bandwidth; These parameters can refer to Table A.8.2-2 for differernt channel bandwidth. As PCell and SCell can be configured with different channel bandwidth, TBS per slot for PCell and SCell could be different.. Allocated resource blocks. Information Bit Payload (TBS) per Slot. Number of Code Blocks per Slot. Number of Code Blocks per Slot. Max. Throughput averaged over 4 frame; Mbps; (18*TBSPCell + 18*TBSSCell) / 0.04 / 10^6. ∆TRTD, P2S; µs; 0. ∆TRTD, S2P; µs; 0. NTA; Tc; 0. NTA,offset; Tc; 25600. CSI-RS for tracking; First subcarrier index in the PRB used for CSI-RS (k0); For PCell: 0 for CSI-RS resource 1,2,3,4,5,6,7,8 For SCell: 0 for CSI-RS resource 1,2,3,4,5,6,7,8. OFDM symbols in the PRB used for CSI-RS; For both PCell and SCell: l0 = 6 for CSI-RS resource 1, 3, 5, 7 l0 = 10 for CSI-RS resource 2, 4, 6, 8. Number of CSI-RS ports; For both PCell and SCell: 1 for CSI-RS resource 1,2,3,4,5,6,7,8. CDM Type; For both PCell and SCell: 'No CDM' for CSI-RS resource 1,2,3,4,5,6,7,8. Density (ρ); For both PCell and SCell: 3 for CSI-RS resource 1,2,3,4,5,6,7,8. CSI-RS periodicity; Slots; 15 kHz SCS: 40 for CSI-RS resource 1,2,3,4,5,6,7,8. CSI-RS offset; Slots; 15 kHz SCS: (One swtich-to slot per frame) For PCell: Mod(i, 40) = 5 for CSI-RS resource 1 and 2 Mod(i, 40) = 13 for CSI-RS resource 3 and 4 Mod(i, 40) = 23 for CSI-RS resource 5 and 6 Mod(i, 40) = 35 for CSI-RS resource 7 and 8 For SCell: Mod(i, 40) = 3 for CSI-RS resource 1 and 2 Mod(i, 40) = 15 for CSI-RS resource 3 and 4 Mod(i, 40) = 25 for CSI-RS resource 5 and 6 Mod(i, 40) = 33 for CSI-RS resource 7 and 8. Frequency Occupation; Start PRB 0 Number of PRB = BWP size. QCL info; TCI state #0. PTRS configuration; PTRS is not configured. NOTE 1: Slot i is slot index per four frames.. Table A.8.2-2 DL TBS for different channel bandwidth under the inter-band CA via switching Parameter; Unit; Value. Channel bandwidth; MHz; 5; 10; 15; 20; 25. Allocated resource blocks; 25; 52; 79; 106; 133. Information Bit Payload per Slot. For Slots i in which PDSCH is not allocated, If Mod(i, 20) = 0 or 1; Bits; NA; NA; NA; NA; NA. For Slots i in which PDSCH is allocated, if Mod(i, 20) = 2,3,4,5,6,7,8,9,10,11,12,13,14,15,16,17,18,19; Bits; 176; 368; 576; 768; 984. Transport block CRC; 16; 16; 16; 16; 16. LDPC base graph; 2; 2; 2; 2; 2. Number of Code Blocks per Slot. For Slots i in which PDSCH is not allocated,; CBs; NA; NA; NA; NA; NA. For Slots i in which PDSCH is allocated,; CBs; 1; 1; 1; 1; 1. Binary Channel Bits per Slot. For Slots i in which PDSCH is not allocated,; Bits; NA; NA; NA; NA; NA. For Slots i in which PDSCH is allocated,; Bits; 600; 1248; 1896; 2544; 3192. NOTE 1: Additional parameters are specified in Table A.8.2-1. NOTE 2: If TB size is larger than 3824, the CRC sequence of L = 24 Bits is attached to each Code Block. NOTE 3: SS/PBCH block is transmitted in slot 0 and 20 of each period of 40 slots NOTE 4: Slot i is slot index per four frames.. - Option 3: Bitmap (0000000000, 1111111111, 0000000000, 1111111111) 0 as FDD carrier, 1 as SDL carrier 1 switch within 10 slots UL TA assume as 0 us for RMC definition and test. - Option 4: other options are not precluded Issue 2: Uplink reference measurement channels Agreement: - RAN4 can further discuss whether an UL RMC is needed for this feature Issue 3: Reference measurement channels: general aspects Agreement: - In RMC, NTA=0 is considered. Issue 4: UE capability update Agreement: - The following FG description is agreed: Issue 5: Whether a liaison to RAN2 is useful to clarify the impact of low band switching on BandCombinationList Agreement: - Send a liaison to RAN2 to clarify the impact of low band switching on BandCombinationList

Liaison to other working groups

-	A liaison to RAN2 was approved in [96] with the following content:

# 1 Overall description RAN4 has discussed the impact of the low NR band carrier aggregation via switching feature on BandCombinationList and would like to share the following information for RAN2 consideration: - There are CA band combinations (BCs) in TS38.101-1, but there are no legacy UEs supporting those in the market, e.g., CA_n5-n29 and CA_n29-n71. - There are, however, low band CA switching to CA BCs whose traditional CA are already in the market. And, there are UEs supporting only low band CA switching, only traditional CA BCs, or both low band CA switching and traditional CA BCs for the said CA BCs. - Further, when a UE supports both low band CA switching and traditional CA, at least following can be different. - The UE can have different maximum MIMO layers per CC depending on low band CA switching or traditional CA - When specifying the UE requirements, RAN4 has assumed that the network supporting any release of the NR specification should be able to distinguish these UEs # 2 Actions To RAN2 ACTION: RAN4 respectfully asks RAN2 to consider the above RAN4 considerations in its work.

RAN4 #116 – RRM (Core)

General aspects

-	The topic summary was prepared in [93]

-	Adhoc minutes are captured in [99]

RRM aspects

-	The draft CRs introducing the feature to the RRM specification in TS 38.133 were endorsed in [104], [105], [106], [107], [108], [109], [110], and [111]

-	The endorsed draft CRs are merged into a single CR in [113], which is undergoing the RAN4 post-meeting email approval process

-	The text proposal to TR 38.768 on RRM aspects was approved in [103] and is incorporated into the v1.1.0 update of the TR

-	The following agreements were captured in [100]:

# Topic #1: Core part for R19 LB CA via switching (7.3.4) ### Issue 1-1: General Issue 1-1-1: RRM requirement applicability for R19 LB CA via switching - Agreement in online session: - for gap-less based L3 measurement and L1 measurement of a serving carrier which can be either FDD PCC or activated SDL SCC: - a. If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed. - b. If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions. - c. If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply. Issue 1-1-2: switching pattern applicability for dormant SCell - Proposal 1 (vivo): Switching pattern is not applied for dormant SCell. - [Agreement]: - No requirement is defined for dormant SCell in LB-CA via switching. ### Issue 1-2: Interruption or scheduling restriction requirement Issue 1-2-1: Necessity of interruption requirement in RRM between FDD CC and SDL CC Agreement: RAN4 to confirm the agreement from the last RAN4 meeting - For SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier. Issue 1-2-2: Necessity of interruption requirement in RRM when more carriers are considered Agreement: - According to the WID, RANA only defines requirements for CA with FDD PCC and one SDL SCC. Issue 1-2-3: Interruption or scheduling restriction requirement (if needed) for carrier switching between FDD CC and SDL CC Agreement: - This issue is closed. ### Issue 1-3: SDL SCell related requirement #### Activated SDL SCell L3 measurement Issue 1-3-1: Requirement design of activated SDL SCell L3 measurement Agreement in online session: - if the activated SCC measurement is MG-based in LB-CA, the legacy intra-frequency identification/measurement requirement with MG can be applied. - reuse the existing measurement requirements with gap for inter-frequency measurement and inter-RAT measurement in LB-CA. - Not consider the scenario and not define the requirement for measurement without gap for inter-frequency measurement and inter-RAT measurement in LB-CA. Agreement in online session: agree on the following approach, and further update on the wording is not precluded. Further discuss on the CR drafting. - if the activated SCC measurement is MG-less in LB-CA, Kp factor can be updated for activated SCC identification/measurement requirement (intra-frequency measurement without MG) for LB-CA. - Kp factor for MG-less activated SCC identification/measurement requirement in LB-CA can be updated as following: - Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows: - For a window W of duration max(SMTC period, xRP_max, switch pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and switch pattern periodicity is the periodicity of the NW configured switch pattern for LB CA via switching, and starting from the beginning of any SMTC occasion: - Ntotal is the total number of SMTC occasions within the window, including those overlapped with MG and those overlapped with FDD PCC duration based on the switching pattern within the window, and - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions and not overlapped with FDD PCC duration based on the switching pattern within the window W. - No requirement when Navailable = 0. #### Deactivated SDL SCell L3 measurement Issue 1-3-2: Interruption requirement design of deactivated SDL SCell measurement [Agreement]: - For R19 LB CA via switching, interruption length on PCC due to measurements on deactivated SDL is (tuning time in slot level + TSMTC_duation + retuning time in slot level), - Tuning time from PCell to SCell is equal to the RRC configured switching gap from PCell to SCell LBCA-SwitchingGap-Duration-PCelltoSCell-r19 - Returning time from SCell to PCell is equal to the RRC configured switching gap from SCell to PCell, i.e., LBCA-SwitchingGap- SCelltoPCell-r19 - Missed ACK/NACK rate For R19 LB CA via switching, missed ACK/NACK rate due to measurements on deactivated SDL are allowed with up to X= YmeasCycleSCell, where Y=2* interruption length. #### SDL SCell activation/deactivation Issue 1-3-3: interruption or scheduling restriction during SDL SCell activation/deactivation [Agreement]: No requirement for interruption or scheduling restriction during SDL SCell activation/deactivation. This issue is closed. Issue 1-3-4: CQI/L1-RSRP reporting during SDL SCell activation in LB-CA - [Agreement]: - FFS this issue in maintenance stage. Issue 1-3-5: Activation of Switching Pattern [Agreement]: - Revise the previous agreement in R4-2504895 issue 1-1-3 (RAN4 #114bis) for the SCell which is in activation procedure: - For the SCell which is in activation procedure, - The RRC configured switching pattern for SDL SCell is applied. - For MAC CE based SCell activation, UE shall apply the RRC configured switching pattern at slot n+(THARQ+3ms+TLBCA)/slot-length - For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - In maintenance stage, RAN4 to FFS how UE performs switching according to the following two options. - For MAC CE based SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(THARQ+3ms+ TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(THARQ+3ms+ TLBCA)/slot-length. - Other options are not precluded. - For RRC based direct SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(TRRC_Process + TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(TRRC_Process + TLBCA)/slot-length. - Other options are not precluded. Note: if the timeline for application and for switching has ambiguity, RAN4 may revise the above application timeline. Issue 1-3-5a: Deactivation of Switching Pattern [Agreement]: - The switching pattern is NOT applied after UE completed the SCell deactivation (n+(THARQ+3ms+ TLBCA)/slot-length). - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - FFS in maintenance stage: If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately. The carrier switching, if applicable, shall be placed at the end of a slot. #### SDL SCell L1 requirement Issue 1-3-6: BFD/CBD requirements for SDL SCell - Proposal 1 (CATT): The BFD/CBD measurement requirement for SDL SCell in LB CA via switching can be defined based on the legacy requirement with the number of measurement sample being updated. The new measurement sample equals to the legacy value plus X, where X is the number of SSB/CSI-RS occasions that are not covered by SDL duration in switch pattern. - Proposal 2 (Apple, HW): - follow the principle in issue 1-1-1 to define the BFD/CBD requirement for SDL SCell. - New P_LB factor is used to replace the P factor in the existing BFD/CBD requirement, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P_LB= Ntotal_LB/ Navailable_LB - Ntotal_LB is total number of BFD-RS or CBD-RS in W window. - Navailable_LB is number of available BFD-RS or CBD-RS in W window. - Proposal 3 (LGE): RAN4 to consider not to apply switching pattern until link recovery is completed after beam failure is declared for SDL SCell, and allow scheduling restriction/interruption for CBD evaluation after beam failure is triggered. - Proposal 4 (vivo, MTK): L1 measurement requirements for BFD/CBD are not defined for SDL SCell via switching. - Proposal 5 (Ericsson): For L1 measurements, RAN4 to define new sharing factor, PLB to account for the non-availability of SSBs due to the switch pattern and SSB overlap with MG. [Agreement]: - RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching - follow the principle in issue 1-1-1 to define the BFD/CBD requirement for SDL SCell. - P factor in the existing BFD/CBD requirement can be updated for LB-CA via switching, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P = Ntotal/ Navailable - Ntotal is total number of BFD-RS or CBD-RS in W window. - Navailable is number of available BFD-RS or CBD-RS in W window. - No requirement when Navailable = 0. - FFS in maintenance stage for P3. Issue 1-3-7: TCI state switch delay requirement for SDL SCell [Agreement]: - RAN4 to define the TCI state switch delay requirement for LB-CA via switching - In TCI state switch delay requirement for SDL SCell, Tfirst-SSB_LB is time to first available SSB reception based on switching pattern after MAC CE command is decoded by the UE. Issue 1-3-8: L1-RSRP/SINR measurement requirements for SDL SCell [Agreement]: - follow the principle in issue 1-1-1 to define the L1-RSRP/SINR requirement for SDL SCell. - P factor in the existing L1-RSRP/SINR measurement requirements can be updated for LB-CA via switching, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P = Ntotal / Navailable - Ntotal is total number of L1-RSRP/SINR RS of SCell in W window. - Navailable is number of available L1-RSRP/SINR RS overlapping with SCell duration according to the RRC configured switching pattern in W window. - No requirement when Navailable = 0. #### Others for SDL SCell Issue 1-3-9: BWP switching for SDL SCell - Proposal 1 (Ofinno): - Clarify the UE behavior by adding the following text in clause 8.6.2: - If the UE is configured with LBCA-SwitchingPattern then the first DL or UL slot refers to the first DL or UL slot, which: - is available after the completion of the BWP switching and - is assigned to the same serving cell on which the active BWP switching occurs. - Clarify the UE behavior by adding the following text in clause 8.6.3: - If the UE is configured with LBCA-SwitchingPattern then the first DL or UL slot in the RRC based BWP switching delay requirements defined in this clause refers to the first DL or UL slot, which is available after the completion of the BWP switching and is assigned to the FDD PCell. [Agreement]: FFS this issue in maintenance stage. ### Issue 1-4: Impact on FDD PCell RRM Issue 1-4-1: RLM requirement for PCell [Agreement]: - For RLM in LB-CA, follow principle in issue 1-1-1 to define RLM requirements for PCell in LB CA. - P factor in the existing RLM measurement requirements can be updated, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P = Ntotal/ Navailable - Ntotal is total number of RLM-RS in W window. - Navailable is number of available RLM-RS in W window. - No requirement when Navailable = 0. - FFS in maintenance stage: RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritize to receive RS for faster link recovery. Principle of scheduling restriction or interruption requirement agreed for the SCell activation to be reused for the link recovery procedure. Issue 1-4-2: others RRM requirement of FDD PCell in LB CA via switching [Agreement]: - The principle of agreed activated SDL SCell requirement in LB-CA can be reused for the corresponding PCell requirement in LB-CA. # Topic #2: Performance part for R19 LB CA via switching (7.3.5) ### Issue 2-1: General Issue 2-1-1: Accuracy requirement impact of LB-CA [Agreement]: Existing measurement accuracy requirements apply for LBCA operation through switching. Issue 2-1-2: Work plan for performance part (TC design) for LB-CA via switching [Agreement]: - Work plan in R4-2509990 is agreed.

#### 2.4.2	Remaining Open issues

Core part:

None

Performance part:

-	Specify the testing parameters in the reference measurement channel definition [RAN4]

-	Specify RRM test cases [RAN4]

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

RAN1 #122

- R1-2505230	Huawei, HiSilicon	Maintenance of Rel-19 low band CA via switching

- R1-2505249	Spreadtrum, UNISOC	Remaining issues on low band carrier aggregation via switching

- R1-2505393	vivo	Maintenance on Low band carrier aggregation via switching

- R1-2505441	Xiaomi	Remaining issues on low band CA via switching

- R1-2505558	Samsung	Remaining issues on low band carrier aggregation via switching

- R1-2505614	Ericsson	Maintenance on Low band carrier aggregation through switching

- R1-2505666	Ofinno	Discussion on low-band CA with switching

- R1-2505726	OPPO	Maintenance on low-band CA via switching

- R1-2505832	Nokia	Maintenance of low-band CA via switching

- R1-2505849	LG Electronics	Remaining issues on low band CA operation via switching for Rel-19

- R1-2505891	Moderator (Apple)	FL summary #1 of Low band carrier aggregation via switching

- R1-2505892	Moderator (Apple)	FL summary #2 of Low band carrier aggregation via switching

- R1-2506032	MediaTek Inc.	Remaining issues on low band carrier aggregation via switching

- R1-2506105	Google	On remaining issues for Low band Carrier Aggregation via switching

- R1-2506127	ZTE Corporation, Sanechips	Maintenance on Low band carrier aggregation via switching

- R1-2506193	Qualcomm Incorporated	Low band carrier aggregation via switching

- R1-2506283	NTT DOCOMO, INC.	Maintenance on Low band carrier aggregation via switching

- R1-2506514	Moderator (Apple)	FL summary round 1 of Low band carrier aggregation via switching

- R1-2505198	Nokia	UE features for Low Band Carrier Aggregation via Switching

- R1-2505350	Huawei, HiSilicon	UE features for low band CA via switching

- R1-2505404	vivo	Discussion on UE features for Low band carrier aggregation via switching

- R1-2505451	Xiaomi	Discussion on UE feature for low band CA via switching

- R1-2505571	Samsung	UE features for low band CA via switching

- R1-2505691	Ericsson	UE features for low band carrier aggregation via switching

- R1-2505727	OPPO	Discussion on UE feature for low-band CA via switching

- R1-2505901	Apple	Views on UE features for Rel-10 LB CA via switching

- R1-2506129	ZTE Corporation, Sanechips	Discussion on UE features for low band CA via switching

- R1-2506293	NTT DOCOMO, INC.	Discussion on UE features for Low band carrier aggregation via switching

- R1-2506422	Moderator (NTT DOCOMO, INC.)	Summary of discussion on Rel-19 NR UE features for low band CA via switching

- R1-2506423	Ad-Hoc Chair (NTT DOCOMO, INC.)	Session Notes of AI 9.12

RAN2 #131

- R2-2505487	Apple, Telus, Nokia	Introduction of low NR band carrier aggregation via switching

- R2-2505488	Apple, Telus, Nokia (Rapporteur)	Introduction of low NR band carrier aggregation via switching

- R2-2506249	Apple, Telus, Nokia	Introduction of low NR band carrier aggregation via switching

- R2-2506250	Apple, Telus, Nokia (Rapporteur)	"Introduction of low NR band carrier aggregation via switching

- R2-2506502	Apple, Telus, Nokia	"Introduction of low NR band carrier aggregation via switching

RAN4 #116

- R4-2509249	CATT	draft CR for TS 38.101-1 to introduce LB-LB CA via switching

- R4-2509965	Apple	TP to TR38.768 on low NR band aggregation via switching

- R4-2511766	Apple, Samsung, LG Electronics, Nokia, Huawei	TP to TR38.768 on low NR band aggregation via switching

- R4-2511901	Apple, Samsung, LG Electronics, Nokia, Huawei	TP to TR38.768 on low NR band aggregation via switching

- R4-2509246	CATT	Discussion on LB-LB CA via switching

- R4-2509247	CATT	Replied LS on Low NR band carrier aggregation via switching

- R4-2509569	Apple, Telus	(NR_LBCA_Sw-Core) Introducing low NR band aggregation via switching to TS38.101-1

- R4-2509821	Xiaomi	View on LB CA with switching

- R4-2510012	LG Electronics	Discussion on low band carrier aggregation via switching

- R4-2510057	LG Electronics UK	LS for parameter update request with switching gap

- R4-2510058	LG Electronics UK	Draft CR for updating MRTD

- R4-2510272	vivo	Discussion of UE RF impact for LB-LB switching

- R4-2510316	Ericsson	Remaining RF requirements for low-band CA via switching

- R4-2510317	Ericsson	Draft CR for 38.101.1: time masks for DL-only CA via switching

- R4-2510322	Spreadtrum, UNISOC	Remaining issues on UE RF requirements for LB-LB CA switching

- R4-2510465	Huawei, HiSilicon	Discussion on LB-LB switching

- R4-2510466	Huawei, HiSilicon	draftCR to 38.101-1: Introduction for LB-LB CA via switching

- R4-2510968	Qualcomm Incorporated	LB CA Switching

- R4-2511066	MediaTek Inc.	draftCR to 38.101-1: Time mask for LB-LB switching

- R4-2511072	Samsung	Views on remaining issues on low band carrier aggregation via switching

- R4-2511073	Samsung	TP for 38.768 on switching period

- R4-2511228	Nokia	Considerations for UL time mask for NR_LBCA_Sw

- R4-2511767	Apple, TELUS, AT&T, CATT, Samsung, Xiaomi, Murata, Huawei, MediaTek	(NR_LBCA_Sw-Core) Introducing low NR band aggregation via switching to TS38.101-1

- R4-2509315	CATT	Discussion on the impacts of low band CA via switching on RRM core requirements

- R4-2509497	Apple	On RRM requirement for R19 LB CA

- R4-2509498	Apple	TP to TR38.768 on RRM requirements for low band aggregation via switching

- R4-2509499	Apple	Big CR for R19 NR_LBCA_Sw RRM

- R4-2509632	LG Electronics	Discussion on RRM requirements for Low Band CA operation

- R4-2509633	LG Electronics Inc.	Draft CR on active TCI state switch delay requirements for Low band CA via switching

- R4-2509685	OPPO	Discussion on RRM requirements for low band carrier aggregation via switching

- R4-2509686	OPPO	Draft CR on activated and deactivated SDL SCell measurement delay requirements

- R4-2509801	Xiaomi	Discussion on LB-CA RRM requirements

- R4-2509802	Xiaomi	draftCR on Rel19 LBCA (Interruption for deactivated SDL SCell measurement)

- R4-2509989	Ofinno	Analysis of RRM Requirements for Low Band CA via Switching

- R4-2510323	vivo	Further discussion on RRM requirements for LB CA via switching

- R4-2510581	Huawei, HiSilicon	Discussion on low band carrier aggregation via switching

- R4-2510582	Huawei, HiSilicon	DraftCR on BFD/CBD requirements for SDL SCell and PCell for low band carrier aggregation via switching

- R4-2510687	ZTE Corporation, Sanechips	Discussion on RRM aspects of R19 Low band carrier aggregation

- R4-2510695	Nokia	Discussion on LBCA requirements

- R4-2510696	Nokia	DraftCR for LBCA SCell activation

- R4-2510728	ZTE Corporation, Sanechips	Draft CR on PL-RS switching delay for R19 LB-CA

- R4-2510895	Ericsson	RRM requirements for low band carrier aggregation via switching

- R4-2510896	Ericsson	Draft CR on L3 measurements and RLM for UE supporting LB CA via switching

- R4-2511085	Samsung	Discussion on RRM requirements for LB CA switching

- R4-2511087	Samsung	Draft CR on L1-RSRP/SINR measurement requirement for SDL SCell

- R4-2511211	MediaTek inc.	Discussion on low-band CA switching

- R4-2511572	Qualcomm Incorporated	RRM requirements for low band carrier aggregation via switching

- R4-2512243	OPPO	Draft CR on activated and deactivated SDL SCell measurement delay requirements

- R4-2509316	CATT	Discussion on the impacts of low band CA via switching on RRM performance requirements

- R4-2509687	OPPO	Discussion on RRM performance requirements for low band carrier aggregation via switching

- R4-2509990	Apple, TELUS	Work plan of performance part for R19 NR_LBCA_Sw RRM

- R4-2510324	vivo	On RRM performance requirements for LB CA via switching

- R4-2510583	Huawei, HiSilicon	Discussion on performance requirements for low band carrier aggregation via switching

- R4-2510691	ZTE Corporation, Sanechips	Discussion on Perf part of R19 SBFD

- R4-2510697	Nokia	Discussion on LBCA test cases

- R4-2510897	Ericsson	Performance requirements for low-band carrier aggregation via switching

- R4-2511086	Samsung	Discussion on RRM performance requirement for LB CA

- R4-2509049	Moderator (Apple)	Topic summary for [116][206] NR_LBCA_Sw

- R4-2509093	Moderator (Apple)	Topic summary for [116][126] NR_LBCA_Sw

- R4-2511862	Apple	WF on UE RF requirements for LB CA via switching

- R4-2511863	Huawei	"[draft] LS on capability of NR_LBCA_Sw

- R4-2511894	Apple, TELUS, AT&T, CATT, Samsung, Xiaomi, Murata, Huawei, MediaTek	"(NR_LBCA_Sw-Core) Introducing low NR band aggregation via switching to TS38.101-1

- R4-2511905	Apple, Samsung, LG Electronics, Huawei	TP to TR38.768 on low NR band aggregation via switching

- R4-2512118	Moderator (Apple)	"AdHoc minutes for [116][206] NR_LBCA_Sw

- R4-2512119	Moderator (Apple)	WF on RRM requirements for NR_LBCA_Sw

- R4-2509248	CATT	TP for TR 38.768 to modify the general description

- R4-2510467	Huawei, HiSilicon	TP to TR38.768 Switching time mask

- R4-2512241	Apple	TP to TR38.768 on RRM requirements for low band aggregation via switching

- R4-2512242	LG Electronics Inc.	"Draft CR on active TCI state switch delay requirements for Low band CA via switching

- R4-2512244	Xiaomi	"draftCR on Rel19 LBCA (Interruption for deactivated SDL SCell measurement)

- R4-2512245	Huawei, HiSilicon	"DraftCR on BFD/CBD requirements for SDL SCell and PCell for low band carrier aggregation via switching

- R4-2512246	Nokia	"DraftCR for LBCA SCell activation

- R4-2512247	ZTE Corporation, Sanechips	"Draft CR on PL-RS switching delay for R19 LB-CA

- R4-2512248	Ericsson	"Draft CR on RLM for UE supporting LB CA via switching

- R4-2512249	Samsung	"Draft CR on L1-RSRP/SINR measurement requirement for SDL SCell

- R4-2512324	OPPO	"Draft CR on activated and deactivated SDL SCell measurement delay requirements

- R4-2509202	Apple	Draft TR 38.768 v1.1.0 Low NR Band Carrier Aggregation via Switching

- R4-2509499	Apple	Big CR for R19 NR_LBCA_Sw RRM

- R4-2511883	CMCC	Rel-19 RAN4 UE feature list for NR (version 2)

04.02.2025		minor adaptations for RAN #107

17.10.2024		minor adaptations for RAN #106 and 2.6 for RAN6 removed

12.08.2024		minor adaptations for RAN #105

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template