3GPP TSG RAN meeting #109			RP-252906

Beijing, China, September 15-18, 2025

Title:	Moderator's summary for other RAN4 REL-20 5G-Adv topics

Source:	Huawei

Agenda:	9.1.4.7

Document for:	Report

Introduction

This document summarizes the companies’ proposals of other topics for RAN4 Rel-20 5G-Adavanced and recommend the candidate WIDs as well as objectives based on companies’ proposals and comments for approval in RAN#109.

Cross RF and baseband topics

Endorsed way forward in RAN#108

The following is the endorsed way forward for other topics in RAN#108 meeting [1].

The following two WI are agreed to be introduced in R20. - Extension of LBCA (the exact scope and WID draft will be discussed and decided in RAN#109): - Further discussion on the possible scoping in RAN#109 based on the following two objectives - Study and if feasible, specify non-RRC based switching solution, e.g. dynamic switching, by taking the RAN1 impacts into consideration - Dual DL solution - RAN1/2 impact is expected and will be further discussed in RAN#109 - Target 0.5 TU (0.25 RF and 0.25 RRM) in Q4’25 and 0.75 TU allocation (0.25 RF and 0.5 RRM) from Q1’26 - UAV (the exact scope and WID draft will be discussed and decided in RAN#109): - UAV UE with antenna array with up to 33dBm maximum output power on TDD spectrum - RAN1/2 scope is not expected - Target 0.5 TU allocation (0.25 RF and 0.25 RRM) The following two proposals will be discussed at RAN#109, where it will be decided whether or not to include either of the proposals - Fragmented DL: - Introducing RF/RRM requirements related to new UE behavior for reducing the number of UE Rx RF chains - RAN1/2 scope is TBC - Intra-band non-collocated EN-DC/NR-CA deployment - Type 3 UE requirements assuming MRTD<CP and power imbalance < [xx] dB for handheld - RAN1/2 scope is TBC

General discussions of candidate objectives for other topics for 5G-A Rel-20

The proposals from companies are summarized in Section 4, which is briefly highlighted below:

- Extension of low band CA

  - Support: Samsung (2072), Vivo (2193), Qualcomm (2489), Apple (2841), MediaTek (2516), LGE (2527), Ericsson (2574), Sony (2585), TELUS et al (2799), CHTTL (2726), Intel (2752)

- UAV

  - Support: ZTE (2039), Vivo (2193), Qualcomm (2489), Ericsson (2574), Sony (2585), CMCC et al (2600), Intel (2752)

- Fragmented DL

  - Support: Apple (2841), MediaTek (2516), LGE (2527), TELUS et al (2699), CATT (2716), Intel (2752), CHTTL (2726)

  - De-prioritize: Samsung (2072), Qualcomm (2489), Ericsson (2574)

- Intra-band non-collocated EN-DC/NR-CA deployment

  - Support: KDDI et al (2528)

  - De-prioritize: Samsung (2072), Qualcomm (2489), Ericsson (2574)

Considering the available RAN4 TU for 5G-A in Rel-20, companies’ views on which topics to be included in the package, and the impacts on RAN1/RAN2, RAN has to down-select the topics for cross area in 5G-A.

Following the previous agreements in RAN#108 and companies’ comments, the moderator suggests to focus on the following two items.

- Extension of low band CA

- UAV

Discussions:

Mediatek: we do not want to only consider LBCA and UAV. A lot of operators support fragmented DL. We do not allow the PCell degradation. We can reuse the existing measurement method, which lead to no RAN1/RAN2 impact.

Nokia: We only have a limited TU. We have already down-selected to two. There is strong preference from RAN chair to focus on LBCA and UAV. There is no need to spend time for additional topic.

Apple: Given the limited capacity, we need make difficult choice. Telus cannot attend the meeting. They do see the business need. What MediaTek proposed to further reduce is a balanced approach. We understand that we have to prioritize.

Ericsson: the SI did not identify the simple solution. It cannot help operator need. We do not have more time.

AT&T: I would like to emphasize that LBCA is in high priority.

Samsung: Like Ericsson said we had a lot of study for fragmented DL including the fallback. There is no easy solution. We see the potential more study to be needed even if we have WI. It is not an easy job. LBCA and fragmented come from the same group of operators. We need see the priority.

Qualcomm: Agree with Ericsson and Samsung. We need focus on the LBCA and UAC.

LGE: The request of operator is clear. Considering RAN1 and RAN2 has limited time, we could set up the checking point for it.

Xiaomi: Support the proposal here. LBCA is our first priority. We need further discuss the solutions including fallback. It is a big work and we can work in 6G.

Mediatek: Capture it in the moderator that fragmented DL can be captured for 6G study.

Companies’ comments:

| Companies | Comments |
| --- | --- |

Extension of low band CA

Based on companies’ contributions, the moderator suggests the following justification and objectives for discussions.

Justifications

The following text is proposed for discussions:

---------------------------------------- Justifications for enhancement of co-location requirements ----------------------------

In Rel-19, 3GPP approved new WI (NR_LBCA_Sw) to enable low band-low band CA via switching. TR 38.768 lists the operator-requested scenarios (CA_n5A-n29A, CA_n12A-n29A, CA_n14A-n29A, CA_n28A-n71A) and the relevant technical background. Semi-static RRC-based switching solution was specified for FDD + SDL low-band carrier aggregation in Rel-19.

Based on Rel-19 discussions, the enhancement of low band-low band CA via switching for the case of switching between FDD and SDL bands with enabling dual downlink is beneficial to improve downlink performance for low band CA combinations.

---------------------------------------- Justifications for enhancement of co-location requirements ----------------------------

Companies’ comments:

| Companies | Comments |
| --- | --- |

Moderator recommendation:

Objectives

According to moderator understanding, there would mainly be a number of issues which need be discussed for the objectives. Firstly, most contributions preferred to limit to semi-static switching scheme, while other companies proposed the dynamic switching scheme (e.g., DCI based switching). Secondly, two companies proposed more scenarios than the switching for FDD+SDL dual DL, while more companies proposed to focus on the FDD+SDL dual DL scenario.

Based on companies’ contributions and considering the guidance to minimize the RAN1/RAN2 impact, the moderator recommends to focus on: 1) semi-static switching first; 2) FDD+SDL dual DL scenario.

The following text is proposed for discussions:

---------------------------------------- Objectives for enhancement of co-location requirements ----------------------------

Core requirement:

- Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 2} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4].

    - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

    - Case 2: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

    - Example band combinations: [CA_n12A-n29A and CA_n28-n67], additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed.

    - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible

- Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching

    - If the clarification is needed, make the necessary modification by sending LS to RAN1/RAN2

- Define the corresponding UE capabilities [RAN4, RAN2].

- Consider the following deployment constraints:

  - The carrier frequency for all cases is <1GHz.

  - Co-located and synchronized network deployment for both carriers.

  - Both carriers are in a single TAG.

  - SCS 15Khz on both carriers.

  - No in-gap interference between DL carriers is expected

Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible .

[Note 2: Further consider in RAN#112 (June, 2026) with clear and compact RAN1/2 scope, if RAN1/2 can accommodate additional dual DL CA non-semi-static scheduling.]

[Note 3: Further consider in RAN#112 (June, 2026) with clear and compact RAN1/2 scope, if RAN1/2 can accommodate the scenario of switching between FDD band and FDD band.]

Performance part:

- Specify RRM test case(s) corresponding to the RRM core requirements.

---------------------------------------- Objectives for enhancement of co-location requirements ----------------------------

Companies’ comments:

| Companies | Comments |
| --- | --- |

Moderator recommendation:

- The following objectives are recommended for LBCA

Core requirement: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 2} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 2: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: [CA_n12A-n29A and CA_n28-n67], additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - If the clarification is needed, make the necessary modification by sending LS to RAN1/RAN2 - Define the corresponding UE capabilities [RAN4, RAN2]. - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible . [Note 2: Further consider in RAN#112 (June, 2026) with clear and compact RAN1/2 scope, if RAN1/2 can accommodate additional dual DL CA non-semi-static scheduling.] [Note 3: Further consider in RAN#112 (June, 2026) with clear and compact RAN1/2 scope, if RAN1/2 can accommodate the scenario of switching between FDD band and FDD band.] Performance part: - Specify RRM test case(s) corresponding to the RRM core requirements.

After the online discussions on Wednesday morning, the following objectives are agreed.

Agreement online:

Core requirement: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 2} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 2: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: [CA_n12A-n29A and CA_n28-n67], additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - If the clarification is needed, make the necessary modification by sending LS to RAN1/RAN2 - Define the corresponding UE capabilities [RAN4, RAN2]. - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible. Performance part: Specify RRM test case(s) corresponding to the RRM core requirements.

UAV

Based on companies’ contributions, the moderator suggests the following justification and objectives for discussions.

Justifications

The following text is proposed for discussions:

---------------------------------------- Justifications for enhancement of co-location requirements ----------------------------

In recent years, the low altitude economy has been burgeoning with remarkable momentum, emerging as a new economic growth engine and attracting widespread attention. Many cities are planning UAV routes for technical trial.

The main application scenarios of UAV include low altitude logistics, travel, tourism, control and supervision, medical rescue, etc. so UL throughput and UL coverage is the key target compared with DL. To reduce the network construction costs, the transmission power of the UAVs should be further increased to enhance the UL coverage for UAV. In China, the upper power limit of ATG UE is 33dBm for trial as issued by MIIT. Besides, to avoid severe interference to ground networks due to the increased uplink power limit, beamforming or directional antenna should be supported.

In Rel-18 NR_UAV WID, directional antenna at UE side has been discussed but without conclusion and Rel-18 NR_UAV UE only support 23dBm without simulations. In NR ATG WID, two ATG UE antenna types are introduced considering the implementation freedom, one is omni-directional antenna and the other is antenna array. Besides, ATG UE could support up to 40dBm rated maximum output power from 23dBm with 1dB granularity when considering the varying in different aircraft types and also in different frequency ranges.

Since existing technologies cannot meet the target use cases of UAV, a new WID is recommended to expand UAV market with higher power and beamforming within 3GPP systems, which will enlarge UL coverage and reduce interference to ground networks.

---------------------------------------- Justifications for enhancement of co-location requirements ----------------------------

Companies’ comments:

| Companies | Comments |
| --- | --- |

Moderator recommendation:

Objectives

Based on companies’ proposals, the following text is proposed for discussions:

---------------------------------------- Objectives for enhancement of co-location requirements ----------------------------

Core part objective

- Scenario:

  - BSs for UAV service are deployed on the ground by adjusting the tilt angle upwards. The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km.

  - A direct radio link between BS on the ground and UAV UE.

- Specify RF requirements for UAV UE to support higher UL output power.

  - Example bands:

    - TDD with same TDD pattern, including n41, [n77,] n78, n79,

    - FDD: n3

  - Consider UAV UE with omni-direction antenna or with antenna array

  - Consider UAV UE with up to 33dBm max total conductive output power

  - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed.

  - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM.

- Specify RRM requirements, if needed

Note 1: No RAN1 impact are expected

Performance part:

- Specify the RRM test cases corresponding to the RRM core requirements.

---------------------------------------- Objectives for enhancement of co-location requirements ----------------------------

Companies’ comments:

| Companies | Comments |
| --- | --- |

Moderator recommendation:

- The following objectives are recommended for LBCA

Core part objective - Scenario: - BSs for UAV service are deployed on the ground by adjusting the tilt angle upwards. The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km. - A direct radio link between BS on the ground and UAV UE. - Specify RF requirements for UAV UE to support higher UL output power. - Example bands: - TDD with same TDD pattern, including n41, [n77,] n78, n79, - FDD: n3 - Consider UAV UE with omni-direction antenna or with antenna array - Consider UAV UE with up to 33dBm max total conductive output power - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed. - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM. - Specify RRM requirements, if needed Note 1: No RAN1 impact are expected Performance part: - Specify the RRM test cases corresponding to the RRM core requirements.

After the online discussions on Wednesday morning, the following objectives are agreed.

The following notes were not agreed as part of the WID, but they can be captured in the moderator summary:

Note 2: Further consider in RAN#112 (June, 2026) with clear and compact RAN1/2 scope, if RAN1/2 can accommodate additional dual DL CA non-semi-static scheduling.

Note 3: Further consider in RAN#112 (June, 2026) with clear and compact RAN1/2 scope, if RAN1/2 can accommodate the scenario of switching between FDD band and FDD band.

Agreement online:

Core part objective - Scenario: - BSs for UAV service are deployed on the ground by adjusting the tilt angle upwards. The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km. - A direct radio link between BS on the ground and UAV UE. - Specify RF requirements for UAV UE to support higher UL output power. - Example bands: - TDD with same TDD pattern, including n41, n77, n78, n79, - FDD: n3 - Consider UAV UE with omni-direction antenna or with antenna array - Consider UAV UE with up to 33dBm max total conductive output power - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed. - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM. - Specify RRM requirements, if needed Note 1: No RAN1 impact are expected Performance part: - Specify the RRM test cases corresponding to the RRM core requirements.

Fragmented DL

Based on companies’ contributions, the moderator suggests the following justification and objectives for discussions. The discussions may not be needed if no consensus was reached for this item.

Justifications

The following text is proposed for discussions:

---------------------------------------- Justifications for enhancement of co-location requirements ----------------------------

DL fragmented carriers containing more than one non-contiguous CC in the same band consuming multiple Rx RF chains. It may limit the maximum supported number of CCs due to the limited number of the Rx RF chains as well as the baseband processor capabilities in the UEs. And this limitation prevents operators from fully utilizing their fragmented spectrum holdings.

The SI [RP-243002] was approved in Rel-19 to investigate solutions with the focus of single DL band with 2CC NCCA within frequency span ≤ 100 MHz, under the assumption of inter-operator co-located scenario. The scope includes the impacted requirements such as ΔRIBNC, ACS, in-band blocking, and the potential new mechanism for a UE to inform network on the need to enable semi-statical switching of UE hardware resources.

Conclusion of the study item is that

- There are potential benefits when using fewer Rx chains than number of component carriers

- It is feasible under certain conditions for a NR UE to operate one Rx RF chain to receive two DL intra-band non-contiguous carriers.

Following the outcome of the SI, it is important to start the nominal work item phase in Rel-20 to really resolve the concern of global operators on fragmented carrier.

---------------------------------------- Justifications for enhancement of co-location requirements ----------------------------

Companies’ comments:

| Companies | Comments |
| --- | --- |

Moderator recommendation:

Objectives

Based on companies’ proposals, the following text is proposed for discussions:

---------------------------------------- Objectives for enhancement of co-location requirements ----------------------------

Objectives (core part)

- This work aims at introducing the mechanism and UE requirements to enable DL fragmented carrier with the following assumptions:

  - Single DL band with 2CC co-located NCCA within frequency span ≤ 100 MHz

  - Up to 6 dB DL received PSD imbalance between the two non-contiguous CCs

  - Semi-statically switch hardware resources

- Under co-located inter-operator scenario

  - Specify allowable RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

  - To manage the impact of Pcell performance degradation and minimize RAN4 workload, the feature is enabled only for SCellsfor FDD bands

  - Determine and specify UE behaviour of CA fallback with/without semi-statically architecture switch hardware resources (i.e. Rx RF chains) based on the SI outcome [RAN4];

    - Re-use current fallback procedure to avoid RAN2 impact

Note 1: No RAN1, RAN2 impacts is foreseen

Note 2: The feature is not enabled with UL tx switching simultaneously

Objectives (Perf part)

- Depending on the down-selected approach, specify the corresponding RRM test cases, if needed.

- Specify UE demod requirements to ensure zero impact to performance degradation under concerned scenarios.

---------------------------------------- Objectives for enhancement of co-location requirements ----------------------------

Companies’ comments:

| Companies | Comments |
| --- | --- |

Intra-band non-collocated EN-DC/NR-CA deployment

Based on companies’ contributions, the moderator suggests the following justification and objectives for discussions. The discussions may not be needed if no consensus was reached for this item.

Justifications

The following text is proposed for discussions:

---------------------------------------- Justifications for enhancement of co-location requirements ----------------------------

In the 5G era, the non-collocated scenarios for B42 and n77/n78 as shown in the previous slides still occur in Japanese, Korean and European operators. Tx antenna co-location at network side is not always available since the co-existence conditions are not always same for all the bands.

In Release 18, FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and FR1 intra-band non-contiguous NR-CA for 2-layer case (Type 2 UE for handheld) are  developed.

In Release 19, Intra-band non-contiguous non-collocated EN-DC/NR-CA for 4-layer case (Type 4 UE for FWA) are developed currently.

On the other hand, the similar functionality target for Type 3 UE for handheld was postponed for the future release. From operators’ point of view, it is desirable to standardize some enhancements in Release 20, i.e., Intra-band non-contiguous non-collocated EN-DC/NR-CA with Type 3 UE.

---------------------------------------- Justifications for enhancement of co-location requirements ----------------------------

Companies’ comments:

| Companies | Comments |
| --- | --- |

Moderator recommendation:

Objectives

Based on companies’ proposals, the following text is proposed for discussions:

---------------------------------------- Objectives for enhancement of co-location requirements ----------------------------

The core part of the work item includes:

- Phase I:

  - Study the feasibility to support non-co-located scenarios for both FR1 inter-band EN-DC with overlapping and FR1 intra-band NR-CA for 4-layer case with shared Rx-chains (Type 3 for handheld).

    - Discuss and decide reference UE architecture considering the UE capability of 4-layer MIMO case for both EN-DC/NR-CA

    - Investigate the tolerable [power] imbalance between carriers

    - Investigate the required arrival time difference between CCs

    - Evaluate the UE performance under the [power] imbalance and arrival time difference

    - Work is limited to EN-DC/NR-CA for bands B42, n77/n78

    - Investigate whether the [power] imbalance should be explicitly (e.g. as an RF requirement) or implicitly specified (e.g. through a demodulation performance test). Specify the [power] imbalance based on the outcome of the investigation.

    - If any change in RAN1 or RAN2 spec is needed, it will be triggered by RAN4 LS

- Phase Ⅱ:

  - Phase Ⅱ work will get started after the feasibility in phase Ⅰ is confirmed

  - Specify MRTD and MTTD requirements and other identified RRM requirement impact(s) (if any) in non-collocated deployment

  - Specify additional UE RF requirement(s) except for the [power] imbalance (if any) in non-collocated deployment

The performance part of the work item includes:

- Phase Ⅱ:

  - Phase Ⅱ work will get started after the feasibility in phase Ⅰ is confirmed

  - Specify PDSCH demodulation requirements for non-collocated scenarios for intra-band non-contiguous EN-DC and NR-CA

    - Define PDSCH demodulation performance requirement based on the applicable MRTD and [power] imbalance values.

---------------------------------------- Objectives for enhancement of co-location requirements ----------------------------

Companies’ comments:

| Companies | Comments |
| --- | --- |

Conclusions

This document summarized the contributions and discussions for 5G-A Rel-20 other topics, and based on them the recommendations for candidate WIs as well as objectives are provided.

Online agreements for LBCA

- The following objectives are recommended for LBCA

Core requirement: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 2} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 2: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: [CA_n12A-n29A and CA_n28-n67], additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - If the clarification is needed, make the necessary modification by sending LS to RAN1/RAN2 - Define the corresponding UE capabilities [RAN4, RAN2]. - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible. Performance part: Specify RRM test case(s) corresponding to the RRM core requirements.

The following notes were not agreed as part of the WID, but they can be captured in the moderator summary:

Note 2: Further consider in RAN#112 (June, 2026) with clear and compact RAN1/2 scope, if RAN1/2 can accommodate additional dual DL CA non-semi-static scheduling.

Note 3: Further consider in RAN#112 (June, 2026) with clear and compact RAN1/2 scope, if RAN1/2 can accommodate the scenario of switching between FDD band and FDD band.

Online agreements for UAV

- The following objectives are recommended for UAV

Core part objective - Scenario: - BSs for UAV service are deployed on the ground by adjusting the tilt angle upwards. The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km. - A direct radio link between BS on the ground and UAV UE. - Specify RF requirements for UAV UE to support higher UL output power. - Example bands: - TDD with same TDD pattern, including n41, n77, n78, n79, - FDD: n3 - Consider UAV UE with omni-direction antenna or with antenna array - Consider UAV UE with up to 33dBm max total conductive output power - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed. - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM. - Specify RRM requirements, if needed Note 1: No RAN1 impact are expected Performance part: - Specify the RRM test cases corresponding to the RRM core requirements.

Annex reference contributions

The Tdocs of other topics submitted for RAN4 Rel-20 5G-Advanced in RAN#109 are listed in the table below.

| TDoc | Title | Source | Type | Agenda ittem |
| --- | --- | --- | --- | --- |
| RP-251927 | Moderator's summary for other RAN4 REL-20 5G-Adv topics | Huawei | report | 9.1.4.7 |
| RP-252039 | Views on Rel-20 other topics for 5G-A | ZTE Corporation, Sanechips | discussion | 9.1.4.7 |
| RP-252072 | Views on 5G-A Rel-20 other topics | Samsung | discussion | 9.1.4.7 |
| RP-252193 | Views on Rel-20 cross area topics in 5G-A | vivo | discussion | 9.1.4.7 |
| RP-252443 | Considerations on RAN4-led REL-20 5G-Adv Cross area topics | Nokia | discussion | 9.1.4.7 |
| RP-252489 | Qualcomm views on Other RAN4-led topics | Qualcomm Incorporated | discussion | 9.1.4.7 |
| RP-252498 | Apple’s view on Rel-20 cross-area topics | Apple | discussion | 9.1.4.7 |
| RP-252516 | Rel-20 RAN4 Cross-Area | MediaTek Inc. | discussion | 9.1.4.7 |
| RP-252527 | Views on 5G-A Rel-20 cross-area topics | LG Electronics Deutschland | discussion | 9.1.4.7 |
| RP-252528 | Motivation on supporting Intra-band non-collocated EN-DC/NR-CA deployment in Release 20 (Type 3 UE for handheld) | KDDI, NTT DOCOMO, SoftBank, KT, LG Uplus | WID new | 9.1.4.7 |
| RP-252574 | Views of RAN4 5G-A cross-area topics | Ericsson | discussion | 9.1.4.7 |
| RP-252585 | Sony’s view on RAN4 Rel-20 5G-A | Sony | discussion | 9.1.4.7 |
| RP-252600 | Support of high power UAV UE in Rel-20 | CMCC | discussion | 9.1.4.7 |
| RP-252698 | Low Band CA via Switching Enhancement | Ofinno, LLC | discussion | 9.1.4.7 |
| RP-252699 | Motivation for Fragmented Carrier WI in Rel-20 | TELUS, Bell Mobility, Apple, AT&T, MediaTek, T-Mobile USA, Southern Linc, Telstra, Vodafone | discussion | 9.1.4.7 |
| RP-252702 | Motivation for low-band carrier aggregation via switching enhancement in Rel-20 | TELUS, Bell Mobility, Apple, BT plc, Skyworks Solutions, Inc, Murata, AT&T | discussion | 9.1.4.7 |
| RP-252716 | Views on cross-area works for Rel-20 5G-A | CATT | discussion | 9.1.4.7 |
| RP-252726 | Views on the other Rel.20 RAN4 topics | CHTTL | discussion | 9.1.4.7 |
| RP-252752 | Views on RAN4 Rel-20 5G-A - Other topics | Intel Corporation | discussion | 9.1.4.7 |
| RP-252799 | Motivation for low-band carrier aggregation via switching enhancement in Rel-20 | TELUS, Bell Mobility, Apple, BT plc, Skyworks Solutions, Inc, Murata, AT&T | discussion | 9.1.4.7 |

|  | Tdoc | Proposals |
| --- | --- | --- |
| 1 | ZTE (2039) | Proposal 1: propose to consider the following objective for UAV enhancement in Rel-20 - UAV (the exact scope and WID draft will be discussed and decided in RAN#109): - UAV UE with antenna array with up to 33dBm maximum output power on TDD spectrum - Example bands: n41, n78, n79, other TDD or FDD bands which depends on the operators’ input - Consider the UL MIMO for UAV UE if phase array implementation is considered. - Co-existence study in the uplink direction only if DL is assumed as omini antenna as the same as Rel-18 UAV device assumption - Specify RF requirements including maximum output power, configured transmitted power, SEM; - RAN1/2 scope is not expected - Target 0.5 TU allocation (0.25 RF and 0.25 RRM if necessary) |
| 2 | Samsung (2072) | 5G-A scope on LBCA Proposal 1: For dual DL, overall principle is to reuse Rel-19 LB-CA switching scheme with minimized RAN1 impact, and the expected RAN1 work includes RRC-based semi-static switching pattern, for which RAN1 workload should be limited if Rel-19 semistatic LB-CA switching scheme is reused. Observation 1: If dynamic switching is specified, it will lead to greater diversity in UE implementation schemes, further exacerbating product type fragmentation and ultimately reducing the likelihood of commercialization. Observation 2: Dynamic switching has substantial RAN1 impact relevant aspects such as DCI type/format, DCI size alignment and PDCCH monitoring configuration (if new DCI format is introduced), details of the DCI indication, DCI application delay and impacts on UCI multiplexing, HARQ-ACK design for DCI, DCI missing impact, etc. The detailed analysis on RAN1 impact for dual DL and dynamic switching can be found in the table in the next slide. Proposal 2: The non-RRC based switching solution shall not be included in Rel-20 5G-A scope, unless the minimized RAN1 scope can be clarified and targeted. Core requirement: - Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap, and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN1, RAN4] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary. - Specify time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15Khz on both carriers Note 1: Specify requirements for the feature with the example band combinations: CA_n12A-n29A and CA_n28-n67 in this WI, and additional FDD+SDL band combinations with proximity frequency ranges similarly to CA_n12A-n29A and CA_n28-n67 can be further added. Note 2: Aiming for reusing Rel-19 LB-CA switching mechanism as much as possible with minimum RAN1 impact Performance part: - Specify RRM test case(s) corresponding to the specified RRM requirements. 5G-A scope on UAV Core requirement: - Specify RF requirements for UAV UE to support higher UL output power - Specify RF requirements including maximum output power, configured transmitted power, SEM, etc. - FFS the necessity of additional co-existence simulation is required by taking account multi-cell interference. - UAV UE with antenna array with up to 33dBm maximum output power on TDD spectrum - Example bands: only consider TDD (with same TDD pattern as TN), including n41, n77, n78 and n79 Note 1: only define RF requirements for regions/areas without A-MPR and additional spurious requirement Note 2: Reuse requirements for ATG UE with antenna array as much as possible Proposal 3: For UAV, FFS the necessity of additional co-existence simulation is required by taking account multi-cell interference. 5G-A Scope on other potential scope Proposal 4: Preclude both items (fragmented carrier and intra-band non-collocated EN-DC/NR-CA deployment) from Rel-20 scope. |
| 3 | Vivo (2193) | LB CA objectives - Specify UE requirements to allow switching between {case 1, case 2} [RAN4] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and Rx on FDD carrier 1 - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2] [Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach] UAV objectives - Specify RF requirements for UAV UE to support 33dBm output power - Example bands: only TDD bands with same TDD pattern, including n41, n78/n79 - Antenna array with up to 33dBm maximum output power - Specify RF requirements including output power, configured power, SEM, etc. - Co-existence simulation analysis is required - Specify necessary RRM requirement |
| 4 | Nokia (2443) | Proposal 1: RAN can consider not to agree more cross RF and baseband WIs than the two endorsed. Remaining proposals can be considered in the relation to 6G. Proposal 2: Great care needs to be taken not to overload the 5G-Adv track, nor to underestimate the TUs required for individual items. |
| 5 | Qualcomm (2489) | Extension of LB-LB CA - “Dual DL” solution - Reuse the same switching pattern and have both DLs active during the period in which only SCell DL was active, - applicable to only some band combinations (e.g. n12+n29) - Minimal RAN1 impact to clarify that PCell DL is on all the time but the interruption time needed for switching - Reuse the interruptions defined in Rel-19 - RF requirements for different filter implementations - Minimal RRM impact as procedures are simpler when PCell DL is always available - Non-RRC based switching solution (dynamic switching) - RAN1/2 impact to specify DCI based switching procedure - Reuse the interruptions defined in Rel-19 - RRM impact due to uncertainty in amount of time spent on SCell/PCell UAV - Actual scope and scenarios need to be clarified by the proponents - Antenna array options should be clarified, co-ex will depend on implementation - Testability should be clarified - Co-existence study will be needed considering the higher power and different scenario compared to ATG and/or TN - At least a short co-existence study is needed (RAN4 should check whether co-existence study is needed or not) - We expect the work to be more complicated than stated in the contributions which proposed this work and claimed no co-ex is needed Other topics - Considering the commercial need and the time constraints of Rel-20, no other item should be approved - Items were captured in the WF from RAN#108 (RP-251879) to be further discussed, they should not be approved - Fragmented DL - No need to approve a WI, the SI conclusions are not very clear and show possible degradations from such an implementation - No unanimous understanding on the control architecture - Possible solutions with network control of the UE will be complicated - Will require a significant amount of time for both RF and RRM to agree requirements and side conditions - Intra-band non-collocated EN-DC/NR-CA - Requirements for type 3 UE with MRTD>CP and higher power imbalance will be difficult to define - Evaluation/definition of degradation when MRTD>CP is very complicated - Need is not clear after defining all the other requirements |
| 6 | Apple (2498 2841) | Enhancements of the low NR band carrier aggregation via switching: framework are proposed as follows - FDD+SDL dual DL - Consider {case 1, case 3} shown at right, where the aggregation of FDD+SDL DL only operation is a switching state - FDD+FDD - Consider {case 4, case 5} shown at right, where non-simultaneous operation between two FDD bands is considered - 1UL2DL/2UL1DL - Consider {case 6, case 7} shown at right, where 1 UL + 2 DL and 2 UL + 1 DL operation is considered as switching states (example with CA_n5A-n8A is used explicitly here to illustrate the solution) - RAN4 has additionally identified potential aspects related to band combination capability signaling and configuration when band pairs supported via the switching framework appear inside a higher order band combination, and an objective to address this is needed to help RAN4 structure the related discussion - As part of the Rel-19 work item on simultaneous Rx/Tx enhancements, RAN4 is discussing CA_n5A-n8A (see agreement at right) [R4-2507910] - The key observation is that the Rel-19 scheme is based on RRC reconfiguration - The Rel-20 enhancement proposed in this document aims to improve the efficiency of the switching operation for this scenario [R4-2505451] Fragmented carriers - Specify method(s) for reducing the number of UE Rx RF chains (e.g. from separate Rx RF chains per CC to one Rx RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario - Up to 6 dB DL received PSD imbalance between the two non-contiguous CCs - Semi-statically switch hardware resources - To manage the impact of PCell performance degradation for FDD bands, fragmented carriers are limited to SCells - Decide the options in the following aspects - Triggering condition to enable ”One Rx RF chain” - CA fallback and RF architecture switching - Indication of one Rx RF chain mode and UE capability - Specify RRM requirements for the new measurement and reporting mechanism - TU consideration - [0.5] TU is proposed, considering the SI has carried out comprehensive study |
| 7 | MediaTek (2516) | DL fragmented carriers Objectives (core part) - This work aims at introducing the mechanism and UE requirements to enable DL fragmented carrier with the following assumptions: - Single DL band with 2CC co-located NCCA within frequency span ≤ 100 MHz - Up to 6 dB DL received PSD imbalance between the two non-contiguous CCs - Semi-statically switch hardware resources - Under co-located inter-operator scenario - Specify allowable RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4]; - To manage the impact of Pcell performance degradation and minimize RAN4 workload, the feature is enabled only for SCellsfor FDD bands - Determine and specify UE behaviour of CA fallback with/without semi-statically architecture switch hardware resources (i.e. Rx RF chains) based on the SI outcome [RAN4]; - Re-use current fallback procedure to avoid RAN2 impact Objectives (Perf part) - Depending on the down-selected approach, specify the corresponding RRM test cases, if needed. - Specify UE demod requirements to ensure zero impact to performance degradation under concerned scenarios. Note 1: No, RAN1, RAN2 impacts is foreseen Note 2: The feature is not enabled with UL tx switching simultaneously Low band CA - Specify UE RF requirements for semi-static switching mechanism or Study phase for non-semi static switching mechanism without DCI updates/changes, including at least switching gap, and corresponding physical layer procedures to switch between {Case 1, Case 3} [RAN4, RAN1] - Case 1: 1 Tx/2 Rx on FDD carrier 1 and 0 Tx/0 Rx on SDL carrier 2 - Case 3: 0 Tx/2 Rx on FDD carrier 1 and 0 Tx/2 Rx on SDL carrier 2 - Example Bands for LBDL CA: CA_n12A-29A, CA_n14A-n29A and CA_n28A-n67A. - Cell specific switching pattern can be considered. - Study UE co-existence emission requirements to protect each single carrier (i.e., n12 NR UE needs to protect the n29 NR UE) - Note: Do not consider cross-carrier scheduling in Rel-20 to minimize other WGs impacts. |
| 8 | LGE (2527) | Low band CA via switching - RF/RRM requirements based on following scenarios and configuration - LB CA FDD + SDL via switching between {Case 1, Case 2, Case 3} - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: no Tx/Rx on FDD carrier 1 and Rx on SDL carrier 2 - Case 3: no Tx on FDD carrier 1 and Dual Rx on FDD carrier 1+SDL carrier 2 - LB CA FDD + FDD via switching between {Case 4, Case 5} - Case 4: Tx/Rx on FDD carrier 1 and no Tx/Rx on FDD carrier 2 - Case 5: no Tx/Rx on FDD carrier 1 and Tx/Rx on FDD carrier 2 - Dynamic switching configuration - Consider both MAC-CE and DCI based switching configuration - Consider slot-based switching DL fragmented carriers - RF requirements for DL fragmented carriers - REFSENS, ACS, Blocking characteristics - RRM requirements - In-gap interference measurement, - Interruption caused by switching from single CC to Non-Contiguous CA (NCCA), vice versa. - Measurement behavior during NCCA |
| 9 | KDDI, NTT DOCOMO, SoftBank, KT, LG Uplus (2528) | Intra-band non-collocated EN-DC/NR-CA deployment - Proposal - Type 3 UE requirements assuming MRTD<CP and power imbalance < [xx] dB for handheld - Without intended impacts on the network side - Assumption/Background - Type 4 : UE and NW sides - Type 2 requirements (Pwr imbalance:25dB, MRTD:33us) will be reused considering similar separated UE Rx chain. - Type 3 : UE sides - The LNA will be shared between 2CCs for Type 3. It means any gain change initiated by one CC will affect another CC. Then it is important that the timing between 2CCs is aligned to some extent. - If MRTD<CP, no performance impact is expected. However, power imbalance might be almost same as Type 1. - If MRTD>CP, the gain change due to one CC will impact the performance of another CC within the received symbol. - Type 3 : NW sides - If MRTD<CP and additionally TAE/MRTD is tightened, it will impact the network deployment. - If MRTD>CP, there seems to be no solutions for NW to avoid impacts for UE performance now. |
| 10 | Ericsson (2574) | Proposal 1: For LB-LB CA, consider non-RRC based switching that is similar to UL switching and does not have large RAN1 impact, e.g. uplink activity based. Proposal 2: For UAV, co-existence study is required. The co-existence study needs to take into account the special circumstances of UAV with accumulated interference over long distances. Proposal 3: Study of co-existence degradations to UAV receivers is also necessary. Proposal 4: RRM impacts of UAV based UE beamforming should be considered. Proposal 5: There is no room to accommodate additional work on intra-band non-co-located or fragmented carriers on top of the other parts of the package discussed at RAN4#108. |
| 11 | Sony (2585) | Currently, 4 objectives are proposed under “cross RF and baseband” areas. In our view, the TU budget may only be able to accommodate 2 out of the 4 objects in Rel-20 5GA track. Therefore, it is proposed to focus on - Extension of LBCA: focus on the dual DL solution in Rel-20. - UAV UE with antenna array with up to 33dBm maximum output power on TDD spectrum |
| 12 | CMCC, China Telecom, China Unicom, [SK Telecom], Verizon, APPLE, CATT, Huawei, HiSilicon, MTK, OPPO, ViVo, xiaomi, ZTE (2600) | UAV Work item leadership: RAN4 (Lead WG) without RAN1/2/3 scope: Objectives for Rel-20 - Scenario: - BSs are deployed on the ground by adjusting the tilt angle upwards. The coverage height is up to 600~1000m. The ISD is about 2~4km. - A direct radio link between BS on the ground and UAV UE - Specify RF requirements for UAV UE/BS to support higher UL output power - Example bands: - TDD with same TDD pattern, including n41, n77, n78, n79, - FDD: n3 - Consider UAV UE with omni-direction antenna and antenna array with up to 33dBm max output power - Specify RF requirements including output power, configured power, SEM, etc. - Analyze co-existence requirements, using calculation as baseline - Note: only define RF requirements for regions/areas without A-MPR and additional spurious requirement - Specify RRM requirements, if needed. |
| 13 | Ofinno et al (2698) | Observation 1: Dual DL solution is suitable for SDL+FDD (with SDL within FDD duplex gap) but may not usable/optimal for other types of band combinations. Observation 2: The non-RRC based switching solution (i.e., based on dynamic switching) can be applied for various types of low band combinations (e.g., with appropriate DCI design). Observation 3: The non-RRC based switching solution (i.e., based on dynamic switching) is also less restricted, enabling the network to more efficiently handle different types of physical layer procedures (e.g., scheduling, RACH procedure, CSI reporting, etc.). Proposal 1: It is proposed to limit the scope of Rel-20 WI on enhancement of low band CA via switching to non-RRC based switching solution (i.e., based on dynamic switching). |
| 14 | TELUS, Bell Mobility, Apple, AT&T, MediaTek, T-Mobile USA, Southern Linc, Telstra, Vodafone (2699) | Fragmented Carrier Proposed Rel-20 Work Item: - Initiate a Rel-20 work item, including both FDD and TDD bands with minimum impact to RAN1 - To manage the impact of PCell performance degradation for FDD bands, consider scenarios where fragmented carriers are SCells |
| 15 | TELUS, Bell Mobility, Apple, BT plc, Skyworks Solutions, Inc, Murata, AT&T (2799) | Low-Low CA - Device needs to support inter-carrier scheduling: UE monitors PDCCH DCI of FDD Carrier 1, which has scheduling information for both carriers - Device switches to Carrier 2 based on DCI instruction - Device switches back to Carrier 1 after scheduled transmission interval of Carrier 2 is finished |
| 16 | CATT (2716) | Fragmented Carrier Observation 1: Rel-19 study item on NR FR1 DL fragmented carriers concluded the feasibility under certain conditions for an NR UE to operate one Rx RF chain mode to receive two DL non-contiguous carriers. - A follow-up WI in Rel-20 may be required to further explore the following aspects: - In-gap interference rejection - Signaling aspects and fallback behavior - Pcell degradation - UE requirement relaxation Observation 2: For intra-band non-collocated EN-DC/NR-CA deployment, Rel-18 and Rel-19 already specified requirements of Type 1, Type 2 and Type 4 UEs, with only Type 3 unspecified. • The support for Type 1/2/4 UEs establishes a strong foundation for deploying intra-band noncollocated EN-DC/NR-CA Proposal 1: If TU budget in Rel-20 5G-A is not sufficient to accommodate both fragmented DL carrier and intra-band non-collocated EN-DC/NR-CA, the former can be included with a carefully tuned scope. |
| 17 | CHTTL (2726) | Proposal 1: For the extension of LBCA, suggest to focus on the dual DL solution first, and consider a reduced time unit for the dynamic switching. Proposal 2: Consider a follow-up NR FR1 DL fragmented carrier WI in Rel.20 with limited TU. - No need to restrict to SCells only for FDD bands - Consider the objectives and the justification provided below NR FR1 DL fragmented carrier WI Objectives: - Specify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to one RF chains) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario [RAN4, RAN2] - Specify the indication of configurations supporting one Rx RF chain mode [RAN4, RAN2] - Specify the needed relaxation for the RF requirements under one Rx RF chain mode, including ΔRIBNC, ACS and in-band blocking at least [RAN4] - Specify UE behaviour including trigger condition to enable one Rx RF chain mode, and the CA fallback and architecture switching, which can be down-select from the SI outcome - Study and specify the needed RRM requirements based on the outcome of triggered condition NOTE 1: example band combination(s) to be discussed during working group NOTE 2: up to 6 dB DL received power level imbalance between the two non-contiguous CCs |
| 18 | Intel (2752) | Proposal #1: Approve the following RAN4 Rel-20 5G-A cross-area topics - Extension of Low Band CA Switching - Introduce RF/RRM requirements for dual DL solution implementing switching between the 2 cases - Case X (FDD band DL + FDD band UL) and Case Y (SDL band DL + FDD band DL) - Exclude “non RRC-based switching” solutions from the work scope - No or minimum RAN1/2 scope - High power UAV UE - Introduce necessary UE RF requirements for high power UAV UE with directional beamforming (antenna array) with up to 33dBm maximum output power - No RAN1/2 scope - Fragmented DL CA - Specify necessary solutions and requirements to allow NR UE to operate one Rx RF chain mode to receive two DL non-contiguous carriers in a single DL band - Solutions are limited to the case of FR1 intra-band CA with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario - Solutions to ensure no or minimum PCell performance degradation - No RAN1/2 scope |

Reference

[1] RP-251879, Way forward on RAN4 R20 package, RAN4 Chair (Apple), RAN TSG meeting #108.