3GPP TSG RAN Meeting #109						RP-251913

Beijing, China, September 15-18, 2025

3GPP TSG-CT WG1 Meeting #156	C1-255319

# Goteberg, Sweden 25-29 August 2025

# Title:	Reply LS on UE usage of the RAT restrictions

# Response to:	R2-2504931 (C1-254578)

# Release:	Release 19

# Work Item:	ECRATU

Source:	CT1

To:	RAN2

Cc:	CT4, RAN

Contact Person:

Name:	Roland Gruber

Tel. Number:

E-mail Address:	rgruber at apple dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	--

1. Overall Description:

CT1 thanks R2 for the reply LS on UE usage of the RAT restriction.

CT1 has discussed the question raised by R2 whether separate NAS capabilities for 2G/3G RAT restriction and 4G/5G RAT restriction would be introduced in CT1 specification or not.

CT1 does not see a need to introduce separate NAS capabilities for 2G/3G RAT, because:

- Such capabilities would result in savings in the NAS signalling only in a network where the access technology restrictions are applicable to 2G/3G RATs only, and CT1 does not consider this a likely use case.

- If  2G/3G access technology restrictions are configured in the UE and if the UE does not support the 2G/3G access technology restrictions during cell selection/re-selection, the UE can camp on 2G/3G. In that case, the 2G/3G core network will reject the registration procedure with cause code #15, which is mandatory for the UE to handle. This will not cause any interoperability problems, but just unnecessary NAS signalling.

2. Actions:

To RAN WG2 group.

ACTION:

CT1 kindly asks RAN2 to take the above into consideration.

3. Date of Next CT1 Meetings:

CT1#157	13th October- 17th October 2025	Sophia Antipolis, France

CT1#158	17th November- 21st November 2025	Dallas, US

CT1#159 	9th February- 13th February 2026	India