3GPP TSG RAN WG1 #122	R1-2505129

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:		11.4.1

Source:	Nokia

Title:	Channel Coding in 6G Radio Air Interface

WI code:	FS_6G_Radio

Release:	Rel-20

Document for:		Discussion and Decision

# Introduction

The 6G study item has been approved in [1]. Concerning the Physical Layer structure for 6GR, the following can be noted:

- Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1

This contribution discusses aspects of channel coding for 6G. Section 2 briefly summarises 5G NR channel coding for control and data channels. Section 3 shows our views on 6G channel coding in different aspects. Section 4 concludes the document with observations and proposals.

# 5G NR channel coding

When transitioned from 4G to 5G, it was decided to switch from tail-biting convolutional codes (TBCCs) to polar codes for control channels, where the main target is very high reliability relative to the previous standard, and from turbo codes to low-density parity-check (LDPC) codes for data channels, which require larger throughput (TP). In the following, we will summarize the existing codes briefly.

## Control channels

Polar codes are used as the channel coding scheme for encoding the uplink control information (UCI) in PUCCH and PUSCH as well as for the downlink control information (DCI) in PDCCH and PBCH. In addition to polar codes, block codes are used to encode very short UCI messages, i.e., 1- to 11-bit of messages. In the following, we summarize the encoding of both UCI and DCI, briefly, which had also different design choices.

The encoding chain for polar coding is specified in [2] for both UCI and DCI. Let’s denote the message size and the length of the final codeword as A and E, respectively. The intervals for A and E change depending on the application. For instance, the length of the final codeword satisfies 18≤E≤8192 for UCI encoding while its minimum value changes to Emin=25 in the case of DCI encoding. If it is used for PBCH, then its value is fixed to E=864, which is achieved using repetition. Regarding the message size, it varies between A=1 bit to A=140 bits in the case of DCI message for PDCCH while its value is fixed to A=32 if it will be used in PBCH. For UCI message size, it should satisfy 1≤A≤1706, where below-outlined polar codes are not used if the message size is below A<12 as mentioned before.

The message is segmented into two roughly even-sized segments if either A≥1013 or (A≥360∩E≥1088) for encoding UCI, where each segment is encoded independently as a separate message, which reduces encoding/decoding complexity/latency. An L-bit cyclic redundancy check (CRC) is added to the information bits, generating a K=A+L-bit sequence. The length of the CRC polynomial is defined by the application and the message length, where it is always L=24 bits for DCI, which require a low undetected error target, and L=6 if A<20 or L=11 otherwise, for UCI. In the case of DCI, CRC-encoded message is interleaved prior to polar code encoding, which places some of the CRC bits into earlier positions instead of placing all of them at the end to help for blind decoding at the UE. The size of this interleaver is limited to 164 since the payload of DCI message is limited to 140 bits. The CRC-aided polar code decoding provides better overall error probability compared to the polar codes without CRC, where the CRC bits is also used for providing error detection capability.

For a given message size A, the length of the final codeword, E, determines the overall code rate. Note that the block length N of a mother polar code is limited to powers of 2 and the existing 5G design is supported for any mother code length of N≤1024. For the DCI, the mother code length is limited by N≤512. The rate adaptation is obtained via applying puncturing, shortening, or repetition to the polar-encoded word, which is obtained after encoding the CRC-appended input bits via the mother polar code. Note that the rate-matching scheme is applied according to the desired rate R=A/E and the desired codeword length at the end of the encoding chain to optimize the performance of the polar codes under the canonical successive decoding algorithm. Prior to the modulation a channel interleaver is applied for the encoded UCI codeword, which is skipped in the case of DCI.

For encoding UCI with 1≤A≤11, block codes are used. In the case of A≤2, the output of the encoder depends on the adopted modulation order. For the other cases, the generator matrix can be obtained as the subset of a pre-defined fixed matrix, which outputs N=32 for any value of A. Rate-matching is obtained via either repetition or puncturing to obtain the desired E bits. CRC is skipped for very small blocks due to the incurred rate loss.

## Data channels

5G NR uses quasi-cyclic low-density parity-check (QC-LDPC) codes for data channels PDSCH and PUSCH, which aimed at improving TP compared to using turbo codes. The encoding chain for QC-LDPC coding is specified in [2]. For error detection, a CRC is first attached to a transport block (TB). A base graph (BG) is selected for encoding the CRC attached TB. There are two BGs in 5G NR, which target different regimes. Roughly speaking, the first BG (BG1) is used for larger blocks and higher rates and the second BG (BG2) targets smaller block lengths and lower rates. Figure 1 shows how the selection between BG1 and BG2 is done in 5G NR, where BG1 supports a maximum code block size of 8448 and BG2 supports up to 3840 bits. BG2 is used if the coding rate is less than or equal to 0.25, or if TB size (TBS) is less than or equal to 292 bits or if TBS is less than or equal to 3824 bits and coding rate is less than or equal to 0.67. BG1 is used in all other cases.

Figure 1. BG selection between BG1 and BG2 in 5G NR.

Depending on the selected base graph (BG) and the TB size, code block segmentation (and further CRC attachment per code block) may be performed if the TBS is greater than the maximum code block size supported by the selected BG. Channel coding is then performed for each code block using the selected base graph.

# Views on 6G channel coding

## Hardware commonality as a basic requirement

Learning from the past challenge of transitioning from 4G to 5G, when considering applicable extensions to the 5G channel coding design to satisfy additional requirements for 6G scenarios, it is important to target a design that can keep the hardware commonality between 5G NR and 6G. This would help 6G UEs and gNBs to also support 5G without the need to implement two hardware blocks for channel coding which is not a cost-effective solution. Hence, 5G LDPC and polar codes should be considered as the foundation for 6G channel coding. In this regard, at least the following are expected:

- For data channels, the 6G LDPC decoder and encoder blocks should be able to process 5G LDPC codes.

- For control channels, the 6G Polar decoder and encoder blocks should be able to process 5G polar codes.

In other words, 5G channel coding should be a subset of 6G channel coding in case extensions are introduced. Moreover, to allow software updates of 5G NR gNBs to support 6G radio, the following needs to be true as well:

- For data channels, the 5G LDPC decoder and encoder blocks should be able to process 6G LDPC codes at least for similar use cases/scenarios.

- For control channels, the 5G Polar decoder and encoder blocks should be able to process 6G polar codes at least for similar use cases/scenarios.

With the above design targets in mind, during the study item, RAN1 should focus only on proposals that demonstrably enhance key performance indicators such as power consumption on top of throughput or reliability, while avoiding impact on existing 5G channel coding hardware for cost efficiency. In addition, the complexity of implementing extensions to the existing coding schemes should be carefully evaluated against the potential benefits, ensuring that the gains outweigh the cost and effort required for integration. Also, given the need for RAN1 to provide an early conclusion latest by RAN#112 on fundamental aspects such as channel coding [1], it is recommended that RAN1 avoids considering/studying channel coding proposals that do not offer a significant performance improvement to justify the added implementation complexity on top of existing 5G baseline.

Proposal 1: For 6G channel coding, LDPC codes are used for data channels, while polar codes are used for control channels as in 5G.

Proposal 2: For the study of 6G channel coding, the scope of potential extensions of the 5G channel coding schemes is to satisfy requirements for 6G use cases.

Proposal 3: For the study of 6G channel coding, RAN1 should study a design that keeps hardware commonality between 5G NR and 6G.

Proposal 4: Given the need for an early conclusion latest by RAN#112 on fundamental aspects such as channel coding, RAN1 should avoid considering/studying channel coding proposals that do not offer significant performance improvement to justify the added implementation complexity on top of existing 5G baseline.

## Views on LDPC codes for data in 6G

As mentioned in the SID, RAN1 should focus on “applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off”. The 6G requirements and characteristics may refer to higher TP and/or better performance (e.g., for UL). For LDPC codes, the aspects that can be studied to satisfy these requirements and characteristics could be LDPC BG and decoder enhancements.

### LDPC base graph

Given the design target of maintaining hardware commonality, it is crucial to maintain the existing 5G NR BGs BG1 and BG2 in 6G. These BGs represent fundamental building blocks for the system and are optimized for specific functionalities and performance characteristics.

Proposal 5: The existing 5G NR base graphs BG1 and BG2 should be used for 6G.

The necessity of any applicable extensions to satisfy 6G requirements should be first assessed during the study. The study should focus on the requirements of LDPC code performance/complexity for data channels in 6G and whether the existing BG1 and BG2 can already satisfy the requirements. It is worth noting that 6G requirements are applied to the whole system and not only for channel coding. Therefore, even if 6G requires higher TP and lower latency, this does not necessarily imply that channel coding should be optimized to satisfy these requirements independently. Indeed, these requirements can also be fulfilled by other design aspects such as MIMO, frame structure, etc. For channel coding, the trade-off between performance and complexity always exists and this should be taken into account to avoid an overdesign in 6G.

Proposal 6: RAN1 to study the necessity of any applicable extension on top of BG1 and BG2 considering acceptable performance/complexity trade-off.

If the study concludes that an extension is necessary, the extension should be studied on top of the existing BG1 and BG2. Introducing an extension on top of (but not replacing) BG1 and BG2 allows for flexibility and expansion without disrupting the core functionality. This approach ensures that the existing hardware infrastructure can be leveraged while accommodating new features or requirements. The introduction of a new BG as a possible extension of the 5G channel coding design should be carefully considered and justified based on a thorough analysis of the potential benefits and impact on system performance, complexity, and cost.

Proposal 7: Any study of applicable extension of base graph, if justified, should be considered in addition to BG1 and BG2 but not replacing any of them.

For 5G NR base graphs, a parity check matrix consists of five sub-matrices (A, B, C, D, E) as follows.

A

C

D

E

B

These matrices are designed according to the above structure, wherein A corresponds to systematic bits, B corresponds to parity bits (B is a square matrix whose first column has weight three while the columns after the weight-three column have a dual diagonal structure, i.e., main diagonal and off diagonal), C is a zero matrix, E is an identity matrix and no specific constraint for D.

The 5G BG design, particularly the dual diagonal structure, offers a significant advantage in terms of computational efficiency. This structure effectively eliminates the need to calculate the inverse of matrix B during the encoding process. Calculating the inverse of a matrix is a computationally intensive operation, requiring significant processing power and time. By avoiding this step, the dual diagonal structure significantly reduces the computational complexity of the encoding process, leading to faster processing times and lower power consumption. This efficiency gain is crucial for real-time communication systems where latency and cost efficiency are critical factors.

Therefore, if the introduction of a new BG for 6G is deemed necessary, it is recommended that RAN1 keeps the same design principles established for 5G. This means continuing to utilize the proven and efficient QC-LDPC codes with the dual-diagonal structure. This approach offers several advantages:

- Proven Performance: The QC-LDPC codes with dual-diagonal structure have demonstrated excellent performance in 5G, achieving high coding gains and efficient error correction capabilities.

- Reduced Complexity: As aforementioned, the dual-diagonal structure significantly simplifies the encoding processes, reducing computational complexity and thus latency. This is particularly important for 6G, which is expected to handle even higher data rates and more complex communication scenarios.

- Interoperability: Maintaining the same design principles promotes interoperability between 5G and 6G systems. This allows for seamless integration and backward compatibility, facilitating a smooth transition to the next generation, leveraging existing knowledge and expertise.

By keeping these design principles, RAN1 can ensure that the new BG for 6G (if any) inherits strengths and efficiencies while still being consistent with its 5G predecessor, paving the way for a robust 6G network.

Proposal 8: Any study of applicable extension of base graph, if justified, should keep the same design principles as in 5G, i.e. using QC-LDPC with dual-diagonal structure.

The 6G requirements and thus the BG extension may be applied in certain scenarios (e.g., when there is a need for higher peak TP), while for other scenarios such requirements may not be applied. The design of the BG extension should target optimizing only the scenarios where the 6G requirements are applied. This means that for other scenarios, the optimal BGs would be the existing BG1 and BG2, as already optimized in 5G. Therefore, RAN1 should also study the conditions for utilization of a potential BG extension.

Proposal 9: For the study of applicable extension of base graph, if justified, RAN1 should study the conditions for utilization of a potential base graph extension.

### Decoding algorithms

In 5G NR, min-sum algorithm (MSA) was adopted as decoding algorithm for performance evaluation thanks to its simplicity compared to sum-product algorithm (SPA), which back then was known as offering better performance at the expense of higher complexity. In the past decade, with the development of hardware implementation capabilities, some decoding algorithms that were deemed too complex in the past may now be implemented more easily. For example, SPA can be considered as decoding algorithm for evaluation.

Proposal 10: RAN1 to discuss decoding algorithms for evaluation.

Proposal 11: RAN1 to consider decoder enhancements targeting higher throughput and/or better performance compared to the baseline with acceptable complexity trade-off for the evaluation of potential extensions of LDPC codes.

## Views on polar codes for control in 6G

6G control channel requirements and characteristics are likely to include higher reliability. However, the 5G polar codes were also designed with this goal in mind. The 5G polar codes design is a combination of a reliability sequence, which determines the encoding procedure for the inner polar code, and the outer systematic CRC encoding. The reliability sequence compactly defines the inner polar code for any length and dimension to be used up to 1024 bits. As mentioned in Section 2.1, the length of the CRC encoding is determined according to the application (DCI or UCI) and the length of the message. Therefore, at least for block lengths up to 1024 bits, the 5G Polar Code should be kept.

Proposal 12: 5G Polar Code should be used for 6G for the block length up to 1024 bits.

For evaluation, the underlying assumption was that CRC-aided successive cancellation list (SCL) decoding with a list size of L=8 can be used at the receiver side as baseline and important metrics are the overall and undetected error probabilities. Since the hardware implementation capabilities improved over the past years, improved decoding algorithms, e.g., SCL decoding with list sizes larger than 8, might be considered to improve the key performance metrics for 6G polar codes. Nevertheless, the complexity requirements should still be kept in consideration.

Proposal 13: For 6G Polar Codes evaluation, a baseline receiver should use successive cancellation list (SCL) decoding with a list size of L=8 or L=16. The key performance metrics for this evaluation are the overall and undetected error probabilities.

For polar codes with block length greater than 1024 bits, the motivation/requirement is still unclear. In principle, the main motivation is to be able to accommodate more UCI bits, especially CSI bits when wider bandwidth is supported. However, it is unclear how many additional bits are needed for UCI and whether they would exceed the current 5G capacity, which considers code block segmentation up to two segments already. On the other hand, based on 5G extended studies, enhancements for short block lengths can be marginal and at the cost of additional standardization and implementation effort.

Observation: Motivation/requirement for polar codes with block length greater than 1024 bits is unclear given that the increase of UCI/DCI bits is still unknown. Enhancements for short block lengths can be marginal and at the cost of additional standardization and implementation effort.

Given the above observation, the necessity of any applicable extension on top of 5G polar codes should be carefully studied considering acceptable performance/complexity trade-off.

Proposal 14: RAN1 to study the necessity of any applicable extension on top of 5G polar codes considering acceptable performance/complexity trade-off.

The codes for encoding the small blocks, i.e., those for smaller than or equal to 11 bits of message, were optimized to have a good minimum distance in the final small codes, e.g., LTE-RM codes. Given this consideration, the gain to be achieved even under maximum-likelihood (ML) decoding might be very limited given their compact representation.

Proposal 15: For 6G, block codes for small block lengths should be kept the same as in 5G.

# Conclusion

In this document, the following observation can be noted:

Observation: Motivation/requirement for polar code with block length greater than 1024 bits is unclear given that the increase of UCI/DCI bits is still unknown. Enhancements for short block lengths can be marginal and at the cost of additional standardization and implementation effort.

The following are proposed:

Proposal 1: For 6G channel coding, LDPC codes are used for data channels, while Polar codes are used for control channels as in 5G.

Proposal 2: For the study of 6G channel coding, the scope of potential extensions of the 5G channel coding schemes is to satisfy requirements for 6G use cases.

Proposal 3: For the study of 6G channel coding, RAN1 should study a design that keeps hardware commonality between 5G NR and 6G.

Proposal 4: Given the need for an early conclusion latest by RAN#112 on fundamental aspects such as channel coding, RAN1 should avoid considering/studying channel coding proposals that do not offer significant performance improvement to justify the added implementation complexity on top of existing 5G baseline.

Proposal 5: The existing 5G NR base graphs BG1 and BG2 should be used for 6G.

Proposal 6: RAN1 to study the necessity of any applicable extension on top of BG1 and BG2 considering acceptable performance/complexity trade-off.

Proposal 7: Any study of applicable extension of base graph, if justified, should be considered in addition to BG1 and BG2 but not replacing any of them.

Proposal 8: Any study of applicable extension of base graph, if justified, should keep the same design principles as in 5G, i.e. using QC-LDPC with dual-diagonal structure.

Proposal 9: For the study of applicable extension of base graph, if justified, RAN1 should study the conditions for utilization of a potential base graph extension.

Proposal 10: RAN1 to discuss decoding algorithm for evaluation.

Proposal 11: RAN1 to consider decoder enhancements targeting higher throughput and/or better performance compared to the baseline with acceptable complexity trade-off for the evaluation of potential extensions of LDPC codes.

Proposal 12: 5G polar code should be used for 6G for the block length up to 1024 bits.

Proposal 13: For 6G Polar Codes evaluation, a baseline receiver should use successive cancellation list (SCL) decoding with a list size of L=8 or L=16. The key performance for this evaluation are the overall and undetected error probabilities.

Proposal 14: RAN1 to study the necessity of any applicable extension on top of 5G Polar considering acceptable performance/complexity trade-off.

Proposal 15: For 6G, block codes for small block lengths should be kept the same as in 5G.

# References

[1] 	RP-251881, “New SID: Study on 6G Radio”, NTT Docomo, June 2025.

[2] 	3GPP TS 38.212 v18.6.0 (2025-03) “Multiplexing and channel coding”.