3GPP TSG RAN WG1 #122	 R1-2506090

Bengaluru, India, Aug 25th – 29th, 2025

Source:	CMCC

Title:	Discussion on inter-vendor training collaboration for CSI compression

Agenda item:	10.1.2

Document for:	Discussion & Decision

Introduction

In RAN#108 meeting, the New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements[1] was approved, including the following objectives of enhancements for inter-vendor training collaboration for two-sided AI/ML models:

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

In this contribution, potential spec impact for standardized dataset format/content plus dataset exchange for two-sided model will be discussed.

Discussion on dataset format/content for inter-vendor training collaboration for CSI compression

Applying a two-sided model at the physical layer, such as CSI compression use case, can significantly improve information transmission efficiency between NW and UE. As early as the Rel-18 study item phase, three training methodologies for two-sided model had been identified and discussed.

-	Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided.

-	Type 2: Joint training of the two-sided model at network side and UE side, respectively.

-	Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the NW-side CSI reconstruction part are trained by UE side and network side, respectively.

However, all these approaches are only feasible from the perspective of single gNB and single UE, failing to address the substantial engineering implementation challenges arising from bilateral vendor-vendor collaboration.

In Rel-19, to resolve the issues related to the inter-vendor collaboration, five options were identified.

Agreement @RAN1#116 To alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, study the following options: - Option 1: Fully standardized reference model (structure + parameters) - Option 2: Standardized dataset - Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side - Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side - Option 5: Standardized model format + Reference model exchange between NW-side and UE-side Note 1: The above options may not be mutually exclusive and may be used together. Note 2: Other options are not precluded. Note 3: The study should consider how different methods of exchanging the parameters / dataset / reference model would affect the feasibility and collaboration complexity of options 3 / 4 / 5 respectively, e.g., over the air-interface, offline delivery, etc. Note 4: “Dataset” refers to a set of data samples of CSI feedback and associated target CSI.

Among these, despite the substantial data volume required for dataset exchange, Option 4-1 are still considered by RAN1 as a viable and effective solution to address inter-vendor training collaboration issues.

For Option 4-1 in Direction A, in RAN1#118bis meeting [4], we have agreed that the dataset shared between NW side and UE side is {target CSI, CSI feedback}, the following is the procedure of Option 4-1. We think it is straightforward to support the dataset content is {target CSI, CSI feedback} for Option 4-1 in Direction A in AI/ML based CSI compression.

[118bis] Agreement For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression, - For Direction A: - Further study the following options with potential down-selection - 4-1: dataset is {target CSI, CSI feedback}, - 3a-1: encoder parameter exchange, with {target CSI} - Note: Additional information beyond what’s listed above may need to be exchanged if necessary.

Figure 1. Procedure of Option 4-1

Proposal 1: For Option 4-1 in Direction A in AI/ML based CSI compression, support {target CSI, CSI feedback} as the standardized dataset content.

As illustrated in our company’s contribution discussing on inference related aspects of CSI compression [5], in Rel-19 study on AI/ML based CSI compression, when discussing the standardized model structure for Option 3a-1 and Direction A, we have agreed to take precoding matrix in the spatial-frequency domain in a subband granularity as the input, and the reconstructed precoding matrix in the spatial-frequency as the output, for the study of standardized model structure. So, to make sure that the standardized model structure can work well for Option 3a-1, at least precoding matrix in the spatial-frequency domain in a subband granularity should be supported as the target CSI type for inference with the assumption of inter-vendor collaboration training Option 3a-1.

Although the target CSI used in inference with the inter-vendor collaboration training assumption of Option 3a-1 and Option 4-1 can be different, to ensure that we have unified design for inference aspects for different inter-vendor collaboration training methods. It is suggested that the precoding matrix in the spatial-frequency domain in a subband granularity is also supported as the target CSI type for inference with the assumption of inter-vendor collaboration training Option 4-1.

The format for target CSI for inter-vendor collaboration training should be aligned with that for model input in inference. Then also at least precoding matrix in the spatial-frequency domain in a subband granularity should be supported as the standardized dataset format/content of target CSI for inter-vendor collaboration training Option 4-1.

Proposal 2: At least precoding matrix in the spatial-frequency domain in a subband granularity should be supported as the standardized dataset content of target CSI for inter-vendor collaboration training Option 4-1.

As for the standardized data set format of target CSI, we also think there should be a common design for target CSI for inference, monitoring and inter-vendor collaboration training.For the data format of target CSI, we can use codebook-based Rel-16 eType2 or Rel-18 eType2 for PMI prediction as baseline. Furthermore, to get much higher quantized ground-truth CSI, the traditional Rel-16 eType-II or Rel-18 Doppler codebook may be enhanced for more accurate channel characterization, like much larger number of beam-delay basis vectors reporting (W1) or much higher resolution quantization for coefficient feedback (W2).

And to increase the accuracy of target CSI, we can consider R16 eType-II codebook with new/larger parameter(s). We think the basic structure of R16 eType II-like codebook could be reused for the target CSI, along with the basic concept of spatial domain, frequency domain and Doppler domain basis, but the exact supported values of codebook parameters will be enhanced to make sure high resolution data transfer. Besides, in Rel-18 MIMO, the Doppler domain compression also has been introduced to obtain PMIs over multiple slots, which will highly improve the compression ratio due to one additional dimension is introduced, Rel-18 Doppler codebook also can be considered as ground truth CSI format for NW-side data collection.

Proposal 3: There should be a common design for target CSI for inference, monitoring and inter-vendor collaboration training.

Proposal 4: In CSI compression using two-sided model use case, regarding the standardized dataset format of target CSI for inter-vendor collaboration training Option 4-1, codebook-based Rel-16 eType2 and Rel-18 eType2 for PMI prediction can be supported.

Proposal 5: In CSI compression using two-sided model use case, regarding the standardized dataset format of target CSI for inter-vendor collaboration training Option 4-1, the basic codebook structure could be reused, along with the basic concept of spatial domain, frequency domain and Doppler domain basis.

Proposal 6: In CSI compression using two-sided model use case, regarding the standardized dataset format of target CSI for inter-vendor collaboration training Option 4-1, the exact supported values of codebook parameters can be studied to make sure high resolution data transfer.

Discussion on additional information for inter-vendor training collaboration for CSI compression

Based on the evaluation results in Rel-18 [2], for the case of separate training, i.e. Option 4-1, if the backbones are not aligned between the two sides, it will lead to minor or moderate performance degradation. So, it is preferred to align some model related information, such as model backbone, to achieve better performance for Option 4-1.

TR 38.843 Section 6.2.2.8 - From the perspective of training collaboration types, compared to 1-on-1 joint training, both multi-vendor joint training and separate training with procedures given in clause 6.2.1 may suffer performance loss. o In particular, for multi-vendor joint training, minor or moderate degradation is observed. o In particular, for separate training with procedure given in clause 6.2.1, the performance loss depends on the factors such as backbone alignment, and multi-vendor training behavior:  For separate training of 1 NW part model and 1 UE part model, under both NW first training and UE first training, if backbones are aligned between the two sides, minor degradation is observed; otherwise, additional degradation is observed, leading to minor or moderate performance degradation.  For NW first training with 1 UE part model to N>1 NW part models, or UE first training with 1 NW part model to M>1 UE part models, additional degradation is observed, leading to minor, moderate, or significant performance degradation, depending on the training approach.  As a note, other procedures of separate training are not extensively evaluated.

- Proposal 7: Some necessary model related information, such as model backbone, can be aligned between NW side and UE side to achieve better performance for Option 4-1.

In RAN1#120bis meeting [3], we have confirmed that SGCS and NMSE can be as the type of performance metric as the additional information for Option 3a-1 and 4-1, while how and which one to use the metric need further study:

Agreement For inter-vendor-collaboration Options 3a-1 and 4-1 in Direction A, confirm SGCS and NMSE as the type of performance metric that may be used for the performance target shared as additional information along with the exchanged dataset or the model parameters. - FFS: when to use SGCS, NMSE, and which one to use or both, and relationship with the inter-vender collaboration sub-options. - FFS: details of the format of the performance target - Option 1: Average performance target, e.g. average SGCS and/or average NMSE - Option 2: distribution of the performance target, e.g., SGCS / NMSE for 5, 10, 20, 30 percentiles, etc. - FFS: whether multiple performance targets should be exchanged for different configurations, such as antenna ports configuration, subband configuration and payload configuration, etc., along with each exchanged dataset or model parameters

In the evaluation study in Rel-18/19, most evaluation results are in the form of SGCS, and the value range of SGCS is 0，1, which is a limited range. It means that if the SGCS is closer to 1, then the AI/ML predicts the CSI more accurately. However, the value range of NMSE is -∞, +∞, which makes it harder to identify whether the NMSE value is good enough for the AI/ML model.

Besides, in Rel-19 AI/ML based CSI prediction [3], we have agreed to take SGCS as the performance metric and SGCS have already been supported as a new CSI report quantity. And the deification of it is also agreed in Rel-19.

Agreement For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric. Agreement For the definition of SGCS, Note: How to handle layer mapping mismatch, if any, is up to UE implementation.

So, we prefer SGCS as the performance metric for performance target in Direction A, and the definition of SGCS used for performance metric for AI/ML based CSI prediction can be reused as much as possible.

- Proposal 8: For inter-vendor-collaboration Options 4-1 in Direction A, for the type of performance metric, prioritize SGCS, and the definition of SGCS used for performance metric for AI/ML based CSI prediction can be reused as much as possible.

Conclusions

In this contribution, we discussed potential spec impact for standardized dataset format/content plus dataset exchange for two-sided model, and the following proposals are made.

Proposal 1: For Option 4-1 in Direction A in AI/ML based CSI compression, support {target CSI, CSI feedback} as the standardized dataset content.

Proposal 2: At least precoding matrix in the spatial-frequency domain in a subband granularity should be supported as the standardized dataset content of target CSI for inter-vendor collaboration training Option 4-1.

Proposal 3: There should be a common design for target CSI for inference, monitoring and inter-vendor collaboration training.

Proposal 4: In CSI compression using two-sided model use case, regarding the standardized dataset format of target CSI for inter-vendor collaboration training Option 4-1, codebook-based Rel-16 eType2 and Rel-18 eType2 for PMI prediction can be supported.

Proposal 5: In CSI compression using two-sided model use case, regarding the standardized dataset format of target CSI for inter-vendor collaboration training Option 4-1, the basic codebook structure could be reused, along with the basic concept of spatial domain, frequency domain and Doppler domain basis.

Proposal 6: In CSI compression using two-sided model use case, regarding the standardized dataset format of target CSI for inter-vendor collaboration training Option 4-1, the exact supported values of codebook parameters can be studied to make sure high resolution data transfer.

- Proposal 7: Some necessary model related information, such as model backbone, can be aligned between NW side and UE side to achieve better performance for Option 4-1.

- Proposal 8: For inter-vendor-collaboration Options 4-1 in Direction A, for the type of performance metric, prioritize SGCS, and the definition of SGCS used for performance metric for AI/ML based CSI prediction can be reused as much as possible.

References

- RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, Prague, Czech Republic, June 9-13, 2025

- RP-233133, TR 38.843 v2.0.0, Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface, Qualcomm

- Chairman's Notes RAN1#120bis, Wuhan, China, April 7th – 11th, 2025.

- Chairman's Notes RAN1#118bis, Hefei, China, October 14th – 18th, 2024.

- R1-2503823, Discussion on AI/ML for CSI compression, St Julian’s, Malta, May 19th – 23rd, 2025