3GPP TSG RAN WG1 #122bis	R1-2507944

Prague, Czech Republic, October 13th – October 17th, 2025

Agenda item:       10.3.1

Source:	IIT Kanpur

Title:	Evaluations for Rel-20 AIoT

Document for:     Discussion and Decision

# Introduction

In Rel-19, Ambient IoT (A-IoT) was specified as a new IoT technology with ultra-low complexity, ultra-low power consumption, and capability for battery-less operation, targeting very low-end IoT applications in indoor scenarios and supporting passive device types. In RAN#108, a new SID [1] was approved to study support for outdoor deployment and active A-IoT device types, with RAN1-related aspects as follows:

A-IoT Devices These are the A-IoT Devices referred to in the objectives: - Device 1: As standardized in the Rel-19 WI Ambient_IoT_Solutions. ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - Device C: ≤ 1 mW to ≤ 10 mW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - SFO value 10Y is assumed to be better than any SFO value considered for Device 1 standardized in Rel-19. Objectives - Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2. RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b - Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage.

In this contribution, we share the evaluations and early findings on the coverage performance of Device type 2b/C for the Rel-20 outdoor scenario.

# Discussion

As per the SID, the study scope for the active device type is limited to the D4T1 deployment scenario, addressing use cases such as outdoor inventory, outdoor sensing, and outdoor command. In Rel-19, a framework for studying indoor deployment scenarios was developed, which could be reused; however, the requirements for outdoor deployment differ, and further study is required to evaluate the necessary changes for outdoor scenarios. The following agreements were reached in the previous meeting on the evaluation assumption and methodologies [3].

Agreement For Rel-20 study, “Link budget template” is as followings, - Note: additional rows (e.g. for reporting assumptions on energy storage/harvesting) can be further discussed Link budget template for Rel-20 study No.; Item; Reader-to-Device; Device-to-Reader. (0) System configuration. [0A]; Scenarios; Deployment scenario 4 with topology 1 with no external carrier wave; Deployment scenario 4 with topology 1 with no external carrier wave. [0B]; Device 2b/C; Device 2b/C; Device 2b/C. [0C]; Center frequency (MHz); 900MHz (M), 2GHz (O); 900MHz (M), 2GHz (O). [0D]; Pathloss model; UMa NLOS defined in TR38.901 RMa NLOS defined in TR38.901 Note: companies can also report to use RMa LOS defined in TR38.901 UMi NLOS defined in TR38.901; UMa NLOS defined in TR38.901 RMa NLOS defined in TR38.901 Note: companies can also report to use RMa LOS defined in TR38.901 UMi NLOS defined in TR38.901. (1) Transmitter. [1D]; Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS; Refer to LLS table [0h1] and [0h2]; 1. [1E]; Total Tx Power (dBm); For standalone: 43 dBm For in-band: 38 or 33 dBm over R2D transmission BW Note: companies should report any PSD limitation they assume in their RAN1 evaluations; Device 2b: {-10, -20} dBm Device C: {-3, 0, 5} dBm. [1F]; Transmission Bandwidth used for the evaluated channel (Hz); Refer to LLS table [1a]; Refer to LLS table [2a1]. [1G]; Tx antenna gain (dBi); 17dBi; For A-IoT device, 0dBi. [1J]; Ambient IoT on-object antenna penalty; N/A; 0dB or 0.9dB or 4.7dB. [1N]; Cable, connector, combiner, body losses, etc. (dB); 3dB; N/A. [1M]; EIRP (dBm); Calculated (see Note 1); Calculated (see Note 1). (2) Receiver. [2A]; Number of receive antenna elements / TxRU / chains modelled in LLS; Same as [1D]-D2R; Same as [1D]-R2D. [2B]; Bandwidth used for the evaluated channel (Hz); Refer to LLS table [1b] ED bandwidth; Refer to LLS table [2a3]. [2C]; Receiver antenna gain (dBi); same as [1G]-D2R; Same as [1G]-R2D. [2X]; Cable, connector, combiner, body losses, etc. (dB); N/A; Same as [1N]-R2D. [2D]; Receiver Noise Figure (dB); Device 2b: 15 dB FFS: 40dB as additional assumption for Device 2b with IF-ED Device C: 13 dB; For BS as reader: 5dB. [2E]; Thermal Noise power spectrum density (dBm/Hz); -174; -174. [2E1]; Receiver interference density (dBm/Hz); [-169.3 dBm/Hz], [-999 dBm/Hz]; [-165.7 dBm/Hz], [-999 dBm/Hz]. [2F]; Effective Noise Power (dBm); Calculated (see Note 1); Calculated (see Note 1). [2G]; Required SNR/CNR; Reported by companies for Budget-Alt2; Reported by companies for Budget-Alt2. [2H]; Ambient IoT on-object antenna penalty; 0dB or 0.9 dB or 4.7 dB; Not applicable. [2J]; Budget-Alt1/ Budget-Alt2; Budget-Alt2; Budget-Alt2. [2L]; Receiver Sensitivity (dBm); Calculated (see Note 1); Calculated (see Note 1). (3) System margins. [3A]; Shadow fading margin (dB); For UMa NLOS: 6dB For RMa NLOS: 8dB For UMi NLOS: 7.82dB; For UMa NLOS: 6dB For RMa NLOS: 8dB For UMi NLOS: 7.82dB. [3B]; polarization mismatching loss (dB); 3 dB; 3 dB. [3C]; BS selection/macro-diversity gain (dB); 0 dB; 0 dB. [3D]; Other gains (dB) (if any please specify); Reported by companies with justification; Reported by companies with justification. [3E]; Penetration margin (dB); {0, 20, 80} dB; {0, 20, 80} dB. (4) MPL / distance. [4A]; MPL (dB); Calculated (see Note 1); Calculated (see Note 1). [4B]; Distance (m); Calculated (see Note 1); Calculated (see Note 1). （5）Other. [5A]; Other notes; Companies to report; Companies to report. Note 1: Calculated values are derived according to the following. [1M]: - For R2D, - [1M] = [1E] + [1G] - [1N] - For D2R - [1M] = [1E] + [1G] - [1J] [2F]: - [2F] = lin2dB(dB2lin([2D] + [2E]) + dB2lin([2E1])) +lin2dB([2B]) [2G] - For the R2D LLS for ED, CINR/CNR is reported, where CINR/CNR is defined as the ratio of signal power spectral density in the transmission bandwidth to the noise and interference (if any) power spectral density in the device ED channel bandwidth. - For R2D ZIF receiver, report the same metrics (i.e., CNR/CINR, signal transmission bandwidth, ED bandwidth) as agreed for RF-ED/IF receiver. - For the D2R LLS, the SINR/SNR is reported and it is defined as the ratio of signal power to noise and interference (if any) power in the receiver bandwidth. Receiver bandwidth is the bandwidth used at the reader side to filter the D2R signals for calculating noise and interference (if any) power. [2L]: - For R2D, - [2L] = [2G] - lin2dB([2B] / [1F]) + [2F] Note E: The term ‘lin2dB([2B] / [1F])’ is applied due to scaling from CNR/CINR to SNR/SINR. - For D2R, - [2L] = [2G] + [2F] [4A] - For R2D - [4A] = [1M] + [2C] - [2H] - [2L] - [3A] - [3B] + [3C] + [3D] - [3E] - For D2R - [4A] = [1M] + [2C] - [2X] - [2L] - [3A] - [3B] + [3C] + [3D] - [3E] [4B] - [4B] is derived by assuming pathloss is [4A] and using the pathloss formula as agreed. ### Agreement For Rel-20 study, “Coverage evaluation assumptions for link-level simulation” is as followings, Coverage evaluation assumptions for link-level simulation for Rel-20 study No.; Parameters; Assumptions. R2D/D2R common parameters. [0a]; Carrier frequency; Refer to link budget template. [0b]; SCS; 15 kHz as baseline. [0c]; Block structure; Blocks as agreed in Rel-19 with update reported by companies. [0d]; Channel model; TDL-C. [0e]; Delay spread; an RMS delay spread of 300 ns. [0f]; Device velocity; 3 km/h Companies can also report 10 km/h. [0g]; Number of Tx/Rx chains for Ambient IoT device; 1. [0h1]; BS; Number of antenna elements; 2 or 4. [0h2]; Number of TXRUs; 2 or 4. [0m]; Reference data rate; 1 kbps (M) 5 - 7 kbps (M) 48 - 60 kbps (O) 0.1 kbps for message size of 20 bits or 96 bits (O) Other data rates can be reported by companies Note 1: Companies to report the exact data rate. Note 2: The exact data rate is close to the values listed above. Note 3: The exact data rate is calculated by dividing the total message size (excluding CRC) by the total transmission time including applicable overheads(e.g., CRC, pre/mid/post-ambles if present). Note 4: The exact data rate may be related to coding scheme, repetition and etc. Note 5: All data rates considered are for evaluation purpose only FFS whether to add additional value(s). [0n]; Message size; {20 bits, 96 bits, 400 bits} are considered for message size. Note 1: companies to report the M value and chip length used for each message size Note 2: CRC is not included for the message size FFS whether to add value of 800 bits.. [0p]; BLER target; 1%, 10%. [0q]; Sampling frequency; Companies to report the sampling frequency (e.g., 1.92Msps or other feasible values if any) Initial SFO (Sampling Frequency Offset) (Fe): For Device 2b: Randomly select a value from the range of 10^3~10^4 ppm For Device C: [50] ppm SFO after clock sync/calibration: For Device 2b: 10^3 (M) ppm, 10^2 (O) ppm For Device C: [10] ppm Note: For random selection, the value is randomly selected per simulation drop, according to a uniform distribution Note: Above values are only for sampling purpose. Note: Above assumptions are only for LLS evaluation purpose only for R2D and D2R. The timing drift ΔT over a time T is modelled as ΔT = ±Fe * T. Note: SFO after clock calibration can be applied to Fe. Initial CFO: For Device 2b: Randomly select a value from the range of 10^3~10^4 ppm For Device C: [50] ppm CFO after clock sync/calibration: For Device 2b: 100 (M) ppm, 200 (O) ppm, 50 (O) ppm For Device C: [10] ppm Drift rate of 0.1 or 1 ppm/s. Companies to report which value they used. Note: Above assumptions are for LLS evaluation purpose only. [0r]; Device 2b/C; ZIF/IF-ED. R2D specific parameters. [1a]; Transmission bandwidth; 180kHz Other values e.g. 360kHz, 540kHz or other multiples of 180kHz are not precluded and up to company report. [1b]; ED bandwidth; Up to company report with considering: R2D transmission bandwidth Device Rx LO uncertainty (e.g. CFO ppm assumption). [1c]; BB LPF; [X]-order Butterworth/RC filter with cutoff frequency at half of ED bandwidth. Companies to report X = {3, 5}.. [1d]; Waveform; OOK waveform generated by OFDM modulator. [1e]; Modulation; OOK-4 Companies to report M chips per OFDM symbol. [1f]; Line code; Manchester. [1g]; FEC; Companies who report evaluations using FEC should also report evaluations without FEC. [1h]; ADC bit width; 4-bit. [1j]; Detection/decoding method for Line code; Companies to report. [1k]; Repetition; Companies who report R2D evaluations using repetition should also report R2D evaluations without repetition. D2R specific parameters. [2a1]; Transmission bandwidth; 15kHz Other values are not precluded and up to company report Companies to report which of 2SB or 1SB modulation is assumed for D2R transmission bandwidth. [2a2]; Chip rate; Companies to report. [2a3]; Receiver bandwidth; D2R receiver bandwidth is the bandwidth used at the reader side to filter out the D2R signals for calculating noise and interference (if any) power. Assume the receiver matches the transmitter's modulation, i.e., to receiver uses 1SB when transmitter uses 1SB, receiver uses 2SB when transmitter uses 2SB. Companies to report the value, and further down-selection of the values and 2SB/1SB is not precluded.. [2d]; Modulation; Companies to report modulation, e.g., OOK, BPSK. [2e]; Line code; Companies to report, e.g., Manchester encoding, no line coding. [2g]; FEC; Companies to report, e.g., CC, No FEC. [2h]; ADC bit width; Companies to report, e.g., 11-bit. [2j]; D2R receiver; Companies to report, e.g., coherent receiver / non-coherent receiver. [2k]; Repetition; Companies to report based on update of Rel-19 if any. Other assumptions. [3a]; Other assumptions; To be reported by company. [3b]; Note: Companies to report required SINR/SNR/CINR/CNR according to BLER target.. Agreement For Rel-20 study, the antenna height for BS and Device used for evaluation purpose are as follows: - BS antenna height - UMa: 25m - RMa: 35m - UMi: 10m - Device antenna height - 1.5m Agreement For Rel-20 study, the following layout assumption is used for evaluation purpose: - Hexagonal grid, 19 macro sites, 3 sectors per site - Note: the same layout assumption is used for UMi

In this contribution, we present our results on the analysis of both device 2b and device C in the outdoor deployment scenario for the in-band system. The study is performed using the following assumptions:

| Parameters | Values |
| --- | --- |
| BS transmit power (R2D) | 43 dBm |
| Transmission Bandwidth (R2D) | 180 kHz spanning across 1 PRB |
| M Value (R2D) | 2 |
| FEC code rate (R2D) | 1/3 |
| Bit level type-2 repetition (R2D) | 3 |
| Receiver interference density (R2D and D2R) | -169 dBm/Hz |
| D2R receiver architecture | Coherent receiver |
| Block size (in bits) | 96 |
| 1% BLER SNR (R2D) | 16 dB |
| 1% BLER SNR (D2R) | 0 dB |
| Data rate | 1 kbps |

2.1	Device 2b

The following observations were made for device 2b for UMa, UMi, RMa path loss models and penetration margins when deployed in an outdoor environment.

| Pathloss Model | UE Transmit Power (dBm) | Penetration Margin (in dB) | Range (in m) |
| --- | --- | --- | --- |
| 'UMi NLOS' | -10 | 0 | 499.07 |
| 'UMa NLOS' |  |  | 729.53 |
| 'RMa NLOS' |  |  | 1046.2 |
| 'UMi NLOS' | -20 |  | 242.48 |
| 'UMa NLOS' |  |  | 338.62 |
| 'RMa NLOS' |  |  | 576.46 |
| 'UMi NLOS' | -10 | 20 | 117.81 |
| 'UMa NLOS' |  |  | 157.17 |
| 'RMa NLOS' |  |  | 317.64 |
| 'UMi NLOS' | -20 |  | 57.24 |
| 'UMa NLOS' |  |  | 72.95 |
| 'RMa NLOS' |  |  | 175.02 |
| 'UMi NLOS' | -10 | 80 | 1.55 |
| 'UMa NLOS' |  |  | 1.57 |
| 'RMa NLOS' |  |  | 8.89 |
| 'UMi NLOS' | -20 |  | 0.75 |
| 'UMa NLOS' |  |  | 0.73 |
| 'RMa NLOS' |  |  | 4.90 |

2.2	Device C

The following observations were made for device C for UMa, UMi, RMapath loss models, and penetration margins when deployed in an outdoor environment.

| Pathloss Model | UE Transmit Power (dBm) | Penetration Margin (in dB) | Range (in m) |
| --- | --- | --- | --- |
| 'UMi NLOS' | -3 | 0 | 827.17 |
| 'UMa NLOS' |  |  | 1248.5 |
| 'RMa NLOS' |  |  | 1578.8 |
| 'UMi NLOS' | 0 |  | 1027.2 |
| 'UMa NLOS' |  |  | 1571.7 |
| 'RMa NLOS' |  |  | 1898.7 |
| 'UMi NLOS' | 5 |  | 1473.6 |
| 'UMa NLOS' |  |  | 2307.0 |
| 'RMa NLOS' |  |  | 2557.9 |
| 'UMi NLOS' | -3 | 20 | 195.27 |
| 'UMa NLOS' |  |  | 268.9 |
| 'RMa NLOS' |  |  | 482.1 |
| 'UMi NLOS' | 0 |  | 242.5 |
| 'UMa NLOS' |  |  | 338.6 |
| 'RMa NLOS' |  |  | 576.5 |
| 'UMi NLOS' | 5 |  | 347.9 |
| 'UMa NLOS' |  |  | 497.0 |
| 'RMa NLOS' |  |  | 776.6 |
| 'UMi NLOS' | -3 | 80 | 2.6 |
| 'UMa NLOS' |  |  | 2.7 |
| 'RMa NLOS' |  |  | 13.5 |
| 'UMi NLOS' | 0 |  | 3.2 |
| 'UMa NLOS' |  |  | 3.4 |
| 'RMa NLOS' |  |  | 16.1 |
| 'UMi NLOS' | 5 |  | 4.6 |
| 'UMa NLOS' |  |  | 5.0 |
| 'RMa NLOS' |  |  | 21.7 |

Observations

Observation 1: The minimum coverage for feasible deployment of the AIoT device 2b and device C should be at least equal to half the ISD as per the deployment scenarios defined in TR 38.901.

| Scenario | ISD (in m) as per TR 38.901 | Minimum Coverage Distance (in m) |
| --- | --- | --- |
| UMi | 200 | 100 |
| UMa | 500 | 250 |
| RMa | 1732 | 866 |

Table 1

Observation 2: For the UMi and UMa deployment scenarios with 0 dB penetration margin, the required coverage as per Table 1 can be achieved using both device 2b and device C across all possible UE transmit power configurations.

Observation 3: For the RMa deployment scenario with 0dB penetration margin, device 2b needs to transmit the signal at -10 dBm to achieve the required coverage as per Table 1, whereas this limitation does not apply to device C.

Observation 4: When a penetration loss of up to 20 dB is considered, device 2b can be deployed only in the UMi scenario. The device C, unlike device 2b, depending upon the deployment scenario, can achieve the required coverage as per Table 1  by transmitting at a higher power.

Observation 5: A penetration loss of 80 dB would impose an excessive load on the receiver in both D2R and R2D cases, to the extent that no AIoT device could be deployed in any environment, making such deployment infeasible. Since the 80 dB penetration loss was considered for studying scenarios with AIoT devices inside metallic containers, which cannot be supported by existing techniques, alternative feasible methods must be explored to enable this use case.

- Conclusion

This paper provides the following early observations from the evaluation on AIoT outdoor deployment:

Observation 1: The minimum coverage for feasible deployment of the AIoT device 2b and device C should be at least equal to half the ISD as per the deployment scenarios defined in TR 38.901.

| Scenario | ISD (in m) as per TR 38.901 | Minimum Coverage Distance (in m) |
| --- | --- | --- |
| UMi | 200 | 100 |
| UMa | 500 | 250 |
| RMa | 1732 | 866 |

Table 1

Observation 2: For the UMi and UMa deployment scenarios with 0 dB penetration margin, the required coverage as per Table 1 can be achieved using both device 2b and device C across all possible UE transmit power configurations.

Observation 3: For the RMa deployment scenario with 0dB penetration margin, device 2b needs to transmit the signal at -10 dBm to achieve the required coverage as per Table 1, whereas this limitation does not apply to device C.

Observation 4: When a penetration loss of up to 20 dB is considered, device 2b can be deployed only in the UMi scenario. The device C, unlike device 2b, depending upon the deployment scenario, can achieve the required coverage as per Table 1  by transmitting at a higher power.

Observation 5: A penetration loss of 80 dB would impose an excessive load on the receiver in both D2R and R2D cases, to the extent that no AIoT device could be deployed in any environment, making such deployment infeasible. Since the 80 dB penetration loss was considered for studying scenarios with AIoT devices inside metallic containers, which cannot be supported by existing techniques, alternative feasible methods must be explored to enable this use case.

# 4. References

[1] RP-251884, “New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”, Moderator (RAN1 Vice-Chair), RAN#108.

[2] TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”.

[3] Chair notes RAN1#122