3GPP TSG RAN WG1 #122bis			R1-2507947

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	11.5

Source:	CAICT

Title:	Discussion on energy efficiency and energy saving

Document for:	Discussion and Decision

- Introduction

In RAN #108 meeting, New SID: Study on 6G Radio is approved [1]:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

In this contribution, we discuss on energy efficiency from Network, UE and joint network and UE perspectives.

- Network energy saving

Considering the energy saving from network perspective, it can be summarized as the adaptation from multiple domain resources.

In the frequency domain, the carriers and bandwidths can be adapted according to traffic load and power saving requirements. Additionally, to better enable the dynamic on/off of carriers, cells and channels, two or more layers of network can be considered, for example, the first layer can be the coverage carrier supporting low data rate, wide coverage with continuous mobility, and the second layer can be the capacity carrier supporting high data rate and small coverage. By using this architecture, the coverage carrier can transmit always on RS, and the capacity carrier can transmit on demand RS to achieve high data rate, which can save network energy.

In the time domain, different from 5G NR, the SSB signal can be transmitted in a longer periodicity with on demand SSB. In 5G NR, the periodicity of SSB UE assumed when performing initial cell selection is 20ms, the network has to transmit no larger than 20ms, even there is not one UE in the cell or the traffic load is low, which cannot effectively save network energy. So the network can transmit SSB with longer periodicity such as 40ms, 80ms or 160ms to achieve better network energy efficiency. And UE can trigger on demand SSB when needed to get equally good initial access performance.

Jointly considering with frequency domain, the relatively long periodicity always on SSB can be transmitted in the coverage carrier, and on demand SSB with or without the longer periodicity always on SSB can be transmitted in the

capacity carrier, and the SIB1 transmission can also be on demand transmission to save energy. In this case, the trade-off between network energy saving and UE complexity needs to be considered, and also the impact to UE initial access needs to be evaluated when defining the long periodicity of SSB and on demand SSB/SIB1.

In the spatial domain, dynamic switching can be implemented across different dimensions such as cell/TRP/transceiver channels/beam, etc.

- Proposal 1: the coverage and the capacity carrier can be defined to enable dynamic on/off carrier, the relative long periodicity always on SSB can be transmitted in the coverage carrier, and the on demand SSB with or without SSB signal with longer periodicity and can be transmitted in the capacity carrier.

- UE energy saving

In 5G-Advanced，there are several UE energy saving technologies introduced in different releases starting from Rel-16. Downlink wake up signals have been introduced in Rel-16 for connected mode UE to indicate whether the UE needs to skip the C-DRX active time. The paging early indication has been introduced in Rel-17 for idle mode UE to indicate whether the UE needs to skip receiving paging signals. The Low power WUS has been introduced in Rel-18 and Rel-19 to indicate whether the connected mode UE needs to skip PDCCH monitoring and whether the idle mode UE needs to skip receiving paging signals. We can see that in 5G, there are too many UE energy saving technologies even addressing the same type of issues, so in Rel-20 6G study item, we could focus on one effective UE energy saving method, for example whether the LP-WUS scheme can be used to replace C-DRX to provide maximum energy saving gain, since we can see that LP-WUS can achieve similar function to C-DRX with more energy saving gain of UE. In this case, the measurement needs to be enhanced for LP-WUR.

For UE energy saving, the reduction of unnecessary PDCCH monitoring is important to save energy, considering the different types of 6G traffic, the PDCCH monitoring needs to be adapted, when there is no or little traffic, the UE needs not to monitor PDCCH, and the UE need to timely get the downlink control information when the traffic arrives.  Two stage control indication can be specified, the detection of DCI can be indicated by low power pre-indication, which is carried by low power receiver to simply indicate ON/OFF.

- Proposal 2: the two stage PDCCH can be specified, low-power pre-indication before full control decode.

- Joint NW and UE energy saving

In 5G-Advanced, the network energy saving and UE energy saving are enhanced separately, we think in Rel-20 6G study item, they can be jointly considered to achieve the best energy saving gain. The first example is that the coordination of base station DTX and UE DRX may be needed to align the active and dormant periods between network and UE to ensure that both network and UE can enter low-power states simultaneously during periods of inactivity. The second example is on-demand transmission of downlink signals by the base station, the UE can send an uplink wake-up signal to trigger the base station to transmit signals such as SSB, SIB1, or CSI-RS only when needed. In this case, the UE can save energy by requesting signals only when required for its operations, and the base station gains more opportunities to save energy by avoiding unnecessary periodic transmissions. The third example is that the network and UE can cooperate to dynamically turn on or turn off the different cells of coverage cell and capacity cell based on the UL WUS of UE.

- Proposal 3: the joint NW and UE energy saving method needs to be considered in 6G study item.

- Conclusion

In this contribution, we give following proposals of energy saving of 6G Rel-20 SI:

- Proposal 1: the coverage and the capacity carrier can be defined to enable dynamic on/off carrier, the relative long periodicity always on SSB can be transmitted in the coverage carrier, and the on demand SSB with or without SSB signal with longer periodicity and can be transmitted in the capacity carrier.

- Proposal 2: the two stage PDCCH can be specified, low-power pre-indication before full control decode.

- Proposal 3: the joint NW and UE energy saving method needs to be considered in 6G study item.

- References

- RP-251881 New SID: Study on 6G Radio