3GPP TSG RAN WG1 Meeting #123	R1-2508331

Dallas, USA, November 17-21, 2025

Agenda Item:	10.3.2.2

Source:	Futurewei

Title:	Discussion on D2R Air Interface for Device 2b/C

Document for:	Discussion and Decision

# Introduction

The objectives of the Ambient IoT study item [1] and work item [2] are listed in Appendix A. The RAN1-related objectives are being treated across four agenda items. The description for this agenda item is:

Including necessary and feasible change to D2R waveform and modulation, FEC, CRC and repetition, bandwidth, timing and Sync signals, multiplexing/multiple access, and scheduling. Topics include:

- D2R waveform and modulation

- Timing and synch signals

- CDMA (for data)

- D2R FEC

- Scrambling

This contribution provides observations and proposals based on the objectives of the Rel-20 study and work items, the agreements from RAN1#122 [3]. RAN1#122bis [4], and the feature lead summary [5].

# Discussion

## D2R waveform and modulation

In Rel-19, OOK, FSK, and BPSK modulation schemes were examined in the study phase. For the normative phase, both BPSK and OOK were standardized for D2R transmission. It is up to UE implementation to use either OOK or BPSK for Msg1, but the same modulation scheme is maintained by the UE for subsequent D2R transmissions.

For Rel-20, other modulation schemes such as MSK and DPBSK are being considered in the study for D2R modulation enhancement as well as OOK and BPSK.

The following agreement was reached at RAN1#122b.

Agreement Regarding D2R modulation for device 2b and device C, further study the following: - Option 1: BPSK - FFS the corresponding preamble and midamble design for coherent detection at reader side - Option 2: OOK - FFS the corresponding preamble and midamble design for coherent or non-coherent detection at reader side - Option 3: DBPSK - FFS the corresponding preamble and midamble design for non-coherent detection at reader side - Option 4: MSK - o FFS the corresponding preamble and midamble design for coherent or non-coherent detection at reader side

BPSK is the simplest form of phase shift keying (PSK). It uses two phases which are separated by 180° and has a constant envelope. Modulation is performed using the two phases. Coherent detection is needed at the receiver side to detect the phase of each bit. It offers a 3 dB advantage over OOK modulation due to its constant envelope. However, it suffers detection performance degradation when there is a carrier frequency offset (CFO) between the carrier frequency at the transmitter and the carrier frequency at the receiver. Note that in Rel-19 CFO is not an issue for Device 1 since the CW source is external to the device. In Rel-20, A-IoT devices are active devices and have their own local oscillators. The RF signal is generated by the devices.

The phase rotation caused by the CFO can be expressed as follows:

φrotation=ej(2π∆ft)

where ∆f is the frequency difference between the carrier frequencies at the transmitter and the receiver. Using Device C as an example, the CFO after calibration is 10 ppm based on the agreement from the previous meeting. At 900 MHz, the phase rotation caused by the CFO for a D2R bit duration of 133.3 μs is 1.2π. Therefore, preamble and midamble sequences must be designed to support CFO estimation at the receiver to maintain BPSK reception performance.

On the other hand, OOK modulation offers robust detection performance against CFO due to its non-coherent detection nature. OOK transmitter offers lower complexity and low power consumption. The modulation scheme is especially suited for Device 2b whose calibrated CFO is at 100 ppm and is high.

Considering OOK and BPSK offer different solutions for Device 2b and Device C, we have the following proposals.

Proposal 1: OOK can be supported as the D2R modulation scheme for Device 2b in Rel-20.

Proposal 2: BPSK is supported as the D2R modulation scheme for Device C in Rel-20.

To deal with the CFO impact on BPSK reception, DBPSK was proposed in previous meetings. DBPSK is a non-coherent detection scheme and uses the relative phase change between the adjacent D2R chips. The idea for DBPSK modulation is similar to, in our opinion, Manchester coding scheme where the relative amplitude difference is used to decode the information. Our view is that further study is needed for Device 2b to evaluate the transmitter complexity and power efficiency, performance comparison with other modulation such as OOK and the impact on preamble and midamble design.

Minimum-shift keying (MSK) is a type of continuous-phase frequency-shift keying is encoded with bits alternating between quadrature components, with the Q component delayed by half the symbol period. In terms of detection performance, MSK is the same as BPSK. However, the higher complexity in transmitter implementation makes it an unattractive solution for low power A-IoT devices. In addition, the detection performance with large CFO is unclear. Considering that MSK does not offer better performance than BPSK, we propose no further study on MSK.

Proposal 3: MSK is not supported as a D2R modulation scheme for Device 2b and Device C in Rel-20.

In Rel-19, D2R transmission is performed by backscattering an external CW source. To support FDMA, a so-called small frequency shift (SFS) is used. Bits are modulated by a square waveform with an SFS factor where the SFS factor determines where frequency location of the bit. SFS is no longer needed in Rel-20 as active devices have their own local oscillators, and these devices can modulate the bits to a desired frequency location by adjusting the local oscillator. In another words, there is no need to use the square wave modulated waveform.

Proposal 4: Small frequency shift used in Rel-19 is not supported for Device 2b and Device C in Rel-20.

At RAN1#122b, the following agreement was reached.

Agreement The study assumes single-carrier waveform for D2R. - FFS whether to use 2SB or 1SB for D2R

In Rel-19, 1SB and 2SB were discussed with the context of backscattering on D2R. Some additional hardware is needed to implement 1SB. To reduce the amount of frequency resources, it is desirable to support 1SB. 1SB is feasible now with the active devices.

Proposal 5: 1SB for D2R can be supported for Device 2b and Device C in Rel-20.

## CDMA (for data)

Several companies proposed studying CDMA in both RAN1#122 and RAN1#122bis. The following study agreements are from RAN1#122bis [4].

D2R: CDMA Agreement Study necessity and feasibility of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission and potential impact to random access procedures. - FFS: other cases, except for data transmission Agreement For the study of binary sequence-based Msg1 for the study of necessity and feasibility of D2R CDM(A) for Device 2b and for Device C, study at least the following aspects: - Sequence type (e.g. Gold sequence, m-sequence) - Length of the sequence - Number of sequences

The contention based access procedures in Rel-19 support both FDMA and TDMA. A device transmits a 16-bit random number (RN16) and the 6 bit CRC in Msg1. The device would proceed to transmit Msg3 if it detected that its transmitted RN16 number is echoed back in Msg2. The CRC eliminates echoing an RN16 that was received in error.

One of the motivations for CDMA is to increase the capacity of Msg1 transmission. In reality, the limiting factor for contention based access is the capacity for Msg3 as this message is about 6-8 times longer than Msg1 and possibly 15-20 times longer for security purposes.

With CDMA, the contention based access procedures would have to change. Instead of RN16 being transmitted, a code from a set of codes is used. The reader indicates which codes it received in Msg2. In addition, it may be necessary to indicate which time domain resource and frequency domain resource the code was received on. This can complicate the design of Msg2. Furthermore, the number of codes available may be limited as the reader has to process the received signal against the number of codes.

Other considerations include sensitivity to the range of power levels in the received CDMA signals. Without power control (e.g., open loop), the near / far issues can impact detection. In addition, the effects of CFO need to be evaluated for code orthogonality.

Proposal 6: Capture the following observations in the TR regarding necessity and feasibility of D2R CDM(A):

- For the contention based access procedure, CDMA could increase the number of Msg1 transmissions. However, the bottleneck for contention based access procedure is the transmission of Msg3, whose size is an order of magnitude larger than Msg1.

- Using CDMA would change the contention based access procedure for Msg2 and scheduling Msg3.

- The sensitivity to near – far and CFO needs additional study.

## D2R FEC

Two study agreements about D2R FEC were captured in RAN1#122bis [4].

FEC Agreement Study interleaving for PDRCH with FEC, including the necessity and feasibility of the following: - Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme - Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver - Alt 3: based on LTE bit collection scheme with some update (e.g., for memory reduction) without LTE sub-block interleaver - Combination of alternatives can be studied Agreement For R2D and D2R, study the order of channel coding (including interleaver if supported) and block-level repetition (if supported) for complexity reduction, considering necessity and feasibility.

The discussion of interleaving, repetition, and encoding order is inter-related. The dimensions (memory) of the sub-block interleaver are related to the number of constituent polynomials in the encoder and the number of bits (smallest integer multiple of 32 of the output of each constituent polynomials). If r represents the coding rate of the encoder, and N is the number of input bits into the encoder, then the total amount of memory for the interleaver is

32rN32

It is clear that having a repetition block before or after the TBCC encoder will increase the memory requirements of the subblock interleaver.

In LTE, since the sub-block interleaver is memory, the output sequence can be read repeatedly for repetition in the bit collection. The same concept can be applied to A-IoT. It is possible to generate various rates such as 1/2, 1/3, and 1/4 without modifying the TBCC encoder.

At the receiver, the reader would route soft bits to appropriate sub-block interleaver. It is possible to combine repeated soft bits as they are stored in the deinterleaver, as shown in Fig. 1.

CRC

Sub-block interleaver

CC encode

Bit collect / repeat

route

combine

Sub-block deinterleaver

CC decode

CRC

Fig. 1. Preferred encoding and decoding steps.

Proposal 7: Capture the following observation in the TR for interleaving:

- Reusing both LTE sub-block interleaver and LTE bit collection scheme enables a device to eliminate the repetition block and associated memory as bit collection scheme can use the memory for the sub-block interleaver for repeated bits. Another benefit is the various coding rates are achievable without modifying the encoder specification.

Fig. 1 shows the preferred ordering that reduces memory and processing complexity.

Proposal 8: For D2R, the preferred ordering for encoding is CRC encoding, convolutional encoding, LTE sub-block interleaving (if supported), and bit collection (which replaces block repetition) from a complexity perspective.

## Scrambling

One agreement about scrambling on D2R link was captured in RAN1#122bis [4].

Agreement For PDRCH, further study the necessity of scrambling with following details: - Scrambling applied after block-level repetition (if applied) and channel coding (if applied, including interleaving if supported) - Scrambling sequence type and generation method

While scrambling is a low complexity operation, the points of discussion are its benefits, how to generate the scrambling pattern (e.g., the seed), and when it is applied.

Scrambling tends to keep the number of transmitted ones and zeros (before modulation) approximately equal and can randomize the modulation symbols used. Following the approach in NR, scrambling is applied just before modulation (long after encoding / rate matching). If scrambling is supported for A-IoT, scrambling should be applied just before modulation.

One question is whether scrambling is applied to the Msg1 content. As the Msg1 content is a random number, applying a scrambling code makes no sense.

In NR, once the seed is provided, the PN generator advances 1600 clocks before scrambling is applied. Although there are mathematical approaches to advance the 1600 clocks using a lookup table and ~30 exclusive ORs, it is preferable to reduce the order of the generator polynomial (log2 of the maximum number of bits transmitted) and to avoid having a clock advance for complexity considerations. For 1000 bits and an extreme value for the coding rate of 1/16, a degree 16 generator polynomial is more than adequate.

Proposal 9: Capture the following observation into the TR regarding the details for scrambling for the PDRCH

- When enabled, scrambling should be after all the bits are encoded/repeated (i.e., just before modulation)

- The degree of the generator polynomial should be no larger than 16

- The output of the scrambler should not be advanced

- The necessity of applying scrambling to Msg1 contents is unclear.

# Conclusion

The following observations and proposals were discussed in this contribution.

D2R Modulation

Proposal 1: OOK can be supported as the D2R modulation scheme for Device 2b in Rel-20.

Proposal 2: BPSK is supported as the D2R modulation scheme for Device C in Rel-20.

Proposal 3: MSK is not supported as a D2R modulation scheme for Device 2b and Device C in Rel-20.

Proposal 4: Small frequency shift used in Rel-19 is not supported for Device 2b and Device C in Rel-20.

Proposal 5: 1SB for D2R can be supported for Device 2b and Device C in Rel-20.

CDMA

Proposal 6: Capture the following observations in the TR regarding necessity and feasibility of D2R CDM(A):

- For the contention based access procedure, CDMA could increase the number of Msg1 transmissions. However, the bottleneck for contention based access procedure is the transmission of Msg3 as the message 6-8 times longer.

- Using CDMA would change the contention based access procedure for Msg2 and scheduling Msg3.

- The sensitivity to near – far and CFO needs additional study.

FEC

Proposal 7: Capture the following observation in the TR for interleaving:

- Reusing both LTE sub-block interleaver and LTE bit collection scheme enables a device to eliminate the repetition block and associated memory as the bit collection scheme can use the memory for the sub-block interleaver for repeated bits. Another benefit is the various coding rates are achievable without modifying the encoder specification.

Proposal 8: For D2R, the preferred ordering for encoding is CRC encoding, convolutional encoding, LTE sub-block interleaving (if supported), and bit collection (which replaces block repetition) from a complexity perspective.

Scrambling

Proposal 9: Capture the following observation into the TR regarding the details for scrambling for the PDRCH

- When enabled, scrambling should be after all the bits are encoded/repeated (i.e., just before modulation)

- The degree of the generator polynomial should be no larger than 16

- The output of the scrambler should not be advanced

- The necessity of applying scrambling to Msg1 contents is unclear.

# References

RP-252964, “Revised SID: Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”, LGE, RAN#109, Sep. 15-18, 2025.

RP-252894, “Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2”, Huawei, RAN#109, Sep. 15-18, 2025.

R1-2506567, “Session notes for 10.3 Study of Enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”, Chair, RAN1#122, Aug. 25-29, 2025.

R1-2508157, “Session Notes of AI 10.3”, Chair, RAN1#122bis, Oct. 13-17, 2025.

R1-2508175, “FL summary #9 for 10.3.2 'Study of air interface for Device 2b/C'”, LG Electronics, RAN1#122bis, Oct. 13-17, 2025.

# Appendices

## SID objectives [1]

A-IoT Devices These are the A-IoT Devices referred to in the objectives: - Device 1: As standardized in the Rel-19 WI Ambient_IoT_Solutions. ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - Device C: ≤ 1 mW to ≤ 10 mW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - SFO value 10Y is assumed to be better than any SFO value considered for Device 1 standardized in Rel-19. Objectives 1. Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2. This objective begins after RAN#108 (June 2025) RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage. RAN4-led - Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios. - Use band n8 as an example band. NOTE: Strive to minimize evaluation cases in RAN1 and RAN4. 2. Identify D2R RSRP-like measurement, and the involved A-IoT signal(s)/channel(s), which are feasible for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID [RAN1]. - RAN1 is not expected to conclude on the positioning accuracy as part of the identification of feasible D2R RSRP-like measurement - Note: Device localization based on Reader ID as specified in Rel-19 can be supported for active device(s) in Rel-20 with no additional work. - The design of D2R signal(s)/channel(s) for active devices is not impacted by the identification of the D2R RSRP-like measurement - This objective does not include the study of dedicated Device procedures for Device localization - The objective assumes no dedicated positioning architecture will be specified in Rel-20 - The objective assumes no specification of inter-Reader coordination for Device localization in Rel-20 - This objective will start at RAN1#124

## WID objectives [2]

A-IoT Devices These are the A-IoT Devices referred to in the objectives: - Device 1: As standardized in the Rel-19 WI Ambient_IoT_Solutions. ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - Device C: ≤ 1 mW to ≤ 10 mW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - SFO value 10Y is assumed to be better than any SFO value considered for Device 1 standardized in Rel-19. Work on objectives for active devices and outdoor scenarios will not start until the completion of the study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices. Objectives 1. Specify necessary support for active Device(s), taking the Rel-19 specifications as the starting point, in indoor scenarios under the following conditions [RAN1]: - Deployment scenario 1 with topology 1, according to D1T1-C. - Traffic types DO-DTT, DT, DO-A. - For DO-A traffic, WGs begin discussions from Clause 6.10 “DO-A assessment” in TR 38.769. - Representative use cases rUC1 (indoor inventory), rUC2 (indoor sensor), and rUC4 (indoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing indoor NR micro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. This objective begins after RAN#111 (March 2026) This objective includes: RAN1-led - In specifying support for the above, re-use the evaluation assumptions of TR 38.769 - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Air interface for active device(s) studied for outdoor will be reused for supporting active device(s) in indoor scenarios RAN2-led - DO-A specific procedure design. - Paging and random access for active device(s). RAN3-led - Specify DO-A specific NGAP procedure. RAN4-led - Study coexistence between active Device(s) and NR/LTE in indoor scenarios [RAN4-led] - Specify RF requirements for Type 1-C Ambient IoT BS where differences are needed from Rel-19. - Specify RF requirements for active Device(s) with IF/ZIF receiver architecture - Specify RRM core requirements for active Device(s) in D1T1 - Use band n8 as an example band - Study and specify the test methodology for AIoT active devices NOTE: Strive to minimize evaluation cases in RAN1 and RAN4. 2. Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify RRC-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1. The work is subject to the following constraints: RAN1 - The objective includes specification of necessary power control and collision handling procedures between intermediate UE and BS. - No physical layer impact for signaling A-IoT resources to the intermediate UE. - No additional specification change on R2D and D2R for Device 1 in Rel-20 RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling. RAN3 - Specify signaling for Topology 2 - The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection. - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized. - Inter-gNB resource coordination is not specified. - Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility RAN4 - Specify RF requirements for intermediate UE towards the A-IoT device. - Specify necessary updates to RF requirements for A-IoT device 1. - Specify A-IoT active device(s) RF requirements in D2T2 - Specify RRM requirements for intermediate UE due to A-IoT operation, if any. Combination with existing RRM procedures/requirements is excluded. - Specify RRM requirements for Device(s) in D2T2 if necessary - Use band n8 as an example band. 3. Specify device (un)availability via Direction 2 in TR 38.769, [RAN2-led, RAN1]. - This objective is applicable only for active Device(s) - Solutions shall be unified for Topology 1 and Topology 2. - This objective begins after RAN#111 (March 2026) - NOTE: Device for outdoor scenarios may be added here, pending RAN#111 decision. Note: Coexistence between Device 1 and Device 2b/C is not considered in the same deployment in the same band.

## Agreement [3]

Agreement For R2D, study necessity of addition/removal of M values from Rel-19 M value set for Device 2b and for Device C for at least the following M values: - Addition of M=1 - Addition of M=32 - Removal of M=24 Agreement Study necessity and feasibility of FEC for R2D for Device 2b and for Device C including the following aspects: - Coding schemes: convolutional code (e.g TBCC), RM code - Note: focus on coding schemes already specified in LTE or NR - FEC code rates: at least 1/2, 1/3 - Interleaver or LTE bit collection scheme for TBCC Agreement Study necessity and feasibility of R2D repetitions for R2D for Device 2b and for Device C considering the following repetition types: - Block-level repetitions - Bit-level Type-2 repetitions - Chip-level repetitions (in case of M=1 with FEC if applied) Agreement For R2D signal(s), study at least the following functionalities (if needed), and study whether to reuse/enhance existing R2D signal(s) or to introduce new R2D signal(s) for the following functionalities (if needed): - R2D chip duration determination - indication of the start of R2D - indication of the end of PRDCH - frequency synchronization (including frequency acquisition) - CFO estimation / LO calibration - SFO calibration - timing synchronization/tracking - measurement (e.g., signal strength) - reader identification/differentiation Agreement For D2R signal(s), study at least the following functionalities (if needed), and study whether to reuse/enhance existing D2R signal(s) or to introduce new D2R signal(s) for the following functionalities (if needed): - CFO estimation - SFO estimation - timing acquisition - channel estimation - measurement (e.g., signal strength, interference estimation) - Device differentiation e.g. for collision resolution, for interference randomization Agreement Study necessity and feasibility of reuse, modifications or enhancements of existing Rel-19 D2R modulations, and the potential introduction of new modulations schemes, for Device 2b and for Device C, including at least the following aspects: - introduction of new modulation schemes (e.g., MSK or DBPSK) - down-selection of existing modulation schemes - design considerations related to spectrum shaping and D2R signals Agreement Study the following aspects to support DO-A traffic (periodic and event-triggered) for Device 2b and for Device C in Rel-20. - Timing and frequency synchronization for DO-A transmission - Resource allocation/determination method for DO-A transmission - Configuration/signaling of necessary information Agreement Study necessity and feasibility of enhancements on existing Rel-19 D2R FEC for Device 2b and for Device C including the following aspects: - additional FEC code rate(s) (e.g., 1/2, 1/4) - interleaver, or LTE bit collection scheme for TBCC - Note: no change to Rel-19 TBCC polynomial Agreement For PDRCH, study necessity and feasibility of larger (>2) number of repetitions for block-level repetitions for Device 2b and for Device C. Agreement For PDRCH, study necessity of scrambling for Device 2b and for Device C. Agreement At least for CBRA for DT and DO-DTT, study necessity and feasibility of enhancements of D2R TDM(A) for Device 2b and for Device C based on the following: - X1 > 2 for Msg1 (X1: number of time domain resources for Msg1 transmission indicated by the corresponding R2D message) - X3 > 1 for Msg3 (X3: number of time domain resources for Msg3 transmission indicated by the corresponding R2D message) Agreement Study necessity and feasibility of enhancements of D2R for Device 2b and for Device C, considering the following aspects or potential enhancements: - Considering that Device 2b and Device C can adjust the LO towards a desired frequency, whether to also support baseband small FS for Device 2b and Device C - D2R frequency resources for FDM(A) (e.g., considering D2R transmission bandwidth, guard-band size between FDMed D2R transmissions, CFO, SFO, etc.) - How a device determines the frequency resource for D2R transmission - FDM(A) for D2R transmissions other than Msg1 and Msg3 Agreement At least for DT and DO-DTT, study necessity and feasibility of changes to existing timing offsets for Device 2b and for Device C, e.g., - timing offset value between R2D and corresponding subsequent D2R (e.g., Toffset1/2/3/4) - timing offset value between D2R and corresponding subsequent R2D (e.g., T_D2R_min) - Note: RAN1 only discusses potential reasons that may impact the value of T_D2R_min, and if any provides those reasons to RAN4 - timing offset between R2D and corresponding subsequent D2R provided by a reader - timing offset between D2R and corresponding subsequent R2D provided by a reader At least for DT and DO-DTT, study necessity and feasibility of introduction of new timing offsets for Device 2b and for Device C, e.g., - timing offset between two consecutive R2D or D2R transmissions for the same Device, if such scenario exists