3GPP TSG RAN WG1 #123		                   R1-2508384

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	10.5.1

Source:	Spreadtrum, UNISOC

Title:	Discussion on evaluation assumption and performance evaluation for ISAC

Document for:	Discussion and decision

Introduction

The SID of Study on Integrated Sensing And Communication (ISAC) for NR was approved in RAN#108 [1].

In this contribution, we will present initial evaluation results and some discussion on remaining issues based on current agreements obtained in RAN1#122 [2] and RAN1#112 bis [3].

Discussion

Evaluation assumptions and initial results

Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. BS antenna configuration; For 4GHz, 4.9GHz: - Option 1 as optional - Tx: (12,16, 2,1,1;2,16) - Rx: (12,16,2,1,1;2,16) - Option 2 - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8); For 6GHz: - Option 1 as optional - Tx: (16,16, 2,1,1;4,16) - Rx: (16,16,2,1,1;4,16) - Option 2: - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8). Mandatory: dH,dV = (0.5, 0.8)λ, +45°/-45° polarization Optional: dH,dV = (0.5, 0.5)λ, +45°/-45° polarization. - Note: Tx antenna elements and Rx antenna elements are operating simultaneously Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. BS antenna radiation pattern; Table 9 in Report ITU-R M.2412. BS antenna mechanic tilt (downtilt angle without electrical tilt); 90° in GCS (pointing to horizontal direction). BS antenna electrical tilt (downtilt angle after mechanic and electrical tilt); Option 1: no electrical tilt Option 2: 102° in GCS. Polarized antenna model; Model-2 in clause 7.3.2 in TR 38.901. Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. gNB-target link; gNB-aerial UE of UMa/RMa parameters in 36.777 with Alternative 3. Concatenation of TX-target and target-RX links; Up to company choice between two options for concatenation defined in Step 9 in section 7.9.4.1, TR 38.901. The power threshold for path dropping after concatenation for target channel; -25dB and -40dB are respectively used for the two options for concatenation. Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Co-site inter-sector interference; Not modelled. Adjacent channel interference; Not modelled. BS receiver noise figure; 5dB. Agreement When Tx/Rx operates simultaneously (baseline case), the following parameters are used for the evaluation on NR ISAC: Antenna isolation; - Option 1: 65dB - Option 2: 80dB. Maximum BS Tx power; - Option 1: 37dBm - Option 2: 52dBm The above options are calculated with BS_maxpower = BS Rx saturation power + antenna isolation by assuming the BS Rx saturation power = -28dBm and the antenna isolation = 65dB and 80dB, respectively. - Optionally: companies should report the maximum BS Tx power when it is assumed that Tx and Rx don’t operate simultaneously. Companies should report how the maximum BS Tx power is derived. Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. CPI (coherent processing interval); Companies to report the CPI value assumed. Waveform; CP-OFDM as baseline, other waveform is up to companies report. - Agreement When Tx/Rx operates simultaneously, the following evaluation parameters are agreed for the evaluation on NR ISAC. Self-interference; The residual leakage interference/noise is modelled e.g. by additional additive white Gaussian noise, -94+X dBm in 100 MHz, X is up to company report. Companies to provide details on their modelling. Note: X = -Infinity corresponds to not modelling self-interference. Agreement Companies are encouraged to report the high-level sensing signal/data processing method used in the evaluation, e.g., 2D FFT, MUSIC, and any other methods. Agreement Up to company to model low power cluster in the evaluation of NR ISAC Agreement Companies should report details of the Tx beam information (number of Tx beams, wide/narrow Tx beam) being used at TRP. - The details of the Tx beam information (number of Tx beams, wide/narrow Tx beam) being used at TRP will be captured along with the corresponding evaluation results in the Rel-20 TR Agreement The following performance objectives are adopted for evaluation purpose of NR ISAC. Metric; Value. Missed detection Probability; [5]%. False Alarm Probability Type 1; [5]%. False Alarm Probability Type 2; [5]%. Horizontal Positioning Accuracy; [10] m with confidence level 90%. Vertical Positioning Accuracy; [10] m with confidence level 90%. Velocity Accuracy; [5] m/s with confidence level 90%.

In RAN1#122bis meeting [3], some basic simulation assumptions had reached the agreements, some other assumptions are up to company report. In the simulation, the maximum BS Tx power was set to 52dBm, the coherent processing interval (CPI) was set to 25ms, and a CP-OFDM waveform was utilized. For interference and noise modeling, residual leakage was modelled by an additional white Gaussian noise (AWGN) level of -91 dBm (defined as -94 + 3 dBm) within a 100 MHz bandwidth. Range and velocity estimation were conducted using 2D-FFT algorithm, while angle estimation was performed via the MUSIC algorithm. Regarding the beam configuration, the Tx beam was set as a wide beam, i.e., BS antenna configuration option 2, and aimed at the target. Finally, the selection of the Tx and Rx was determined based on the geographical location of target.

The CDF of horizontal localization error, vertical localization error and radial velocity error are given in Figure 1 (a), (b), (c), respectively.

| (a) | (b) | (c) |
| --- | --- | --- |

Figure 1 Simulation results

Table 1 summarizes the sensing performance at 4.9 GHz based on the simulation assumptions.

Table 1 Sensing performance

| 4.9GHz |  | Performance |
| --- | --- | --- |
| Horizontal localization error [m] | @90% | 7.429 m |
|  | @95% | 17.356 m |
| Vertical localization error [m] | @90% | 9.619 m |
|  | @95% | 23.292 m |
| Radial velocity error [m/s] | @90% | 5.384 m/s |
|  | @95% | 8.849 m/s |
| Missed detection probability [%] |  | 12.15 % |
| False alarm probability type 2 [%] |  | 27.84 % |

Observation 1: The initial evaluation performance are summarized in Table 1.

Table 1 Sensing performance

| 4.9GHz |  | Performance |
| --- | --- | --- |
| Horizontal localization error [m] | @90% | 7.429 m |
|  | @95% | 17.356 m |
| Vertical localization error [m] | @90% | 9.619 m |
|  | @95% | 23.292 m |
| Radial velocity error [m/s] | @90% | 5.384 m/s |
|  | @95% | 8.849 m/s |
| Missed detection probability [%] |  | 12.15 % |
| False alarm probability type 2 [%] |  | 27.84 % |

Measurement metrics

Based on the discussions in previous meetings [4][5], the issues on measurement metrics has been preliminarily studied and the following proposal has been obtained:

[FL5][H] Proposal 7.1-1-rev3 - The following measurements are identified in the study of NR ISAC. - Level 1: raw data, which is the time-domain samples or subcarrier-domain samples of the channel response from each antenna port, or anything equivalent - Level 2: (partial raw data) - Amplitude and phase profile of delay, and/or Doppler, and/or angle - E.g., Amplitude and phase profile of delay-angle - Level 3 per path (subset of Level 2): - Option 1: Delay, Doppler, angle, and power per detected path for a given time stamp - Definition: one path is associated with one delay, and one or multiple Doppler, and one or multiple angles - A path can correspond to one or multiple detected points - A path can correspond to one or multiple detected objects - Level 4 per point: - Option 1: Range, velocity, angle, and power per detected point for a given time stamp - Definition: A point is defined by one range/delay, one Doppler and one angle - Option 2: Position, velocity, and power per detected point for a given time stamp - Definition: A point is defined by one location and one velocity - Level 5: e.g., {position, velocity, no target ID} - Definition: Position and velocity for a detected object/target for a given time stamp - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is not reported - Level 6: e.g., {position, velocity, target ID} - Definition: Position and velocity for a detected object/target for a given time stamp - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is reported - SINR, RSRP from perspective of sensing - Time stamp - Study the following levels of measurements for potential down-selection within Rel-20 SI - Level 2 - Level 3 - Level 4 - Level 5 - Level 6 - SINR, RSRP from perspective of sensing

The framework established in [FL5][H] Proposal 7.1-1-rev3 offers a comprehensive foundation for studying ISAC measurements. In terms of down-selection, the criterion such as reporting overhead and evaluation suitability should be considered.

Level 1 provides raw channel data and Level 2 offers amplitude and phase profiles, both retaining maximum information which is valuable for advanced accuracy estimation and deep analysis. However, their extreme data volumes present significant complexity for efficient performance evaluation.

Observation 2: Level 1 and Level 2 can offer maximum information but their extreme data volumes present significant complexity for efficient performance evaluation.

Level 3 reports measurements per path, effectively reducing overhead while preserving key parameters. Despite this improvement, its fundamental limitation lies in evaluating performance, as the "path" is not directly equivalent to a sensing target, thereby requiring non-standardized post-processing. Level 4 addresses this by reporting per point, creating a more direct link to physical scatters and proving highly useful for point-cloud-based applications. Nevertheless, its inherent ambiguity in mapping multiple points to a single target complicate the KPI definition and assessment.

Observation 3: Level 3 and Level4 provide valuable intermediate data but are not the most direct metric for evaluating final sensing performance.

Level 5 delivers measurements per target without an assigned ID. It provides the essential output of target position and velocity in a highly efficient data format, enabling direct and unambiguous calculation of performance metrics like detection probability and position accuracy. This approach allows the TRP to focus on target detection while leveraging the core network's superior capability for multi-source data fusion and trajectory management. While Level 6 enhances this by including a target ID for immediate tracking, it inherently associates the evaluation with specific tracking algorithms at the TRP, which presents challenges for fair assessment at this stage.

Observation 4: Level 6 will associate the evaluation with specific tracking algorithms at the TRP, which presents challenges for fair assessment at this stage.

Therefore, it is recommended that Level 5 is selected as the primary level for down-selection, with Level 2 retained as an optional capability. Besides, Level 6 can be considered for future work.

Proposal 1: Recommend to select Level 5 as the primary level for measurement metrics.

Conclusion

In this contribution, we have the following observations and proposals:

Observation 1: The initial evaluation performance are summarized in Table 1.

Table 1 Sensing performance

| 4.9GHz |  | Performance |
| --- | --- | --- |
| Horizontal localization error [m] | @90% | 7.429 m |
|  | @95% | 17.356 m |
| Vertical localization error [m] | @90% | 9.619 m |
|  | @95% | 23.292 m |
| Radial velocity error [m/s] | @90% | 5.384 m/s |
|  | @95% | 8.849 m/s |
| Missed detection probability [%] |  | 12.15 % |
| False alarm probability type 2 [%] |  | 27.84 % |

Observation 2: Level 1 and Level 2 can offer maximum information but their extreme data volumes present significant complexity for efficient performance evaluation.

Observation 3: Level 3 and Level4 provide valuable intermediate data but are not the most direct metric for evaluating final sensing performance.

Observation 4: Level 6 will associate the evaluation with specific tracking algorithms at the TRP, which presents challenges for fair assessment at this stage.

Proposal 1: Recommend to select Level 5 as the primary level for measurement metrics.

Reference

- RP-251861, “New SID: Study on Integrated Sensing And Communication (ISAC)”, AT&T

- Draft_Minutes_report_RAN1#122

- Draft_Minutes_report_RAN1#122b

- R1-2506480, “Summary #5 on evaluations for NR ISAC”, Moderator (Xiaomi)

- R1-2507427, “Summary #5 on evaluations for NR ISAC” , Moderator (Xiaomi)