3GPP TSG RAN WG1 #123 	                                            R1-2508452

Dallas, USA, Nov 17th – 21st, 2025

Source: 	CMCC

Title:	Discussion on NR-NTN GNSS resilience

Agenda item:	10.6.1

Document for:	Discussion & Decision

- Introduction

During the RAN#108 meeting, a new study item was proposed for supporting GNSS-resilient operation of NR-NTN system in scenarios where GNSS is temporarily unavailable or performance degraded [1]. The objective is to initially focus on studying the impacts of the scenario where GNSS information is temporarily unavailable, which is shown as below.

Objective The study will be carried assuming that - The GNSS information in UE with GNSS capability may be 1/ temporarily unavailable or 2/ available but with GNSS position accuracy degradation or 3/ available but with increased GNSS measurement periodicity for power saving purpose - This study will initially focus on 1/ (where GNSS information is temporarily unavailable) - Note: This study does not include any enhancement on positioning for the above scenario Study GNSS resilient NR-NTN operation [RAN1, RAN4] - Assess impact on initial access and connected mode procedures for NR-NTN, including potential implications if any on RAN4 RRM specifications - Aim to minimize physical layer procedures impact - Avoid physical layer channel/signal changes - Backward compatible issues with legacy NTN capable UEs will be assessed and should be prevented - Applicable to NGSO and GSO constellations, NTN operating in below and in above 10 GHz frequency bands, transparent and regenerative satellites - Check in RAN#112 (June, 2026) to decide whether to proceed with normative work, extend the study item, or postpone normative work to Release 21

In this contribution, we will discuss and share our views on the evaluation and potential enhancements of GNSS resilient NR-NTN operation for both initial access and connected mode procedures, and provide evaluation results on timing/frequency error in different scenarios.

- Discussion

For NR-NTN UE in Rel-17/Rel-18/Rel-19, it is supported that UE can derive its position, a reference time and frequency based on its GNSS implementation. An NR-NTN UE in RRC_IDLE, RRC_INACTIVE states or RRC_CONNECTED states is capable to calculate timing advance and frequency adjustment by using the acquired GNSS position and satellite ephemeris. However, in Rel-20, the issue on GNSS maybe temporarily unavailable or degraded due to GNSS jamming, spoofing and other effects (e.g. poor geometry, local blockages, solar storm activity or simply reduce GNSS measurement periodicity to save UE power) has been identified. Therefore, the study item has been launched to define GNSS-resilient operation to allow the UE to maintain initial access, timing and frequency synchronization, and connected-mode tracking.

  - 2.1 Consideration on the evaluation of GNSS-resilient operation

According to the timing and frequency enhancements for NR-NTN in Rel-17, NR-NTN UE in both idle and connected states shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate timing/frequency pre-compensation on the service link. During initial access, UE has not been configured with uplink resources and relies solely on GNSS-based time/frequency pre-compensation, loss of GNSS for a duration would lead to access failure or repeated random-access attempts. In connected mode, UE with invalid GNSS position fix may risk large timing/frequency drift beyond synchronization requirement, causing unnecessary handovers, or radio-link failure. To guarantee the service continuity even when GNSS position fix is temporarily unavailable, the operations to maintain time and frequency synchronization for both initial access and connected mode procedures in NR-NTN needs to be studied.

There are different assumptions regarding the GNSS positioning acquisition in NR-NTN deployment scenarios, namely GNSS information 1) temporarily unavailable or 2) available but with GNSS position accuracy degradation or 3) available but with increased GNSS measurement periodicity for power saving purpose. As mentioned in the objective of the SID, it is initially focused on the case 1 that GNSS information in UE with GNSS capability may be temporarily unavailable (due to blockage, interference, or spoofing, etc.) during a period of time, to evaluate the impact on initial access/connected state and feasibility of GNSS resilient NR-NTN operation due to GNSS is unavailable, two scenarios listed below are considered for the study [2].

RAN1#122 Agreement For the study on GNSS resilient operation, scenarios should be considered where: - Scenario 1: The UE cannot rely on its GNSS for timing and frequency compensation on the service link. - Scenario 2: There is a previously acquired GNSS based position - UE has not received GNSS information for time duration T from the last acquired GNSS position and hence, the GNSS accuracy is degraded - FFS: value of T - FFS: GNSS based UE location validity duration - FFS: GNSS accuracy Note: Two scenarios above belong to description as in SID. RAN1#122 Agreement For evaluation regarding GNSS-resilient operation, based on GNSS information or the lack of it, it is assumed that the UE has a horizontal location uncertainty within an area - Case a: the location uncertainty area is the area served by the cell or beam. - Case b: the location uncertainty area is a circle of radius X km - X to be reported UE altitude error should be taken into account. - UE altitude error to be reported. For both Case a and Case b above, RAN1 to evaluate at least: - Differential delay values - Differential frequency offset/Doppler values

According to the table of satellite scenario parameters listed below, different deployment environments and satellite orbits (GEO, LEO, S-band, Ka-band, Ku-band) are recommended for evaluation.

RAN1#122 Agreement: For the study on GNSS resilient operation, the following scenarios are considered for evaluation purposes: Satellite orbit; GSO; LEO-1200; LEO-600. Satellite altitude; 35786 km; 1200 km; 600 km. Satellite scenario parameters; At least Table 6.1.1.1-1 in TR 38.821, Parameters in Table 6.1.1.1-2 in TR 38.821could be considered.. Beam size (note 3); At least values captured in Table 6.1.1.1-1/2 are considered,. Minimum elevation angle; 30° (LEO), 12.5° (GSO). Frequency ranges/bands; FR1 NTN (2GHz/Ku: 14 GHz) and FR2 NTN (30 GHz). UE type; Handheld (L/S band), VSAT (Ku/Ka). UE speed (note 1); L/S band: 3 km/h, 120km/h, 1500 km/h, Ku/Ka: 3 km/h , 120 km/h, 1500 km/h, (note 2). Beam/cell type (note 4); (Quasi)-Earth-fixed beams/cells and Earth-moving. Note 1: UE altitude is considered Note 2: 10 km altitude in case of aircraft scenario. 1500 km/h is for aircraft scenario. Note 3: Same values can be reused for other elevation angle than nadir. Beam size of the edge beam can be reported by companies. Note 4: Footprint of the beam could be: case 1: within the orbital plane, case 2: at 90° with respect to orbital plane. Companies to report the elevation angle of the beam. Note 5: Other parameters can be reported by companies.. Note: The cyan highlighted parameters are prioritized for evaluation purposes.

The Table 6.1.1.1-1/2 in TR38.821 [3] only provide the satellite payload characteristics of S-band and Ka-band. The evaluation parameters for Ku-band are still not clear or aligned within the group. The specific parameters assumed for evaluation, such as satellite payload parameters, beam size, should be identified first. And we think LEO based deployment for S-band can be prioritized for study.

Proposal 1: For the study on GNSS resilient operation, the detailed satellite parameters of Ku-band for evaluation purpose need to be defined.

Proposal 2: It is suggested to prioritize the LEO based deployment in S-band for the study on GNSS resilient operation.

In the last meeting, two alternative methodologies for calculating timing and frequency differential were discussed, and the corresponding agreement was reached.

Currently, there are certain discrepancies in scenario modeling among different companies, leading to variations in simulation results. These differences may impact the assessment of issues such as the usability of the current PRACH format. Therefore, it is necessary to establish a simulation modeling approach that is agreed upon by all companies.

The Alt1 modeling only considers the two-dimensional plane. The goal of our simulation is to determine the range, or the maximum value, of time and frequency differential. Thus, modeling UE mobility speed as currently done is acceptable (velocity components in other directions relative to the UE-satellite vector would only be smaller). The overall idea of the Alt1 method is correct, however, there are some errors in the formulas that need to be corrected. For example, the subscript of distance d is not consistent across different formulas.

Considering whether the minimum elevation is at farthest w.r.t satellite point of the uncertainty area or at the center of the uncertainty area, our point is that the farthest w.r.t satellite point of the uncertainty area should be where the minimum elevation is. The elevation at farthest w.r.t satellite point of the uncertainty area is smaller than the center of the uncertainty area. The UE at farthest w.r.t satellite point may not see the satellite when the elevation at center the minimum elevation is.

For the UE altitude uncertainty, we can perform a simple analysis: when the elevation angle is 45 degrees, the impact of the UE's horizontal position uncertainty and vertical position uncertainty on the variation of the positional relationship between the UE and the satellite is equivalent. The magnitude of horizontal position uncertainty is on the order of tens of kilometers, while the vertical position uncertainty, influenced by terrain, is generally not as significant. Moreover, it can be further reduced through measurements such as barometric sensors in the UE. Therefore, it can be concluded that the vertical position uncertainty has a relatively minor impact on the final calculation results. To avoid further complicating the computational process, we consider the vertical position uncertainty negligible.

Proposal 3: For alternatives representing two calculation methodologies, Alt1 with some modifications is selected, such as revising the subscripts of some variables in the formulas. The version in the FL recommendation in R1-2508178 can be used as a reference.

Proposal 4: The minimum elevation should be calculated at farthest w.r.t satellite point of the uncertainty area.

Furthermore, according to the Rel-19 WID of further enhancements for NR NTN Phase 3, three sets of reference satellite parameters for LEO-600 in FR1 band are assumed for different NTN deployment scenarios, referred to Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1 listed in the Annex I [4].

Three sets of reference satellite parameters present different satellite capabilities and using scenarios. LEO-600km Set 1-1 FR1 represents the case which has a relatively large number of simultaneously active beams, i.e. 106 beams, to cover 1058 planned beam footprints on the earth. Set 1-2 FR1 has the same DL transmit power as Set 1-1, but with a smaller number of simultaneously active beams, i.e. 16 beams, due to the limitation of satellite payload. For the three satellite parameter sets, the ratio of active beam is limited by 10% or worse, by 1.5% due to limitation of the hardware and the payload of satellites. Since it cannot satisfy 100% coverage ratio of SSB and common channel with the default SSB periodicity, increasing SSB periodicity to improve DL coverage ratio and reduce control channel overhead has been studied. While considering the above methods investigated for GNSS resilient operation, the reference signals or reference locations that need to be broadcast by network still face the coverage issue caused by limited number of simultaneously active beams.

Proposal 5: If considering the limitation of simultaneously active beams in different NR NTN deployment scenarios, the feasibility of network broadcasting the configuration of reference signals or the reference location need to be studied.

  - 2.2 Evaluation on PRACH performance

Regarding the case that GNSS information in UE with GNSS capability is temporarily unavailable in NR-NTN scenarios, the key aspects need to be studied include the impacts on DL synchronization, random access procedure, and maintaining UL timing and frequency synchronization. And according to the objective of the SID, it aims to minimize physical layer procedures impact and avoid physical layer channel/signal changes, the operation to maintain synchronization with the network needs to consider reusing existing DL/UL signals and synchronization mechanisms as much as possible.

It has been supported that UE calculates timing advance and frequency adjustment based on acquired GNSS position in NR NTN. While timing advance value calculated based on GNSS-acquired UE position may be not available or sufficiently accurate due to the GNSS becomes outdated/unavailable. Regarding the restriction in the SID that avoid physical layer channel/signal changes, if network assisted methods are used for the UE calculating its location and maintaining UL timing/frequency synchronization, one key issue of random-access procedure is to evaluate whether current PRACH formats design, such as SCS, CP length, sequence length and sequence repetition number, are sufficient for dealing with timing errors/frequency shift caused by positioning errors of different scenarios.

With the assumption of reusing existing PRACH preamble formats, the evaluation methods, parameter assumptions on PRACH performance and criteria of tolerable timing error and frequency shift have been achieved, which are listed below.

RAN1#122 Agreement: For PRACH performance evaluation for existing PRACH formats, adopt the following methods: - Baseline: Analytical characterization of performance based on e.g. properties of ZC sequences and values of differential Delay / Doppler. - Optional: Link level evaluations - The following table is proposed for LLS parameters Configurations; S/L-band; Ka/Ku-band. Carrier Frequency; 2 GHz; For Ka: UL 30GHz, for Ku: UL 14 GHz. Channel Model; Baseline NTN TDL-C rural model in TR38.811. Antenna Configuration at the TRP (satellite); 1 Rx; 1 Rx. Antenna Configuration at the UE; A single omni-directional antenna element; VSAT with 60 cm equivalent aperture diameter. PRACH format; To be reported by companies.; To be reported by companies. PRACH configuration; To be reported by companies.. Metric; PRACH detection rate, FAR (Based on the preamble pool size is not less than 64), CDF of estimation error for frequency/timing. Receiver; Companies to report the receiver for PRACH detection.. RAN1#122bis Agreement: For the evaluation of GNSS resilient NR-NTN operation, consider the following assumptions: - For scenario 1 and 2 - The following time/frequency differences should be evaluated: - The differential one-way delay (in µs) as calculated between the largest UE-Sat delay and smallest UE sat delay within the uncertainty area. - For UL performance evaluation 2 times differential one-way delay should be considered. - The differential one-way Doppler/frequency offset (in ppm) is calculated between the largest UE-sat Doppler/frequency offset and smallest UE-sat Doppler/frequency offset within the uncertainty area. - For UL performance evaluation, scaled one-way differential Doppler/frequency offset to determine UL differential Doppler/frequency offset should be considered. Scaling factor should be reported. RAN1#122bis Agreement: For PRACH performance evaluation for existing PRACH preamble formats based on analytical characterization, adopt the following criteria: - PRACH tolerance is exceeded if - min (Ncp, Sequence duration×maxNcsL) - 2 Te Differential RTT, where; - Ncp is cyclic prefix duration - max Ncs is given by Tables 6.3.3.1-5, 6.3.3.1-6 and 6.3.3.1-7 of TS 38.211 - L is the ZC sequence length - Te is an additional timing error. - Companies to report how Te is derived. - Differential RTT is 2 time one-way delay. - Note: the delay spread is neglected - GP can be considered and reported for the case of consecutive RO. - Or (scaling factor*(Differential Doppler) +2* fe×fc) ≥ (1+γ)ΔfRA - For restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0 - Note: In practice, the detection performance degrades at frequency offsets exceeding 1 SCS and 2 SCS with restricted set A and B, respectively. - Othe values of γ can be used and reported with justification - fe=0.1 ppm - fc is the uplink carrier frequency - Note: The scaling factor*Differential Doppler is equal to the UL differential Doppler/frequency offset. - The same scaling factor as per the previous agreement is to be reported Note: the above is applicable to single PRACH transmission.

### 2.2.1 Tolerable positioning errors under current PRACH formats

Considering the restriction that legacy PRACH preamble formats shall not be modified, the criteria of the tolerable timing error based on the CP length of preambles and the value of cyclic shift NCS of existing PRACH formants is used for evaluation. Regarding the additional timing error Te mentioned in the above agreement, the value needs to be further discussed. As specified in TS 38.133 [5], the UE initial transmission timing error shall be less than or equal to Te_NTN where the timing error limit value Te_NTN is specified in table 7.1C.2-1 for FR1-NTN and in table 7.1C.2-2 and table 7.1C.2-3 for VSAT UE in FR2-NTN. However, the timing error requirements for NTN are derived by taking the GNSS positioning error, error of UE determining the satellite location and extra margin into consideration, resulting in larger values compared with the timing error limits for TN. Form our perspective, since the case that GNSS information is temporarily unavailable is studied in Rel-20 GNSS resilience operation, at least the relaxation of GNSS positioning error requirements will not be considered for determining Te in the criteria. For simplicity, it is recommended to refer to the following timing error limit for TN in Table 7.1.2-1 of TS38.133 for calculating the tolerable timing error.

Table 7.1.2-1: Te Timing Error Limit Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te; Te (μs). 1; 15; 15; 12*64*Tc; 0.39. 30; 10*64*Tc; 0.33. 60; 10*64*Tc; 0.33. 30; 15; 8*64*Tc; 0.26. 30; 8*64*Tc; 0.26. 60; 7*64*Tc; 0.23. 2-1; 120; 60; 3.5*64*Tc; 0.11. 120; 3.5*64*Tc; 0.11. 240; 60; 3*64*Tc; 0.098. 120; 3*64*Tc; 0.098. NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6].

Furthermore, the requirement for frequency shift error also depends on whether the restriction set is considered, it needs to ensure that the one-way Doppler frequency shift will not exceed 0.4*SCS of PRACH formats without restricted set, and not exceed 1*SCS or 2*SCS of PRACH formats with restricted set [6,7]. 	Therefore, in the criteria for tolerable frequency shift of existing PRACH preamble formats, it is assumed for restricted set A,γ=1; for restricted set B,γ=3; for non-restricted set,γ=0.

Proposal 6: For PRACH performance evaluation, the scaling factor can be assumed as 2 in the criteria for tolerable frequency shift of existing PRACH preamble formats.

For PRACH performance evaluation, the long PRACH preamble formats are configured with restricted sets to limit the set of usable cyclic shifts, different maximum values of Ncs for PRACH formats with different configurations are given by Tables 6.3.3.1-5, 6.3.3.1-6 and 6.3.3.1-7 of TS 38.211 separately [8]. The additional timing error Te of 0.39μs for S-band and 0.11μs for Ka/Ku-band is considered. And the tolerable frequency error based on SCS restriction can be different depending on whether restricted sets are configured for PRACH preamble. By referring the detailed preamble numerology in Table 6.3.3.1-1 and Table 6.3.3.1-2 in TS 38.211, the tolerable timing error/frequency shift of each PRACH preamble format are listed in Table 2.2.1-1 for long preambles and in Table 2.2.1-2/2.2.1-3 for short preambles respectively.

Table 2.2.1-1 Tolerable timing/frequency errors under current PRACH long formats in S-band

| Format | LRA | ΔfRA | Nu | NCPRA | Tolerable timing errors w/o restricted set (μs) | Tolerable timing errors with restricted set A (μs) | Tolerable timing errors with restricted set B (μs) | Tolerable frequency errors w/o restricted set (kHz) | Tolerable frequency errors with restricted set A (kHz) | Tolerable frequency errors with restricted set B (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 839 | 1.25 kHz | 24576κ | 3168κ | 102.35 | 102.35 | 102.35 | 0.85 | 2.1 | 4.6 |
| 1 | 839 | 1.25 kHz | 2⋅24576κ | 21024κ | 398.74 | 225.20 | 129.85 | 0.85 | 2.1 | 4.6 |
| 2 | 839 | 1.25 kHz | 4⋅24576κ | 4688κ | 151.82 | 151.82 | 129.85 | 0.85 | 2.1 | 4.6 |
| 3 | 839 | 5 kHz | 4⋅6144κ | 3168κ | 99.10 | 55.72 | 31.88 | 4.6 | 9.6 | 19.6 |

Table 2.2.1-2 Tolerable timing/frequency errors under current PRACH short formats in S-band

| Format | LRA | ΔfRA | Nu | NCPRA | Tolerable timing errors (μs) | Tolerable frequency errors (kHz) |
| --- | --- | --- | --- | --- | --- | --- |
| A1 | 139 | 15 kHz | 2⋅2048κ | 288κ | 8.60 | 14.6 |
| A2 | 139 | 15 kHz | 4⋅2048κ | 576κ | 17.97 | 14.6 |
| A3 | 139 | 15 kHz | 6⋅2048κ | 864κ | 27.35 | 14.6 |
| B1 | 139 | 15 kHz | 2⋅2048κ | 216κ | 6.25 | 14.6 |
| B2 | 139 | 15 kHz | 4⋅2048κ | 360κ | 10.94 | 14.6 |
| B3 | 139 | 15 kHz | 6⋅2048κ | 504κ | 15.63 | 14.6 |
| B4 | 139 | 15 kHz | 12⋅2048κ | 936κ | 29.69 | 14.6 |
| C0 | 139 | 15 kHz | 2048κ | 1240κ | 32.31 | 14.6 |
| C2 | 139 | 15 kHz | 4⋅2048κ | 2048κ | 32.31 | 14.6 |

Table 2.2.1-3 Tolerable timing/frequency errors under current PRACH short formats (120kHz SCS) in Ka-band

| Format | LRA | ΔfRA | Nu | NCPRA | Tolerable timing errors (μs) | Tolerable frequency errors (kHz) |
| --- | --- | --- | --- | --- | --- | --- |
| A1 | 139 | 120 kHz | 2⋅2048κ⋅2-3 | 288κ⋅2-3 | 0.95 | 114 |
| A2 | 139 | 120 kHz | 4⋅2048κ⋅2-3 | 576κ⋅2-3 | 2.12 | 114 |
| A3 | 139 | 120 kHz | 6⋅2048κ⋅2-3 | 864κ⋅2-3 | 3.30 | 114 |
| B1 | 139 | 120 kHz | 2⋅2048κ⋅2-3 | 216κ⋅2-3 | 0.66 | 114 |
| B2 | 139 | 120 kHz | 4⋅2048κ⋅2-3 | 360κ⋅2-3 | 1.24 | 114 |
| B3 | 139 | 120 kHz | 6⋅2048κ⋅2-3 | 504κ⋅2-3 | 1.83 | 114 |
| B4 | 139 | 120 kHz | 12⋅2048κ⋅2-3 | 936κ⋅2-3 | 3.59 | 114 |
| C0 | 139 | 120 kHz | 2048κ⋅2-3 | 1240κ⋅2-3 | 4.83 | 114 |
| C2 | 139 | 120 kHz | 4⋅2048κ⋅2-3 | 2048κ⋅2-3 | 8.11 | 114 |

### 2.2.2 Scenario 1 and Case a

As in Scenario 1 and in Case a, the timing error/deviation and the frequency shift due to the uncertainty of UE location is calculated and listed in Annex II. The radius of beam footprints can be considered as radius of the location uncertainty areas.

Observation 1: The required CP lengths and SCSs (with restricted set A, with restricted set B and without restricted set) under LEO-600, LEO-1200, GEO with different beam footprint sizes are evaluated in Annex II.

The evaluation results of timing /frequency error and required CP/SCS for LEO-600 FR1 NTN S-band excerpted from Annex II are listed below, it can be observed that only CP of PRACH format 1 can cover the timing difference due to beam size level positioning uncertainty. Restricted set of PRACH would be used to deal with variant Doppler shifts due to the positioning error. If restricted set of PRACH is not used, larger subcarrier spacing beyond 5kHz is required. If the restricted set for the long format is used, the required SCS would be below 1.25kHz.

Observation 2: Without GNSS facilitating the timing and frequency pre-compensation, under the beam footprint size-level positioning error, larger CP and subcarrier spacing are required at satellite or gNB to receive preamble correctly.

Table 2.2.2-1 Timing/frequency error and required CP/SCS for S-band/2GHz

| Scenario | Frequency band | Beam radius | Elevation angle | UE speed（km/h） | one-way Maximum timing error (us) | Require CP length/differential RTT (us) | one-way Maximum differential Doppler shift (Hz) | differential RTD (Hz) | Required SCS without restricted set（kHz） γ = 0 | Required SCS with restricted set A（kHz） γ= 1 | Required SCS with restricted set B（kHz） γ= 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LEO-600 | 2GHz | 25km | 30 | 3 | 142.99 | 285.98 | 763.095 | 1526.19 | 1.52619 | 0.763095 | 0.3815475 |
|  |  |  |  | 120 | 142.99 | 285.98 | 766.6841 | 1533.3682 | 1.5333682 | 0.7666841 | 0.38334205 |
|  |  |  |  | 1500 | 143.19 | 286.38 | 821.3225 | 1642.645 | 1.642645 | 0.8213225 | 0.41066125 |
|  |  |  | 90 | 3 | 1.8987 | 3.7974 | 4196.5 | 8393 | 8.393 | 4.1965 | 2.09825 |
|  |  |  |  | 120 | 1.8987 | 3.7974 | 4216.2 | 8432.4 | 8.4324 | 4.2162 | 2.1081 |
|  |  |  |  | 1500 | 1.9339 | 3.8678 | 4531 | 9062 | 9.062 | 4.531 | 2.2655 |

The speed of 1500km/h would induce additional Doppler shift, which even in the area where Doppler shift is not that sensitive, e.g. in the elevation angle of 30o. It can be observed that most high Doppler shifts happen in the elevation angle of 90 degree. And in the elevation angel, the Doppler shifts are not that high for the speed of 3km/h and 120km/h. But for the speed of 1500km/h, the single side Doppler shifts would be as high as 2672.74Hz, which is even higher than that at the elevation 90 degree area. It should be further discussed whether a unified solution should be strived to solve both low speed mobility (3km/h, 120km/h) and high speed mobility (1500km/h) of the air traffic. Since the Doppler shift due to high speed of 1500km/h may need additional enhancements, such as additional carrier spacing. Separated design or enhancements could bring more streamlined design for each system.

Proposal 7: It should be further discussed whether a unified solution should be strived to solve both low speed mobility (3km/h, 120km/h) and high speed mobility (1500km/h) of the air traffic.

As illustrated in the Annex II, lager beam footprint size and uncertain positioning areas would bring higher requirements for CP and SCS. Under current assumption of beam footprint size, current CP design in NR cannot support the LEO-1200 and GEO in S-band.

Observation 3: Under current assumption of beam footprint size, the current CP design in NR cannot support the LEO-1200 and GEO in S-band.

Regarding FR2 NTN (30 GHz) for LEO-600 and LEO-1200, though the beam size is smaller than that of S-band, and the required CP is smaller than S-band. The CP of short format preamble still cannot cover the timing error due to the uncertainty area.

Observation 4: Current short format preamble cannot be used in the LEO-600, LEO-1200 and GEO in Ka-band, if the positioning uncertainty area is in a beam footprint size level.

### 2.2.3 Scenario 2 and Case b

As agreed in the last meeting, Scenario 2 is that UE has a previously acquired GNSS based position and UE has not received GNSS information for a time duration. Case b is assuming the location uncertainty area is circle of radius X km. Based on UE’s speed, the location uncertain area will increase with the UE speed increasing. And beyond a certain distance or time, the current PRACH may or may not be used with previously acquired GNSS. With this consideration, we also evaluate the positioning error or location uncertainty which can be tolerated by current PRACH design. The tolerable timing errors and tolerable frequency errors of different PRACH formats are calculated in Section 2.2.1, which will result in different poisoning errors tolerated by each PRACH format, and the corresponding tolerated positioning errors listed in the table are determined based on the minimum value derived by tolerable timing error and tolerable frequency error. Taking LEO-600km with S-band and Ka-band as an example, both long format and short format PRACH are evaluated. In the case of long format, both restricted set and un-restricted set are evaluated. Additional Doppler shift due to the moving speed, such as 3km/h, 30km/h, 1500km/h is not considered in the Tables below.

Table 2.2.3-1 Tolerable positioning errors under current PRACH long format in LEO-600 S-band

| Format | LRA | ΔfRA | Restricted set | Tolerable timing errors (us) | Tolerable frequency errors (kHz) | Corresponding positioning error/ radius (km) |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 839 | 1.25 kHz | w/o | 51.18 | 0.425 | 2.53 |
| 0 | 839 | 1.25 kHz | Type A | 51.18 | 1.05 | 6.25 |
| 0 | 839 | 1.25 kHz | Type B | 51.18 | 2.3 | 8.89 |
| 1 | 839 | 1.25 kHz | w/o | 199.37 | 0.425 | 2.53 |
| 1 | 839 | 1.25 kHz | Type A | 112.6 | 1.05 | 6.25 |
| 1 | 839 | 1.25 kHz | Type B | 64.93 | 2.3 | 11.29 |
| 2 | 839 | 1.25 kHz | w/o | 75.91 | 0.425 | 2.53 |
| 2 | 839 | 1.25 kHz | Type A | 75.91 | 1.05 | 6.25 |
| 2 | 839 | 1.25 kHz | Type B | 64.93 | 2.3 | 11.29 |
| 3 | 839 | 5 kHz | w/o | 49.55 | 2.3 | 8.61 |
| 3 | 839 | 5 kHz | Type A | 27.86 | 4.8 | 4.83 |
| 3 | 839 | 5 kHz | Type B | 15.94 | 9.8 | 2.76 |

Table 2.2.3-2 Tolerable positioning errors under current PRACH short format in LEO-600 Ka-band

| Format | LRA | ΔfRA | Tolerable timing errors (us) | Tolerable frequency errors (kHz) | Corresponding positioning error/ radius (km) |
| --- | --- | --- | --- | --- | --- |
| A1 | 139 | 120 kHz | 0.475 | 57 | 0.08 |
| A2 | 139 | 120 kHz | 1.06 | 57 | 0.18 |
| A3 | 139 | 120 kHz | 1.65 | 57 | 0.29 |
| B1 | 139 | 120 kHz | 0.33 | 57 | 0.06 |
| B2 | 139 | 120 kHz | 0.62 | 57 | 0.11 |
| B3 | 139 | 120 kHz | 0.915 | 57 | 0.16 |
| B4 | 139 | 120 kHz | 1.795 | 57 | 0.31 |
| C0 | 139 | 120 kHz | 2.415 | 57 | 0.42 |
| C2 | 139 | 120 kHz | 4.055 | 57 | 0.7 |

In the scenario 2 and Case b, UE should be capable of sending PRACH to the satellite to resume the uplink synchronization or request further assistance from the satellite. Then tolerable positioning error under current PRACH format can be studied to determine the radium of the location uncertainty area.

Proposal 8: The tolerable positioning error under current PRACH format can be studied to determine the radius of the location uncertainty area.

  - 2.3 Potential enhancement

According to the observed solutions summarized in the RAN1#122bis meeting, some potential enhancements are proposed, including the following

Observation: For the study on GNSS resilient NR-NTN operation, the following solutions for initial access to increase PRACH tolerance and reducing time/frequency uncertainty are listed based on inputs/discussions in RAN1#122bis: - Solution 1: To improve robustness: - Solutions based on modified random access procedures, using existing PRACH formats: - Solution 1A: Multiple PRACH transmissions (e.g. with different roots or different formats or with different time/frequency pre-compensation using multiple reference locations within the uncertainty area) using existing PRACH formats - Solution 1B: New PRACH restricted sets. - Solution 1C: Single PRACH transmission with multiple TAC/FAC commands in RAR - Solution 1D: Signalling enhancements for Msg2 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command) - Solution 1E: Selection of a set of existing PRACH root - Solution 1F: Using TA margin and RO masking. - Other solutions are not precluded - Solution 2: Solutions based on reducing time-frequency uncertainty - Solution 2A: Single/multi-satellite DL-TDOA based on current specifications. - Solution 2B: multiple PRACH attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area) - Solution 2C: Solutions based on broadcasting DL timestamp(s) - Solution 2D: UE side time/frequency pre-compensation based on reference location. - Solution 2E: service link time/frequency pre-compensation based on last acquired GNSS position - Solution 2F: UE side time/frequency pre-compensation based on reference signal. - Other solutions are not precluded - Solution 3: Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay. - Other Solutions are not precluded

### 2.3.1 RAT-dependent positioning

From our perspective, without available GNSS information, the UE can still obtain coarse position information by re-using legacy terrestrial RAT dependent positioning methods applied to NR-NTN system. It is noted that this study does not include any enhancement on positioning, for Rel-20 NR-NTN. Measuring DL reference signal PRS and UL reference signal SRS can be considered to estimate UE location or perform timing/frequency compensation. Whether the methods are sufficient to compute timing-advance and Doppler pre-compensation values that allow UE to initiate random access and maintain UL synchronization even when GNSS is unavailable needs to be evaluated.

For evaluating positioning performance of RAT dependent positioning method, we think the evaluation metric and parameter assumptions studied in Rel-18 NR-NTN WID can serve as references for evaluation GNSS independent positioning mechanisms. Specifically, for initial access procedure, no dedicated uplink configuration has yet been delivered via RRCReconfiguration during the initial-access phase, the UE lacks dedicated resource for uplink reference signal transmission. To assess the impact of GNSS resilient NR-NTN operation and evaluate the positioning performance of GNSS independent positioning method with invalid GNSS position fix, the DL-TDOA method via downlink reference signals PRS measurement can be prioritized for the study.

Proposal 9: For evaluating positioning performance of GNSS resilient NR-NTN operation, the evaluation metrics and assumptions of RAT dependent positioning methods study in Rel-18 NR NTN can be considered as a start point.

A preliminary evaluation based on PRS was carried out. The simulation assumption refers to Rel-18 UE location verification, which can be referred from the Annex III. Both single satellite and multiple satellite are considered in the simulations. The timing error of PRS is evaluated from the link level simulation. Two slot PRS with Comb 2 are used to evaluate the timing errors. The positioning error is simulated based on the exact UE’s location and SINR from the system level simulation. Based on the PRS timing error, the positioning algorithm is used to calculate the UEs position under the satellites. The methodology is also similar to that in Rel-18 UE location verification. In the simulation of multiple satellites, the synchronization errors between different satellites are considered.

Table 2.3.2-1 Single satellite positioning error of DL-TDOA via PRS with 3 RSTDs

(corresponding to 4 measurement occasions)

|  | Time interval(s) | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- |
| Positioning error (km) | 2 | 5.93 | 10.11 | 16.78 | 30.73 | 47.48 |
|  | 4 | 1.51 | 2.56 | 4.48 | 9.15 | 14.96 |
|  | 8 | 0.39 | 0.67 | 1.17 | 2.28 | 4.23 |
|  | 12 | 0.17 | 0.29 | 0.52 | 1.03 | 2.02 |

Table 2.3.2-2 Multiple satellite positioning error of DL-TDOA via PRS

|  | Time interval(s) | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- |
| Positioning error (km) | 2 | 1.31 | 1.85 | 2.52 | 3.5 | 4.34 |
|  | 4 | 0.66 | 0.92 | 1.25 | 1.76 | 2.23 |
|  | 8 | 0.33 | 0.46 | 0.62 | 0.87 | 1.11 |
|  | 12 | 0.22 | 0.31 | 0.42 | 0.57 | 0.74 |

In the single satellite positioning, 4 measurements and 3 RSTDs are considered. Multiple time intervals between measurements are simulated. With different time intervals, the performance would be different. With 8 seconds and 12 seconds time interval, around 4km and around 2km positioning errors can be observed. The consumed time would be 24 seconds and 36 seconds. The performance with multiple satellites would have better positioning accuracy. Two satellites with at least two measurement occasions are considered. Then the consumed time per satellite would be 2 seconds or 4 seconds to achieve a similar positioning accuracy as that in single satellites.

Observation 5: Single satellite with 4 measurement occasions and 3 RSTDs can achieve 4.23km and 2.02km positioning accuracy (or positioning uncertainties) in 24s and 36s.

Observation 6: Multiple satellites with at least two measurement occasions can achieve 4.34km and 2.23 km positioning accuracy in 4s and 8s. And with larger time intervales, the positioning errors would be even lower to 1km and less than 1km.

With less than 1km positioning accuracy, the requirement for CP and SCS of PRACH would be much lower.

Proposal 10: It is proposed to study the RAT-dependent positioning to improve the GNSS resilience.

- DL-TDOA positioning method via PRS measurement with single satellite can be prioritized for the evaluation of GNSS resilient NR-NTN operation.

### 2.3.2 Consideration on TA adjustment

In the NTN quasi-earth-fixed scenario, the distance between the UE and the satellite varies during the satellite's service period, resulting in the variation of propagation delay. Taking a LEO-600 satellite as an example, one UE can receive the satellite's signal when the elevation angle is 30o. The distance between the satellite and the UE decreases from 1075 km to 600 km as the elevation angle increases from 30o to 90o. Consequently, the one-way propagation delay varies from 3.58ms to 2ms. If a CP length of 4.7 us is set as the trigger for TA adjustment, as the elevation angle changes from 30o to 90o, the gNB needs to send approximately 300 TA update commands to the UE. The satellite moving from 30 degrees elevation angle to 90 degrees would cost 2 minutes, during which the 300 TA commands would be sent to the UE. It means that 2 TA commands would be sent in one second for one UE. Without GNSS positioning and timing pre-compensation, a large amount of TA commands would be needed. It is proposed to study the enhancements to reduce the transmission of TA commands when GNSS is not available.

Figure 1. Illustration of one-way propagation delay with a fitted model of the TA change

Observation 7: As the elevation angle changes from 30o to 90o, the gNB needs to send approximately 300 TA update commands to the UE. This high rate imposes a significant signaling overhead and degrades system performance.

Proposal 11: It is proposed to study the enhancements to reduce the transmission of TA commands when GNSS is not available.

### 2.3.3 Assistance information report

Considering Scenario 2 where UE has not received GNSS information for a time duration, the previously acquired GNSS based position can be used for timing and frequency pre-compensation, the degradation of GNSS positioning accuracy will lead to a decrease in synchronization accuracy, or requiring relaxation of positioning accuracy requirements. While it is not clear on the acceptable range of GNSS position accuracy degradation, which needs to be further evaluated. And from our understanding, the tolerable time duration T is related to some key factors, including UE velocity, timing/frequency error requirements, etc.

Regarding the GNSS based UE location validity duration, since when to activate GNSS and perform GNSS measurement are up to UE implementation, it is preferred that UE can report assistance information to inform gNB when the previous acquired GNSS position becomes invalid. Especially when the network assisted mechanisms, such as close-loop time and frequency adjustments, are considered to maintain the time and frequency synchronization if the GNSS position is invalid due to exceeding the GNSS validity duration.

Proposal 12: The assistance information on GNSS based UE location validity duration can be reported by UE.

### 2.3.4 Other aspects

Considering the observed potential enhancements based on modified random access procedures. Since this is a 5G-A study, any further enhancements to the physical layer channels/signals should be avoided, which is also illustrated in the SID. From our perspective, enhanced PRACH formats and/or preamble sequences will not be specified in the WI phase of GNSS resilient operation, such as solution 1A. And it needs to be further clarified that the enhancements on PRACH configuration, such as changing the existing resource allocation for preamble transmission, are not considered in the evaluation, which may have impact on the PRACH performance.

Proposal 13: Enhanced PRACH formats and/or preamble sequences, including update of PRACH resource configuration, will not be specified in the WI phase in the Rel-20 GNSS resilient NR-NTN operation.

Furthermore, in the cases that the UE cannot rely on the pre-acquired GNSS information for timing and frequency compensation during a time duration, one method under consideration is that the network may provide the reference location of the serving cell or beam, which can be used for UE to perform time/frequency pre-compensation. This approach assumes that the UE can estimate propagation delay and Doppler shift based on the network-indicated reference point and pre-compensate them for the uplink transmission. While the actual UE location may be different from the reference location, this discrepancy introduces residual timing and frequency errors. But the closed loop of timing adjustment and frequency adjustments can be used to correct the residual timing and frequency errors. Solution 2E is similar to the solution 2D, in which the reference location can be considered as the last acquired GNSS position in Scenario 2 and Case B.

Regarding other potential solutions, such as solution 2E that service link time/frequency pre-compensation based on last acquired GNSS position, more specific mechanisms are needed. Last required GNSS information is still available but the GNSS positioning accuracy is degraded, leading to a decrease in synchronization accuracy, or requiring relaxation of positioning accuracy requirements. Mechanism such as close-loop time and frequency adjustments can be considered to maintain the time and frequency synchronization if the GNSS position is invalid due to extending the GNSS measurement period.

Proposal 14: For the GNSS resilient NR-NTN operation, the following methods can be studied when the UE cannot rely on its GNSS for timing and frequency compensation on the service link,

- The UE performs timing and frequency compensation based on the reference location provided by network

- Service link time/frequency pre-compensation based on last acquired GNSS position

- Conclusions

In this contribution, we share our views on studying GNSS resilient NR-NTN operation. The observations and proposals are summarized as follows.

Observation 1: The required CP lengths and SCSs (with restricted set A, with restricted set B and without restricted set) under LEO-600, LEO-1200, GEO with different beam footprint sizes are evaluated in Annex II.

Observation 2: Without GNSS facilitating the timing and frequency pre-compensation, under the beam footprint size-level positioning error, larger CP and subcarrier spacing are required at satellite or gNB to receive preamble correctly.

Observation 3: Under current assumption of beam footprint size, the current CP design in NR cannot support the LEO-1200 and GEO in S-band.

Observation 4: Current short format preamble cannot be used in the LEO-600, LEO-1200 and GEO in Ka-band, if the positioning uncertainty area is in a beam footprint size level.

Observation 5: Single satellite with 4 measurement occasions and 3 RSTDs can achieve 4.23km and 2.02km positioning accuracy (or positioning uncertainties) in 24s and 36s.

Observation 6: Multiple satellites with at least two measurement occasions can achieve 4.34km and 2.23 km positioning accuracy in 4s and 8s. And with larger time intervales, the positioning errors would be even lower to 1km and less than 1km.

Observation 7: As the elevation angle changes from 30o to 90o, the gNB needs to send approximately 300 TA update commands to the UE. This high rate imposes a significant signaling overhead and degrades system performance.

Proposal 1: For the study on GNSS resilient operation, the detailed satellite parameters of Ku-band for evaluation purpose need to be defined.

Proposal 2: It is suggested to prioritize the LEO based deployment in S-band for the study on GNSS resilient operation.

Proposal 3: For alternatives representing two calculation methodologies, Alt1 with some modifications is selected, such as revising the subscripts of some variables in the formulas. The version in the FL recommendation in R1-2508178 can be used as a reference.

Proposal 4: The minimum elevation should be calculated at farthest w.r.t satellite point of the uncertainty area.

Proposal 5: If considering the limitation of simultaneously active beams in different NR NTN deployment scenarios, the feasibility of network broadcasting the configuration of reference signals or the reference location need to be studied.

Proposal 6: For PRACH performance evaluation, the scaling factor can be assumed as 2 in the criteria for tolerable frequency shift of existing PRACH preamble formats.

Proposal 7: It should be further discussed whether a unified solution should be strived to solve both low speed mobility (3km/h, 120km/h) and high speed mobility (1500km/h) of the air traffic.

Proposal 8: The tolerable positioning error under current PRACH format can be studied to determine the radius of the location uncertainty area.

Proposal 9: For evaluating positioning performance of GNSS resilient NR-NTN operation, the evaluation metrics and assumptions of RAT dependent positioning methods study in Rel-18 NR NTN can be considered as a start point.

Proposal 10: It is proposed to study the RAT-dependent positioning to improve the GNSS resilience.

- DL-TDOA positioning method via PRS measurement with single satellite can be prioritized for the evaluation of GNSS resilient NR-NTN operation.

Proposal 11: It is proposed to study the enhancements to reduce the transmission of TA commands when GNSS is not available.

Proposal 12: The assistance information on GNSS based UE location validity duration can be reported by UE.

Proposal 13: Enhanced PRACH formats and/or preamble sequences, including update of PRACH resource configuration, will not be specified in the WI phase in the Rel-20 GNSS resilient NR-NTN operation.

Proposal 14: For the GNSS resilient NR-NTN operation, the following methods can be studied when the UE cannot rely on its GNSS for timing and frequency compensation on the service link,

- The UE performs timing and frequency compensation based on the reference location provided by network

- Service link time/frequency pre-compensation based on last acquired GNSS position

- References

- RP-251933, Study on GNSS (Global Navigation Satellite System) resilient NR-NTN (Non-Terrestrial Networks) operation, September 15-18th, 2025.

- Chairman's Notes RAN1#122, August 25th – 29th, 2025.

- 3GPP TR38.821, Solutions for NR to support non-terrestrial networks (NTN).

- Chairman's Notes RAN1#116, February 26th - March 1st, 2024.

- TS 38.133, NR; Requirements for support of radio resource management.

- R1-1908049, Discussion on Doppler compensation, timing advance and RACH for NTN, 3GPP TSG RAN WG1 Meeting #98, Prague, Czech Republic, August 26 – 30, 2019.

- R1-1913017, Doppler Compensation, Uplink Timing Advance and Random Access in NTN, 3GPP TSG RAN WG1 #99, Reno, USA, November 18th – 22nd, 2019.

- 3GPP TS38.211 V18.5.0, “Physical channels and modulation”.

- Annex

  - Annex I Satellite parameters

RAN1#116 Agreement For DL coverage study, consider the following additional reference satellite parameters scenarios for LEO600km Set1 in FR1 (i.e., S-band), referred to as Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1: LEO600km Set1-1 FR1 (i.e., S-band). Maximum Bandwidth per beam; 5 MHz. SCS; 15 kHz. Beam size(Note 1); 50km. Satellite EIRP density /beam (dBW/MHz); 34. Payload Total DL power level (dBW); 31.24. Aggregated EIRP (Total) (dBW); 61.24*. Satellite Tx max Gain; 30 dBi. Maximum EIRP per Satellite beam (dBW); 41. Total number of beam footprints***; 1058. Total number of simultaneously active beams **; 106. % simultaneously active beams**; 10.02 %. *Note: EIRP limit is 61.24 dBm for the reference configuration. **Assuming 100 % Resource Block utilization within the same beam at max power. Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) *** For a constellation design at 600km with low elevation angle with 30° and selected (i.e Set 1 parameters) beam size Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. LEO600km Set1-2 FR1 (i.e., S-band). Maximum Bandwidth per beam; 5 MHz. SCS; 15 kHz. Beam size (note 1); 50km. Satellite EIRP density /beam (dBW/MHz); 34. Payload Total DL power level (dBW); 23. Aggregated EIRP (Total) (dBW); 53*. Satellite Tx max Gain; 30 dBi. Maximum EIRP per Satellite beam (dBW); 41. Total number of beam footprints; 1058. Total number of simultaneously active beams**; 16. % simultaneously active beams**; 1.5 %. *Note: EIRP limit is 53 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 16 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. LEO600km Set 1-3 FR1 (i.e., S-band). Maximum Bandwidth per beam; 5 MHz. SCS; 15 kHz. Beam size (note 1); 50km. Satellite EIRP density /beam (dBW/MHz); 26. Payload Total DL power level (dBW); 23.24. Aggregated EIRP (Total) (dBW); 53.24*. Satellite Tx max Gain; 30 dBi. Maximum EIRP per Satellite beam (dBW); 33. Total number of beam footprints; 1058. Total number of simultaneously active beams**; 106. % simultaneously active beams**; 10.02 %. *Note: EIRP limit is 53.24 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. Note: RAN1 will aim to identify necessary enhancements for these scenarios in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential enhancements will be specified within Rel-19 framework.

  - Annex II Requirements of CP and SCS in different deployment scenarios

Table 5.2-1 Timing/frequency error and required CP/SCS for S-band/2GHz

| Scenario | Frequency band | Beam radius | Elevation angle | UE speed（km/h） | one-way Maximum timing error (us) | Require CP length/differential RTT (us) | one-way Maximum differential Doppler shift (Hz) | differential RTD (Hz) | Required SCS without restricted set（kHz） γ = 0 | Required SCS with restricted set A（kHz） γ= 1 | Required SCS with restricted set B（kHz） γ= 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LEO-600 | 2GHz | 25km | 30 | 3 | 142.99 | 285.98 | 763.095 | 1526.19 | 1.52619 | 0.763095 | 0.3815475 |
|  |  |  |  | 120 | 142.99 | 285.98 | 766.6841 | 1533.3682 | 1.5333682 | 0.7666841 | 0.38334205 |
|  |  |  |  | 1500 | 143.19 | 286.38 | 821.3225 | 1642.645 | 1.642645 | 0.8213225 | 0.41066125 |
|  |  |  | 90 | 3 | 1.8987 | 3.7974 | 4196.5 | 8393 | 8.393 | 4.1965 | 2.09825 |
|  |  |  |  | 120 | 1.8987 | 3.7974 | 4216.2 | 8432.4 | 8.4324 | 4.2162 | 2.1081 |
|  |  |  |  | 1500 | 1.9339 | 3.8678 | 4531 | 9062 | 9.062 | 4.531 | 2.2655 |
| LEO-1200 | 2GHz | 45km | 30 | 3 | 256.94 | 513.88 | 828.41 | 1656.82 | 1.65682 | 0.82841 | 0.414205 |
|  |  |  |  | 120 | 256.94 | 513.88 | 832.64 | 1665.28 | 1.66528 | 0.83264 | 0.41632 |
|  |  |  |  | 1500 | 257.33 | 514.66 | 889 | 1778 | 1.778 | 0.889 | 0.4445 |
|  |  |  | 90 | 3 | 3.3408 | 6.6816 | 3777.3 | 7554.6 | 7.5546 | 3.7773 | 1.88865 |
|  |  |  |  | 120 | 3.3408 | 6.6816 | 3796.6 | 7593.2 | 7.5932 | 3.7966 | 1.8983 |
|  |  |  |  | 1500 | 3.3742 | 6.7484 | 4063.9 | 8127.8 | 8.1278 | 4.0639 | 2.03195 |
| GEO | 2GHz | 125km | 12.5 | 3 | 809.7 | 1619.4 | 0.0534 | 0.1068 | 0.0001068 | 0.0000534 | 0.0000267 |
|  |  |  |  | 120 | 809.7 | 1619.4 | 26.7174 | 53.4348 | 0.0534348 | 0.0267174 | 0.0133587 |
|  |  |  |  | 1500 | 810.96 | 1621.92 | 61.34 | 122.68 | 0.12268 | 0.06134 | 0.03067 |
|  |  |  | 90 | 3 | 4.815 | 9.63 | 0.2568 | 0.5136 | 0.0005136 | 0.0002568 | 0.0001284 |
|  |  |  |  | 120 | 4.815 | 9.63 | 10.2714 | 20.5428 | 0.0205428 | 0.0102714 | 0.0051357 |
|  |  |  |  | 1500 | 4.823 | 9.646 | 128.43 | 256.86 | 0.25686 | 0.12843 | 0.064215 |

Table 5.2-2 Timing/frequency error and required CP/SCS for Ka-band/30GHz

| Scenario | Frequency band | Beam radius | Elevation angle | UE speed（km/h） | one-way maximum timing error (us) | Require CP length/differential RTT (us) | one-way maximum differential Doppler shift (Hz) | differential RTD (Hz) | Required SCS without restricted set（kHz） γ= 0 | Required SCS with restricted set A（kHz） γ= 1 | Required SCS with restricted set B（kHz） γ= 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LEO-600 | 30GHz | 10km | 30 | 3 | 57.52 | 115.04 | 4407.3 | 8814.6 | 8.8146 | 4.4073 | 2.20365 |
|  |  |  |  | 120 | 57.52 | 115.04 | 4428 | 8856 | 8.856 | 4.428 | 2.214 |
|  |  |  |  | 1500 | 57.61 | 115.22 | 4740.5 | 9481 | 9.481 | 4.7405 | 2.37025 |
|  |  |  | 90 | 3 | 0.304 | 0.608 | 25199 | 50398 | 50.398 | 25.199 | 12.5995 |
|  |  |  |  | 120 | 0.304 | 0.608 | 25318 | 50636 | 50.636 | 25.318 | 12.659 |
|  |  |  |  | 1500 | 0.3096 | 0.6192 | 27208 | 54416 | 54.416 | 27.208 | 13.604 |
| LEO-1200 | 30GHz | 20km | 30 | 3 | 114.92 | 229.84 | 1647.32 | 3294.64 | 3.29464 | 1.64732 | 0.82366 |
|  |  |  |  | 120 | 114.92 | 229.84 | 4461.9 | 8923.8 | 8.9238 | 4.4619 | 2.23095 |
|  |  |  |  | 1500 | 115.09 | 230.18 | 37672.78 | 75345.56 | 75.34556 | 37.67278 | 18.83639 |
|  |  |  | 90 | 3 | 0.6601 | 1.3202 | 12598.25 | 25196.5 | 25.1965 | 12.59825 | 6.299125 |
|  |  |  |  | 120 | 0.6601 | 1.3202 | 12598.25 | 25196.5 | 25.1965 | 12.59825 | 6.299125 |
|  |  |  |  | 1500 | 0.6667 | 1.3334 | 12704.09 | 25408.18 | 25.40818 | 12.70409 | 6.352045 |
| GEO | 30GHz | 55km | 12.5 | 3 | 357.25 | 714.5 | 0.3355 | 0.671 | 0.000671 | 0.0003355 | 0.00016775 |
|  |  |  |  | 120 | 357.25 | 714.5 | 13.42 | 26.84 | 0.02684 | 0.01342 | 0.00671 |
|  |  |  |  | 1500 | 357.81 | 715.62 | 167.76 | 335.52 | 0.33552 | 0.16776 | 0.08388 |
|  |  |  | 90 | 3 | 0.9322 | 1.8644 | 1.695 | 3.39 | 0.00339 | 0.001695 | 0.0008475 |
|  |  |  |  | 120 | 0.9322 | 1.8644 | 67.797 | 135.594 | 0.135594 | 0.067797 | 0.0338985 |
|  |  |  |  | 1500 | 0.9339 | 1.8678 | 847.7 | 1695.4 | 1.6954 | 0.8477 | 0.42385 |

Table 5.2-3 Timing/frequency error and required CP/SCS for Ku-band/14GHz

| Scenario | Frequency band | Beam radius | Elevation angle | UE speed（km/h） | one-way Maximum timing error (us) | Require CP length/differential RTT (us) | one-way Maximum differential Doppler shift (Hz) | differential RTD (Hz) | Required SCS without restricted set（kHz） γ= 0 | Required SCS with restricted set A（kHz） γ= 1 | Required SCS with restricted set B（kHz） γ= 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LEO-600 | 14GHz | 15km | 30 | 3 | 86.124 | 172.248 | 3124.3 | 6248.6 | 6.2486 | 3.1243 | 1.56215 |
|  |  |  |  | 120 | 86.124 | 172.248 | 3139 | 6278 | 6.278 | 3.139 | 1.5695 |
|  |  |  |  | 1500 | 86.253 | 172.506 | 3361.3 | 6722.6 | 6.7226 | 3.3613 | 1.68065 |
|  |  |  | 90 | 3 | 0.6837 | 1.3674 | 17636 | 35272 | 35.272 | 17.636 | 8.818 |
|  |  |  |  | 120 | 0.6837 | 1.3674 | 17719 | 35438 | 35.438 | 17.719 | 8.8595 |
|  |  |  |  | 1500 | 0.6964 | 1.3928 | 19042 | 38084 | 38.084 | 19.042 | 9.521 |
| LEO-1200 | 14GHz | 30km | 30 | 3 | 171.95 | 343.9 | 3783.7 | 7567.4 | 7.5674 | 3.7837 | 1.89185 |
|  |  |  |  | 120 | 171.95 | 343.9 | 3803 | 7606 | 7.606 | 3.803 | 1.9015 |
|  |  |  |  | 1500 | 172.21 | 344.42 | 4060 | 8120 | 8.12 | 4.06 | 2.03 |
|  |  |  | 90 | 3 | 1.4852 | 2.9704 | 17636 | 35272 | 35.272 | 17.636 | 8.818 |
|  |  |  |  | 120 | 1.4852 | 2.9704 | 17726 | 35452 | 35.452 | 17.726 | 8.863 |
|  |  |  |  | 1500 | 1.5 | 3 | 18974 | 37948 | 37.948 | 18.974 | 9.487 |
| GEO | 14GHz | 80km | 12.5 | 3 | 519.13 | 1038.26 | 0.2319 | 0.4638 | 0.0004638 | 0.0002319 | 0.00011595 |
|  |  |  |  | 120 | 519.13 | 1038.26 | 9.2754 | 18.5508 | 0.0185508 | 0.0092754 | 0.0046377 |
|  |  |  |  | 1500 | 519.95 | 1039.9 | 115.95 | 231.9 | 0.2319 | 0.11595 | 0.057975 |
|  |  |  | 90 | 3 | 1.9723 | 3.9446 | 1.15 | 2.3 | 0.0023 | 0.00115 | 0.000575 |
|  |  |  |  | 120 | 1.9723 | 3.9446 | 46.02 | 92.04 | 0.09204 | 0.04602 | 0.02301 |
|  |  |  |  | 1500 | 1.9759 | 3.9518 | 575.4 | 1150.8 | 1.1508 | 0.5754 | 0.2877 |

  - Annex III UE positioning evaluation assumptions

Table 5.3-1 Common simulation assumption

| Parameter | Description/Value |
| --- | --- |
| Scenarios | Rural, LOS |
| Satellite Orbit | 600km |
| Satellite parameters | Reuse Set-1satellite parameters as in table 6.1.1.1-1/2 of TR38.821 |
| Channel model/ Delay spread | Based on section 6.7.2 of TR 38.811 |
| FR/Carrier frequency | FR1: 2GHz, S-band (n256). |
| Subcarrier spacing, kHz | 15 for FR1 |
| Number of satellite in view | 1 for single satellite case, |
| Orbit inclination | 90 |
| Satellite location at t=0 | 0°N, 0°E |
| UE type | Handheld terminal |
| UE related parameters | Handheld UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration as agreed under AI 9.12.1 |
| Interference modelling (ideal muting, or other) | Ideal muting |
| Delay between symbol boundary and first path | DL: Uniform in [0,500]ns |
| Network synchronization | Truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1. T1 = 50ns [2]. |
| UE/gNB Tx/Rx calibration error | Truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1. gNB Rx/Tx Time error T1 = 1.4ns UE Rx/Tx Time error T1 = 5.6ns |
| Reference point for timing measurement | Satellite |
| UE speed | 3km/h |